3GPP TSG RAN WG1 Meeting #87	R1-1613092

Reno, USA, 14th - 18th November 2016

Source:	Ericsson

Title:	Channel Coding Candidates for Control Channel

Agenda Item:	7.1.5.1

Document for:	Discussion

# Introduction

In RAN#71, a new study item, “Study on New Radio Access Technology,” has been approved. The initial work of the study item is expected to focus on fundamental physical layer signal structure for new RAT, of which channel coding schemes is listed as an area to investigate. In RAN1#86bis, the following agreements for the eMBB control channel coding evaluations were reached [1]:

- Simulation Assumptions for eMBB control channel coding

- Evaluate the block error rate (BLER) performance versus SNR

- Evaluate the false alarm rate versus SNR

| Channel | AWGN |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | QPSK |  |  |  |  |  |  |
| Coding Scheme | Repetition | Simplex | TBCC | Turbo | LDPC | Reed-Muller | Polar |
| Decoding algorithm** | ML | ML | List-Viterbi | Scaled max log MAP | Adjusted min-sum | FHT | SC list |
| Info. block length (bits w/o CRC) (for evaluation purposes) *** | 1, 2, 4, 8, 16, 32, 48, 64, 80, 120, 200 |  |  |  |  |  |  |

* Code rate 1/24 is valid for info block length of 1-2 bits

** Other variants of agreed algorithms can be used for encoding and decoding (Complexity details should be illustrated)

*** Each of these info. block lengths shall be evaluated at at least one of the code rates. Other info. block lengths and code rates are not precluded. Similar info. and encoded block lengths should be used for the evaluation. Total number of coded bits is derived accordingly.

In this contribution, we study channel coding candidates for the control channel of NR. The study compares the performance of tail-biting convolutional codes (TBCC), tail-biting turbo codes (TBTC), and Polar codes for the purpose of protecting the control channels of NR. Simulation results are provided for short information block sizes to be used for control channel coding as listed above.

# Transmission and Reception of DCI, UCI

## DCI

For DCI, blind decoding is necessary in the receiver (i.e., UE) in LTE.

In a primary cell:

| UE-specific search space | 16 blind decodes × 2 DCI formats = 32 blind decodes |
| --- | --- |
| Common search space | 6 blind decodes × 2 DCI formats (1A, 1C) = 12 blind decodes |
| Total | 44 |

In a secondary cell (no need to monitor Common search space):

| UE-specific search space | 16 blind decodes × 2 DCI formats = 32 blind decodes |
| --- | --- |
| Total | 32 |

Without accounting for UL-MIMO:

Max number of blind decodes, when CA of 5 cells and no UL MIMO:

44 (blind decodes in primary cell) + 4 (secondary cell) × 32 (blind decodes in a SCell)

= 172 (blind decodes per subframe)

It is noted that if the maximum carrier aggregation of 32 cells are considered, then the amount of blind decoding increases. Also, starting from Rel-10, with uplink spatial multiplexing, an additional UL DCI format needs to be monitored in UE-specific search space. If UL-MIMO is considered, the amount of blind decoding of DCI also increases.

For NR, blind decoding of DCI is also expected, similar to LTE. Considering the potential of requiring a large number of DCI monitoring in a subframe, the decoding latency of DCI is a very important design metric. Short decoding latency is as important as the BLER performance of the decoder,

## UCI

For UCI, no blind decoding is necessary at the receiver. On the other hand, decoding of one instance of UCI can be more challenging than the decoding of one instance of DCI. This is due to the potentially large info block size of UCI, for carrying CSI back to the eNodeB.

Also it is possible that multiple UEs send UCI simultaneously on the uplink, which requires the eNodeB to be able to simultaneously decoding the UCI of each UE. Hence the channel coding scheme for UCI should also allow efficient decoding algorithm at the receiver, while achieving good BLER performance.

- For both DCI and UCI, channel coding technique should be selected considering the requirements of low decoding complexity and low decoding latency.

# Performance Comparison: TBCC vs. TBTC

In this section, we compare the performance of TBCC, TBTC, and Polar code according to the simulation assumption of eMBB control channel.

The agreement states information block lengths without CRC, so the additional nCRC=16 CRC bits have been added to the information block that the encoder/decoder see. The same number of CRC bits are assumed for all code types. However, when calculating the code rate, the CRC bits should be counted as additional parity bits, not seen as information bits. This gives an increased code rate seen by the TBCC/TBTC/Polar decoder which is given by

Rdecoder=Rinfo×kinfo+nCRCkinfo

Note that the rates shown in the legends are the rates Rinfo agreed for the information blocks, before adding CRC bits. The actual rate that the TBCC/TBTC/Polar encoder/decoder see is then calculated according to Rdecoder in the equation above. The agreed info block lengths Kinfo are shown in the titles, even though the encoder/decoder sees an info block length (Kinfo+nCRC) since the CRC bits are added before encoding.

Here, the results for Polar codes are based on the new variant, termed Parity-Check (PC) Polar code as described in [6], with a list size of 8.  The results for TBCC are those of constraint length 9 (i.e. 256-state) TBCC in [5] with lowest native code rate 1/9. (Note that in [7], the TBCC results are those of constraint length 7 (i.e. 64-state) TBCC with lowest native code rate 1/9.)  As shown in these figures, 256-state TBCC outperforms both TBTC and PC Polar codes at short information block length (e.g. Kinfo≤32).  256-state TBCC delivers comparable performance to TBTC and PC Polar codes for Kinfo in the middle range, for example, Kinfo =48 bit to 80 bit. As the information block length increases further to Kinfo=120 and beyond, TBTC and Polar codes show advantage over TBCC, especially at low code rate Rinfo <1/3.

- TBCC outperforms both TBTC and PC-Polar codes at shorter information block lengths.

- TBCC is outperformed by both TBTC and PC-Polar codes at larger information block lengths.

- TBTC and PC-Polar codes perform comparably for the short block sizes of control channel.

# Conclusion

In this contribution, we study channel coding candidates for the control channel of NR.  Based on the analysis and obtained results, we have the following proposals and observation.

- TBCC outperforms both TBTC and PC-Polar codes at shorter information block lengths.

- TBCC is outperformed by both TBTC and PC-Polar codes at larger information block lengths.

- TBTC and PC-Polar codes perform comparably for the short block sizes of control channel.

- For both DCI and UCI, channel coding technique should be selected considering the requirements of low decoding complexity and low decoding latency.

# References

Chairman Notes, 3GPP TSG-RAN WG1 #86, Aug. 2016.

R1-166926, “Further Discussion on TBCC Complexity and Enhancements,” Ericsson, August 2016.

R1-166373 “Short block-length design”, Qualcomm Inc., 3GPP TSG-RAN WG1 #86, Aug. 2016

R1-166772 “Performance Analysis of Polar Codes”, Samsung, 3GPP TSG-RAN WG1 #86, Aug. 2016.

R1-166926 “Further Discussion on TBCC Complexity and Enhancements”, Ericsson, 3GPP TSG-RAN WG1 #86, Aug. 2016.

R1-1608862, “Polar code construction for NR”, Huawei, HiSilicon, October 2016.

R1-1611316, “Performance Comparison of Channel Coding Candidates for Short Block Sizes,” Ericsson, November 2016.