3GPP TSG RAN WG1 #88bis	R1-1705005

April 3rd – 7th, 2017

Spokane, USA

Agenda item:	7.2.4.2

Source: 	Qualcomm Incorporated

Title: 	Enhancement to control signaling for NCJT

Document for:	Discussion and Decision

# 1	Introduction

In RAN #75, non-coherent joint transmission (NCJT) is approved in WID of feCoMP [2].

In the scope of NCJT, the following are included

- Specify enhancements to support non-coherent joint transmission (JT) schemes [RAN1]

  - Support of a new QCL assumption for DM-RS antenna ports

  - Support of control signalling enhancements to assist QCL,PDSCH REs mapping and resource allocation

  - Support possible CSI feedback enhancement

In this paper, we focus on control signaling enhancement.

# 2	Discussion

In [1], the following categories of NCJT are identified.

- Mapping of codewords (CWs) to TPs:

Case 1: Different CWs are transmitted from different TPs. Each TP perform adaptive precoding independently

Case 2a: The same CW is transmitted from different TPs with spatial diversity (e.g. SFBC) / spatial multiplexing

Case 2b: The same CW is transmitted from different TPs using SFN

- Resource allocations on different TPs:

Scheme 1: The resource allocations from different TPs for a UE are fully overlapped

Scheme 2: The resource allocations from different TPs for a UE are partially overlapped

Scheme 3: The resource allocations from different TPs for a UE are not overlapped

Out of the three cases of codeword mapping, case 1 has the best gain from simulation studies.

Out of the three resource allocation schemes, scheme 3 is not realistic as it implies the restriction that a UE cannot be assigned full bandwidth from both TPs. Scheme 2 can be considered a super set of scheme 3. However, overlapping parts and non-overlapping parts of the codeword from each TP typically have different interference, which will lead to complicated CSI reporting and MCS selection design. Consider the limited availability of study time, we propose to focus on case 1 and scheme 1 for NCJT.

Proposal 1.  NCJT in feCoMP only support different CWs from different TPs with independent precoding, fully overlapped resource allocation from different TPs.

On the UE side, to reuse UE receiver capability, we can restrict to support up to 2 codewords for NCJT. This 2-codeword restriction implies that we can support up to 2 TPs in NCJT with one codeword transmitted from each TP. We believe this can already achieve most of the system gain from NCJT. For better flexibility, transmission from one TP only should be supported as well, in which case, up to two codewords from the only TP is supported. To limit the PDCCH blind decoding count, the two types of scheduling should be supported using a single DCI format.

Proposal 2.  Under NCJT, support joint transmission from two TPs with one codeword from each TP, and support transmission from one TP with up to two codewords. The two operation modes share the same DCI format.

Since we restrict the total number of codewords in NCJT to be two, most of the structure of TM10 DCI format 2D can be reused (such as HARQ fields, per codeword MCS and NDI, etc), with the exception of the following:

In legacy SU-MIMO case, the rank combination of the two codewords is deterministic given the total rank (RI). If the total rank is even, each codeword gets half of the total rank. If the total rank is odd, the codeword 2 rank is 1 larger than the codeword 1 rank. For NCJT with two TPs, we may want to have higher flexibility for ranks. The following options can be considered:

-	Option 1: Follow the legacy design with TP1 transmits codeword 1 and TP2 transmits codeword 2

-	Option 2: Slightly improve from the legacy design and also allow TP1 rank to be 1 higher than TP2

-	Option 3: Flexible design to allow ranks for TP1 and TP2 to be specified separately in DCI

The supported rank combinations of TP1 and TP2 are summarized in Table 1, assuming each TP has 4 CRS ports.

Table 1. Supported Rank Combinations of TP1 and TP2 under Different Options

| Options | (TP1 rank, TP2 rank) | Notes |
| --- | --- | --- |
| 1 | (1,1), (1,2), (2,2), (2,3), (3,3), (3,4), (4,4) | Totally 7 combinations |
| 2 | (1,1), (1,2), (2,1), (2,2), (2,3), (3,2), (3,3), (3,4), (4,3), (4,4) | Totally 10 combinations |
| 3 | (1,1), (1,2), (1,3), (1,4), (2,1), (2,2), (2,3), (2,4), (3,1), (3,2), (3,3), (3,4), (4,1), (4,2), (4,3), (4,4) | Totally 16 combinations |

Comparing the 3 options, option 1 is the most restrictive, but no antenna ports, scrambling identity and number of layers field (Table 5.3.3.1.5C-1 and Table 5.3.3.1.5C-2 in [1]) change is needed in DCI format 2C and 2D. For option 2, we have more flexibility and potentially higher performance gain. To support this option while avoid re-defining the antenna ports, scrambling identify and number of layers field, we can introduce a bit in the DCI to flip the role of TP1 and TP2 as shown in Table 2. Option 3 has the best flexibility and potential has larger gain. The cost to pay is to we need to define a new antenna ports, scrambling identity, and number of layers field in DCI.

Compare the three options, the option 2 is a preferable compromise.

Table 2. Codeword Swap Indicator for NCJT

| CW Swap | TP1 | TP2 |
| --- | --- | --- |
| 0 | CW1 | CW2 |
| 1 | CW2 | CW1 |

Proposal 3.  When both TPs participate in the NCJT, the allowed rank combinations for the codewords from two TPs need further study.

# 3	Conclusions

For the control signaling enhancement for NCJT, we propose the following:

Proposal 1.  NCJT in feCoMP only support different CWs from different TPs with independent precoding, fully overlapped resource allocation from different TPs.

Proposal 2.  Under NCJT, support joint transmission from two TPs with one codeword from each TP, and support transmission from one TP with up to two codewords. The two operation modes share the same DCI format.

Proposal 3.  When both TPs participate in the NCJT, the allowed rank combinations for the codewords from two TPs need further study.

# 4	References

[1]. TR 36.741 v2.0.1 “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on further enhancements to Coordinated Multi-Point (CoMP) Operation for LTE”, 3GPP

[2]. RP-170750, “New WID: Further enhancements to Coordinated Multi-Point (CoMP) Operation for LTE”, Intel, ZTE