3GPP TSG RAN WG1 Meeting #89                                            	R1-1708583

Hangzhou, CN 15th  - 19th May 2017

Agenda item:	7.1.2.1.5

Source: 	Qualcomm Incorporated

Title: 	On UL diversity transmission scheme

Document for:	Discussion/Decision

# Introduction

In RAN1#88 [1] and RAN1#88bis [3], the following agreement has been made for uplink transmit diversity for DFT-S-OFDM based data channel

- For DFT-S-OFDM, following schemes can be candidates for UL diversity schemes for UL data: CDD, precoder cycling, antenna port switching, SFBC, and STBC.

  - RAN1 should down select them in WI phase.

  - FFS the corresponding spec impact (if any)

- For DFTsOFDM in data channel, the following schemes are candidates for transmit diversity:

  - Low PAPR Alamouti-based transmit diversity applied in frequency or time domain, transparent transmit diversity (e.g. short delay CDD, panel selection), time domain beam/precoder cycling

The transmit diversity schemes for PUCCH and CP-OFDM based PUSCH are not decided yet. Even for the DFT-S-OFDM based PUSCH channel, RAN1 still needs to down select transmit diversity scheme from five proposals.

In this contribution, the diversity scheme for PUCCH and PUSCH channels are discussed. We address the diversity for PUCCH channel first, followed by the discussion on diversity scheme for PUSCH channel.

The PUCCH channel utilizes DFT-S-OFDM waveform to improve the performance for cell edge users which are link budget limited. A transmit diversity scheme which can explore the Tx diversity while maintain low PAPR DFT-S-OFDM waveform at the same time is definitely desirable to increate the coverage of PUCCH channel. However, not all transmit diversity schemes can maintain low PAPR waveform. For example, the traditional SFBC transmit diversity scheme, which applies space frequency block coding over two adjacent frequency tones, cannot be applied to PUCCH, because the generated waveform with SFBC is not single carrier anymore.

In LTE, SORTD (space orthogonal resource transmit diversity) is adopted for PUCCH. SORTD scheme utilizes two PUCCH resources to transmit signals from two antennas. Therefore, SORTD can maintain the single carrier waveform for PUCCH. However, SORTD requires twice the amount of PUCCH resources to achieve transmit diversity.

In this contribution, for PUCCH with DFT-S-OFDM waveform, we focus on the transmit diversity schemes that can maintain good PAPR properties. Specifically, a scheme of STBC before DFT is proposed for PUCCH transmit diversity. The proposed STBC before DFT scheme has the following advantages over other diversity schemes

- maintains the single carrier waveform

- works with any number of OFDM symbols

- does not increase PUCCH resource consumption

- achieves full diversity gain

- Alamouti receiver processing can be done in frequency domain rather than time domain for simplicity.

For PUSCH with DFT-S-OFDM waveform, STBC can also provide a gain over small delay CDD. It is proposed to use adapt the proposed STBC for PUSCH with DFT-S-OFDM waveform.

For PUSCH with OFDM waveform, the agreement on DL PDSCH transmit diversity scheme can be applied, in order to maintain the DL/UL symmetricity.

The rest of the contribution is organized as following. The details of STBC based transmit diversity scheme for PUCCH is discussed in Section 2. The transmission diversity schemes for PUSCH is discussed in Section 3. The conclusions are listed in Section 4.

# STBC for PUCCH diversity

## STBC before DFT

The proposed PUCCH transmit diversity scheme is a STBC scheme on modulated symbols before DFT operation. The time domain signal construction cross two OFDM symbols before DFT is given in Figure 1. The modulated symbols in the first OFDM symbol is denoted as where. The modulated symbols in the second OFDM symbol is denoted as where. For the first Tx antenna, as shown in Figure 2 and Figure 3, the modulated symbols and are transmitted in the first OFDM symbol and second OFDM symbol, respectively, by going through the standard DFT-S-OFDM waveform generation chain without any special operation. For the second Tx antenna, the modulated symbols transmitted in the first OFDM symbol are . The modulated symbols transmitted in the second OFDM symbol are . The reason of flipping the order of modulated symbols with operation mod-n, M is because of the DFT property of, , which allows per tone de-Alamouti combiner in frequency domain at receiver.

The manipulated modulated symbols and are then fed to the same waveform generation chain for the first and second OFDM symbols respectively, going through the same standard DFT-S-OFDM waveform generation chain as for the first Tx antenna. It is clear to see that conjugate and sign flip operations do not change the single carrier property of the generated waveform for the second antenna. Therefore, the proposed STBC scheme maintains the single carrier waveform. Furthermore, unlike SORTD scheme, two waveforms are send based on the same PUCCH resource without extra resource requirement.

Figure 1: Time domain signal construction over two OFDM symbols.

Figure 2: PUCCH transmit chain with OFDM symbol 1.

Figure 3: PUCCH transmit chain with OFDM symbol 2.

The receiver diagram for PUCCH with STBC can be illustrated in Figure 4. At gNB receiver side, after FFT and tone extraction, two sets of signals on frequency domain can be obtained

whereand are the DFT transform of and respectively. and  are the channel from the first and second Tx antennas.

It is clear to see that based on the signals and , on each tone k, a very simple de-Alamouti combiner can be applied as below, which explores the full diversity of channels and .

After the de-Alamouti combining, the FDE (frequency domain equalizer) is performed, followed by the IDFT to obtain the time domain samples to do demodulation and decoding.

Figure 4: PUCCH receiver chain with STBC before DFT based diversity.

## STBC before DFT with virtual symbol splitting

STBC described above need even number of symbols. With odd number of symbols, STBC can still be applied with virtual symbol splitting. Without changing the subcarrier spacing, one OFDM symbol can be split into two half-symbols before DFT.

Now, after virtual symbol splitting, there are even number of half-symbols available. Then we treat each half-symbol as if it is a regular OFDM symbol and apply STBC, as shown in Figure 5. The only different between Figure 5 and Figure 1 is that the length of modulated symbols for and is reduced to , i.e., .

Figure 5: Time domain signal construction over two virtually split OFDM symbols.

The receiver structure for PUCCH with virtually split symbol based STBC is given in Appendix 5.1.

Also, it is very interesting that this virtual split symbol based STBC scheme has some similarity comparing to the PAPR preserving SC-SFBC (Single carrier SFBC) scheme proposed in [2]. The similarity and difference between the single carrier SFBC and virtual split based STBC is described in Appendix 5.2.

## Performance of STBC scheme

In this section, we compare the performance of STBC with small delay CDD for 2-symbol PUCCH. RS and UCI are TDMed where RS is in the first symbol and UCI is in the second symbol.  For STBC before DFT scheme, virtual symbol splitting as described in Figure 5 is applied to the UCI symbol. Besides the regular CP for each OFDM symbol, no extra CP is added between the virtually split half-symbols. The detailed simulation parameters are listed in Table 1.

As shown in Figure 6, STBC has about 2 dB gain over the SCDD scheme, which is expected because STBC can explore full Tx diversity gain. Comparing with other diversity schemes, because both precoder cycling and antenna port switch can not explore full Tx diversity, we expect their performance are worse than STBC as well. On the other hand, although SFBC can explore full diversity gain too, it can not maintain single carrier waveform, which will cause a few dB loss to STBC due to bad PAPR. A modified version of SFBC which can preserve low PAPR is proposed in [2]. However, this low PAPR SFBC may suffer in frequency selective fading channel, because of the large spacing between paired non-adjacent SFBC tones.

Table 1 Simulation parameters for STBC vs SCDD comparison

| Parameters | Settings |
| --- | --- |
| PUCCH duration | 2 symbols |
| RS/UCI MUX | TDM (1 RS symbol, 1 UCI symbol) |
| PUCCH bandwidth | 48 tones |
| UCI code rate | ½ with 24 bit CRC |
| Modulation | QPSK |
| Channel estimation | MMSE |
| FFT size | 2048 |
| CP length | 4.6uS |
| Subcarrier Spacing | 15kHz |
| Antenna Configuration | 2 Tx, 1 Rx |
| Channel model | TDL-C with 30ns RMS delay |
| Added delay for SCDD | 2.77uS |

Figure 6: Performance comparison between virtual symbol split STBC with small delay CDD for small RB (4 RB) allocation

Observation 1: for small RB allocation, e.g. PUCCH, STBC outperforms small delay CDD.

Proposal 1: For PUCCH, adopt STBC as described in Section 2 for the transmission diversity scheme.

- Virtual split of symbols can be considered if symbol pairing is not available.

# PUSCH transmit diversity

The proposed STBC scheme can provide a gain over the SCDD scheme even for large RB allocations. Figure 7 shows the link level simulation result with the same simulation parameters as Table 1 except that the number of allocated PRBs increases to 20, 40, 80, 120 RBs. The simulation results shows that the proposed STBC still has 2 dB gain over the SCDD. Therefore, for simplicity, it is preferable to use STBC also for PUSCH with DFT-S-OFDM waveform.

Figure 7 Performance comparison between virtual symbol split STBC with small delay CDD for large RB allocation

Observation 2: for large RB allocation, e.g. PUSCH, STBC outperforms small delay CDD.

For PUSCH with CP-OFDM based waveform, the agreement for transmit diversity scheme for downlink PDSCH can be reused, in order to keep symmetric UL and DL data channel structure such as DMRS pattern.

Proposal 2: For PUSCH with DFT-S-OFDM, adopt STBC as described in Section 2 for the transmission diversity scheme.

- Virtual split of symbols can be considered if symbol pairing is not available.

Proposal 3: For PUSCH with CP-OFDM waveform, adopt the same transmit diversity scheme as PDSCH.

# Conclusions

The following observation is for UL transmit diversity

Observation 1: for small RB allocation, e.g. PUCCH, STBC outperforms small delay CDD.

Observation 2: for large RB allocation, e.g. PUSCH, STBC outperforms small delay CDD.

The following proposals are for UL transmit diversity.

Proposal 1: For PUCCH, adopt STBC as described in Section 2 for the transmission diversity scheme.

- Virtual split of symbols can be considered if symbol pairing is not available.

Proposal 2: For PUSCH with DFT-S-OFDM, adopt STBC as described in Section 2 for the transmission diversity scheme.

- Virtual split of symbols can be considered if symbol pairing is not available.

Proposal 3: For PUSCH with CP-OFDM waveform, adopt the same transmit diversity scheme as PDSCH

# Appendix

## STBC receiver with virtual split symbols

### Option 1

Figure 8: PUCCH receiver chain option 1 with virtual split symbol STBC.

As shown in Figure 8, at gNB receiver side, N point FFT and tone extraction are performed to obtain the PUCCH signals  for desired UE in frequency domain. Equivalently, from time domain perspective, the received signal is denoted as

where and are the received signals in the first half-symbol and second half-symbol.

Next step, we extract and separately from . We then run two smaller DFT with size M/2 to convert and  back to frequency domain signals and . The rest operations are straightforward by doing de-Alamouti combining and frequency domain equalization (DFE), followed by two smaller IDFT with size M/2 to convert the equalized signals to time domain for demodulation and decoding.

Option 1 requires quite a few IDFT/DFT operations, which increase the receiver complexity. To simplify receiver design, another option is considered as below.

### Option 2

Figure 9: PUCCH receiver chain option 2 with virtual split symbol STBC.

As shown in Figure 8, at gNB receiver side, N point FFT and tone extraction are performed to obtain the PUCCH signals  for desired UE in frequency domain. Equivalently, from time domain perspective, the received signal is denoted as

where and are the received signals in the first half-symbol and second half-symbol.

where  and  are the channel from the first Tx antenna and the second Tx antenna respectively.

In frequency domain, the PUCCH tones are then multiplied with and respectively, where  and  are the channel from the first and second Tx antennas. The two streams of channel scaled signals are then pass through two IDFT blocks to obtain the signals in time domain. We denote the two steams of time domain signals as  and  where  are concatenated in time domain, are concatenated in time domain as well.

Given the IDFT property that , it can be shown that

The de-Alamouti combiner is conducted in time domain as follow

,

We can see that the signals after combining can be expressed as below

It is clear to see that full transmit diversity can be achieved with STBC with split half-symbols.

## Comparison between virtual split based STBC before DFT and PAPR preserving SC-SFBC

Figure 10: virtual split based STBC before DFT with 8 tones

Figure 11: PAPR preserving SC-SFBC [2] with 8 tones

Figure 9 shows a transmitter diagram for virtual split based STBC before DFT which is proposed in this contribution when 8 tones are used. Figure 10 shows a transmitter diagram for PAPR preserving SC-SFBC [2] when 8 tones are used.

As we explained above, in the virtual split based STBC of Figure 9, the time domain sequence a0, …, a7 is split into two subsequences with length 4, and Alamouti coding is introduced to two split sequences. Here, we can notice the following two:

- Descending time indexing for the second antenna transmission

- Discontinuous phase for the second antenna transmission

The time indices for the first half of the time domain sequence for antenna 2 in Figure 9, (a4 *, a7 *, a6 *, a5 *), is (4,7,6,5), which can be thought as a cyclic shift of (7,6,5,4). Likewise, the time indices for the second half can be thought as a cyclic shift of (3,2,1,0). Therefore, we can notice that time indexing for the second antenna in virtual split based STBC is decreasing. Due to this descending time indexing, the proposed scheme result in wideband Alamouti coding in frequency domain as discussed in Section 2.

Furthermore, the signs for the second antenna’s sequence is given as (1,1,1,1,-1,-1,-1,-1) which can be thought as the following phase rotation on each elements, (ej0,ej0,ej0,ej0,ejπ, ejπ, ejπ, ejπ). Therefore, we can notice a discontinuous phase rotation between the first and the second half of the sequence. Due to this discontinuity, the tone-wise Alamouti coding shown in [2] is not achievable in virtual split STBC of Figure 9.

In PAPR preserving SC-SFBC of Figure 10, the time domain sequence a0, …, a7 is cyclic shifted by 4. From the second antenna’s sequence in Figure 10, we can notice that the second antenna’s transmitted waveform is also DFT-s-OFDM. We can further notice the following two:

- Ascending time indexing for the second antenna transmission

- Continuous phase for the second antenna transmission

The time indices for the time domain sequence for antenna 2 in Figure 10 is (4,5,6,7,0,1,2,3), which can be thought as (0,1,2,3,4,5,6,7) with a cyclic shift of 4. Therefore, we can notice that time indexing for the second antenna in PAPR preserving SFBC is increasing. It is known that frequency domain conjugate corresponds to time domain conjugate with reversed index. Due to this ascending time indexing in Figure 10, the PAPR preserving SFBC scheme does not result in wideband Alamouti coding in frequency domain shown in Section 2.

Furthermore, the phase for the second antenna’s sequence is given as

(ej0,ej 6/8 π,ej 12/8 π,ej 18/8 π,-ej0,-ej 6/8 π,-ej 12/8 π,-ej 18/8 π)

which can be thought as the following phase rotation on each elements:

(ej0,ej 6/8 π,ej 12/8 π,ej 18/8 π,ej 24/8 π,ej 30/8 π,ej 36/8 π,ej 42/8 π).

Therefore, we can notice a continuously increasing phase rotation by ej 6/8 π for each element in the second antenna’s sequence. Thanks to this continuous phase rotation, the tone-wise Alamouti coding shown in [2] can be achievable in PAPR preserving SFBC of Figure 10.

In sum, even though both schemes in Figure 9 and Figure 10 provides Alamouti coding with DFT-s-OFDM waveforms, their frequency domain properties are fundamentally different due to distinct arrangement and phase-rotation of the second antenna’s sequence.

# References

- 3GPP RAN1 88 “RAN1 Chairman’s Notes”, Athens, Greece 13th – 17th February 2017.

- 3GPP R1-1700648, “PAPR preserving transmit diversity for DFTsOFDM,” Mitsubishi Electric, Spokane, Washington, January 16-20th 2017.

- 3GPP RAN1 88bis “RAN1 Chairman’s Notes”