3GPP TSG RAN WG1 #89					 R1- 1709641

Hangzhou, China 15th – 19th May 2017

Agenda Item:	7.1.4.1.2

Source: 	LG Electronics

Title: 	LDPC Codes Design for eMBB data channel

Document for:	Discussion

Introduction

In last RAN1#88bis meeting, we made following working assumptions:

Working Assumption:

- The largest info block size supported by LDPC encoder Kmax and the largest shift size Zmax defined is {8448, 384} => Kbmax = 22

- To be confirmed automatically at RAN1#89 if no significant implementation or performance issues are identified.

- The base graph supporting Kmax should support the following set of shift sizes Z, where Z= a×2j:

| Z |  | a |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2 | 3 | 5 | 7 | 9 | 11 | 13 | 15 |
| j | 0 | 2 | 3 | 5 | 7 | 9 | 11 | 13 | 15 |
|  | 1 | 4 | 6 | 10 | 14 | 18 | 22 | 26 | 30 |
|  | 2 | 8 | 12 | 20 | 28 | 36 | 44 | 52 | 60 |
|  | 3 | 16 | 24 | 40 | 56 | 72 | 88 | 104 | 120 |
|  | 4 | 32 | 48 | 80 | 112 | 144 | 176 | 208 | 240 |
|  | 5 | 64 | 96 | 160 | 224 | 288 | 352 |  |  |
|  | 6 | 128 | 192 | 320 |  |  |  |  |  |
|  | 7 | 256 | 384 |  |  |  |  |  |  |

- FFS by RAN1#89 whether some values can removed from the above table.

- FFS by RAN1#89 whether some of {272, 304, 336, 368} can be added to the above table.

Conclusion:

- Companies to submit almost-final proposals for one base graph by RAN1#89

  - Fine-tuning still permitted until submission deadline for June adhoc.

  - Final base graph to be finalized by the June NR Ad-Hoc meeting

- If it is agreed to support a second base graph, aim also to finalise it by the June adhoc.

Based on the working assumption and conclusion above [1], we propose LDPC codes for eMBB data channel.

Proposed LDPC Codes

2.1 Structure of proposed QC-LDPC

The coding scheme for NR requires supporting wide range of code rates, variable code blocks as well as fine granularity. In order to satisfy these requirements, we propose two base codes and the associated set of lifting sizes. (The details of BC2 was provided in latest contribution [2]).

| Base code(BC) | Mb,min × Nb,min (Mb,max × Nb,max) | Kb,max | Pb | Max. code rate | Max. information | Lifting Set (Z) |
| --- | --- | --- | --- | --- | --- | --- |
| BC1 | 5×27 (46×68) | 22 | 2 | 0.89 | 8448 | Set1: 384, 320, 192, 160, 96, 48, 24 Set2: 256, 224, 128, 64, 32, 16, 8, 4 |
| BC2 | 6×16 (42×52) | 10 | 2 | 0.71 | 2560 | 256, 224, 192, 160, 128, 112, 96, 80, 64, 56, 48, 40, 32, 28, 24, 20, 16, 14, 12, 10, 8 |

- Mb,min means the minimum number of rows in base code.

- Nb,min means the minimum number of columns in base code.

- Kb,max means the maximum number of column for information in base code.

- Pb means the number of systematic information puncturing in base code, not transmitted.

Table 1 Base Code parameters

The red-colored lifting values in Table 1 indicate the representative lifting values whose full description of parity check matrix (H-matrix) are enclosed as circulant-value table format. H-matrixes for the other Zs can be calculated from the representative circulant-value tables.

2.2 Supporting flexibility of LDPC Codes

In order to calculate circulant permutation sub-matrixes (CPM) for a Z-lifted parity check matrix, simple formula is provided as follow. HZi,j is the circulant values for Z-lifted CPM of (i-th row, j-th column) position in base code.

HZi,j=HZmaxi,j &           if HZmaxi,j<0(HZmaxi,j mod Zt) & mod Z,     otherwise

where HZmax(i,j) presents circulant value of (i-th row, j-th column) position in a base code for Z-lifting, which is provided in the xlsx file. Zt presents pre-scaling values for BC1 shown at Table 2.

| Z | 320 | 224 | 160 | Others |
| --- | --- | --- | --- | --- |
| Zt | 384 | 256 | 192 | Z |

Table 2 pre-scaling Zt

2.3 Performance evaluation

The simulation parameters are briefly given in the Table 3.

| Channel | AWGN |
| --- | --- |
| Modulation | QPSK |
| Coding Scheme | QC-LDPC code |
| Code rate | BC1: 1/3, 2/5, 1/2, 2/3, 3/4, 5/6, 8/9 |
| Decoding algorithm | Standard flooding with SPA |
| # of decoding iteration | 50 |
| Info. Block Length | 100, 400, 1000, 2000, 4000, 6000, 8000 |

Table 3. Simulation Parameters

According to the performance results in the following Figure 1~7, the proposed LDPC codes have very stable BLER performance at least down to BLER 1e-4.

Observation 1: The proposed LDPC codes show consistent performance at least down to BLER 1e-4, according to the variable information block size and code rates.

Figure 1 BLER performance for K=100

Figure 2 BLER performance for K=400

Figure 3 BLER performance for K=1000

Figure 4 BLER performance for K=2000

Figure 5 BLER performance for K=4000

Figure 6 BLER performance for K=6000

Figure 7 BLER performance for K=8000

Additionally, Figure 8 presents the required SNR variation at target BLER as information block length changes. It is observed that the proposed LDPC codes shows stable performance tendency across all information size.

Figure 8 Required SNR at target BLER

Conclusions

In this contribution, we propose new LDPC code design and present some performance results.

Observation 1: The proposed LDPC codes show good error floor performance at least down to BLER 1e-4, according to the variable information block size and code rates.

References

[1]	Chairman’s Notes, RAN#88-bis, USA

[2] R1-1704928, RAN#88-bis, LG Electronics