3GPP TSG RAN WG1 Meeting #89	 R1-1709707

Hangzhou, China, 15-19 May 2017

Agenda Item:	6.2.8.4

Source:	Huawei, HiSilicon

Title:	TP on potential problems for study on enhanced support of aerial vehicles

Document for:	Discussion and decision

- Introduction

This text proposal is to provide potential problems for study on enhanced support of aerial vehicles [1].

- Text proposal

------------------------------------------------------------- Text start -----------------------------------------------------

- 6	Potential problem statements

Editor’s note:	Potential issues to cater for aerial vehicles using the existing LTE network are to be captured will be captured in this section.

a)	High interference received by the terrestrial UEs from the UL transmission of aerial vehicles

The radio propagation characteristics experienced by the aerial vehicles are different from those experienced by terrestrial UEs. Once an aerial vehicle is flying well above the BS antenna height, the UL signal from the aerial vehicle becomes more visible to multiple cells due to line-of-sight propagation conditions. The UL signal from an aerial vehicle increases interference in the neighbour cells. The increased interference gives a negative impact to the terrestrial UE, e.g. smartphone, IoT device, etc.

b) High interference received by aerial vehicles from the DL transmission in neighbour cells

The radio propagation characteristics experienced by the aerial vehicles are different from those experienced by terrestrial UEs. Once an aerial vehicle is flying well above the BS antenna height, the aerial vehicle becomes more visible to multiple cells due to line-of-sight propagation conditions. The DL transmission from multiple neighbouring cells increases interference to the DL transmission of aerial vehicle in serving cell.

------------------------------------------------------------- Text end -----------------------------------------------------

- Conclusion

Proposal: Capture the text proposal for potential problems for study on enhanced support of aerial vehicles.

- References

- [1] 	RP-170779, “New SID on Enhanced Support for Aerial Vehicles”, NTT DOCOMO INC, Ericsson, Dubrovnik, Croatia, March 6 - 9, 2017.