3GPP TSG RAN WG1 Meeting #90	R1-1713448

August 21st – 25th, 2017

Prague, Czech Republic

Agenda item:	6.1.3.3.5.1

Source: 	Qualcomm Incorporated

Title: 	HARQ feedback timing in FDD

Document for:	Discussion/Decision

# Introduction

Currently most of NR discussion in RAN1 has been focusing on TDD design with FDD/TDD design compatibility in mind. This contribution provides overview on NR FDD design focusing on HARQ, based on the agreements so far made in 3GPP.

# Slot structure

The slot structure for FDD is compared with that of TDD in Figure 1. It can be seen that FDD full slot for DL and UL can be considered equivalent to TDD DL only slot and TDD UL only slot respectively. It should be noted that the length of PDCCH/PDSCH/PUCCH and PUSCH can vary depending on the configuration. Given flexible design in NR for each control and data channel, it is worth pointing out that we can also achieve the DL portion of TDD DL centric slot and the UL portion of TDD UL centric slot in FDD, as illustrated in Figure 2. Therefore, it can be concluded that no new slot structure needs to be introduced for FDD.

Observation 1: No new slot structure needs to be introduced for FDD.

Figure 1: TDD and FDD slot structure comparison

Figure 2: FDD partial slot comparison with TDD DL and UL centric slot

# PHY channels

For both DL and UL PHY channels, TDD and FDD can share the same design. No new channel design is expected for FDD compared to TDD. Even synchronization signals share the exactly the same design between TDD and FDD unlike LTE. Therefore, the detection of PSS/SSS will not provide information on TDD/FDD. RMSI can provide this information.

Observation 2: For both DL and UL PHY channels, TDD and FDD can share the same design.

Observation 3: RMSI can provide TDD/FDD information.

# HARQ timeline

Considering very flexible design in HARQ design, we can assume that the same framework can be utilized for both TDD and FDD. Current HARQ framework allows configurable timing for:

- DL grant to DL data

- DL data to ACK on UL

- UL grant to UL data

- UL data to ACK on DL

Next, we will take a look at HARQ timeline for FDD.

## TDD-like timeline

In order to fully reuse the implementation in TDD, exactly the same HARQ timeline can be achieved with FDD partial slots shown in Figure 2. Both DL and UL HARQ timeline is shown with short PUCCH in Figure 3 and with long PUCCH in Figure 4. It can be assumed that the timing in Figure 3 and Figure 4 is at gNB (i.e., after TA and propagation delay). The blank part in both DL and UL spectrum can be utilized by mini-slots (for same or different UEs) to maximize spectral efficiency of the spectrum. It should be noted that lax timeline via cross slot scheduling/acknowledgement with more HARQ processes can be also supported.

Figure 3: TDD-like HARQ timeline with short PUCCH in FDD (2 DL and 2 UL HARQ processes)

Figure 4: TDD-like HARQ timeline with long PUCCH in FDD (3 DL and 2 UL HARQ processes)

## Additional HARQ with FDD full slots

HARQ timeline with FDD full slots are shown with short PUCCH in Figure 5 and with long PUCCH in Figure 6. Compared to TDD-like HARQ timeline in subsection 3.1.1, One additional HARQ is needed for both DL and UL with one slot delay. Lax timeline via cross slot scheduling/acknowledgement with more HARQ processes can be also supported.

Figure 5: FDD full slot HARQ timeline with short PUCCH in FDD (3 DL and 3 UL HARQ processes)

Figure 6: FDD full slot HARQ timeline with long PUCCH in FDD (4 DL and 3 UL HARQ processes)

Observation 4: TDD-like HARQ timeline can be achieved in FDD with aligned UL.

## Other consideration

For FDD, an offset between UL slot and DL slot can be introduced via TA given there are parallel DL and UL spectrum. With an appropriate offset, it can provide more evenly distributed processing time between UE and gNB. Considering evenly distributed processing time, this may appear more attractive compared to aligned UL case. However, it will introduce deviation from TDD design and there would be issues with half-duplex FDD as discussed in the next section. It also should be noted that if UE and gNB is already budgeted for TDD timeline, no additional margin for processing time may be necessary with the same implementation.

Observation 5: More balanced processing time can be achieved with introducing an offset in UL slot in FDD.

# Half Duplex FDD

This section looks into half duplex FDD (HD-FDD) operation for NR. Similar to LTE, we assume the same rule of operation for HD-FDD UE in NR, i.e., UE listens to DL unless transmitting on UL. Figure 9 and Figure 10 shows DL and UL HARQ timeline for HD-FDD with aligned UL. For DL HARQ, 2 DL HARQ processes can be used. With short PUCCH, TDD-like timeline can be achieved. With long PUCCH, toggling of DL slot and UL slot can be naturally adopted. Blank portion can be utilized by full-duplex UEs. For UL HARQ, 2 UL HARQ processes with TDD-like timeline can be fulfilled. Blank portion can also be utilized by full-duplex UEs.

On the other hand, in the case of UL with an offset relative to DL slot, it would be trickier to operate HD-FDD compared to the timeline shown in Figure 9 and 10. Due to the nature of mis-alignment, it may be necessary to puncture data when ACK is transmitted. This may require additional signalling to indicate the puncturing. Also the operation is not quite clean compared to the aligned UL case.

Figure 9: HD-FDD DL HARQ timeline with aligned UL

Figure 10: HD-FDD UL HARQ timeline with aligned UL

Observation 6: HD-FDD can be easily adopted for FDD with aligned UL.

# Conclusion

This contribution has provided overview on NR FDD design focusing on HARQ procedure, based on the agreements so far made in 3GPP. The following observations have been made:

Observation 1: No new slot structure needs to be introduced for FDD.

Observation 2: For both DL and UL PHY channels, TDD and FDD can share the same design.

Observation 3: RMSI can provide TDD/FDD information.

Observation 4: TDD-like HARQ timeline can be achieved in FDD with aligned UL.

Observation 5: More balanced processing time can be achieved with offseted UL in FDD.

Observation 6: HD-FDD can be easily adopted for FDD with aligned UL.