3GPP TSG RAN WG1 Meeting #90                             	R1-1714397

Prague, Czech Republic, 21th – 25th August 2017

Source:	WILUS Inc.

Title:	Discussion on UE behavior by pre-emption indication

Agenda item:	6.1.3.3.7.1

Document for:	Discussion/Decision

- Introduction

It has been agreed that NR supports pre-emption indication to tell the UE(s) which DL physical resources has been pre-empted and the downlink pre-emption indication is transmitted using a group-common DCI in PDCCH [1]-[3].

Agreements: (RAN1 #88) - Indication can be dynamically signaled to a UE, whose assigned downlink resources have partially been pre-empted by another downlink transmission, to increase the likelihood of successful demodulation and decoding of the TB(s) transmitted within the above mentioned assigned resource - The indication may be used to increase the likelihood of successful demodulation and decoding of the transport block based on the pre-empted transmission and/or subsequent (re)-transmissions of the same TB Agreements: (RAN1 #89) - For DL CBG-based (re)transmission, - Following information can be configured to be included in the same DCI: - Which CBG(s) is/are (re)transmitted. - Which CBG(s) is/are handled differently for soft-buffer/HARQ combining. - FFS: whether/how UE behavior is specified, e.g., part/whole of soft-buffer of indicated CBG(s) is flushed. - FFS: timing of CBG-based (re)transmission. - For pre-emption indication; - When configured, the indication tells the UE(s) which DL physical resources has been pre-empted. - The pre-emption indication is transmitted using a PDCCH. - The pre-emption indication is not included in the DCI that schedules the (re)transmission of the data transmission. - FFS: the granularity of the time and/or frequency resources. - FFS: what DCI is used. - FFS: timing of the pre-emption indication. Agreements: (NR Ad-hoc#2) - For downlink preemption indication - It is transmitted using a group common DCI in PDCCH - FFS: This group common DCI is transmitted separately from SFI - Whether a UE needs to monitor preemption indication is configured by RRC signaling - The granularity of preemption indication in time domain can be configured - Details of granularity are FFS

In this contribution, we discuss UE’s HARQ-ACK feedback generation and gNB’s retransmission for pre-emption recovery when the pre-emption indication is configured

- UE’s HARQ-ACK Feedback Generation and gNB’s Retransmission

As RAN1 agreed, NR supports a pre-emption indication to inform to a UE which physical resources are impacted by a different DL transmission. From this pre-emption indication, the impacted UE(s) can know which physical resources are pre-empted for a different UE so that it can increase decoding likelihood by discarding the pre-empted physical resources (i.e., inserting zeros to the corresponding soft-bits). This pre-emption indication could be effective to the case that small parts of CBs are pre-empted and/or the code-rate is quite low. In order to obtain such an advantage, it can be desirable that the impacted UE(s) receives the pre-emption indication as early as possible. If the pre-emption indication is received after HARQ-ACK feedback generation timing, the UE cannot report appropriate HARQ-ACK feedback to gNB even if the TB can be correctly decoded. In this case, the UE would surely rely on the HARQ retransmission of the impacted TB, which would result in unnecessary retransmission.

- Proposal 1. In order to use the pre-emption indication for demodulation and decoding steps, the pre-empted UE should receive the pre-emption indication before own HARQ-ACK feedback generation timing.

The effectiveness of the pre-emption indication can be quite limited. In case that most of physical resources of a CB(s) are pre-empted, the UE would usually fail to decode the impacted CB(s). Also, in case that code-rate is quite high, small parts of resource impacted by pre-emption would be critical to decode the impacted CB(s). Therefore, we can categorize the CBs into two categories:

Cat. 1) No impact on CBs: The CBs with no resource pre-emption or the CBs of which small parts of resources are pre-empted, but sufficiently robust against the pre-emption to be decoded successfully.

Cat. 2) Critical impact on CBs: The CBs impacted by the pre-emption - A UE usually fails to decode these CBs due to the pre-emption.

It should be noted that this classification can be determined after receiving the pre-emption indication, depending on how many resources are pre-empted and whether the CBs with resource pre-emption are decodable or not. On the other hand, all of CBs are classified into “No-impact-on-CBs” in the case that the pre-emption indication is not received.

For example, as illustrated in Figure 1, where 10 CBs are mapped to the 8 OFDM symbols, when DL resources in the 4th and 5th OFDM symbols have been partially pre-empted, CB3, CB4, and CB4 may be impacted by the pre-emption. After receiving the pre-emption indication, CB4 and CB5 would not be decoded correctly due to a large portion of resource pre-emption. However, CB3 may be successfully decoded because of a small portion of resource pre-emption. Therefore, we can categorize the CBs as follows: {CB0, CB1, CB2, CB3, CB6, CB7, CB8, CB9} as “No-impact-on-CBs” and {CB4, CB5} as “Critical-impact-on-CBs”.

One interesting point is that both UE and gNB can know the critically impacted CBs may not be decoded at the UE with high probability (one example is that the critically impacted CBs are the CBs of which all resources are punctured so that the decoding likelihood is nearly zero). When all CBs including critically impacted CBs in a TB are handled by the same way as in current HARQ operation with no consideration of the effect of pre-emption, the UE’s HARQ-ACK feedback is almost always NACK. In this case, the spectral efficiency is much lower because all CBs corresponding to the TB need to be re-transmitted as in current LTE HARQ design. Thus, it is necessary to design the UE’s HARQ-ACK feedback and gNB’s HARQ retransmission based on the HARQ-ACK feedback by considering the effect of the pre-emption.

Figure 1. An illustration of resource pre-emption with multiple CBs in a TB

- Observation 1: In cases that almost all physical resources of a CB(s) are pre-empted or the CBs have high code-rate with small portion of resource pre-emption, the UE usually fails to decode the pre-empted CB(s). However, CBs in a same TB can be decodable successfully with no resource pre-emption or small portion of resource pre-emption by using the pre-emption indication.

- Proposal 2: NR should consider an efficient HARQ-ACK feedback generation rule and retransmission rule in case of presence of the pre-emption indication.

Based on the above observations, as shown in Figure 1, some of CBs in a TB are not decodable by the pre-emption indication with high probability. The recovery of the TB relies on the retransmission of gNB. To re-transmit the pre-empted CBs, the UE send a feedback NACK for the TB even when the remaining CBs (e.g., CB0-CB3, CB6-CB9) are successfully decoded. In that case, all CBs in the TB are retransmitted, which would result in unnecessary retransmission.

Rather than reporting ACK/NACK feedback for all CBs in the TB, the UE can report ACK/NACK only for the “No-impact-on-CBs.”As shown in Figure 2, the UE can report ACK if CRCs of the not-critically impacted CBs are correctly checked. Otherwise, the UE can report NACK. gNB can determine which CBs are not critically impacted (if the selection rule is pre-defined – One simple rule is to consider CBs with no resource pre-emption). In that case, gNB can retransmit the critically impacted CBs only if the UE’s HARQ-ACK feedback is ACK. Also, the gNB retransmits all CBs in the TB as normal HARQ retransmission when the UE’s HARQ ACK feedback is NACK. This HARQ-ACK feedback design is enable to provide more opportunities to know which CBs are successfully decoded and to avoid the retransmissions of the successfully decoded CBs. One drawback is that gNB may retransmit the critically impacted CBs even if these CBs may be successfully decoded. Thus, the selection rule for non-critically impacted CBs is important to reduce the unnecessary retransmission.

- Proposal 3: In case of TB-based retransmission, when a UE is aware that some of the allocated physical resources are impacted by the different DL transmission from the pre-emption indication, the UE can generate HARQ-ACK feedback based on the CB-level CRCs of “no-impact-on-CBs.”

  - If not critically impacted CBs is all of CBs, the UE can generate HARQ-ACK feedback based on the TB-level CRC.

  - FFS: detail selection rule of the critically impacted CBs

- with preemption                 (b) without preemption

Figure 2. HARQ-ACK generation according to the presence of pre-emption

One of possible issues is whether the HARQ-ACK process works well in the case that a UE misses the pre-emption indication. When a UE misses the pre-emption indication, the UE’s HARQ-ACK feedback is generated based on all CBs, but the gNB consider that the UE’s HARQ-ACK feedback is generated based on non-critically impacted CBs. If all CBs are non-critically impacted CBs, there is no other issue. However, when at least one critically impacted CB(s) exists in a TB and the UE sends the HARQ-ACK feedback as “NACK,” the gNB retransmit all of CBs as normal HARQ operation. In the case of ACK feedback, the gNB retransmits critically impacted CBs, then the UE reports ACK again since the UE already decodes the TBs successfully. Therefore, the uplink HARQ-ACK feedback can be wasteful but the HARQ-ACK process still works well.

- Conclusion

In this contribution, we discussed the HARQ-ACK design and gNB’s retransmission. Our views are summarized as follows:

- Observation 1: In cases that almost all physical resources of a CB(s) are pre-empted or the CBs have high code-rate with small portion of resource pre-emption, the UE usually fails to decode the pre-empted CB(s). However, CBs in a same TB can be decodable successfully with no resource pre-emption or small portion of resource pre-emption by using the pre-emption indication.

- Proposal 1. In order to use the pre-emption indication for demodulation and decoding steps, the pre-empted UE should receive the pre-emption indication before own HARQ-ACK feedback generation timing.

- Proposal 2: NR should consider an efficient HARQ-ACK feedback generation rule and retransmission rule in case of presence of the pre-emption indication.

- Proposal 3: In case of TB-based retransmission, when a UE is aware that some of the allocated physical resources are impacted by the different DL transmission from the pre-emption indication, the UE can generate HARQ-ACK feedback based on the CB-level CRCs of “no-impact-on-CBs.”

  - If not critically impacted CBs is all of CBs, the UE can generate HARQ-ACK feedback based on the TB-level CRC.

  - FFS: detail selection rule of the critically impacted CBs

References

[1] Final Report of 3GPP TSG RAN WG1 RAN1 #88

[2] Draft Report of 3GPP TSG RAN WG1 RAN1 #89

[3] Draft Report of 3GPP TSG RAN WG1 NR Ad-hoc #2