3GPP TSG RAN WG1 Meeting #90	R1-1714793

Prague, Czech Republic, 21st – 25th August 2017

Source:	Ericsson

Title:	Information Sequence Design for Polar Codes

Agenda Item:	6.1.4.2.2

Document for:	Discussion and Decision

# Introduction

In RAN1 NR Ad-Hoc#2, a decision procedure was agreed on how to select information sequences among the sequence candidates submitted by the companies.

In this contribution, we discuss the performance and properties of the submitted sequences.

# Discussion

To complete the Polar code desgin work, a total of 7 candidate sequences were submitted to the RAN1 email reflector. The submitted sequences all have the nested property, where the length-N/2 sequence can be otained from the length-N length by extracting entries that are smaller than N/2 while preserving the order of the entries.

The sequence candidates were then evaluated by companies using the agreed simulation assumption as described in Appendix 2. According to the evaluations submitted by participating companies, 3 sequences, [1][2],[3], were identified by one or more companies as survivors of the down selection process. The evaluation results of the 3 surviving sequences are presented in Appendix 1.

Both canidate [1] and [2] are joint designs by five companies. Joint design is usually encouraged in RAN1 compared with single company design. Hence candidate [1] and [2] should be given higher priority than [3]. Between candidate [1] and [2], candidate [1] garnered more support than [2] in the evaluation process. Hence it is appropriate that candidate [1] should be selected as information sequence for NR Polar codes.

Further, it has been identified that candidate [3] does not comply with Universal Partial Order (UPO) property for any N, 64<=N<=1024. In contrast, candidate [1] complies with UPO property for 64<=N<=256, and candidate [2] complies with UPO property for 64<=N<=512.

As UPO property is useful to guarantee good performance for any underlying channel type and any SNR value, UPO property should be incorporated as long as the BLER performance is not degraded for the simulated conditions.

Indeed, candidate [1] can be adjusted to be fully UPO-compliant by rearranging 14 entries (out of 512 entries) for the N=512 sequence. The N=64, 128, 256 sequences are not affected. The N1024 sequence is ajusted by nesting on the UPO-compliant N512 sequence without any other change.The adjusted set of sequences is shown in the companion Excel sheet. Simulations show that the UPO-compliant variation of candidate [1] has the same performance as candidate [1].

Details of example adjustment to achieve UPO for Candidate [1] and Candidate [3] are shown in the section below.

- Candidate [1] is selected as information sequence of NR Polar codes.

- If UPO property is desired, the UPO-compliant variant of candidate [1] should be selected.

# Adjustment to Achieve UPO

Example adjustment to the submitted candidates [1] and [3] are shown below, where the adjusted sequences become fully UPO compliant for all N values, 64<=N<=1024. The resulting sequences are listed in the attached excel sheet. In the following subsections, the re-arranged entries are described.

## Example adjustment to Candidate [1] to achieve UPO:

- N=1024: Same entries to adjust as N=512 sequence, i.e., only entries due to the N=512 sequence nested inside.

- N=512:

Entries in submitted sequence: 260,35,132,264,37,136,272,38,26,67,41,144,288,69,42,49,160,320,74,81,15,52,384,133,192,328,105,200

Corresponding entries in adjusted sequence:

35,132,260,37,136,264,38,26,67,41,144,272,69,42,49,160,288,74,81,15,52,133,192,320,384,105,200,328

- N=256: submitted sequence is UPO compliant.

- N=128: submitted sequence is UPO compliant.

- N=64: submitted sequence is UPO compliant.

## Example adjustment to Candidate [3] to achieve UPO:

Red font highlights the entries that are adjusted in sequence of length N, beyond those due to adjustment in the lower N.

- N = 1024:

Entries in submitted sequence: 26,25,22,74,70,73,304,296,608,352,704,448,678,677,790,789,374,366,429,373,427,251,442,441,247,922,918,502,494,921,501,959,895

Corresponding entries in adjusted sequence:

25,22,26,70,73,74,296,304,352,608,448,704,677,678,789,790,366,373,427,374,429,247,441,442,251,918,921,494,501,922,502,895,959

- N = 512:

Entries in submitted sequence:

26,25,22,74,70,73,304,296,390,174,393,283,122,448,374,366,429,373,427,251,442,441,247,502,494,501

Corresponding entries in adjusted sequence:

25,22,26,70,73,74,296,304,448,390,174,393,283,122,366,373,427,374,429,247,441,442,251,494,501,502

- N = 256:

Entries in submitted sequence:

26,25,22,74,70,73,251,247

Corresponding entries in adjusted sequence:

25,22,26,70,73,74,247,251

- N = 128:

Entries in submitted sequence:

26,25,22,74,70,73,

Corresponding entries in adjusted sequence:

25,22,26,70,73,74,

- N = 64:

Entries in submitted sequence:

26,25,22

Corresponding entries in adjusted sequence:

25,22,26

# Conclusions

In this contribution discussed the information sequence candidates submitted to RAN1 for NR Polar codes. Based on the discussion in this contribution we propose the following:

- Candidate [1] is selected as information sequence of NR Polar codes.

- If UPO property is desired, the UPO-compliant variant of candidate [1] should be selected.

# References

Polar code information sequence submitted by Ericsson to the RAN1 reflector. Joint design by DOCOMO, Ericsson, Intel, Qualcomm, and Samsung. August 04, 2017.

Polar code information sequence submitted by Samsung to the RAN1 reflector. Joint design by DOCOMO, Ericsson, Intel, Qualcomm, and Samsung. August 04, 2017.

Polar code information sequence submitted by Huawei to the RAN1 reflector. August 04, 2017.