3GPP TSG RAN WG1 Meeting #90                                            		R1-1714983

Prague, P.R. Czech Republic 21th – 25th August 2017

Agenda Item:	6.1.4.2.4

Source: 	Intel Corporation, Fujitsu

Title:	Proposed Evaluation assumptions for Polar Channel Interleaver for DL

Document for:	Discussion/Decision

# Introduction

For Polar channel interleaver, it was agreed to check performance for DL based on the following agreement.

Agreement:

- For UL, the channel bit interleaver is a separate stage after rate matching.

- For DL, check performance based on assumptions made in Scheduling/HARQ  session (including assumptions on interference to be proposed by Tim Moulsley by Fri), and using the rate matching interleaver taken as working assumption on Fri (e.g. in R1-1715000); if it is identified that the interleaving is insufficient, the channel bit interleaver will be adopted after rate matching also for the DL – decide at NR AH#3.

In this document, some evaluations assumptions are proposed to enable checking performance of DL Polar channel interleaver proposals.

- Proposed Evaluation Assumptions

| Parameter | Value |
| --- | --- |
| System bandwidth | 10 MHz |
| Waveform | OFDMA |
| Numerology | 15 kHz |
| Payload (not including CRC) | 32, 60 bits |
| FEC type and Modulation | Polar with CRC size =24, QPSK |
| Tx-Rx antenna configuration | 2x2 |
| Transmit diversity scheme | 1-port per REGB precoder cycling |
| Channel estimation | 1/3 DM-RS density, practical channel estimation (MMSE) |
| Channel model | TDL-C 300ns |
| Number of REGs per CCE | 6 |
| Aggregation levels | 1, 8 |
| REG bundle size | 2 REGs, 6 REGs |
| CORESET configuration | 1 symbol, 48 PRBs (i.e. PRB0,PRB1…PRB47) |
| CCE-to-REG mapping | Frequency first |
| Interleaving for CCE-to-REG mapping | For evaluation only, Sub-block interleaver operating on REG bundles |