3GPP TSG RAN WG1 Meeting #92 		         R1-1801707

Athens, Greece, February 26th – March 2nd, 2018

Source:	CATT

Title:	Discussion on latency reduction between packet arrival and resource selection

Agenda Item:	6.2.3.5

Document for:	Discussion and Decision

Introduction

In RAN1 #91, the following RAN 1 objectives were approved in work item on 3GPP Phase2 V2X [1]:

# Agreement - The minimum value of T2 can be reduced to support Layer 1 latency reduction. - (Pre) configuration based selection of minimum value of T2 is supported. - The minimum value of T2 is selected from a set of values. - The set of values includes at least 20ms, and a value lower than 20ms (FFS how many additional values). - FFS: whether the (pre)configuration is per PPPP, CBR range, per carrier, or if it intends to have a similar behaviour as a rel-14 UE, etc.

In this contribution, we discussed the latency reduction between packet arrival at layer 1 and resource selection for transmission. For the convenience, the latency mentioned in this contribution is referred to the latency between the packet arrival at layer 1 and resource selection for transmission.

Discussion

According to the use case in 3GPP TR 22.886 [3], the latency requirements are varied from 3ms to 100ms. It is not feasible to realize the minimum latency requirement of 3ms with Rel-14 LTE-V2X numerology and current UE’s capability. The use cases of latency requirements lower than 20ms is summarized in the following table:

Table 1 The use cases of latency requirements lower than 20ms in TR 22.886

| Type of use cases | Communication scenario |  |  |  |  |  | Payload (Bytes) | Tx rate (Message/ Sec) | Max end-to-end latency (ms) | Reliability (%) | Data rate (Mbps) | Communication range (meters) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Section # | Description |  |  |  | CPR # |  |  |  |  |  |  |
| platooning | 5.1 | Among a group of UEs (or two UEs) supporting V2X application |  |  |  | [CPR.P-004] | 50-1200 (NOTE 1) | 30 | 10 |  |  |  |
|  | 5.5 | Between UEs supporting V2X application | Fully automated driving |  |  | [CPR.P-008] | 1200 |  | 10 | 99.99 |  | 80 |
| advanced driving | 5.9 | Between UEs supporting V2X applications Fully automated driving |  |  |  | [CPR.A-001] | [2000] |  | [10] | [99.99] | [10] |  |
|  | 5.20 | Between UEs supporting V2X application Fully automated driving |  |  |  | [CPR.A-006 |  |  | [3] | [99.999] | [30] | [500] |
|  | 5.23 | Between UEs supporting V2X application |  | Full automated driving |  | [CPR.A-009] | [12000] |  | [10] | [99.99] |  |  |
| extended sensors | 5.3 | Between UEs supporting V2X application Fully automated driving |  |  |  | [CPR.E-001] |  |  | 10 | 95 | Peak data rate [25] |  |
|  | 5.6 | Between UEs supporting V2X application |  |  | Fully automated driving | [CPR.E-003] |  |  | 3 | 99.999 |  | 200 |
|  |  |  |  |  |  | [CPR.E-004] |  |  | 10 | 99.99 |  | 500 |
|  | 5.16 | Between UEs supporting V2X application |  |  | Fully automated driving | [CPR.E-008] |  |  | [10] | 99.99 | [700] | [500] |

From the Table 1, the use cases, e.g. 5.20 and 5.23 with large payload size and extremely stringent reliability requirements, are difficult to be fulfilled for LTE-V2X based enhanced scheme. The use case 5.1 and 5.3 may be supported with enhanced latency reduction and other scheme in Rel-15. The performance should be further evaluated.

In Rel-14 LTE-V2X, latency of 100ms is assumed for the baseline resource selection solutions. The lower latency of 20ms and 50ms has been introduced in the design of resource selection scheme. In order to support the 20ms and 50ms requirements, the scaling method based on the 100ms is applied in the process of sensing window, resource selection and resource reservation. If further reduction of T2 is considered, the target use cases should be clarified and the target latency value should be defined.

The service priority can be mapped into PPPP, and PPPP can also be mapped into PDB (Packet Delay Budget). Currently, the mapping of PPPP/PDB to T2 is not clearly defined [4]. In order to support different services of different latency requirements, the mapping relationship between PPPP/PDB and T2 should be provided by upper layer. LS should be sent to RAN2 to define the mapping relationship between PPPP/PDB and T2.

When T2 is decreased, the length of the resource selection window is also decreased. As defined in TS 36.213 [2], considering the skipped subframe and the excluded subframe with PSSCH-RSRP higher than the threshold based on the TX-RX priorities, the number of the available candidate single-subframe resources may be decreased with decreasing T2. Because of the different T2 configurations, the TBs arriving at the layer 1 may not be distributed evenly. If the ratio of the candidate single-subframe resources to the total number of the candidate single-subframe resources is still set as 20%, in the short resource selection window of decreasing T2, the PSSCH-RSRP threshold may be increased iteratively to unreasonable high. If the ratio of to can be decreased lower than 20%, the number of iteration of uplifting the PSSCH-RSRP threshold with 3dB may be reduced and provide the reasonable threshold for the resource selection. However, with decreased ratio, the probability of the resource collision may increase. With the consideration of the unevenly distributed services scenario due to variable T2 value, the ratio (20%) of to  should be re-evaluated, especially for the service with stringent latency requirement.

In legacy Rel-14 V2X mode 4, candidate resources in the resource selection window are selected based on the sensing mechanism and the random mechanism is selected to reduce the resource collision and half duplex impacts. However, if the T2 is decreased, it will reduce the resource selection window in time-domain, and it will increase the probability of resource collision and half duplex impacts. In order to avoid the resource collision, some extra mechanisms shall be considered in resource selection, e.g., the preemption mechanism based on the service priory is feasible and could be re-evaluated in Rel-15.

Proposal 1: When T2 is decreased, the following issues should be considered:

- The target use cases should be clarified and the target latency value should be defined.

- The mapping relationship between PPPP/PDB and T2 should be provided by upper layer. LS should be sent to RAN2 to define the mapping relationship between PPPP/PDB and T2.

- In the unevenly distributed services scenario, the ratio (20%) of the candidate single-subframe resources to the total number of the candidate single-subframe resources  should be re-evaluated, especially for the service with stringent latency requirement.

- In order to avoid the resource collision, some extra mechanisms shall be considered in resource selection, e.g., the preemption mechanism based on the service priory is feasible and could be re-evaluated in Rel-15.

Because the physical layer scheme provides the transmission capability for the packets from upper layer, the enhanced multiple SPS processes and CA scheme should be considered to reduce the buffering time in the upper layer and may improve the performance of the reliability.

In Rel-14, only two SPS processes are defined to support CAM/DENM messages for Mode 4. If the latency lower than 20ms is supported, when CAM/DENM messages occupied the two SPS processes, there may be no available SPS processes for the stringent latency services. The packets of the services with latency lower than 20ms may be restricted to be transmitted with on-shot mechanism. Thus, the performance of the lower than 20ms latency services may be deteriorated of the resource collision. Therefore, extended SPS process shall be provided to support the service with stringent requirement except CAM and DENM message, and the latency reduction could be guaranteed with the decreasing T2 during the process of the resource selection.

Proposal 2: The number of sidelink SPS process for mode 4 shall be extended to support the service with stringent requirement except CAM and DENM message.

Because one service can be mapped into different carriers, if the multiple segments of the large packet can be transmitted on the multiple carriers, the buffering time of the segments can be reduced. Thus, the CA scheme can be utilized to reduce latency.

Figure 1 The example of the impact of the half-duplex in V2X CA scheme

In Figure 1, the example of the impact of the half-duplex in V2X CA scheme is presented. It is assumed that two component carriers as CC1 and CC2. In the resource selection window, T1 is set as 4ms and T2 is decreased to 10ms. When the TBs of the segments from the large packet are arrived at layer 1 in n subframe, the resources of the first transmission and retransmission in the resource selection window are selected on the CC1 and CC2 respectively. Because T2 is decreased, the candidate single-subframe resources may be decreased. As the example in Figure 1, though only two transmissions are in every CC, there are four subframes cannot sense with the impact of the half-duplex. As the analysis in the companion contribution, when the number of the carriers is increased, the performance will be deteriorated further with the impact of the half-duplex. The enhanced CA scheme is also proposed in this companion contribution to mitigate the impact of the half-duplex [5].

Proposal 3: Combining with decreased T2, the enhanced CA scheme should be considered to mitigate the impact of the half-duplex and achieve the reduced latency.

Conclusion

In this contribution, the latency reduction between the packet arrival at layer 1 and resource selection for transmission is discussed. Particularly, we have following observations and proposals:

Proposal 1: When T2 is decreased, the following issues should be considered:

- The target use cases should be clarified and the target latency value should be defined.

- The mapping relationship between PPPP/PDB and T2 should be provided by upper layer. LS should be sent to RAN2 to define the mapping relationship between PPPP/PDB and T2.

- In the unevenly distributed services scenario, the ratio (20%) of the candidate single-subframe resources to the total number of the candidate single-subframe resources  should be re-evaluated, especially for the service with stringent latency requirement.

- In order to avoid the resource collision, some extra mechanisms shall be considered in resource selection, e.g., the preemption mechanism based on the service priory is feasible and could be re-evaluated in Rel-15.

Proposal 2: The number of sidelink SPS process for mode 4 shall be extended to support the service with stringent requirement except CAM and DENM message.

Proposal 3: Combining with decreased T2, the enhanced CA scheme should be considered to mitigate the impact of the half-duplex and achieve the reduced latency.

References

- Draft Minutes 3GPP TSG RAN WG1 Meeting #91, RAN1 Chairman’s Notes;

- 3GPP TS 36.213, V15.3.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures”.

- 3GPP TR 22.886, V15.1.0, “Study on enhancement of 3GPP support for 5G V2X services”.

- 3GPP TS 24.385, V14.2.0, “V2X services Management Object (MO)”.

- R1-1801701, “Discussion on carrier aggregation for mode 4 in V2X phase 2”, 3GPP RAN1 #92, CATT