3GPP TSG RAN WG1 #93	R1-1807647

Busan, Korea, May 21st, 2018 – May 25th, 2018

Source:	vivo

Title:	Summary on Issues of PRB bundling for DL

Agenda Item:	7.1.2.1.4

Document for:	Discussion

Introduction

Based on contributions submitted for RAN1 #93 meeting, a text proposal is given to refine the description in 214.

38.214 editorial issues

Several companies provided editorial update of 38.214 capturing on different parts. The following text proposal is given merging the reasonable ones.

Proposal 2: Adopt the following text proposal for section 5.1.2.3 of 38.214.

----Text Proposal for TS 38.214------------------------------------------------------------------ - 5.1.2.3 Physical resource block (PRB) bundling < Unchanged parts are omitted > A UE may assume that precoding granularity is consecutive resource blocks in the frequency domain for bandwidth part i. can be equal to one of the values among {2, 4, wideband}. If is determined as one of the values among {2, 4}, Precoding Resource Block Group (PRGs) partition the bandwidth part i with consecutive PRBs. Actual number of consecutive PRBs in each PRG could be one to or more. When receiving PDSCH scheduled by PDCCH with CRC scrambled by SI-RNTI, RA-RNTI, P-RNTI or TC-RNTI, the UE shall assume that is equal to 2 PRBs. If is determined as "wideband", the UE is not expected to be scheduled with non-contiguous resource allocation and the UE may assume that the same precoding is applied to the allocated resource. If is determined as one of the values among {2, 4}, Precoding Resource Block Group (PRGs) partition the bandwidth part i with consecutive PRBs. Actual number of consecutive PRBs in each PRG could be one to or more. When is determined as one of the values among {2, 4}, The first PRG size is given by and the last PRG size given by if , and the last PRG size is if . The UE may assume the same precoding is applied for any downlink contiguous allocation of PRBs in a PRG partitioned above. For PDSCH carrying SIB1 scheduled by PDCCH with CRC scrambled by SI-RNTI, PRG is partitioned from the lowest numbered resource block of the CORESET signalled in PBCH. If a UE is scheduled a PDSCH with DCI format 1_0, then the UE shall assume that is equal to 2 PRBs. When receiving PDSCH scheduled by PDCCH with DCI format 1_1 with CRC scrambled by C-RNTI or CS-RNTI, for each carrier bandwidth part i is equal to 2 PRBs unless configured by the higher layer parameter prb-BundlingType given by PDSCH-Config. When receiving PDSCH scheduled by PDCCH with CRC scrambled by SI-RNTI, RA-RNTI, P-RNTI or TC-RNTI, the UE shall assume that is equal to 2 PRBs. In case UE is scheduled by DCI 1_0, the higher layer parameter prb-BundlingType is set to 'static', and when receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI or CS-RNTI, the value of is fixed as 2 PRBs. If a UE is scheduled a PDSCH with DCI format 1_0 scrambled with C-RNTI and CS-RNTI, then the UE shall assume that is equal to 2 PRBs. When receiving PDSCH scheduled by PDCCH with DCI format 1_1 with CRC scrambled by C-RNTI or CS-RNTI, if the higher layer parameter prb-BundlingType is set to 'dynamicBundling', the higher layer parameters bundleSizeSet1 and bundleSizeSet2 configure two sets of values, the first set include can take one or two values among {2, 4, wideband}, and the second set include can take one value among {2, 4, wideband}. The UE is not expected to be configured with (2, 4) in the first set. If the PRB bundling size indicator signalled in DCI format 1_1 as defined in Subclause 7.3.1.2.2 of [2, TS 38.212] - is set to '0', the UE shall use the value from the second set of values when receiving PDSCH scheduled by the same DCI. - is set to '1' and one value is configured for the first set of values, the UE shall use this value when receiving PDSCH scheduled by the same DCI - is set to '1' and two values are configured for the first set of values as n2-wideband' (corresponding to two values 2 and wideband) or n4-wideband' (corresponding to two values 4 and wideband), the UE shall use the value when receiving PDSCH scheduled by the same DCI as follows: - If the scheduled PRBs are contiguous and the size of the scheduled PRBs is larger than , is the same as the scheduled bandwidth, otherwise is set to the remaining configured value of 2 or 4, respectively. When receiving PDSCH scheduled by PDCCH with DCI format 1_1 with CRC scrambled by C-RNTI or CS-RNTI, if the higher layer parameter prb-BundlingType is set to 'staticBundling', the value is configured with the single value indicated by the higher layer parameter bundleSize. When a UE is configured with RBG = 2 for bandwidth part i according to Section 5.1.2.2.1, or when a UE is configured with interleaving unit of 2 for VRB to PRB mapping provided by the higher layer parameter vrb-ToPRB-Interleaver given by PDSCH-Config for bandwidth part i, the UE is not expected to be configured with = 4. < Unchanged parts are omitted >

References

[1] R1-1806041	Remaining issues on PRB bundling	vivo

[2] R1-1806213	Remaining issue on PRB bundling	Ericsson

[3] R1-1806711	Remaining issues on DL PRB bundling	Samsung

[4] R1-1806839	Text Proposal for DL PRB Bundling	OPPO