3GPP TSG-RAN WG1 Meeting #94	R1-1809275

Gothenburg, Stockholm, August 20th – 24th, 2018	(Revision of R1-1806443)

Source:	Ericsson

Title:	IMT-2020 self-evaluation: Reliability in NR

Agenda Item:	7.2.7.2

Document for:	Discussion

# 1	Introduction

The ITU target for reliability in IMT 2020 has been set to 10-5 error within 1ms for a 32B packet [1]. In this paper, we go through the evaluation steps as defined in [1] and relate the results to the NR Rel-15 capabilities.

# 2	Discussion

## 2.1	System-Level Simulations (SLS)

The assumptions for the SLS are given in Table 1, with values that have been aligned with the calibration campaign for ITU self-evaluation.

The results are produced for the URLLC configuration. A (4GHz) and configuration. B (700MHz) while considering list of simulation assumption which are presented in Table 1.

For configuration A, the total gain (including antenna gain) is presented in Figure 1 for UMa model A and B. The resulting SINR at full load (cell utilization 1) is drawn in Figure 2. The cell-edge (5th percentile) SINR is found to be 1.98 dB (in DL) and 0.81 dB (in UL) for channel model UMa A, and 1.98 dB (DL) and 1.77 dB (UL) for channel model UMa B as shown in Figure 3.

For configuration B, the total gain (including antenna gain) is given in Figure 4 for UMa models A and B. The resulting SINR at full load (cell utilization 1) is given in Figure 5. The cell-edge (5th percentile) SINR is found to be 0.16 dB (in DL) and 0.83 dB (in UL) for channel model UMa A, and -0.06 dB (DL) and 0.65 dB (UL) for channel model UMa B as shown in Figure 6.

Table 1. Assumptions for the system-level simulations.

| Configuration Parameters | URLLC configuration A | URLLC configuration B |
| --- | --- | --- |
| Carrier frequency | 4 GHz | 700 MHz |
| Base station Antenna Height | 25 m | 25 m |
| Inter-site distance | 500 m | 500 m |
| Bandwidth | 20 MHz | 20 MHz |
| Device deployment | 80% outdoor, 20% indoor | 80% outdoor, 20% indoor |
| Number of UE antenna elements | 4 | 4 |
| UE noise figure | 7 | 7 |
| UE power | 23 dBm | 23 dBm |
| Path loss model | UMa A/B with SCM (for ZOD) | UMa A/B with SCM (for ZOD) |
| BS antenna VxH (vs x Hs x P) | 4 x8 (2x1x2) | 4 x4 (2x1x2) |
| BS Transmit power | 49 dBm | 49 dBm |
| BS noise figure | 5 | 5 |
| Electrical down tilt | 9 degrees | 9 degrees |
| Traffic model | Full buffer | Full buffer |
| UL power control | Alpha=1, P0=-106dBm | Alpha=1, P0=-106dBm |
| UL allocation | 5PRB (10UEs sharing 50PRBs) | 5PRB (10UEs sharing 50PRBs) |

Figure 1. Total gain for URLLC configuration A

Figure 2. SINR distribution for URLLC configuration A.

Figure 3. SINR distribution at 5th percentile for URLLC configuration A.

The cell-edge SINR for URLLC Conf. A is approximately 1.98 dB (DL) and 0.81 dB (UL) for channel model UMa A, and 1.93 dB (DL) and 1.77 dB (UL) for channel model UMa B.

Figure 4. Total gain for URLLC configuration B.

Figure 5. SINR distribution for URLLC configuration B.

Figure 6. SINR distribution at 5th percentile for URLLC configuration B.

The cell-edge SINR for URLLC Conf. B is approximately 0.16 dB (DL) and 0.83 dB (UL) for channel model UMa A, and -0.06 dB (DL) and 0.65 dB (UL) for channel model UMa B.

## Link-level simulations

The assumptions for LLS are given in Table 2. Two different datasets are used for data and control. For PDCCH a DCI of size 40 bits excluding CRC is assumed. For PUCCH we have assumed 1-bit UCI carried by PUCCH format 0 with 2os duration and frequency hopping.

The resulting BLER as function of SNR for the control channels is shown in Figure 7, and for data channels in Figure 8 and 9.

Table 2. Assumptions for the link-level simulations.

| Channel model | TDL-C with 300ns delay spread |
| --- | --- |
| Carrier | 700MHz |
| Bandwidth | 20 MHz |
| Subcarrier spacing | 30 kHz |
| Antenna setting | 2TX 2RX (data), 1TX 2RX (control) |
| Tx diversity | Rank 1 (TX diversity precoding based on CSI reports with 5 slots periodicity). |
| Speed | 3km/h |
| Channel estimation | Practical: - 4os mini-slot - 1os front-loaded DMRS type 2 - 7os mini-slot - 2os front-loaded DMRS type 2 |
| Frequency allocation | Frequency allocation type 1 (contiguous) |
| Time allocation | 4os and 7os allocations type B |
| PUCCH | 1 A/N bit, PUCCH format 0 with 2- symbol duration and frequency hopping between band edges |
| PDCCH | Polar codes, 40b payload excl. CRC. Distributed CCEs |
| Data | LDPC, BG2, 256b |

Figure 7. Sequence selection Short PUCCH and PDCCH BLER as function of SNR.

Figure 8.  4OS-Data (1st attempt) LDPC BLER for QPSK with different MCS as function of SNR.

Figure 9. 7OS-Data (1st attempt) LDPC BLER for QPSK with different MCS as function of SNR.

## Total reliability

With some exceptions, the discussion here assumes that the retransmissions are uncorrelated, which is reasonable to assume if e.g. retransmissions are done on a different frequency allocation. In the following we will write the success probabilities on the channel level according to Table 3, and find expressions for the total success rate pt=1-ε, where ε is the residual error rate.

Table 3: Success probabilities for calculating total reliability

| Probability | Description |
| --- | --- |
| p0 | SR |
| p1 | PDCCH |
| p2 | PDSCH/PUSCH |
| p3 | PUCCH NACK detection |
| p4 | PUCCH DTX detection |

DL data, HARQ-based

In the DL we can describe the total reliability after N transmission as:

pt=n=1Ni=1nn-1n-i(1-p1)p4n-ip1p2,ij=1i-1p1p3(1-p2,j)

where for any positive integer k,  p2,k is the probability of a data block being correctly received after exactly k transmissions are soft combined. In this expression the DL control transmissions are seen as uncorrelated with each other and with data. This is an approximation but can be motivated by e.g. moving the DL control between attempts. The data attempts are correlated with each other.

UL data, configured grant

With configured grant-based UL scheduling instead we remove the SR step and the first DL control, and the total reliability can be described as:

pt=p2,1+(1-p2,1)n=2Np1p2,ni=2n-1(1-p1p2,i)

Here the PDCCH reliability comes in starting from the first retransmission. Assuming perfect energy detection performance on the PUSCH resource.

### Reliability estimate URLLC conf. B UMa B

Accordingly, based on the above expressions for DL and UL data while considering the link simulations results, we can evaluate the total reliability. By observing at the lower percentiles of the SINR distributions for URLLC conf. B UMa B we find the channel BLER at the corresponding DL and UL SINR points, and compute the total error rates for DL and UL data, respectively. The results are shown in Figures 10-13. We assume AL16 for PDCCH and 1% D2A level on PUCCH. For UL we assume SPS with a configured resource every TTI. For both DL and UL 1-3 transmission attempts (including HARQ retransmissions) are considered. The data transmissions are assumed to be correlated and are soft combined.

Figure 10. Total reliability for 4OS – DL data with 1-3 HARQ transmissions at lowest percentiles assuming correlated transmissions.

Figure 11. Total reliability for 7OS – DL data with 1-3 HARQ transmissions at lowest percentiles assuming correlated transmissions.

Figure 12. Total reliability for 4OS UL data with 1-2 HARQ transmissions at lowest percentiles with SPS-based scheduling assuming correlated transmissions.

Figure 13. Total reliability for 7OS UL data with 1-2 HARQ transmissions at lowest percentiles with SPS-based scheduling assuming correlated transmissions.

With 1 transmission using MCS1 the reliability target of 10-5 error can be met in DL and in UL with configured grant.

## Packet size

The ITU requirement states that a packet of 32B should fulfil the latency and reliability target. With QPSK modulation and a code rate using MCS1 to MCS5 and an overhead of one OFDM symbol, the required #PRBs per such transmission is given in Table 4. Here, the TBS is assumed to be exactly 32B, and CRC is not considered.

Table 4. Required #PRBs for 32B packet and 1OFDM symbol OH, at different code rates.

| #PRBs | 14-os TTI | 7-os TTI | 4-os TTI | 2-os TTI |
| --- | --- | --- | --- | --- |
| Code rate MCS1 | 22 | 46 | 92 | 274 |
| Code rate MCS2 | 17 | 37 | 73 | 219 |
| Code rate MCS3 | 14 | 29 | 57 | 171 |
| Code rate MCS4 | 11 | 24 | 47 | 141 |
| Code rate MCS5 | 9 | 19 | 37 | 111 |

With MCS1 and a 7-os mini-slot, 46 PRBs are required for a 32B packet.

## Total latency

In a companion paper [2] we evaluate the UP latency for a sequence of transmissions. It is found that DL and configured grant UL transmissions with 7-os and 30 kHz SCS are possible within the latency bound of 1ms, see Table 5. Thus, the ITU reliability requirement of 10-5 error within 1ms can be met.

Table 5. Maximum #transmissions including retransmissions in FDD within 1ms.

| #TX within 1ms | 15kHz SCS |  |  |  | 30kHz SCS |  |  |  | 120kHz SCS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 14-os TTI | 7-os TTI | 4-os TTI | 2-os TTI | 14-os TTI | 7-os TTI | 4-os TTI | 2-os TTI | 14-os TTI | 7-os TTI | 4-os TTI | 2-os TTI |
| DL data | 0 | 0 | 0 | 1 | 0 | 1 | 1 | 2 | 1 | 2 | 2 | 3 |
| UL data (SPS) | 0 | 0 | 0 | 1 | 0 | 1 | 1 | 2 | 1 | 1 | 1 | 2 |

With 30kHz SCS and 7-os mini-slot 1 transmission can be made in FDD within 1ms.

# Conclusion

In section 2 we made the following observations:

Observation 1	The cell-edge SINR for URLLC Conf. A is approximately 1.98 dB (DL) and 0.81 dB (UL) for channel model UMa A, and 1.93 dB (DL) and 1.77 dB (UL) for channel model UMa B.

Observation 2	The cell-edge SINR for URLLC Conf. B is approximately 0.16 dB (DL) and 0.83 dB (UL) for channel model UMa A, and -0.06 dB (DL) and 0.65 dB (UL) for channel model UMa B.

Observation 3	With 1 transmission using MCS1 the reliability target of 10-5 error can be met in DL and in UL with configured grant.

Observation 4	With MCS1 and a 7-os mini-slot, 46 PRBs are required for a 32B packet.

Observation 5	With 30kHz SCS and 7-os mini-slot 1 transmission can be made in FDD within 1ms.

# References

- 5D/TEMP/216-E, Preliminary draft new report ITU-R M., [IMT-2020.TECH PERF REQ], ITU-R Working Party 5D, (WP5D-TD-0300).

- R1-1809277, “IMT 2020 Evaluations - UP latency in NR”, Ericsson