3GPP TSG RAN WG1 Meeting #95

# Spokane, USA, November 12 – 16, 2018

Agenda Item:	6.1.4

Source:	Huawei, HiSilicon

Title:	Discussion on flexible UL starting PRB allocation of 3MHz system bandwidth in TDD

Document for:	Discussion and decision

# Introduction

In RAN1 #94 meeting, the following agreements on flexible PUSCH starting PRB in CEMode A were reached:

Agreement Update the size of RA field in TS 36.212 clause 5.3.3.1.10 to log25×NRBUL for TDD.

However, for the case of 3MHz, the size of RA field will consume one more bit than Rel-13 legacy RA field, which is not consistent with the following agreements concluded in RAN1 #92bis meeting.

Agreement At least for BL/CE UE configured without other Rel-14/15 features, DCI size after padding is not increased in order to support flexible starting PRB. Note: For PUSCH and PDSCH, the solution may or may not be the same Note: The solution may or may not be the same for FDD and TDD

In this contribution, we provide our considerations on the specific resource block allocation without increasing bit overhead for 3MHz system bandwidth in TDD.

# Unsupportable RA combinations for Rel-13 RA

For 3MHz system bandwidth, it includes 15 PRBs. Therefore, two narrowbands are defined for 3MHz system bandwidth. In this case, for Rel-13 legacy RA on MTC UE, 1 bit is used for narrowband indication and 5 bits for the RA within the indicated narrowband using UL RA type 0. Totally, there are 6 bits for Rel-13 legacy RA on MTC UEs.

Table 1 below shows the unsupportable RA combinations for Rel-13 RA on MTC. As seen from Table 1, the ratio of unsupportable RA combinations is increased with the increment of RA length (L), which is rather high for L=6.

Table 1: Unsupportable RA combinations for Rel-13 RA on MTC, 3MHz

|  | RA using UL RA type 0 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | L=1 | L=2 | L=3 | L=4 | L=5 | L=6 |
| Full flexible RA | 15 | 14 | 13 | 12 | 11 | 10 |
| Rel-13 MTC RA | 12 (6x2) | 10 (5x2) | 8 (4x2) | 6 (3x2) | 4 (2x2) | 2 (1x2) |
| Unsupportable RA combinations | 3 | 4 | 5 | 6 | 7 | 8 |
| Ratio of unsupportable RA combinations | 20% | 28.5% | 38.4% | 50% | 63.6% | 80% |

# Flexible UL starting PRB allocation of 3MHz

For normal UL type 0 resource allocation, the RIV is numbered for L=1~6 illustrated as Table 2 below. In Table 2, N=15 (the number of PRBs for 3MHz). As seen from Table 2, to fully reuse the normal UL type 0 RA, 85 combinations are needed, which means 7 bits needed.

Table 2: RIV numbering for normal UL type 0 resource allocation

|  | RA Length | RA combinations | RIV index |
| --- | --- | --- | --- |
| 1st N RIVs | L=1 | 15 | 0~14 |
| 2nd N RIVs | L=2 | 14 | 15~28 |
|  | L=15 | 1 | 29 |
| 3rd N RIVs | L=3 | 13 | 30~42 |
|  | L=14 | 2 | 43~44 |
| 4th N RIVs | L=4 | 12 | 45~56 |
|  | L=13 | 3 | 57~59 |
| 5th N RIVs | L=5 | 11 | 60~70 |
|  | L=12 | 4 | 71~74 |
| 6th N RIVs | L=6 | 10 | 75~84 |
|  | L=11 | 5 | 85~89 |

Keeping the same bits (6) of Rel-13 RA on MTC, there are 64 available states used for flexible UL starting PRB allocation. Obviously, the RA of flexible starting PRB should prioritize larger length, which can effectively complement the shortage of legacy Rel-13 MTC RA.

RIV 0 for flexible starting PRB can start from the length 3 as shown in the Table 3 below. In order to comply with normal UL type 0 RA mechanism, 60 RIVs are needed for indicating L=3~6. Therefore, 4 RIVs (64-60=4) are left.

For L=2, the combinations of full RA is 14, and Rel-13 legacy RA already supports 10 combinations. As a result, 4 combinations cannot be supported. To provide full RA flexibility, the remaining 4 RIVs can be used to indicate the remaining 4 combinations.

Table 2: RIV numbering for normal UL type 0 resource allocation

|  | RA Length | RA combinations | RIV index |
| --- | --- | --- | --- |
| 1st N RIVs | L=3 | 13 | 0~12 |
|  | L=14 | 2 | 13~14 |
| 2nd N RIVs | L=4 | 12 | 15~26 |
|  | L=13 | 3 | 27~29 |
| 3rd N RIVs | L=5 | 11 | 30~40 |
|  | L=12 | 4 | 41~44 |
| 4th N RIVs | L=6 | 10 | 45~54 |
|  | L=11 | 5 | 55~59 |
| The remaining 4 RIVs | L=2 | 4 | 60~63 |

Therefore, 6 bits are enough to indicate the resource allocation with L=2~6 for flexible PUSCH starting PRB of 3MHz in TDD. Therefore, we propose:

Proposal 1:

If a BL/CE UE in TDD is configured with higher layer parameter ce-PUSCH-FlexibleStartPRB-AllocConfig and NRBUL=15:

For 3≦≦6,

For =2,

Where, =0,1,2,3, and is derived from Table 3 below.

Table 3:  for LCRBs=2

| 0 | 0 |
| --- | --- |
| 1 | 6 |
| 2 | 7 |
| 3 | 13 |

# Conclusions

In this contribution, we provide our considerations on the specific resource block allocation without increasing bit overhead for 3MHz system bandwidth in TDD, and the following proposal is given.

Proposal 1:

If a BL/CE UE in TDD is configured with higher layer parameter ce-PUSCH-FlexibleStartPRB-AllocConfig and NRBUL=15:

For 3≦≦6,

For =2,

Where, =0,1,2,3, and is derived from Table 3 below.

Table 3:  for LCRBs=2

| 0 | 0 |
| --- | --- |
| 1 | 6 |
| 2 | 7 |
| 3 | 13 |

# Appendix:

## /******** Start of the Text Proposal  *******/

### 8.1.1	Uplink resource allocation type 0

The resource allocation information for uplink resource allocation type 0 indicates to a scheduled UE a set of contiguously allocated virtual resource block indices denoted by . A resource allocation field in the scheduling grant consists of a resource indication value (RIV) corresponding to a starting resource block () and a length in terms of contiguously allocated resource blocks ( 1).

For a BL/CE UE,

-	uplink resource allocation type 0 is only applicable for UE configured with CEModeA, and

-	, if the UE in TDD is configured with higher layer parameter ce-PUSCH-FlexibleStartPRB-AllocConfig; otherwise  and,

-	if the UE is configured with higher layer parameters ce-PUSCH-FlexibleStartPRB-AllocConfig, shall not exceed  and,

-	if the UE is not configured with higher layer parameter ce-PUSCH-FlexibleStartPRB-AllocConfig,  is always set to 6 in this subclause regardless of the system bandwidth.

The resource indication value is defined by

For PDCCH/SPDCCH DCI format 7-0A/7-0B and NRBUL>15, VRB allocations for a UE vary from 4 VRB(s) up to  VRBs with an increment step of 4 VRBs. A type 0 resource block assignment field consists of a resource indication value (RIV) corresponding to a starting resource block  using  and a length in terms of virtually contiguously allocated resource blocks , where  is defined if configured by higher layer parameter resourceAllocationOffset; otherwise set to 0. The resource indication value is defined by:

if  then

else

where , and , and where,

1 and shall not exceed .

Otherwise, the resource indication value is defined by

if  then

if a BL/CE UE in TDD is configured with higher layer parameter ce-PUSCH-FlexibleStartPRB-AllocConfig and  NRBUL>15, then

else

else

If a BL/CE UE in TDD is configured with higher layer parameter ce-PUSCH-FlexibleStartPRB-AllocConfig and NRBUL=15:

For 3≦≦6,

For =2,

Where, =0,1,2,3, and is derived from Table 8.1.1-1.

Table 8.1.1-1:  for LCRBs=2

| 0 | 0 |
| --- | --- |
| 1 | 6 |
| 2 | 7 |
| 3 | 13 |

## /******** End of the Text Proposal  *******/