3GPP TSG RAN WG1 Meeting #95                         R1-1812552

Spokane, USA, November 12th – 16th, 2018

Agenda Item:	7.2.1.1

Source: 	LG Electronics

Title: 	Transmitter side signal processing schemes for NCMA

Document for:	Discussion and decision

Introduction

In general, for the conventional DL/UL multiple access (MA) scheme, three design criteria have been considered as spectral efficiency, energy efficiency and system complexity. In NR (New RAT) design, three use cases are considered as eMBB, mMTC, and URLLC with several KPIs for IMT 2020 RIT submission [1]. Specially, some KPIs such as massive connectivity, spectral efficiency and latency, might be enormously challenging. In this contribution, the non-orthogonal multiple access (NOMA) schemes are discussed.

Followings are the agreements and observations on the multiple access of NR in RAN1 #84bis;

Agreements:

- Non-orthogonal multiple access should be investigated for diversified NR usage scenarios and use cases

- At least for UL mMTC, autonomous/grant-free/contention based non-orthogonal multiple access should be studied

Observations:

- Examples non-orthogonal schemes include (but not limited to):

  - For UL, Multi-user shared access (MUSA) (e.g., R1-162226)

  - Resource spread multiple access (RSMA) (e.g., R1-163510)

  - Sparse code multiple access (SCMA) (e.g., R1-162153)

  - Pattern defined multiple access (PDMA) (e.g., R1-163383)

  - Non-orthogonal coded multiple access (NCMA) (e.g., R1-162517)

  - Low code rate spreading (e.g., R1-162385)

  - Frequency domain spreading (e.g., R1-162385)

  - Non-orthogonal multiple access (NOMA) (e.g., R1-163111)

Followings are the agreements of NOMA SI in RAN1 #92;

Agreements:

- Adopt the following table as the metrics for NOMA study from link level point of view.

  - More metrics may be added in the future

| Performance metrics | BLER vs. per UE SNR at a given pair of {per UE SE, # of UEs} Sum throughput v.s. SNR at given BLER level, for a given pair of {per UE SE, # of UEs} MCL |
| --- | --- |
| Implementation related metrics | PAPR/cubic metric Rx complexity and processing latency FFS: Configuration/Scheduling flexibility |

Non-orthogonal Coded Multiple Access (NCMA)

The NOMA schemes have been proposed for the increase of the connectivity or the system throughput. These schemes are based on the non-orthogonal spreading code or the difference of spatial resource and power. In case of specific channel environments with system optimizations (e.g. power allocation or user scheduling); the NOMA schemes can provide the improved connectivity or the increased system throughput compared to the conventional OMA schemes. However, NOMA schemes have some defeats, such as scheduling complexity, encoding/decoding complexity, loss of BLER, and limited environments.

In the NOMA schemes, multiuser interference (MUI) is inherently induced. Considering this, spreading based non-orthogonal coded multiple access (NCMA) scheme was suggested in [2]. This scheme is one of an approach for theoretically minimizing the MUI based on the spreading codes. Here, we define the spreading code as ‘Grassmannian Sequence’ [3][4] and the sequence and quantized version of the sequence can be obtained as follows.

- Grassmannian Sequence

  - Each complex spreading sequence of this sequence set is generated by Grassmannian line packing problem. Let the Grassmannian sequence set defined by G=s(1)…s(K)=s1(1)…s1(K)⋮⋱⋮sN(1)…sN(K), G ⊂CN×K, where N is the spreading factor and K is the superposition factor. Then, the sequence design problem can be posed in terms of maximizing the minimum chordal distance between sequence pairs: minGmax1≤k<j≤K1-sk*∙sj2, where sk* is the conjugate sequence of sk.

  - Spreading sequence set: G=s(1)…s(K)

- M-QAM quantized Grassmannian Sequence

  - Each complex coefficient of this sequence (which is generated by the Grassmannian sequence) is quantized by M-QAM constellations. Then, the M-QAM quantized Grassmannian sequence set is defined by G=s(1)…s(K)=s1(1)…s1(K)⋮⋱⋮sN(1)…sN(K), G ⊂CN×K, where N is the spreading factor and K is the superposition factor.

  - Complex coefficient: si(j)=c, i=1≤i≤N, i=1≤j≤K, c∈the set of M-QAM constellations

  - Spreading sequence set: G=s(1)…s(K)

Note that the spreading code design mentioned in [2] is one of the NCMA schemes. From another point of view, coding based NCMA scheme can be also considered as one of other approaches for the NCMA schemes. This scheme is one of an approach to disperse the MUI based on the user-specific scrambling codes in coding domain [5][6]. For simplicity, spreading based NCMA (N>1) and coding based NCMA (N=1) are called NCMA.

The transceiver structure of NCMA is assumed and illustrated in Figure 1. Here, UE specific non-orthogonal code cover (NCC) represents the spreading sequence allocated to (or selected by) each UE.

Figure 1. An example of the transceiver structure of NCMA in UL

NCMA can provide the additional throughput or improved connectivity with a small loss of BLER in specific environments, by exploiting additional layers through superposed symbol, while satisfying QoS constraints. The receiver of NCMA system can be applied by matched filter (MF), successive interference cancellation (SIC), parallel interference cancellation (PIC), and maximum likelihood (ML) based receiver for the multiuser detection.

In addition, the MUI level between spreading sequence pairs is always similar due to the correlation characteristics mentioned above. Consequently, NCMA provides the potentials in terms of throughput or connectivity under special scenarios, e.g., huge connections with small packet in mMTC scenario without changing TB size, or for reducing the collision probability in contention based MA.

Hence, in special scenarios, the NOMA schemes may provide valuable alternatives, e.g., CDMA based approaches can efficiently provide low rate services to a large number of users. Nevertheless, to consider NOMA schemes including NCMA for NR, it should be justified for advantages and complemented to defects, such as mutual interference, complexity, processing delay, limited benefit, channel variation effect, and so on. In this sense, it may need to consider benefit from combination of multiple MA schemes, including OMA and NOMA schemes in special scenarios for NR design.

Proposal 1: The spreading based NCMA based on MQAM-quantized Grassmannian Sequence is considered for the NOMA design in NR.

Peak-to-Average-Ratio (PAPR) with DFT-s-OFDM

In NR design, the NOMA system focuses on uplink. Therefore, peak-to-average-ratio (PAPR) at the transmitter side is one of major performance metrics since lower PAPR leads to high PA efficiency. Specially, some cases such as cell-edge located devices and low cost mMTC devices might be enormously challenging in terms of PAPR.

In general, DFT-s-OFDM has lower PAPR compared to CP-OFDM and so DFT-s-OFDM outperforms CP-OFDM in power limited situations [7]. Therefore, DFT-s-OFDM may be more reasonable for link budget limited scenarios. PAPR can be calculated as CCDF function derived by all the time samples. PAPR between CP-OFDM and DFT-s-OFDM with QPSK modulation is represented in Figure 2.

Figure 2. PAPR comparison between CP-OFDM vs DFT-s-OFDM with QPSK

At 1% PAPR level in Figure 8, DFT-s-OFDM has about 2 dB advantage of PAPR. At 0.1% PAPR level, DFT-s-OFDM provides 2.5 dB advantage gain of PAPR. Therefore, when UE uses low MCS in power limited situations, DFT-s-OFDM can provide link performance gain. Thus, DFT-s-OFDM may be more reasonable for link budget limited scenarios.

Specially, in the NOMA system, the characteristics of PAPR among spreading sequences can be different. If the spreading is applied to the time domain, the transceiver structure of NCMA with DFT-s-OFDM is assumed and illustrated in Figure 3.

Figure 3. An example of the transceiver structure of NCMA with DFT-s-OFDM in UL

In the NOMA system, if the DFT process is operated at the transceiver, the spreading sequence (e.g., UE specific NCC) can be multiplied in time domain signal. In this case, PAPR of each spreading sequence can vary. In Figure 4, it is represented to the simulation result of the sample level PAPR of Hadamard sequence based MA and NCMA with DFT-s-OFDM. The details of the simulation parameter are shown in appendix.

Figure 4. PAPR of Hadamard sequence based MA and NCMA with DFT-s-OFDM in UL

As shown in these results, the characteristic of PAPR can be different depending on the codeword (spreading sequence) index. Following the codeword index, its PAPR is widely spread compared to the non-spreading case (such as Figure 8). Thus, for fair comparison between NOMA schemes, PAPR should be considered as one of performance metrics. In addition, the performance of DFT-s-OFDM based NOMA schemes needs to be considered as one of major consideration in NOMA SI.

For cell coverage extension and power limited UE, the PAPR is an important issue. For example, the cell-edge user can be assigned with sequence pool with lower PAPR, while cell-center user can be allocated with relatively higher PAPR compared to sequences with lower PAPR. By doing so, we can get more throughput enhancement for power limited UE (i.e., cell-edge user), and this can be utilized for extending cell coverage especially for MTC scenario.

In order to exploit this enhancement, gNB needs to assign a sequence each UE based on feedback or UL reference signal by using DCI in the control channel. However, in the grant-free MA, the different way needs instead of explicit way via DCI. One of solutions is for example, the NoMA system with two sequence pools where a sequence is selected from UE in an implicit way. One is a set of sequences with relatively low PAPR, the other is a set of sequences with relatively high PAPR. UE can select a sequence from a sequence pool based on UE geometry related to parameters such as power measurement of DL reference signal, or power headroom.

In the case of UEs capable of UL transmission with high spectral efficiency over wideband (e.g., eMBB or URLLC devices), if UL transmission with low spectral efficiency over narrowband is required, the PAPR issue may not be a critical issue, since the margin of power headroom will be affordable. However, in the case of UEs capable of UL transmission with low spectral efficiency over narrowband only (e.g., mMTC dedicated devices), the PAPR issue may be a critical issue, since the margin of power headroom is shortage. In other words, low cost devices that have small margin of power headroom will be sensitive to the characteristic of PAPR, although the allocated bandwidth is small.

Proposal 2: In grant-free transmission, NoMA sequence pool allocation/selection based on PAPR need to be considered.

Summary

In this document, we discussed on MA for the NR. Our suggestions can be summarized as below

Observation 1: Grassmannian sequence provides lower cross-correlation compared to other non-orthogonal spreading sequences.

Proposal 1: The spreading based NCMA based on MQAM-quantized Grassmannian Sequence is considered for the NOMA design in NR.

Proposal 2: In grant-free transmission, NoMA sequence pool allocation/selection based on PAPR need to be considered.

Reference

TR 38.913, “Study on Scenarios and Requirements for Next Generation Access Technologies”

R1-162517, LG Electronics, “Considerations on DL-UL multiple access for NR”

R1-166874, LG Electronics, “Discussion on Non-orthogonal Spreading Sequences for NoMA”

R1-1609230, LG Electronics, “Discussion on Non-orthogonal Spreading Sequence and Codebooks for NoMA”

R1-164557, LG Electronics, “Initial LLS Evaluation Result for NoMA”

R1-166876, LG Electronics, “Performance evaluation of NCMA”

R1-1609222, LG Electronics, “Discussion on DFT-s-OFDM and CP-OFDM for Uplink”

Appendix

- Examples of 64QAM-quantized Grassmannian Sequence based spreading codebook for minimizing the MUI are represented as follows:

Table I. 64QAM-quantized Grassmannian Sequence based codebook for Spreading Factor: N = 2

| # of codewords (Max. # of users: K) | Examples of spreading codebook c1⋯cK |
| --- | --- |
| 2 | 5+5i5+5i5+5i-5-5i*Pno,2,2 |
| 4 | -3+5i-3+7i3+7i-3+1i-1-3i-7-3i7-3i5+3i*Pno,2,4 |
| 6 | -3+1i-7-3i7-3i-5+3i-1-5i-5+3i-1-7i-5-5i-1-7i-7+1i-1-1i3-3i*Pno,2,6 |
| 8 | -3-7i-7-1i-1-3i1-5i-3-1i-1-5i-1+7i-5-7i-3-1i-7+1i1-7i3+1i-3-7i-1+5i-5+1i-1-7i*Pno,2,8 |

Table II. 64QAM-quantized Grassmannian Sequence based codebook for Spreading Factor: N = 4

| # of codewords (Max. # of users: K) | Examples of spreading codebook c1⋯cK |
| --- | --- |
| 4 | 5+5i5+5i5+5i-5-5i5+5i5+5i5+5i-5-5i5+5i5+5i5+5i-5-5i-5-5i-5-5i-5-5i5+5i*Pno,4,4 |
| 6 | -7+3i-3+1i-5+1i-1-5i3+1i-1-7i1-7i-7+3i-1-7i-1+1i-3+1i3+5i-1-3i-1+7i-3+1i-5+5i-7+5i-3+1i-3-7i5+7i3+3i5-7i-1+3i1+3i*Pno,4,6 |
| 8 | -5-3i-7+1i1-5i1+3i-1-3i-7-3i-7+7i5-1i-1-3i-5-3i7-3i3+1i-5-3i-7+7i-1+5i-5+5i-7-3i1-7i1-3i1-7i-1+3i7+1i-5+7i-3+5i-5+7i-3-5i1+1i-7+7i-5+3i-1-1i7-5i3+1i*Pno,4,8 |

NOTE: Pno,N,K is (K×K) normalized matrix for the power constraints, Pno,N,K=Pno,1   0      0   Pno,2   0      0      0      0      0      0      0      0      ⋱      0      0   Pno,K. Here, Pno,k=1ck×N, for k=1, …, K.

NOTE: More detailed examples are presented in R1-1809844.

- Simulation setup for PAPR are represented as follows:

Table II. 64QAM-quantized Grassmannian Sequence based codebook for Spreading Factor: N = 4

| Assumptions | Value |
| --- | --- |
| Carrier frequency | 2 GHz |
| System Bandwidth and FFT size | 10 MHz, 1024 |
| Data transmission bandwidth | 4PRB |
| Data Allocation | Localized mapping |
| Modulation | QPSK |
| Metric | Sample Level PAPR ; CCDF Function is derived by all the time samples |