3GPP TSG-RAN1#96			                  		                    R1-1902123

Athens, GR, Feb 25th – March 1st, 2019

Source: 	Fraunhofer IIS, Fraunhofer HHI

Title:	Enhancements on Type-II CSI reporting

Agenda item:       7.2.8.1

Document for:	Decision

# Introduction

In the RAN1#95 meeting [1] and in the RAN1 NR-AH 1901 meeting [2], the following agreements related to Type-II CSI compression were made:

Agreement

For Rel-16 NR, agree on Alt1 (DFT-based compression) in Table 1 of R1-1813002 as the adopted Type II rank 1-2 overhead reduction (compression) scheme as formulated in Alt1.1 of R1-1813002

- Note: The same DFT-based compression scheme is extended for Type II port selection codebook

- Codebook subset restriction (CBSR) is supported when DFT-based compression is utilized for Type II codebooks with overhead reduction (compression) scheme

  - FFS: detailed signaling mechanism

- Note: Additional compression scheme(s) are not precluded

Agreement

On basis/coefficient subset selection for the first layer, support the following:

- Size-K0 subset design: down select in RAN1#96 from the following alternatives

  - Alt1. Unrestricted subset (size=2LM)

  - Alt2. Polarization-common subset (size=LM)

  - Alt3. Restricted subset (for a given subset of beams and FD basis, size=2L+M)

- The value of K0:   where two values of β are supported

  - Down select in RAN1#96 from

- The UCI consists of two parts:

  - Information pertaining to the number(s) of non-zero coefficients is reported in UCI part 1

    - Note: This does not imply whether this information consists of single or multiple values

  - The payload of UCI part 1 remains the same for different RI value(s)

- Bitmap is used to indicate non-zero coefficient indices

Agreement

Two values of M are supported. In RAN1#96, down select between the following alternatives ():

- Alt1.

- Alt2.

- FFS: support for p=1/8 and/or p=3/4 in addition to 1/4 and 1/2

Agreement:

The value of M is higher-layer configured

- FFS: Whether UE reporting smaller value of M (in addition to the configured M) is supported

Agreement

Values of N3: For  and NSB is # CQI subbands, when ,

Values of N3: For  and NSB is # CQI subbands, when , downselect among the following alternatives in RAN1#96

- Alt1: N3 is smallest multiple of 2, 3, or 5 which is

- Alt2: N3 is a multiple of 2, 3, or 5. Segment into 2 parts with overlapping between 2 parts. Note: no padding is needed to align the DFT size with the multiple of 2, 3, or 5

Offline proposal: On LC coefficient quantization, finalize the scheme based on the following alternatives in RAN1#96:

- Alt1 (per coefficient analogous to Rel.15 Type II W2): Rel.15 3-bit amplitude, N-bit phase where N is configured to either 2 (QPSK), 3 (8PSK), or 4 (16PSK).

- Alt2 (differential): Rel.15 3-bit wideband amplitude for each beam, X-bit differential amplitude TBD, Y-bit phase TBD

  - [X=2 or 3, Y=2 or 3 were mentioned – please confirm the final choice]

- Alt3 (ABC matrix): A and C are real-valued diagonal matrices and B is a coefficient matrix. The amplitude set for each element of B is either 0 or 1. The amplitude sets of A and C TBD

  - [Rel.15 3-bit amplitude for A/C and Rel.15 3-bit amplitude for A and {0, 1/4, 1/2, 1} for C were mentioned – please confirm the final choice]

- Alt4 (two parts with two resolutions): For each beam: 4-bit amplitude and 4-bit phase for the first FD component’s coefficient; 3-bit amplitude and 3-bit phase for the remaining coefficients

  - Alphabets for 4-bit and 3-bit amplitude TBD

  - Alphabets for 4-bit and 3-bit phase TBD

In this contribution, we discuss the configuration, quantization and reporting scheme for the agreed DFT-based compression scheme for Type II overhead reduction.

Also, a compression scheme based on Doppler-domain precoding is discussed in our companion contribution [3].

# Feedback compression for Type II CSI Reporting

In the RAN1#95 meeting [1], it was agreed to adopt a DFT-based compression scheme for Type II overhead reduction. The DFT-based compression scheme is summarized in the following.

Assuming a dual-polarized antenna array at the gNB with configuration (N1, N2, P=2), the conventional Rel.-15 double-stage precoder for a transmission layer is given by

|  | F=F1F2, | (1) |
| --- | --- | --- |

where F1∈C2N1N2×2L is the wideband first-stage precoder containing 2L spatial beams bi identical for N3 subbands, and F2∈C2L×N3 is the second-stage precoder that contains 2L subband (wideband amplitude, subband amplitude and phase) complex frequency-domain combining-coefficients per subband associated with the 2L spatial beams. For a compression of F, the second stage precoder F2 is expressed per beam as a product of  few frequency domain (FD) combining coefficients and associated FD basis vectors. Each FD basis vector is given by a DFT vector and models a linear phase increase over the N3 subbands. Hence, each FD vector is associated with a delay in the transformed (delay) domain. The overall precoder can be expressed by

|  | F=αi=0L-1bid=0M-1k1,i,d fdTi=0L-1bid=0M-1k2,i,d fdT∈C2N1N2×N3 | (1) |
| --- | --- | --- |

where M is the number of FD components/delays for the i-th beam, fd is the d-th FD (DFT) component vector of size N3×1 associated with the i-th beam, kp,i,d is a complex FD coefficient associated with the i-th spatial beam, d-th FD basis vector and p-th polarization, and α is a normalization constant.

  - Quantization of coefficients in W2

In this section, we shortly discuss the agreed quantization schemes at RAN1#95 in terms of feedback overhead. Simulation results of the different quantization schemes are presented in Section 3.

ALT1: In ALT1A, the UE reports not more than K0 quantized combining coefficients (amplitude and phase) and a bitmap to indicate the selected coefficients. Each of the quantized combining coefficient is expressed as kp,i,d=bp,i,dgp,i,d, where bp,i,d is the amplitude which is quantized with 3 bits, and gp,i,d=expj2πn2N; n∈0,1,…,2N-1, N=3  is a complex-valued unit-magnitude coefficient to indicate the phase.

The amplitude set for bp,i,d is identical to the Rel.15 wideband amplitude set.

ALT2A: In ALT2A, the UE reports not more than K0 combining coefficients (amplitude and phase) and a bitmap to indicate the selected coefficients. Each of the selected combining coefficient kp,i,d is expressed as kp,i,d=ap,ibp,i,dgp,i,d, where ap,i is a real-valued coefficient representing a common amplitude across all combining coefficients associated with the i-th beam and p-th polarization, bp,i,d is the differential amplitude quantized with 1 or 2 bits, and gp,i,d=expj2πn2N; ∈0,1,…,2N-1, N=3 is a complex-valued unit-magnitude coefficient to indicate the phase.

The amplitude set for ap,i is identical to the Rel.15 wideband amplitude set. The differential amplitudes bp,i,d are quantized with 2 bits. The amplitude set used for the differential amplitudes bp,i,d is given by 0,116, 14,1.

ALT2B: In ALT2B, the UE reports not more than K0 combining coefficients (amplitude and phase) and a bitmap to indicate the selected coefficients. Each of the selected combining coefficient kp,i,d is expressed as kp,i,d=bp,i,dcdgp,i,d, where cd is a real-valued coefficient representing a common amplitude across all combining coefficients associated with the d-th delay, bp,i,d is the differential amplitude quantized with 2 bits, and gp,i,d=expj2πn2N; ∈0,1,…,2N-1, N=3 is a complex-valued unit-magnitude coefficient to indicate the phase.

The amplitude set for cd is identical to the Rel.15 wideband amplitude set. The differential amplitudes bp,i,d are quantized with 2 bits. The amplitude set used for the differential amplitudes bp,i,d is given by 0,116, 14,1.

ALT3: In ALT3, the combining coefficients kp,i,d are expressed as kp,i,d=ap,ibp,i,dcdgp,i,d, where ap,i is a real-valued coefficient representing a common amplitude across all combining coefficients associated with the i-th beam and p-th polarization, bp,i,d∈{0,1}  is the differential amplitude quantized with 1 bit, cd is a real-valued coefficient representing a common amplitude across all combining coefficients associated with the selected d-th FD basis vector, and gp,i,d=expj2πn2N; ∈0,1,…,2N-1, N=3   is a complex-valued unit-magnitude coefficient to indicate the phase. In contrast to the previous quantization schemes, for ALT3 a bitmap does not need to be additionally reported for the selection of the coefficients since the amplitude set of the differential amplitudes bp,i,d is defined by 0,1. The values of bp,i,d represent the bitmap.

The amplitude set for ap,i is identical to the Rel.15 wideband amplitude set. The coefficient ap,i associated with the leading beam is not reported. The coefficient cd associated with the strongest FD component is not reported. the phase values corresponding to the selected combining coefficients are reported. The differential amplitudes bp,i,d∈{0,1} are quantized with 1 bit. The amplitudes cd are quantized with 3 bits. The amplitude set for cd is also identical to the Rel.15 wideband amplitude set.

Procedure to calculate values of ai, ∀i and cd, ∀d:

The procedure of calculating ai, ∀i and cd, ∀d are explained for an example matrix in the following. First, before doing the quantization, we normalize the matrix W2 with the strongest coefficient in amplitude. By doing this, the strongest coefficient will always be equal to 1.

In the next step, select the first K0≤K strongest coefficients of matrix W2.  The selection of the K0 non-zero indices are indicated by a bitmap of size 2LM which is represented by the matrix B.

Please note that matrix B (bitmap) is not updated during the iterations, it is rather fixed before quantization. Only the values of ai, ∀i and cd, ∀d are calculated sequentially.

Assuming L=2 and M=4, the FD coefficient matrix W2∈ C4×4 having K0=11 non-zero coefficients is given by

W2= w11w12w21000w23w24w31w32w410w330w43w44

From W2, the bitmap is given by B= 1110001111101011.

By using ALT3 scheme, the amplitude values of  W2 is approximately expressed as

amplitudew11w12w21000w23w24w31w32w410w330w43w44=a1c1a1c2a2c1000a2c3a2c4a3c1a3c2a4c10a3c30a4c3a4c4

For simplicity, assume wi,j, ∀i,j are absolute values from now on.

In the following, the procedure to calculate the values of ai, ∀i and cj, ∀j is explained.

In the first step, the values of ai are initialized as follows:

a1=quant(maxw11, w12),

a2=quant(maxw21, w23, w24)

a3=quant(maxw31, w32,w33),

a4=quant(maxw41, w43, w44)

In the second step, the values of cd are calculated using the MSE criterion. For example, the values of cd’s are calculated as

c1=quantpinva1a2a3a4w11w21w31w41

c2=quantpinva10a30w120w320

Similarly,  c3 and c4 are calculated.

In the third step, based on the calculated values of cd,∀d, the values of ai,∀i are updated using the same MSE criterion as

a1=quantw11w1200pinvc1c200

a2=quantw210w23w24pinvc10c3c4

Once the values of ai,∀i are calculated, the values of cj,∀j are updated once again using step 2.

In our observation at most 2 iterations are sufficient to obtain ai's and cj's that minimize the mean square error between the unquantized W2 and ABC matrices.

Advantages of ALT3 scheme:

- The feedback associated with the amplitude values does not depend on the value of K0 for a fixed number of spatial beams and FD components.

- For a large number of FD components, the feedback associated with the amplitude values is significantly less compared to ALT1/ALT2/ALT4.

ALT4: In ALT4, the UE reports not more than K0 quantized combining coefficients (amplitude and phase) and a bitmap to indicate the selected coefficients. Each of the K0 quantized combining coefficient is expressed as kp,i,d=bp,i,dgp,i,d, where bp,i,d is the amplitude, and gp,i,d=expj2πn2N; n∈0,1,…,2N-1, N∈{3,4}  is a complex-valued unit-magnitude coefficient to indicate the phase.

For the evaluations of ALT4, the coefficients (amplitude and phase) for each beam are separated into two sets. The first set contains the coefficient associated with the first FD component of each spatial beam. The amplitude and phase of the coefficients of first set are quantized with 4 bits each. The amplitudes of the coefficients of the first set are quantized using the amplitude set: 0,1214,…,128,…123,122,121,1. The second set contains the remaining coefficients. The amplitude and phase of the coefficient in the second set are quantized with 3 bits each. The amplitude set for the second set of coefficients is identical to the Rel.15 wideband amplitude set.

The numbers of bits to be reported for ALT1, ALT2a, ALT2b, ALT3 and ALT4 are listed in Table I. Also shown is the corresponding compression ratio achieved by each quantization scheme with respect to Type-II CSI reporting (WB and SB amplitudes and phases). For a fair comparison to the Type-II CSI feedback, the feedback overhead for reporting the selected M FD basis vectors is taken into account when calculating the compression ratio.

Table I: Numbers of bits needed for reporting of the combining coefficients per layer for different quantization schemes.

| Quant. Scheme | Numbers of bits for reporting of |  |  |  |  |  | Total number of bits to be reported for configuration (2L,M,K0, N3) vs. compression ratio |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Beam amplitudes (ap,i) | FD component amplitudes (cp,d) | Bitmap | Amplitudes of combining coefficients (bp,i,d) | Phase of combining coefficients (gp,i,d) | Indices of selected M FD basis vectors | (8,4,16,13) | (8,7,28,13) |
| ALT1A | - | - | 2LM | 3K0 | 3K0 | log2N3M | 192/42% | 252/25% |
| ALT2a | 3(2L-1) | - | 2LM | 2K0 | 2K0 | log2N3M | 189/44% | 245/27% |
| ALT2b | - | 3(M-1) | 2LM | 2K0 | 2K0 | log2N3M | 177/47% | 242/28% |
| ALT3 | 3(2L-1) | 3(M-1) | - | 2LM | K0 | log2N3M | 150/55% | 207/38% |
| ALT4 | - | - | 2LM | 4(2L)+ 3(K0-2L) | 4(2L)+ 3(K0-2L) | log2N3M | 208/38% | 268/20% |
| Type-II CSI (reference) | - | - | - | - | - | - | 336/0% | 336/0% |

Considering the above results from Table I, it is observed that ALT3 achieves the highest compression ratio compared to the other quantization schemes.

Observation 1: ALT3 achieves the highest compression ratio compared to the other quantization schemes.

  - Codebook subset restriction

At RAN1#95 meeting [1], it was agreed that Codebook subset restriction (CBSR) is supported when DFT-based compression is utilized for the Type-II codebook. In this section, CBSR for the codebook containing the FD basis vectors is discussed and shown that CBSR can be used to reduce the UE calculation complexity and feedback overhead.

As mentioned above, each FD basis vector is given by a DFT vector which is associated with a delay in the transformed (delay) domain. The value range of the selected delays/FD basis vectors by the UE depends on the delay spread of the 2L beam-formed channels (obtained when combining the 2L beam vectors bi,∀i with the MIMO channel impulse response). The energy of the beam-formed channel impulse responses is mainly concentrated in a single peak or very few peaks and only few dominant delays/FD basis vectors selected from a subset of the codebook are associated with these peak(s). Thus, the codebook size can be reduced to a subset of FD basis vectors. Figure 1 and Figure 2 shows the probability distribution of the indices of selected FD basis vectors from the codebook without oversampling for two different codebook sizes (N3) for 10 MHz and 100 MHz system bandwidths, respectively. The DFT vectors in the codebook are ordered with respect to the phase increase. This means, the first DFT vector is given by the all-one vector (whose phase increase is zero). It is observed that the first P1 and the last P2 FD basis vectors have a significantly higher probability to be selected than other FD basis vectors. Therefore, the entries of the codebook containing the FD basis vectors may be defined by a submatrix of a DFT matrix. The codebook size reduces hence from N3 to P1+P2. Furthermore, the search space of the FD basis vector combinations used by the UE for the optimization of the parameters of the precoder in (1) is reduced as well.

Simulation results in Section 3 show that for 10 MHz system bandwidth and for (2L,M,N3)=(8,4,13) configuration,  a good choice for P1 and P2 are 3 and 3, respectively, and for (2L,M,N3)=(8,7,13) configuration,  a good choice for P1 and P2 are 4 and 4, respectively. For 100 MHz system bandwidth for (2L,M,N3)=(8,5,35) configuration, a good choice for P1 and P2 are 4 and 3, respectively and for (2L,M,N3)=(8,9,35) configuration, a good choice for P1 and P2 are 5 and 5, respectively.

Proposal 1: Codebook subset restriction should be supported for the FD basis vector codebook for reducing the feedback overhead and the UE calculation complexity for the selection of the FD basis vectors. The entries of the codebook containing the FD basis vectors then are defined by a submatrix of a DFT matrix. A good choice for P1 and P2 are

- P1=4 and P2=4 for (2L,M,N3)=(8,7,13) and 10 MHz system bandwidth, and

- P1=5 and P2=5 for (2L,M,N3)=(8,9,35) and 100 MHz system bandwidth.

| - M= 4, N3 =13, P1 =3 and P2=3 | - M= 7, N3 =13, P1 =4 and P2=4 |  |
| --- | --- | --- |
| Figure 1: Probability distribution of selected FD basis vectors by the UE for two different values of M over many channel realizations for configuration (2L,M,N3) = (8,4,13) and (2L,M,N3) = (8,7,13) for 10 MHz bandwidth. The first P1 and the last P2 FD basis vectors from the codebook have a significantly higher probability to be selected than other FD basis vectors. |  |  |
| - M= 5, N3 =35, P1 =4 and P2=3 |  | - M= 9, N3 =35, P1 =5 and P2=5 |
| Figure 2: Probability distribution of selected FD basis vectors by the UE for two different values of M over many channel realizations for the two configurations (2L,M,N3) = (8,5,35) and (2L,M,N3) = (8,9,35) for 100 MHz system bandwidth. The first P1 and the last P2 FD basis vectors from the codebook have a significantly higher probability to be selected than other FD basis vectors. |  |  |

# Simulation Results

For the evaluation of the performance of the DFT-based Type II overhead reduction scheme, simulations were carried out for the Dense Urban (Macro only) channel model for SU-MIMO/MU-MIMO setups with rank adaptation. The results are provided for 32 antenna ports at the gNB and system bandwidths of 10 MHz (SCS of 15 kHz) and 100 MHz (SCS of 30 kHz). The simulation parameters are according to the agreements made in RAN1#95 [1], and listed again in Table 2 in Appendix. For a performance comparison of the proposed overhead reduction scheme, the Type-II CSI scheme using L=4 beams with WB/SB amplitude and 8-PSK/QPSK phase is used as a reference.

For 10 MHz bandwidth, N3 is given by 13 and the value of M is chosen by Alt1: M= p×N3R, where  p∈14,12. Therefore, for the given two values of p, M∈{4,7}. Choosing Alt2: M= p×N3 also results in identical values of M. For 100 MHz bandwidth, N3 is given by 35. By choosing Alt1, for given values of p, M∈{5,9} , whereas using Alt2, M∈{9,18}. Since the number of FD components does not depend on the codebook size N3 and due to the larger M values using Alt2, Alt1 is preferred.

Proposal 2: Support Alt1: M= p×N3R with p∈14,12.

Also, the value of K0 depends on the value of β. The supported values of β are given by {18,14,12,34}. When β=18  a large performance degradation is observed. Therefore, only the last three values of β are used in the simulations.

Quantization of combining coefficients

Figure 3a and Figure 3b show the performance of the DFT-based compression scheme for the different quantization schemes discussed above. Also shown are the compression ratios achieved by each quantization scheme with respect to Type-II CSI reporting in Figure 4. The following setups are considered: (2L,M,N3)=(8,4,13) and 2L,M,N3=8,7,13 with different values of K0 for 10 MHz system bandwidth.

From Figure 3a and Figure 3b, it can be observed that the performance achieved by ALT3 is almost identical to the performance of ALT1 for both configurations, but it achieves a significant reduction in feedback overhead. Although, ALT2a also achieves a comparable performance to ALT1, the performance-overhead trade-off is not on par compared to the performance-overhead tradeoff achieved by ALT3. Also, shown in Figure 4, the compression ratio achieved by ALT3 scheme is the highest compared to the other quantization schemes.

Observation 2: ALT3 achieves almost the same performance than ALT1 by a further reduction of the feedback overhead by 12%, 13% and 26% for the configurations (2L,M,K0)= (8,4,24), (2L,M,K0)=(8,7,28), and  (2L,M,K0)=(8,7,42), respectively.

Figure 3a: Performance achieved by various quantization schemes with respect to Rel. 15 Type-II CSI framework for the configuration (2L, M)=(8,4) for three values of K0=[8,16,24]

Figure 3b: Performance achieved by various quantization schemes with respect to Rel. 15 Type-II CSI framework for the configuration (2L, M)=(8,7) for three values of K0=[14,28,42]

Figure 4: Feedback compression ratios of various quantization schemes for different configurations (M,K0) with respect to Rel.15 Type-II CSI feedback.

As a conclusion, we have the following proposal.

Proposal 3: Consider ALT3 for the quantization of the FD coefficients for the Rel.16 Type-II compression codebook.

Moreover, from Figures 3a and 3b, considering M=4 and ¼ of the 2LM coefficients (β=14) results only in minor performance gain, whereas for M=7, the performance is satisfactory due to the use of large number of FD components. However, when increasing the value of β to 12, a performance improvement of approximately 2% can be seen for both setups. Further increasing the value of β to 34 results in the best performance for both setups. Therefore, larger values of β shall be supported to achieve significant performance gain.

Proposal 4: Support β=12,34.

Codebook subset restriction for FD basis vector codebook

Figure 5a and Figure 5b show the performance of the DFT-based compression scheme obtained when using CBSR on the frequency domain codebook for different values of (P1, P2) and sizes of the codebook N3 for the following setups: (2L,M,N3)=(8,4,13) and (2L,M,N3)=(8,7,13) for 10 MHz system bandwidth, and (2L,M,N3)=(8,9,35) and (8,18,35) for 100 MHz system bandwidth. As observed from Figure 5a (10 MHz system bandwidth), when the codebook is restricted to the first 3 basis vectors and last 3 basis vectors, i.e., when (P1,P2)=(3,3), and when M=4 FD vectors are selected, the performance loss is only 0.5% compared to the case when 4 FD basis vectors are selected using the entire codebook (referred to as “Ideal”).

Figure 5a: Performance loss when the codebook is restricted to the first P1 basis vectors and the last P2 basis vectors. (P1,P2) = (3,3) and (P1,P2) = (4,4).

Figure 5b: Performance loss when the codebook is restricted to the first P1 basis vectors and the last P2 basis vectors. (P1,P2) = (4,3) and (P1,P2) = (5,5).

On the other hand, when (P1,P2)=(4,4)  with M=7 FD basis vectors, the performance loss is only 0.7% compared to the case when M=7 FD basis vectors are selected using the entire codebook (“Ideal”). As observed in Figure 5b (100 MHz system bandwidth), when the codebook is restricted to the first 5 basis vectors and last 5 basis vectors, i.e., when (P1,P2)=(4,3) and when M=5 FD vectors are used, the performance loss in only 0.6% compared to the case when M=5 vectors are selected using the entire codebook. On the other hand, when the codebook is restricted to the first 5 basis vectors and last 5 basis vectors, i.e., when (P1,P2)=(5,5) and when M=9 FD vectors are used, the performance loss in only 0.9% compared to the case when M=9 vectors are selected using the entire codebook.

Observation 3: A marginal performance loss is observed when using codebook subset restriction on the FD basis vector codebook. The codebook size (N3) reduces from 13 to 6 or 8 FD basis vectors and from 35 to 7 or 10 FD basis vectors for the 10 MHz and 100 system bandwidth configurations, respectively.

FD Basis and size-K0 subset selection:

In this subsection, the simulation results are presented for the following two issues for the configuration 2L, M,N3=8,4,13 and 2L, M, N3=8,7,13 for two different values of K0.

- size-K0 subset design for the first layer

  - Alt1: Unrestricted subset (2LM)

  - Alt2: Polarization-common subset (LM)

  - Alt3: Restricted subset (for a given subset of beams and FD basis, size=2L+M)

- FD basis selection and coefficient subset selection for two layers

  - Alt 1A: layer-common FD basis subset selection, layer-common coefficient subset selection

  - Alt 1B: layer-common FD basis subset selection, layer-independent coefficient subset selection

  - Alt 2: layer-independent FD basis subset selection, layer-independent coefficient subset selection

Figure 6 shows the performance achieved without restriction on subset selection and with polarization-common subset for various configurations of M and K0 for a single layer transmission. In the case of no restriction, the bitmap of size 2LM is used to indicate the selection of K0 non-zero coefficients. In the case of polarization common subset, a single bitmap of size LM is used to select not more than K02 combining coefficients per polarization subset.

Figure 6: Performance achieved when using size-K0 subset with no restriction and polarization-common subset for the first layer.

From Figure 6, it can be observed that the difference in the achieved performance using unrestricted subset selection and polarization common subset selection is quite small. However, using polarization-common subset results offers a reduction of feedback overhead saving of LM bits.

Observation 4: Polarization-common subset selection results in minor performance loss over the unrestricted subset.

Proposal 5: Support unrestricted subset and polarization-common subset selection.

Figure 7a and 7b show the achieved performance when using the identical and non-identical FD basis in addition to common and independent size-K0 subset selection over two layers. The results are shown for two different configurations 2L,M,N3=(8,4,13) and 2L,M,N3=(8,7,13) for two different values of K0. The agreed alternatives are listed under issue 2. From Figure 7a and 7b, it can be observed that using layer-common basis results in performance loss compared to that of layer-independent basis. Also, layer-common basis with layer-common subset selection results in minor performance loss compared to the layer-independent subset selection.

Figure 7a: Performance achieved when using layer-common and layer-independent basis and subset selection for the configuration 2L,M,N3=(8,4,13) for two different values of K0.

Figure 7b: Performance achieved when using layer-common and layer-independent basis and subset selection for the configuration 2L,M,N3=(8,7,13) for two different values of K0

Observation 5: Layer-common basis selection and layer-common subset selection results in some performance loss over layer-independent FD basis and layer-independent coefficient subset selection.

Instead of choosing a sub-optimal approach for FD basis and coefficient selection, for example, a layer-common FD basis selection and layer-common coefficient subset selection to achieve feedback reduction and then spending more bits for the quantization of the coefficients to avoid large performance degradations, it would be more meaningful to take advantage of the benefits provided by the optimal approaches (layer-independent basis selection and/or subset selection) combined with a quantization scheme that results in a better performance, but achieves a higher feedback reduction.

Therefore, any decision on the feedback reduction schemes associated with the FD basis and the subset selection should be made in conjunction with the quantization scheme.

Observation 6: Choosing a sub-optimal approach for FD basis and coefficient subset selection to save feedback overhead and then spending more bits for the quantization scheme should be avoided.

Proposal 6: Discuss layer-common/independent FD basis and coefficient subset selection in conjunction with the quantization scheme.

# Conclusions

Based on the above discussion, we have the following observations and proposals.

Observation 1: ALT3 achieves the highest compression ratio compared to the other quantization schemes.

Proposal 1: Codebook subset restriction should be supported for the FD basis vector codebook for reducing the feedback overhead and the UE calculation complexity for the selection of the FD basis vectors. The entries of the codebook containing the FD basis vectors then are defined by a submatrix of a DFT matrix. A good choice for P1 and P2 are

- P1=4 and P2=4 for (2L,M,N3)=(8,7,13) and 10 MHz system bandwidth, and

- P1=5 and P2=5 for (2L,M,N3)=(8,9,35) and 100 MHz system bandwidth.

Observation 2: ALT3 achieves almost the same performance than ALT1 by a further reduction of the feedback overhead by 12%, 13% and 26% for the configurations (2L,M,K0)= (8,4,24), (2L,M,K0)=(8,7,28), and  (2L,M,K0)=(8,7,42), respectively.

Proposal 3: Consider ALT3 for the quantization of the FD coefficients for the Rel.16 Type-II compression codebook.

Proposal 4: Support β=12,34.

Observation 3: A marginal performance loss is observed when using codebook subset restriction on the FD basis vector codebook. The codebook size (N3) reduces from 13 to 6 or 8 FD basis vectors and from 35 to 7 or 10 FD basis vectors for the 10 MHz and 100 system bandwidth configurations, respectively.

Observation 4: Polarization-common subset selection results in minor performance loss over the unrestricted subset.

Proposal 5: Support unrestricted subset and polarization-common subset selection.

Observation 5: Layer-common basis selection and layer-common subset selection results in some performance loss over layer-independent FD basis and layer-independent coefficient subset selection.

Observation 6: Choosing a sub-optimal approach for FD basis and coefficient subset selection to save feedback overhead and then spending more bits for the quantization scheme should be avoided.

Proposal 6: Discuss layer-common/independent FD basis and coefficient subset selection in conjunction with the quantization scheme.

# References

Chairman’s notes, RAN1#95, Spokane, USA, November 12th – November 16th, 2018.

Chairman’s notes, RAN1-AH 1901, Taipei, Taiwan, 21st – 25th January 2019.

Fraunhofer IIS, Fraunhofer HHI, “Enhancements on Type-II CSI: Doppler approach,” R1-1900361, Athens, Greece, Feb 25th – March 1st, 2019.

# Appendix

Table 2: Simulation parameters and setup

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Macro Dense Urban |
| Frequency Range |  | 4GHz (FR1) |
| Inter-BS distance |  | 200m |
| Channel model |  | According to the TR 38.901 |
| Antenna setup and port layouts at gNB |  | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ |
| BS Tx power |  | 41 dBm |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Number of RBs |  | 52 for 15 kHz SCS /275 for 30 kHz SCS |
| Simulation bandwidth |  | 10 MHz,15kHz SCS; 100 MHz, 30 kHz SCS |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU/MU-MIMO with rank adaptation |
| MIMO layers |  | Up to 2 layers |
| Overhead |  | DMRS, CSI-RS, PDCCH |
| UE distribution |  | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC |
| Evaluation Metric |  | Throughput vs CSI feedback overhead (bits) |
| Baseline for performance evaluation |  | Rel-15 Type II Codebook |