3GPP TSG-RAN WG1 Meeting #96	 R1-1902884

Athens, Greece, February 25th – March 1st, 2019

Agenda Item:	7.2.2.2.2

Source:	Ericsson

Title:	Enhancements to initial access procedure

Document for:	Discussion, Decision

# 1	Introduction

In this contribution we discuss further details on procedures for initial access, RRM measurements and radio link monitoring based on agreements during the SI phase [2].

# 2	Discussion

Support of unlicensed operation both stand-alone as well as in combination with licensed spectrum is an essential requirement for NR. Due to specific characteristics and regulatory requirements of using unlicensed spectrum, e.g., coexistence with other systems, uncertainty of channel availability, and transmit power restrictions, etc., some technology components need to be specifically designed as compared with those for licensed spectrum.

In unlicensed spectrum, transmissions are often (depending on regulation) subject to LBT which leads to that signals and channels might be shifted in time or not transmitted at all. This will evidently affect procedures such as initial access, RRM measurements and radio link monitoring (RLM).

When signals and channels are subject to LBT, their relative positions and/or presence cannot be guaranteed.

When operating in unlicensed spectrum it is important to minimize always on transmissions and to concentrate them in time as much as possible. Minimizing always on transmissions reduces interference both within a network and towards other networks. Concentrating transmissions in time instead of spreading them out has the benefit that the number of times a node needs to contend for the medium is minimized.

For efficient operation in unlicensed spectrum, always-on-transmissions should be kept at a minimum and be concentrated in time as much as possible.

In the following sections we give an overview of the respective procedures in NR and highlight aspects where procedures need to be modified to accommodate operation in unlicensed spectrum, given the observations above. Note that in our companion paper [4], we discuss the benefits of grouping signals for initial access and RRM into a Discovery Reference Signal (DRS).

## 2.1	Initial access

The NR the initial access procedure can be split into (at least) the following three steps:

- Cell search

- Reception of (minimum) system information

- Random Access

### 2.1.1	Cell search

In NR, cell search is accommodated by the UE reading at least one SS/PBCH block that provides time and frequency synchronization and the physical layer cell ID. The gNB can transmit up to L SS/PBCH blocks in a half frame, where L depends on the frequency range. The candidate SS/PBCH blocks in a half frame are indexed in ascending order in time from 0 to . The UE determines the SS/PBCH block index based on the detected PBCH DM-RS sequence when L ≤ 8, or a combination of the detected PBCH DM-RS sequence and the PBCH payload when 8 < L ≤ 64. Based on the SS/PBCH index, candidate positions as defined in [1], and the PBCH payload, the UE can establish cell timing.

During the SI phase [2] it was agreed to define a mechanism for SS/PBCH block transmission accounting for LBT failure by expanding the number of candidate SS/PBCH block positions within a DRS transmission window to [Y], for e.g. Y = [64]. It was also agreed to define a mechanism for UE(s) to determine the timing and QCL assumptions from the detected SS/PBCH block.

In NR, the maximum value of the SS/PBCH block index that can be signaled to the UE is 64 corresponding to FR2. It is noted that the maximum value is lower for FR1 (4 or 8), but in principle the signaling mechanism used for FR2 can be reused for NR-U to account for LBT failure. Because there are two SS/PBCH block positions in each slot, the 64 indices can cover 32 slots, which corresponds to 32 or 16 ms for 15 or 30 kHz subcarrier spacing, respectively. Thus, using a subset of the available 64 indices, any SS/PBCH block position within a half-frame (duration = 5 ms) can be addressed.

Using a subset of the 64 available SS/PBCH block indices and the half-frame indicator, the UE can determine the frame timing from a detected SS/PBCH block arbitrarily shifted in time with a shift granularity of a half slot.

In in SI phase [2], the duration of the DRS transmission window was for further study. For Rel-13 LAA, the DRS can shift within a window of 6 ms. For NR-U we expect the UE to be even more dependent on robust DRS transmissions, e.g. to keep time/frequency synch and perform measurements for RLM and mobility. Thus, given Observation 3 we think that the mechanism to signal the timing of the DRS should be designed to allow the UE to determine the timing of an arbitrary shifted SS/PBCH block position. This can be achieved using the half-frame indicator and 10 or 20 SS/PBCH block indices for a SS/PBCH block sub-carrier spacing of 15 kHz or 30 kHz respectively.

During the SI phase, the following agreement regarding the relationship of the DRS duration and the LBT category was made [2]:

|  | Cat 2 LBT | Cat 4 LBT |
| --- | --- | --- |
| DRS alone or multiplexed with non-unicast data (e.g. OSI, paging, RAR) | When the DRS duty cycle ≤1/20, and the total duration is up to 1 ms: 25 µs Cat 2 LBT is used (as in LAA) | When DRS duty cycle is > 1/20, or total duration > 1 ms |

With this agreement, in order to use Cat 2 LBT the duration of the transmitted DRS is limited to 1 ms, and thus the maximum number of SS/PBCH blocks that can be transmitted within the DRS depends on the sub-carrier spacing of the SS/PBCH blocks. In [4] we give an example of a compact DRS design for the case of 30 kHz SCS that has a duration of 1 ms where 4 SS/PBCH blocks and the corresponding CORESET#0 and PDSCH carrying RMSI are multiplexed.

In order to use Cat2 LBT, the duration of the DRS should be less than 1 ms; hence, the maximum number of transmitted SS/PBCH blocks X within the DRS depends on the sub-carrier spacing. For 30 or 15 kHz SCS, X = 4 or 2 results in a DRS duration less than 1 ms, respectively.

In NR the UE may assume that SS/PBCH blocks transmitted with the same SS/PBCH block index on the same center frequency location are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and, when applicable, spatial Rx parameters. However, the UE shall not assume quasi co-location for any other SS/PBCH block transmissions. This implies that SS/PBCH blocks with different indices should be treated independently by the UE.

When the SS/PBCH blocks are allowed to shift in time due to LBT outcome, the SS/PBCH index detected by the UE should change depending on the shift. This will allow the UE to determine the frame timing, but it will also affect other procedures. For example, SS/PBCH block based RRM measurements are done per SS/PBCH block index, and the UE averages measurements only for SS/PBCH blocks detected with the same index. If the indices shift due to LBT, clearly the procedure can be affected if the shift is not taken into account. Similarly, RLM and RACH procedures can be affected since they also depend on the detected SS/PBCH block index.

This was also recognized in the SI phase [2] where it is for further study how to derive the frame timing from detected SS/PBCH blocks and the relationship between transmitted SSB index and QCL assumptions at the UE.

As an example, say that the gNB transmits four different SS/PBCH blocks, e.g., 4 SS/PBCH blocks that are beamformed differently. If LBT succeeded on the first attempt, the gNB will transmit index 0,1,2,3. In the next SS/PBCH block period, say that LBT failed on the first and second attempt, requiring the SSB transmissions to be delayed, the gNB would then transmit index 2,3,4,5. From the UE’s perspective it appears as though index 4 and 5 correspond to  new SS/PBCH blocks and that  index 0,1 could not be detected. However, from the gNB’s perspective it is simply transmitting the same 4 SS/PBCH blocks as in the previous period but shifted in time for the current period.

Thus, the UE needs to know that even if the indices detected for two SS/PBCH blocks are different, the actual SS/PBCH blocks are in fact the same, e.g., beamformed in the same way, and thus may be assumed to be quasi co-located.

One way to achieve this is that the UE computes an effective SS/PBCH block index given by the detected SS/PBCH block index modulo N. N is the number of “distinct” SS/PBCH blocks within an SS/PBCH block burst set, i.e., SS/PBCH blocks that may not be assumed to be QCL’d, e.g., to different beamforming. The effective SS/PBCH block index is then used by the UE instead of the detected SS/PBCH block index for the purposes of RRM and RLM measurements and also for indicating PRACH occasions. The only exception is frame timing for which the UE would use the actual detected SS/PBCH indices, not the effective ones.

A key aspect to consider is the number of candidate SS/PBCH block positions and the granularity of the time shifts. A reasonable starting point is to consider a DRS transmission window of 5 ms, with half-slot granularity for the time shifts. With this assumption, the number of candidate positions is Y = 20/10 for 30 kHz/15 kHz SCS. In NR Rel-15, 8 PBCH DMRS sequences are available. Without making use of the PBCH payload, this would allow only 8 candidate positions. Making use of 2 bits of the PBCH payload in addition to the 8 DMRS sequences, then 32 candidate positions can be indicated. Since only 20 are needed, the 2 bits of the SS/PBCH block index field will take on only 3 distinct values, which limits the number of blind decodes due to the 3 hypotheses that may be needed if soft combining across SS/PBCH block bursts is needed for PBCH decoding. We point out that as long as the timing of a serving or neighbor cell is determined just once, then this blind decoding effort is not needed for future RRM/RLM measurements. Based on the above discussion, we propose the following:

Support a combination of the detected PBCH DMRS sequence and 2 bits of the PBCH payload to determine the SS/PBCH block index  of an SS/PBCH block arbitrarily shifted in time with a shift granularity of a half slot. This allows signaling of at least 20 SS/PBCH block indexes within a 5 ms window in the same way as for FR2 in NR Rel-15.

The UE determines the frame timing in the same way as in NR Rel-15 based on the decoded SS/PBCH block index and the half-frame indicator.

The UE may assume that a detected SS/PBCH block in an SS/PBCH block burst set is QCL’d with a detected SS/PBCH block in a previous SS/PBCH block burst set if the detected SS/PBCH block index modulo N is the same for both detected SS/PBCH blocks. N is the number of distinct SS/PBCH blocks within the SS/PBCH block burst set, i.e., those SS/PBCH blocks that may not be assumed to be QCL’d. FFS: Signaling mechanism for N.

In the SI phase [2] it was for further study if and how to support beam repetition for soft combining of SS/PBCH blocks within the same DRS transmission, i.e., within the same SS/PBCH block burst set. Using the mechanism in Proposal 3 and setting N=1, basic repetition can be achieved. It can be further discussed whether or not soft combining of PBCH within the same DRS transmission is supported given that the SS/PBCH block index in PBCH must change every 8th SS/PBCH block in order for the UE to derive frame timing

Basic SS/PBCH block repetition can be achieved using N=1 where the UE determines the effective SS/PBCH block index as the detected SS/PBCH block index modulo N.

In Figure 1, several examples are shown for different values of N assuming a DRS with 4 SS/PBCH blocks, including basic repetition with N=1. QCLx denotes which SS/PBCH blocks the UE may assume to be QCL’d.

Figure 1: Examples of different time shifted SS/PBCH block positions and the  actual (transmitted) SS/PBCH block index (SSBx). The effective SS/PBCH index, or QCL index, is given by QCLx = SSBx modulo N. SS/PBCH blocks with the same effective index may be assumed to be QCL’d with or between SS/PBCH block bursts.

### 2.1.2	System information

System Information (SI) is divided into Minimum SI and Other SI. Minimum SI is periodically broadcast and comprises basic information required for initial access and information for acquiring any other SI broadcast periodically or provisioned on-demand, i.e. scheduling information. The Other SI encompasses everything not broadcast in the Minimum SI and is broadcast, either triggered by the network or upon request from the UE.

The Minimum SI is transmitted over two different downlink channels using different messages (MasterInformationBlock and SystemInformationBlockType1). The term Remaining Minimum SI (RMSI) is also used to refer to SystemInformationBlockType1. Other SI (OSI) is transmitted in SystemInformationBlockType2 and above.

The MasterInformationBlock (MIB) is always transmitted on the BCH (carried on PBCH) with a periodicity of 80 ms with repetitions made within the 80 ms period. The MIB includes parameters that are needed to acquire SystemInformationBlockType1 (SIB1) from the cell.

The SystemInformationBlockType1 (SIB1) is transmitted on the DL-SCH (carried on PDSCH). SIB1 includes information regarding the availability and scheduling (e.g. periodicity, SI-window size) of other SIBs. It also indicates whether they (i.e. other SIBs) are provided via periodic broadcast basis or only on-demand basis.  If other SIBs are provided on-demand, then SIB1 includes information for the UE to perform SI request.

Considering Observation 2 it is beneficial if SIB1 (RMSI) is transmitted in the same TX burst as the SS/PBCH block(s). Depending on the exact composition and choice of subcarrier spacing for the SS/PBCH block(s) the SIB1 can either be multiplexed in time or frequency. In our companion paper [4] we discuss how grouping of at least SS/PBCH block(s) and SIB1 into a DRS can be achieved.

### 2.1.3	Random access

Support for RACH is relevant not only for the NR-U stand-alone scenario, but also for interworking (dual-connectivity) scenarios such as EN-DC.

In NR Rel-15, a mapping between transmitted SS/PBCH block(s) and a combination of ranges of preambles and random-access occasions (RO) is configured using ssb-perRACH-OccasionAndCBPreamblesPerSSB. This mapping is used for the UE to implicitly indicate to the gNB which SS/PBCH block that had the best reception conditions.

Considering Observation 1, fixed time relations between different signals/channels where LBT between the signals is required should not be assumed. If the mapping of SS/PBCH block index is done so that the UE indicates the best SS/PBCH block by transmitting Msg1 in a specific time domain RO it might happen that the UE cannot access the channel for that specific time occasion, thus it would have to delay Msg1 to the next matching time occasion. This would introduce unnecessary delays in the RACH procedure. Thus, it is beneficial if the mapping between transmitted SS/PBCH blocks and RO is done so that the best SS/PBCH block is not indicated by which time occasion the UE transmits Msg1 in, but rather by used frequency resource and/or preamble.

This can be achieved already in NR Rel-15 using appropriate configuration of ssb-perRACH-OccasionAndCBPreamblesPerSSB and msg1-FDM. For example, if 4 SS/PBCH blocks are transmitted, setting msg1-FDM equal to 4 and ssb-perRACH-Occasion to 1 or msg1-FDM equal to 2 and ssb-perRACH-Occasion to 2 will achieve time-independent mapping. Where the former would allow a maximum of 64 contention-based preambles and the latter 32. Note that this assumes that the UE uses the effective SS/PBCH block index (as defined in Section 2.1.1) in the selection of PRACH occasion for preamble transmission.

Rel-15 NR already supports configurations so that the mapping of transmitted SS/PBCH blocks to RACH occasions is robust to LBT failures.

NR Rel-15, currently allows msg1 of a contention free RACH procedure to be transmitted on SCells, with the corresponding random-access response (RAR) transmitted on the PCell. To increase the robustness of the RACH procedure against LBT failures it is beneficial to allow both contention free and contention-based RACH procedures on SCells.

Two motivations for supporting CBRA on an SCell are as follows: (1) due to LBT failure, it may be more likely in unlicensed operation that the UE reaches the maximum number of PUCCH-SR failures, meaning a CBRA-SR would be triggered. In this case it is beneficial to allow the CBRA on SCells to enhance the number of RA opportunities. (2) Due to LBT failure, a CFRA triggered by a PDCCH order on an SCell may be blocked. It is beneficial if the UE is allowed to fallback to CBRA on the same or different SCell in such a case.

Support transmission of random access response (RAR) on the SCell where MSG1 was transmitted.

In addition to contention free RACH, support also contention-based RACH (CBRA) procedures on SCells.

According to the NR-U TR [2] Section 7.2.1.3.2, several potential RACH enhancements beyond the functionality supported in Rel-15 have been identified for further study:

For potential RACH resource enhancements, the following options have been identified for NR-U, beyond the flexibility already available in Rel-15, but consensus was not achieved. These options may be further considered when specifications are developed:

-	Frequency-domain enhancement: Multiple PRACH resources across multiple LBT sub-bands/carriers for both contention-free and contention-based RA

-	Time-domain enhancements:

-	For connected mode UE, scheduling of PRACH resources via DCI.

-	Triggered PRACH within gNB acquired COT can use a new resource indicated by the DCI

-	For idle mode UE, scheduling of PRACH resources via paging

-	Note: potential inefficiency in network resource due to paging across multiple cells

-	Additional, new RACH resources are used immediately following detection of DRS transmission

-	Multiple PRACH transmissions before Msg2 reception in RAR window for initial access

-	Number of allowed transmissions is pre-defined or indicated, e.g., in RMSI

-	Group wise SSB-to-RO mapping by frequency first-time second manner, where grouping is in time domain

The first listed enhancement is to use multiple PRACH resources across multiple LBT sub-bands/carriers. It should be noted that on per-carrier level, this is already supported for initial access, as further discussed in [5], and with Proposal 5 this would be allowed also for connected mode. In contrast, multiple PRACH resources across LBT sub-bands within the same carrier should not be supported, since it is not clear yet from a RAN4 perspective what potential restrictions on contiguous or non-contiguous LBT sub-bands will be introduced.

Multiple PRACH resources across multiple carriers are already supported for initial access, and will with Proposal 7 also be supported for connected mode.

Multiple PRACH resources across multiple LBT sub-bands should not be supported due to potential RAN4 restrictioins on contiguous and non-contiguous LBT-subands within a wideband carrier.

A second listed enhancement is scheduling of PRACH resources via DCI. This is, however, a potential latency optimization, and our view is that it is not critical for Rel-16. It can instead be considered for a later release in a backwards-compatible manner.

Scheduling of PRACH resources via DCI is an optimization and is not critical for the first release of NR-U. It can be considered for a later release in a backwards-compatible manner.

Scheduling of PRACH resources via DCI should not be supported in Rel-16.

A third listed enhancement is multiple PRACH transmissions before Msg2 reception in RAR window for initial access. This feature was proposed already in Rel-15 standardization, but it was noted that it would create additional interference, and it was not included for CBRA in Rel-15. Note that the potential benefit of this feature would only be in scenarios where LBT failures and/or PRACH collisions are significant issues, but in such scenarios, one should be extra careful not to create additional interference and hence should not use the feature.

Multiple PRACH transmissions before Msg2 reception in RAR window creates additional interference and hence makes the LBT/collision problems it is intended to solve even worse.

Multiple PRACH transmissions before Msg2 reception in RAR window should not be supported.

A fourth listed enhancement is group wise SSB-to-RO mapping by frequency first-time second manner, where grouping is in time domain. The motivation is to provide multiple transmit opportunities in time domain for a PRACH transmission. However, multiple PRACH transmit opportunities can be obtained already in Rel-15 based on cyclic repetition of the SSB-to-RO mapping. See example and further discussion above in conjunction with Observation 6.

Mappings similar to group-wise SSB-to-RO mapping seem already to be supported in NR Rel-15.

## 2.2	RRM and RLM

### 2.2.1	RSSI and Channel Occupancy reporting

During the SI phase [2] it was agreed to support RSSI reporting and define a metric to measure channel occupancy or medium contention and its corresponding reporting. To that end we start by revisiting the RSSI and channel occupancy procedure in Rel-13 LAA. In Rel-13 LAA the UE performs RSSI measurements using an RMTC configuration

RMTC-Config-r13 ::=	CHOICE {

release							NULL,

setup							SEQUENCE {

rmtc-Period-r13					ENUMERATED {ms40, ms80, ms160, ms320, ms640},

rmtc-SubframeOffset-r13			INTEGER(0..639)					OPTIONAL,		-- Need ON

measDuration-r13				ENUMERATED {sym1, sym14, sym28, sym42, sym70},

...

}

}

In addition, the UE can be configured with a channel occupancy threshold given by an RSSI value

MeasRSSI-ReportConfig-r13 ::=	SEQUENCE {

channelOccupancyThreshold-r13			RSSI-Range-r13				OPTIONAL	-- Need OR

}

Based on these configurations, the UE periodically reports the following

MeasResultForRSSI-r13 ::=			SEQUENCE {

rssi-Result-r13							RSSI-Range-r13,

channelOccupancy-r13					INTEGER (0..100),

...

}

where the RSSI result is the average RSSI within the reporting period and channel occupancy is the rounded percentage of samples (OFDM symbols) which are above the channel occupancy threshold during the reporting period.

In our view the Rel-13 configuration provides enough flexibility also for NR-U and thus we propose to use the Rel-13 framework as a baseline also for NR-U.

Support RSSI and channel occupancy measurement and reporting using the Rel-13 LAA RSSI and channel occupancy framework as a baseline.

### 2.2.2	DMTC for RRM and RLM

In the SI phase [2] it was agreed that it is beneficial to configure DMTC(s) (DRS Measurement Timing Configuration) in which UEs can perform measurements. Further it was agreed beneficial that both DRS-based RRM and RLM measurements are performed inside of the (potentially different) DMTC(s). Any similarities with Rel-15 SMTC is FFS.

Our understanding is that the DMTC(s) are conceptually different from the DRS transmission window. In our view, the DRS transmission window is a concept that exists at the gNB and describes the set of possible candidate DRS positions. The DMTC(s) on the other hand is a configuration given to the UE where it should search for and/or measure on the transmitted DRS. Naturally they are tightly connected, but nevertheless not the same.

For DRS-based RLM measurements, the DMTC window will need to cater for LBT failures when the serving gNB attempts to transmit DRS in the DRS transmission window. Thus, the optimal length of the DMTC window will depend on the LBT success rate of the serving gNB.

For DRS-based RRM measurements of neighbor cells, the DMTC window will need to account for both LBT failures at the neighbor gNBs when they attempt DRS transmission as well as any timing difference between the neighbor cells and the serving cell. Note that RRM measurements of the serving cell can be done in the same DMTC window as RLM measurements.

The DMTC window for RLM measurements (and serving cell RRM) needs to account for LBT failures at the serving gNB, whereas the DMTC window for RRM measurements (for neighbor cells) needs to account for both LBT failures in neighboring gNBs and timing differences between neighbor and serving gNBs. Hence, the RLM and RRM windows should naturally be different.

Next turning to the similarities with the Rel-15 NR SMTC window (or SSB-MTC as it is called in 38.331) which is defined as

SSB-MTC ::=                             SEQUENCE {

periodicityAndOffset                    CHOICE {

sf5                                 INTEGER (0..4),

sf10                                    INTEGER (0..9),

sf20                                    INTEGER (0..19),

sf40                                    INTEGER (0..39),

sf80                                    INTEGER (0..79),

sf160                               INTEGER (0..159)

},

duration                                ENUMERATED { sf1, sf2, sf3, sf4, sf5 }

}

The periodicity and offset are already very configurable with periodicities ranging from 5 ms to 160 ms. Thus, we see no need to enhance that configurability. However, the duration field only ranges from 1 ms to 5 ms. For reference, in Rel-13 LAA, the DMTC window used for RRM measurements of SCells is fixed to 6 ms. For NR-U we expect the UE to be even more dependent on robust DRS transmissions to e.g. keep time/frequency synch and perform measurements for RLM and PCell/PSCell mobility. For cases where the UE requires measurement gaps one needs to keep in mind that the current Rel-15 NR maximum gap length is 6 ms. But regardless of that there are other cases such as intra-frequency measurements (both in CONNECTED- and IDLE/INACTIVE mode) and inter-frequency measurements in IDLE/INACTIVE mode where an extension could be useful, to increase robustness against DRS transmission failures. Thus, we make the following proposal

The SMTC window can serve as a baseline for configuring the RLM and RRM DMTC windows. FSS: if extending the duration beyond 5 ms is needed.

### 2.2.3	RLM/RLF

An RRC_CONNECTED UE in NR is provided with up to X configurable and frequency-dependent reference signals that can be used for radio link monitoring (RLM). Both SS/PBCH block and CSI-RS can be used as RLM reference signals. The RLM-RS(s) can be explicitly configured to the UE. If they are not explicitly configured, then the UE uses the (periodic) CSI-RS contained in the TCI state that indicates the QCL source for PDCCH.

Hypothetical block error rate (BLER) is used to determine in-sync (IS) or out-of-sync (OOS) conditions for either of the reference signal types (SS/PBCH block or CSI-RS). A UE assumes to be IS, if the hypothetical BLER estimated on at least one out of the X configured RLM reference signals is below a configurable threshold. On the other hand, the UE assumes to be OOS, if the hypothetical BLER estimated on all configured RLM reference signals is above yet another configurable threshold.

Upon reception of enough OOS indications, the UE starts a timer, denoted by T310. If the radio link problem is not resolved upon T310 expiry, the UE declares radio link failure (RLF). Moreover, a failure in random access and/or RLC failure can also result in RLF. After the UE declares an RLF, it attempts an RRC Connection Re-establishment by trying to find a suitable cell. If that process is not successful within a certain time, the UE will enter RRC_IDLE state.

The intention of the RLM/RLF procedures is to prevent the UE from becoming unreachable by the network. Since hypothetical BLER estimation is performed on each RLM resource to indicate in-sync (IS) or out-of-sync (OOS), we need to consider how this should be reflected when an RLM reference signal is not transmitted due to LBT.

In other words, occasional absence of DL RLM-RS transmissions should not necessarily create an OOS indication, as the UE might still be in the coverage of that RLM reference signal. However, if RLM-RS absence is ignored completely, it can result in unacceptable delays in declaring a legitimate RLF. It is necessary to find the right balance on how responsive the RLM procedure is to LBT failure.

Therefore, it seems important that the procedures to indicate in-sync/out-of-sync are properly revisited, so that not only the BLER estimated on each of the different reference signal samples is considered, but also the possible lack of reference signals.

To capture the impact of missing DL RLM-RS transmissions due to LBT failure, RAN2 together with RAN1 shall  select one of the following two options:

- Option 1: The event of the UE not detecting DL RLM-RS transmission is accounted for in the evaluation of L1 IS/OOS at the UE.

- Option 2: Introduce a separate maximum counter (or timer) for missed RLM- RS detections at the UE.

For option 1, missed RLM-RS can be considered in the L1 evaluation by giving them particular weights into the hypothetical BLER estimations for both IS and OOS. These weights could be up to UE implementation and then performance requirements would be later defined to ensure consistent behavior amongst UEs. That is, we expect the UE to tolerate a certain amount of missed RLM-RSs, but declare RLF when there have been too many. The advantage with this approach is that it will not have impact to RAN2 specifications.

The advantage of option 2 is that the event of missing RLM-RS transmissions is explicitly visible and potentially a separate RLF procedure can be defined. This requires additional interaction between L1 and L3 and impacts RAN2 specifications.

Compared to NR, NR-U operation will likely have fewer DRS transmissions due to LBT failures and/or longer configured DRS periods. As proposed in our companion contribution [4], we don’t see a benefit of periodic CSI-RS for tracking (periodic TRS) in unlicensed systems. Use of aperiodic TRS, and aperiodic CSI-RS in general, is a more natural fit for unlicensed operation. For  RLM, it makes sense that the UE could make use of aperiodic CSI-RS when triggered. Furthermore, the aperiodic CSI-RS does not necessarily need to fall inside the DMTC window. The UE should be able to make use of ap-CSI-RS for RLM outside the DMTC when needed. Increasing the number of samples used in RLM algorithms will invariably increase the measurement accuracy.

Detected DL RLM reference signals (including aperiodic CSI-RS, for example) outside of the DMTC window are used by the UE for both out-of-sync and in-sync evaluations.

A UE periodically assesses the radio link quality, which is evaluated using RLM-RS samples collected within the previous so-called evaluation period. The evaluation period is a sliding window and ends at the time of the assessment and is typically much larger than the assessment period. When considering the evaluation period and assessment period for RLM, there is a tradeoff between measurement accuracy and the decision speed of the IS/OOS algorithms. In NR-U operations, available samples can vary dramatically depending on traffic load, channel load and the DRS period. Typically, the evaluation period is chosen based on a fixed time period such as the DRS period. Hence for larger DRS periods (80 or 160 ms) and high traffic loads, many samples could be available within the evaluation period, for example if RLM-RSs outside the DMTC window are used. In other cases, where there is either a high channel load with high LBT failures or low traffic loads, fewer number of samples can be expected within the evaluation period.

Under low traffic/high channel contention, the number of samples for RLM evaluation will be relatively small.

Under high traffic/low channel contention, the number of samples for RLM evaluation can be relatively high and likely exceed any accuracy requirements.

To ensure measurement accuracy, a minimum number of samples into the RLM algorithm is required. Conversely, when the minimum number of new samples is reached, it should be possible to perform an IS/OOS evaluation and deliver a result to higher layers immediately. This will potentially speed up the RLM process and allow faster IS/OOS decisions while meeting the measurement accuracy requirement. Note that an assessment is still triggered in cases where there are not enough new samples within the assessment time period, i.e. it is triggered whichever condition occurs first.

In addition to evaluating IS/OSS in fixed time periods as in legacy NR, IS/OSS is evaluated when a defined number of fixed samples is received by the UE to accelerate the RLM process.

Like Rel-15 NR, IS/OOS assessment is triggered based on fixed time periods.

In addition, IS/OOS assessment is triggered when a defined number of fixed samples is received by the UE. FFS: The number of samples received by the IS/OOS algorithm to ensure accuracy.

In addition to DL radio link quality and inability of the gNB to transmit due to frequent DL LBT failures as discussed above, LBT failures at the UE for UL transmissions could also render the UE unreachable by the network. Thus, we consider it beneficial to introduce a mechanism that allows the UE to declare RLF due to persistent UL LBT failures. Which type of UL transmissions (e.g. RACH and PUCCH) to include in such a mechanism needs further study.

Define a mechanism (together with RAN2) for the UE to declare RLF if it experiences persistent UL LBT failures.

# Conclusion

In the previous sections we made the following observations:

Observation 1	When signals and channels are subject to LBT, their relative positions and/or presence cannot be guaranteed.

Observation 2	For efficient operation in unlicensed spectrum, always-on-transmissions should be kept at a minimum and be concentrated in time as much as possible.

Observation 3	Using a subset of the 64 available SS/PBCH block indices and the half-frame indicator, the UE can determine the frame timing from a detected SS/PBCH block arbitrarily shifted in time with a shift granularity of a half slot.

Observation 4	In order to use Cat2 LBT, the duration of the DRS should be less than 1 ms; hence, the maximum number of transmitted SS/PBCH blocks X within the DRS depends on the sub-carrier spacing. For 30 or 15 kHz SCS, X = 4 or 2 results in a DRS duration less than 1 ms, respectively.

Observation 5	Basic SS/PBCH block repetition can be achieved using N=1 where the UE determines the effective SS/PBCH block index as the detected SS/PBCH block index modulo N.

Observation 6	Rel-15 NR already supports configurations so that the mapping of transmitted SS/PBCH blocks to RACH occasions is robust to LBT failures.

Observation 7	Multiple PRACH resources across multiple carriers are already supported for initial access, and will with Proposal 7 also be supported for connected mode.

Observation 8	Scheduling of PRACH resources via DCI is an optimization and is not critical for the first release of NR-U. It can be considered for a later release in a backwards-compatible manner.

Observation 9	Multiple PRACH transmissions before Msg2 reception in RAR window creates additional interference and hence makes the LBT/collision problems it is intended to solve even worse.

Observation 10	Mappings similar to group-wise SSB-to-RO mapping seem already to be supported in NR Rel-15.

Observation 11	The DMTC window for RLM measurements (and serving cell RRM) needs to account for LBT failures at the serving gNB, whereas the DMTC window for RRM measurements (for neighbor cells) needs to account for both LBT failures in neighboring gNBs and timing differences between neighbor and serving gNBs. Hence, the RLM and RRM windows should naturally be different.

Observation 12	Under low traffic/high channel contention, the number of samples for RLM evaluation will be relatively small.

Observation 13	Under high traffic/low channel contention, the number of samples for RLM evaluation can be relatively high and likely exceed any accuracy requirements.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Support a combination of the detected PBCH DMRS sequence and 2 bits of the PBCH payload to determine the SS/PBCH block index  of an SS/PBCH block arbitrarily shifted in time with a shift granularity of a half slot. This allows signaling of at least 20 SS/PBCH block indexes within a 5 ms window in the same way as for FR2 in NR Rel-15.

Proposal 2	The UE determines the frame timing in the same way as in NR Rel-15 based on the decoded SS/PBCH block index and the half-frame indicator.

Proposal 3	The UE may assume that a detected SS/PBCH block in an SS/PBCH block burst set is QCL’d with a detected SS/PBCH block in a previous SS/PBCH block burst set if the detected SS/PBCH block index modulo N is the same for both detected SS/PBCH blocks. N is the number of distinct SS/PBCH blocks within the SS/PBCH block burst set, i.e., those SS/PBCH blocks that may not be assumed to be QCL’d. FFS: Signaling mechanism for N.

Proposal 4	Support transmission of random access response (RAR) on the SCell where MSG1 was transmitted.

Proposal 5	In addition to contention free RACH, support also contention-based RACH (CBRA) procedures on SCells.

Proposal 6	Multiple PRACH resources across multiple LBT sub-bands should not be supported due to potential RAN4 restrictioins on contiguous and non-contiguous LBT-subands within a wideband carrier.

Proposal 7	Scheduling of PRACH resources via DCI should not be supported in Rel-16.

Proposal 8	Multiple PRACH transmissions before Msg2 reception in RAR window should not be supported.

Proposal 9	Support RSSI and channel occupancy measurement and reporting using the Rel-13 LAA RSSI and channel occupancy framework as a baseline.

Proposal 10	The SMTC window can serve as a baseline for configuring the RLM and RRM DMTC windows. FSS: if extending the duration beyond 5 ms is needed.

Proposal 11	To capture the impact of missing DL RLM-RS transmissions due to LBT failure, RAN2 together with RAN1 shall  select one of the following two options:

a.	Option 1: The event of the UE not detecting DL RLM-RS transmission is accounted for in the evaluation of L1 IS/OOS at the UE.

b.	Option 2: Introduce a separate maximum counter (or timer) for missed RLM- RS detections at the UE.

Proposal 12	Detected DL RLM reference signals (including aperiodic CSI-RS, for example) outside of the DMTC window are used by the UE for both out-of-sync and in-sync evaluations.

Proposal 13	Like Rel-15 NR, IS/OOS assessment is triggered based on fixed time periods.

Proposal 14	In addition, IS/OOS assessment is triggered when a defined number of fixed samples is received by the UE. FFS: The number of samples received by the IS/OOS algorithm to ensure accuracy.

Proposal 15	Define a mechanism (together with RAN2) for the UE to declare RLF if it experiences persistent UL LBT failures.

# References

3GPP TS 38.213 v15.4.0, “Physical layer procedures for control”, December 2018.

3GPP TR 38.889, “Study on NR-based access to unlicensed spectrum,” v16.0.0, December 2018.

RP-182878, “New WID on NR-based Access to Unlicensed Spectrum,” Qualcomm, RAN#82, December 2018.

R1-1902880, “Initial access signals and channels”, Ericsson, RAN1#96.

R2-1901678, “Discussions on RACH enhancements for NR-U”, Ericsson, RAN2#105.

R1-1812026, “LS on wideband carrier operation for NR-U”, RAN1#94b.

R2-1901751, “Handling DL LBT failures in an integrated RLM framework”, Ericsson, RAN2#105.