3GPP TSG RAN WG1 Meeting #96	R1-1903586

Athens, Greece, February 25 – March 1, 2019

Agenda Item:	7.2.6.4

Source:	Huawei

Title:	Summary of discussion on combinations of Uu QoS characteristics values for V2X services

Document for:	Discussion and decision

# Introduction

The following has been captured in Chairman’s Notes RAN1 Adhoc 1910:

R1-1900012	Combinations of Uu QoS characteristics values for V2X services	SA2, Huawei

Reply LS is necessary

This document summarizes views from the related contributions submitted under agenda item 7.2.6.4 and provides potential way forward.

# Evaluation work in RAN1

According to R1-1900012 from SA2, RAN1 is requested to respond whether, for Uu over E-UTRA and NR, the two new combinations of QoS characteristics values are feasible or not.

Since the proposed reliability and latency requirements are stringent, it may not be possible to comment on the feasibility without performing evaluations according to the specific requirements of [1].

R1-1901602 has the following proposal: after the requirements and simulation assumptions are clarified, RAN1 conduct evaluation work before responding to the SA2 LS [1].

- Offline proposal 1: Performing evaluation for feasibility of these two new specific combination of UU Qos characteristics values is needed in RAN1 for the corresponding LS response.

# Simulation parameters for evaluation on two new Qos combinations

R1-1901564 provides the following simulation assumptions:

Table 1 Requirements for case 1 for performance evaluation in RAN1

|  | Air interface latency | Reliability | Traffic model |
| --- | --- | --- | --- |
| Case 1 | 3 ms | 99.99% | Periodic with a packet size of 1354 Bytes and an arrival interval of 5 ms |

Table 2 Requirements for case 2 for performance evaluation in RAN1

|  | Air interface latency | Reliability | Traffic model |
| --- | --- | --- | --- |
| Case 2 | 0.5 ms | 99.999% | Periodic with a packet size of 1300 Bytes and an arrival interval of 1.5 ms |

Table A Simulation assumptions for R15 NR

| Parameters | Value |
| --- | --- |
| Layout | Single layer - Macro layer: Road configuration in Figure 6.1.9-1 in 38.913 and BS placement as depicted in Figure A.1.3-1 in 36.885. |
| Inter-BS distance | [500 m] |
| Carrier frequency | [4 GHz] |
| Duplex model / Simulation bandwidth | FDD / 40 MHz for both DL and UL |
| SCS / Cyclic Prefix | 30 kHz / NCP |
| Channel model | UMa in TR 38.901 |
| BS antenna configuration | 4 Tx/4 Rx antenna ports, and (dH, dV) = (0.5λ, 0.8λ) (M, N, P, Mg, Ng; Mp, Np) = (8, 4, 2, 1, 1; 1, 2) (+45°, -45°) polarization and 102 degrees antenna tilt |
| UE antenna configuration | 2 Tx/4 Rx antenna ports, and Panel model 1: dH = 0.5λ For 4 Rx, (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1; 1, 2) For 2 Tx: (M, N, P, Mg, Ng; Mp, Np) = (1, 1, 2, 1, 1; 1, 1) (M, N, P, Mg, Ng; Mp, Np) = (1,1,2,1,1;1,1) |
| Transmit power | 49 dBm at BS and 23 dBm at UE |
| Antenna height | 25 m at BS and 3 m at UE |
| Antenna gain | 8 dBi at BS and 0 dBi at UE |
| Noise figure | 5 dB at BS and 9 dB at UE |
| UE distribution | Urban A in 37.885 - Vehicle type distribution: 100% vehicle type 2. - Vehicle speed is 60 km/h in all the lanes. |
| Number of UEs per cell | [10] |
| Scheduling Algorithm | Latency-based SU-MIMO |
| UE power control | Open-loop power control with P0 = -86 dBm, alpha = 0.9 |
| HARQ/repetition | Adaptive HARQ retransmission |
| Channel estimation | Realistic |
| BS receiver | MMSE-IRC |
| URLLC/eMBB Co-existence | Yes |
| Others | All control channels are error-free |

Table B Simulation assumptions for R15 LTE

| Parameters | Value |
| --- | --- |
| Layout | The same with Table A |
| Inter-BS distance | The same with Table A |
| Carrier frequency | The same with Table A |
| Duplex model / Simulation bandwidth | FDD / CA with 2 carriers, 20 MHz for each carrier for both DL and UL |
| Channel model | The same with Table A |
| BS antenna configuration | The same with Table A |
| UE antenna configuration | The same with Table A |
| Transmit power | 49 dBm per 20 MHz at BS and 23 dBm at UE |
| Antenna height | The same with Table A |
| Antenna gain | The same with Table A |
| Noise figure | The same with Table A |
| UE distribution | The same with Table A |
| Number of UEs per cell | The same with Table A |
| UE power control | Open-loop power control |
| HARQ/repetition | Repetition |
| Channel estimation | The same with Table A |
| BS receiver | The same with Table A |
| URLLC/eMBB Co-existence | The same with Table A |
| Others | The same with Table A |

There is following proposal in R1-1901602:

- Use TBS=1370 bytes in evaluations for the two new 5QI values.

There are other parameters that may affect the reliability and latency performance for the eV2X service. For example:

- Inter-BS distance

  - Current simulation assumption for transport industry is: Inter-BS distance = 500m

- Carrier frequency

  - Current simulation assumption for transport industry is: Carrier frequency = 4 GHz

- UE speed

  - Current simulation assumption for transport industry is: UE speed = 60 km/h

- Number of UEs per cell

  - Current simulation assumption for transport industry is: Number of UEs per cell = up to 10

It should be checked if the simulation assumptions for transport industry can be directly reused for the evaluation work requested by [1].

- Offline proposal 2: Take the following simulation parameters as starting point, check the highlight parameters to see if we can get consensus on these highlight parameters.

Table 3 Requirements for case 1 for performance evaluation in RAN1

|  | Air interface latency | Reliability | Traffic model |
| --- | --- | --- | --- |
| Case 1 | [3 ms] | 99.99% | a packet size of [1354,1370] Bytes and an arrival interval of 5 ms |

Table 4 Requirements for case 2 for performance evaluation in RAN1

|  | Air interface latency | Reliability | Traffic model |
| --- | --- | --- | --- |
| Case 2 | [0.5 ms] | 99.999% | a packet size of [1300,1370] Bytes and an arrival interval of 1.5 ms |

Table 5 Simulation assumptions for R15 NR

| Parameters | Value |
| --- | --- |
| Layout | Single layer - Macro layer: Road configuration in Figure 6.1.9-1 in 38.913 and BS placement as depicted in Figure A.1.3-1 in 36.885. |
| Inter-BS distance | [500 m] |
| Carrier frequency | [4 GHz] |
| Duplex model / Simulation bandwidth | FDD / 40 MHz for both DL and UL |
| SCS / Cyclic Prefix | 30 kHz / NCP |
| Channel model | UMa in TR 38.901 |
| BS antenna configuration | 4 Tx/4 Rx antenna ports, and (dH, dV) = (0.5λ, 0.8λ) (M, N, P, Mg, Ng; Mp, Np) = (8, 4, 2, 1, 1; 1, 2) (+45°, -45°) polarization and 102 degrees antenna tilt |
| UE antenna configuration | 2 Tx/4 Rx antenna ports, and Panel model 1: dH = 0.5λ For 4 Rx, (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1; 1, 2) For 2 Tx: (M, N, P, Mg, Ng; Mp, Np) = (1, 1, 2, 1, 1; 1, 1) (M, N, P, Mg, Ng; Mp, Np) = (1,1,2,1,1;1,1) |
| Transmit power | 49 dBm at BS and 23 dBm at UE |
| Antenna height | 25 m at BS and 3 m at UE |
| Antenna gain | 8 dBi at BS and 0 dBi at UE |
| Noise figure | 5 dB at BS and 9 dB at UE |
| UE distribution | Urban A in 37.885 - Vehicle type distribution: 100% vehicle type 2. - Vehicle speed is 60 km/h in all the lanes. |
| Number of UEs per cell | [10] |
| Scheduling Algorithm | Latency-based SU-MIMO |
| UE power control | Open-loop power control with P0 = -86 dBm, alpha = 0.9 |
| HARQ/repetition | Adaptive HARQ retransmission |
| Channel estimation | Realistic |
| BS receiver | MMSE-IRC |
| URLLC/eMBB Co-existence | Yes |
| Others | All control channels are error-free |

Table 6 Simulation assumptions for R15 LTE

| Parameters | Value |
| --- | --- |
| Layout | The same with Table A |
| Inter-BS distance | The same with Table A |
| Carrier frequency | The same with Table A |
| Duplex model / Simulation bandwidth | FDD / CA with 2 carriers, 20 MHz for each carrier for both DL and UL |
| Channel model | The same with Table A |
| BS antenna configuration | The same with Table A |
| UE antenna configuration | The same with Table A |
| Transmit power | 49 dBm per 20 MHz at BS and 23 dBm at UE |
| Antenna height | The same with Table A |
| Antenna gain | The same with Table A |
| Noise figure | The same with Table A |
| UE distribution | The same with Table A |
| Number of UEs per cell | The same with Table A |
| UE power control | Open-loop power control |
| HARQ/repetition | Repetition |
| Channel estimation | The same with Table A |
| BS receiver | The same with Table A |
| URLLC/eMBB Co-existence | The same with Table A |
| Others | The same with Table A |

# Discussion point 1: LS response to SA2 about the detail traffic models

There are two related proposals from these contributions submitted for RAN1#96, both of which propose to send LS response to ask SA2 to provide the detailed information about the new two Qos combinations:

- RAN1 respectfully asks SA2 to provide detailed traffic modelling, e.g., periodic or aperiodic as well as the packet arriving rate or interval, and also other assumptions, e.g., the density of vehicles, for performance evaluation in R1-1901564[1]

- Send a response LS to SA2 to inquire about the delay between a UPF terminating N6 and a 5G-AN for the two new 5QI values in R1-1901602[3].

- Offline proposal 3: send one LS response to SA2

Draft LS response:

Title: Draft response to LS on Combinations of Uu QoS characteristics values for V2X services Response to: S2-1813386 Release: Rel-16 Study Item: FS_NR_L1enh_URLLC，FS_NR_V2X To: SA WG2 Cc: RAN WG2 1. Overall Description: RAN1 has received the LS S2-1813386 from SA2. SA2 has given two new combinations of QoS characteristics values in the LS: 1) PDB = 5ms, PER = 10-4 and MDBV = 1354 bytes for Collision Avoidance and Platooning with high LoA, and 2) PDB ~1.5 ms, PER=10-5 and MDBV ~1300 bytes for Emergency Trajectory Alignment and Sensors information Sharing with high LoA. SA2 proposed the following question to RAN1. SA WG2 would like to ask RAN WG2 and RAN WG1 whether, for Uu over E-UTRA and NR, the two new combinations of QoS characteristics values indicated above are feasible or not. [RAN1 agreed that performing evaluation for feasibility of these two new specific combination of UU Qos characteristics values is needed]. For feasibility evaluation, RAN1 needs SA2 to provide detailed traffic modelling, e.g., periodic or aperiodic, and information about the delay between UPF and 5G-AN for two new Qos combinations, to get air interface latency budget for RAN1 evaluation. 2. Actions: To SA WG2 RAN1 respectfully asks SA2 to provide detailed traffic modelling, e.g., periodic or aperiodic, and information about the delay between UPF and 5G-AN for two new Qos combinations, for performance evaluation.

# Discussion point 2: LS response to RAN3 cc RAN2 about the network interface latency

There is following proposal in R1-1901602: Send a LS to RAN3 (copy RAN2) to inquire about the network interface latency for the two new 5QI values.

- However, in R1-19the latency in core network has been defined as 2 ms for remote driving in RAN1, and we can reuse this setting in R1-1901564.

- Offline proposal 4: Further discuss whether LS response to RAN3 cc RAN2 about the network interface latency is needed or not; if deemed necessary, add one question to RAN3 cc RAN2

Draft LS response:

Title: Draft response to LS on Combinations of Uu QoS characteristics values for V2X services Response to: S2-1813386 Release: Rel-16 Study Item: FS_NR_L1enh_URLLC，FS_NR_V2X To: SA WG2, RAN WG3 Cc: RAN WG2 1. Overall Description: RAN1 has received the LS S2-1813386 from SA2. SA2 has given two new combinations of QoS characteristics values in the LS: 1) PDB = 5ms, PER = 10-4 and MDBV = 1354 bytes for Collision Avoidance and Platooning with high LoA, and 2) PDB ~1.5 ms, PER=10-5 and MDBV ~1300 bytes for Emergency Trajectory Alignment and Sensors information Sharing with high LoA. SA2 proposed the following question to RAN1. SA WG2 would like to ask RAN WG2 and RAN WG1 whether, for Uu over E-UTRA and NR, the two new combinations of QoS characteristics values indicated above are feasible or not. [RAN1 agreed that performing evaluation for feasibility of these two new specific combination of UU Qos characteristics values is needed]. For feasibility evaluation, RAN1 needs SA2 to provide detailed traffic modelling, e.g., periodic or aperiodic, and information about the delay between UPF and 5G-AN for two new Qos combinations, to get air interface latency budget for RAN1 evaluation. 2. Actions: To SA WG2 RAN1 respectfully asks SA2 to provide detailed traffic modelling, e.g., periodic or aperiodic, and information about the delay between UPF and 5G-AN for two new Qos combinations, for performance evaluation. To RAN WG3 RAN1 respectfully asks RAN3 to provide the network interface latency for the eV2X messages (Collision Avoidance, Platooning with high LoA in TS 22.186)) and eV2X messages (Emergency Trajectory Alignment and Sensors information Sharing with high LoA in TS 22.186), therefore RAN1 can know the delay available for physical layer transmission over the Uu interface for evaluation.

# 6 Reference: List of contributions and proposal

- R1-1901563	Draft response to LS on Combinations of Uu QoS characteristics values for V2X services	Huawei, HiSilicon

- R1-1901564	Evaluation results on Combinations of Uu QoS characteristics values for V2X services	Huawei, HiSilicon

- R1-1901602	On Two Proposed 5QI Values	Ericsson

R1-1901563

Draft response to LS on Combinations of Uu QoS characteristics values for V2X services

Huawei, HiSilicon

To SA WG2

RAN1 respectfully asks SA2 to provide detailed traffic modelling, e.g., periodic or aperiodic as well as the packet arriving rate or interval, and also other assumptions, e.g., the density of vehicles, for performance evaluation.

R1-1901564

Evaluation results on Combinations of Uu QoS characteristics values for V2X services

Huawei, HiSilicon

In this contribution, simulation results are provided to evaluate whether the new combinations of QoS characteristics values are feasible or not in RAN1. Observations and proposals include the follows.

Observation 1: For the periodic traffic model with an arrival interval of 5 ms and a packet size of 1354 Bytes in case 1, the performance in 4 GHz assuming FDD system with 40 MHz bandwidth and 10 UEs per cell is as follows:

About 98% UEs could achieve 3 ms latency and 99.99% reliability for the downlink transmission;

About 59% and 65% UEs could achieve 3 ms latency and 99.99% reliability for the uplink transmission when GB PUSCH and GF PUSCH schemes are used respectively.

Observation 2: For the periodic traffic model with an arrival interval of 1.5 ms and a packet size of 1300 Bytes in case 2, the performance in 4 GHz assuming FDD system with 40 MHz bandwidth and 5 UEs per cell is as follows:

No UEs could achieve 0.5 ms latency and 99.999% reliability for the downlink transmission;

About 50% UEs could achieve 0.5 ms latency and 99.999% reliability for the uplink transmission when GF PUSCH scheme is used.

Proposal 1: RAN1 asks SA2 to provide detailed traffic models and requirements on UE density for the combinations of QoS characteristics values, and will further perform performance evaluation to check the feasibility.

Extraction note: 5 pages; total pages=35; total files=6; avg=5.83 pages/file

R1-1901602

On Two Proposed 5QI Values

Ericsson

Based on the discussion in the previous sections we propose the following:

Proposal 1	Send a response LS to SA2 to inquire about the delay between a UPF terminating N6 and a 5G-AN for the two new 5QI values.

Proposal 2	Send a LS to RAN3 (copy RAN2) to inquire about the network interface latency for the two new 5QI values.

Proposal 3	Use TBS=1370 bytes in evaluations for the two new 5QI values.

Proposal 4	After the requirements and simulation assumptions are clarified, RAN1 conduct evaluation work before responding to the SA2 LS [1].