3GPP TSG-RAN WG2 #101Bis	R2-1804794

Sanya, P.R. of China, 16th – 20th April 2018

Agenda Item:	10.4.1.3.9

Source:	Ericsson

Title:	PCell failure handling for Standalone NR

Document for:	Discussion, Decision

# Introduction

In RAN2 #99 meeting the following agreements has been made for the user plane in case the RLF happens on the SCell when CA level duplication is employed:

Agreements:

-	RLC reports maxNumberofRLC retransmissions are reached to RRC.

-	For a logical channel restricted to one or multiple SCell(s) (i.e. logical channel configured for duplication) UE reports the failure to the gNB (e.g. SCell-RLF) but no RRC re-establishment happens

However, same principle may be applied in case of PCell RLF for standalone NR. Therefore, in this contribution we provide a discussion and details regarding the PCell failure handling in case of CA duplication for standalone NR.

# Discussion

When CA is configured, the UE has only one RRC connection with the network. At RRC connection establishment/re-establishment/handover, one serving cell provides the NAS mobility information, and at RRC connection re-establishment/handover, one serving cell provides the security input. This cell is referred to as the Primary Cell (PCell). Depending on UE capabilities, Secondary Cells (SCells) can be configured to form together with the PCell a set of serving cells. The configured set of serving cells for a UE therefore always consists of one PCell and one or more SCells.

CA level PDCP duplication has been introduced in NR, where duplicate PDCP packet can be sent via two RLC entities, where each RLC is channel constrained/associated to only one the carriers (see Figure 1).

Figure 1 CA duplication in standalone NR

In addition to what has been described above, different principles apply to the PCell and SCell. In particular, we have:

- PCell can only be changed with handover procedure (i.e. with security key change and, unless RACH-less HO is configured, with RACH procedure);

- Unlike SCells, PCell cannot be de-activated;

- Re-establishment is triggered when PCell experiences RLF, not when SCells experience RLF;

- RLM is performed only on the PCell.

Thus, when it comes that a radio link failure is experienced on the PCell and PDCP duplication is active, based on the current specification [1] this would trigger RRC connection re-establishment with a consequent connection interruption (i.e., in this case also the SCell needs to be re-established). However, when considering use cases with stringent requirements in terms of reliability and latency, e.g., URLLC, triggering RRC connection re-establishment would cause an interruption time that may not be tolerable.

For use cases such as URLLC, when experiencing RLF on the PCell, triggering RRC connection re-establishment would cause an interruption time that may not be tolerable.

For this reason, in order to avoid interruption in the connectivity, a straightforward solution would be to avoid the RRC connection re-establishment procedure and instead sending a report (e.g., PCell-RLF) via the SCell to the network that eventually can take the necessary actions. In addition, this solution allows an alignment with the agreements made in RAN2 #99 meeting where for a logical channel restricted to one or multiple SCell(s) the UE reports the failure to the gNB (e.g. SCell-RLF) but no RRC re-establishment happens. Thus, we propose:

In case of CA duplication, when a radio link failure is experienced on the PCell, the UE sends a report to the network via the SCell without triggering RRC connection re-establishment.

RAN2 should consider to introduce a new message, e.g., named PCellRLFReport, for indicating the RLF on the PCell during CA duplication.

On the other side, since no RLM is performed on the SCell, if also the SCell is failed (even if this would be a rare case) it may happen that the PCellRLFReport sent by the UE will not reach the network and thus the UE could wait an infinite time for a RRC connection re-configuration message that will never arrive.

Since no RLM is performed on the SCell, if also the SCell is failed (even if this would be a rare case) it may happen that the PCellRLFReport sent by the UE will not reach the network and thus the UE could wait an infinite time for a RRC connection re-configuration message that will never arrive.

In order to avoid this deadlock, a solution would be that UE starts the T310 timer upon the sending of the PCellRLFReport to the network like in normal radio link failure detection procedure. Upon the expiry of timer T310, if no RRC connection re-configuration message has been received, UE shall initiate the RRC connection re-establishment procedure. Therefore, we propose:

Upon the sending of the PCellRLFReport, UE starts the T310 timer and upon its expiry the RRC connection re-establishment procedure is triggered.

# Conclusion

In section 2 we made the following observations:

Observation 1	For use cases such as URLLC, when experiencing RLF on the PCell, triggering RRC connection re-establishment would cause an interruption time that may not be tolerable.

Observation 2	Since no RLM is performed on the SCell, if also the SCell is failed (even if this would be a rare case) it may happen that the PCellRLFReport sent by the UE will not reach the network and thus the UE could wait an infinite time for a RRC connection re-configuration message that will never arrive.

Based on the discussion in section 2 we propose the following:

- In case of CA duplication, when a radio link failure is experienced on the PCell, the UE sends a report to the network via the SCell without triggering RRC connection re-establishment.

RAN2 should consider to introduce a new message, e.g., named PCellRLFReport, for indicating the RLF on the PCell during CA duplication.

Upon the sending of the PCellRLFReport, UE starts the T310 timer and upon its expiry the RRC connection re-establishment procedure is triggered.

# References

3GPP, TS 36.330 V15.0.0, 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2 (Release 15), January 2018.