3GPP TSG-RAN WG2 Meeting #102                                                            	R2-1806970

Busan, Korea, 21st - 25th May 2018

Source:	CATT, CMCC

Title:	Mobility Evaluation Parameters Extension Considering HSDN

Agenda Item:	9.22.2

Document for:	Discussion and Decision

Introduction

In RAN2#101 meeting, agreements about HSDN have been made as following:

Agreement:

1	Introduce 1 new bit indication in SIB1 to indicate the high speed dedicated network cell.

2	Maximum value for the weight for high speed dedicated network cell is 16. New value of MSE cell count for HSDN case can be further considered.

3	When the UE is in high mobility state, the UE prioritize to re-select the high speed dedicated network cell regardless of frequency priority.

4	When the UE supporting the HSDN feature is not in high mobility state, the UE deprioritize (i.e., lowest priority) to re-select the high speed dedicated network cell.

1	HSDN neighbouring cell list can be provided in SIB. FFS the content of the list.

The above agreements gave the number range of equivalent normal cell for the HSDN cell. Considering the current value range of the mobility evaluation parameters, some extension may need to be introduced to cover the HSDN cell for MSE.

Discussion

MSE for normal cell

The current mobility state parameter broadcast in SIB3 as following:

MobilityStateParameters ::=			SEQUENCE {

t-Evaluation						ENUMERATED {

s30, s60, s120, s180, s240, spare3, spare2, spare1},

t-HystNormal						ENUMERATED {

s30, s60, s120, s180, s240, spare3, spare2, spare1},

n-CellChangeMedium					INTEGER (1..16),

n-CellChangeHigh					INTEGER (1..16)

}

For normal cell, the scope of the RRU is 500m usually, as the above shows, the maximum evaluation duration is 240s, and the maximum evaluation number of cell change for high/medium speed is 16. Assuming the velocity of UE is 60km/h, 120km/h and 360km/h respectively, the number of the cell changed when the t-Evaluation is 240s is:

N = V×T÷R

Where: N: number of cell change; V: velocity of UE; T: t-Evaluation, R: scope of cell.

| Velocity | 60km/h | 120km/h | 360km/h |
| --- | --- | --- | --- |
| N | 8 | 16 | 48 |

Observation 1: For normal cell, the number of cell change is more than 16 when t-Evaluation is set to maximum value 240s and UE velocity is above 120km/h.

MSE for HSDN cell

In RAN2#101 meeting, the maximum number of equivalent normal cell for HSDN cell is 16, we take 16 as consideration to evaluate the current MSE parameter range.

The following table lists the number of cell change for normal cell and HSDN cell when the t-Evaluation is 240s with the velocity of the UE is 60km/h, 120km/h and 360km/h respectively.

Table 1 number of cell change with different mobility states

| Velocity | Number of cell change of normal cell | Number of cell change of HSDN cell |
| --- | --- | --- |
| 60km/h | 8 | 0.5 |
| 120km/h | 16 | 1 |
| 360km/h | 48 | 3 |

As above table shows, the number of UE passes the HSDN cell with the current maximum t-Evaluation is less than 1 when the velocity is lower than 120km/h, and the number of the HSDN cell is less than 3 when the velocity is lower than 360km/h.

Observation 2: For current t-Evaluation range, during the maximum t-Evaluation, the number of HSDN cell change is less than 1 when the velocity is lower than 120km/h, and less than 3 when the velocity is lower than 360km/h.

Considering the less number of HSDN cell change reduces the accuracy of the mobility state evaluation, the t-Evaluation should be extended to allow UE with high speed can passes more HSDN cells like for normal cells e.g. 4 HSDN cells during the maximum t-Evaluation.

Proposal 1: The t-Evaluation parameter should be extended to allow UE with high speed can passes more HSDN cells during the duration t-Evaluation.

Assuming UE with 120km/h and one HSDN cell equals 1-16 normal cells respectively, the period of the UE passes 4 HSDN cells is:

T= R×N÷V

Where: T: t-Evaluation; N: the number of the HSDN cells the UE passes; R: the scope of the HSDN cell by the railway; V: velocity of UE.

| equivalent number | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T(s) | 60 | 120 | 180 | 240 | 300 | 360 | 420 | 480 | 540 | 600 | 660 | 720 | 780 | 840 | 900 | 960 |

Observation 3: The t-Evaluation parameter should be extended to 960s for HSDN scenario if considering UE passes 4 HSDN cells with high speed of 120km/h.

Observation 4: the maximum value of t-Evaluation is dependent on the least number of HSDN cells that should be counted during the evaluation duration and the lowest threshold of high speed that operator defined.

Here it uses 4 HSDN cells and high speed of 120km/h in the analysis is to let some flexibility and room to allow operators to decide what the real high speed is. If 60km/h UE can select the HSDN cell, during the t-Evaluation duration, it only can cross 2 HSDN cells.  So the following analysis will be based on the assumption of crossing at least 4 HSDN cells with speed of 120km/h.

Proposal 2: The t-Evaluation parameter should be extended up to 960s for HSDN scenario.

Furthermore, with the t-Evaluation extension, the number of the cell change will increase not only for the UE with high mobility state but also for the normal mobility state or medium mobility state. Thus both n-CellChangeHigh and n-CellChangeMedium should be extended accordingly as well as t-Evaluation extension.

When UE with 120km/h during the t-Evaluation as extended above, the UE passes 4 HSDN cells, the n-CellChangeHigh and n-CellChangeMedium should be extended as shown in the table below considering each equivalent number of one HSDN cell:

| equivalent number | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n cell change | 4 | 8 | 12 | 16 | 20 | 24 | 28 | 32 | 36 | 40 | 44 | 48 | 52 | 56 | 60 | 64 |

Proposal 3: The n-CellChangeHigh and n-CellChangeMedium parameter should be extended to 64 for HSDN scenario.

As for the parameter t-HystNormal which is used to determine whether the UE is in normal speed state, considering in HSDN cell, all the UEs camped on or connected to this cell are in high speed state, when one UE leaves high speed state, the UE will reselect to a normal cell or be handovered to normal cell by network, in HSDN cell UE only needs to judge whether it is in high speed state or not, thus the HSDN cell needn’t broadcast t-HystNormal. So the t-HystNormal needn’t be extended correspondingly with the extension of t-Evaluation.

Proposal 4: The t-HystNormal parameter needn’t be extended and broadcast in HSDN cell.

Proposals

In this contribution, we propose:

Observation 1: For normal cell, the number of cell change is more than 16 when t-Evaluation is set to maximum value 240s and UE velocity is above 120km/h.

Observation 2: For current t-Evaluation range, during the maximum t-Evaluation, the number of HSDN cell change is less than 1 when the velocity is lower than 120km/h, and less than 3 when the velocity is lower than 360km/h.

Observation 3: The t-Evaluation parameter should be extended to 960s for HSDN scenario if considering UE passes 4 HSDN cells with high speed of 120km/h.

Observation 4: the maximum value of t-Evaluation is dependent on the least number of HSDN cells that should be counted during the evaluation duration and the lowest threshold of high speed that operator defined.

Proposal 1: The t-Evaluation parameter should be extended to allow UE with high speed can passes more HSDN cells during the duration t-Evaluation.

Proposal 2: The t-Evaluation parameter should be extended up to 960s for HSDN scenario.

Proposal 3: The n-CellChangeHigh and n-CellChangeMedium parameter should be extended to 64 for HSDN scenario.

Proposal 4: The t-HystNormal parameter needn’t be extended and broadcast in HSDN cell.

Reference

[1]Chairman notes: LTE breakout session minutes RAN2-101_01-03-18-19_30

Text Proposal

- SystemInformationBlockType3

The IE SystemInformationBlockType3 contains cell re-selection information common for intra-frequency, inter-frequency and/ or inter-RAT cell re-selection (i.e. applicable for more than one type of cell re-selection but not necessarily all) as well as intra-frequency cell re-selection information other than neighbouring cell related.

SystemInformationBlockType3 information element

-- ASN1START

SystemInformationBlockType3 ::=		SEQUENCE {

cellReselectionInfoCommon			SEQUENCE {

q-Hyst								ENUMERATED {

dB0, dB1, dB2, dB3, dB4, dB5, dB6, dB8, dB10,

dB12, dB14, dB16, dB18, dB20, dB22, dB24},

speedStateReselectionPars			SEQUENCE {

mobilityStateParameters				MobilityStateParameters,

q-HystSF						SEQUENCE {

sf-Medium						ENUMERATED {

dB-6, dB-4, dB-2, dB0},

sf-High							ENUMERATED {

dB-6, dB-4, dB-2, dB0}

}

}																OPTIONAL		-- Need OP

},

cellReselectionServingFreqInfo		SEQUENCE {

s-NonIntraSearch					ReselectionThreshold		OPTIONAL,		-- Need OP

threshServingLow					ReselectionThreshold,

cellReselectionPriority				CellReselectionPriority

},

intraFreqCellReselectionInfo		SEQUENCE {

q-RxLevMin							Q-RxLevMin,

p-Max								P-Max						OPTIONAL,		-- Need OP

s-IntraSearch						ReselectionThreshold		OPTIONAL,		-- Need OP

allowedMeasBandwidth				AllowedMeasBandwidth		OPTIONAL,		-- Need OP

presenceAntennaPort1				PresenceAntennaPort1,

neighCellConfig						NeighCellConfig,

t-ReselectionEUTRA					T-Reselection,

t-ReselectionEUTRA-SF				SpeedStateScaleFactors		OPTIONAL		-- Need OP

},

...,

lateNonCriticalExtension				OCTET STRING (CONTAINING SystemInformationBlockType3-v10j0-IEs)	OPTIONAL,

[[	s-IntraSearch-v920					SEQUENCE {

s-IntraSearchP-r9					ReselectionThreshold,

s-IntraSearchQ-r9					ReselectionThresholdQ-r9

}																OPTIONAL,		-- Need OP

s-NonIntraSearch-v920				SEQUENCE {

s-NonIntraSearchP-r9				ReselectionThreshold,

s-NonIntraSearchQ-r9				ReselectionThresholdQ-r9

}																OPTIONAL,		-- Need OP

q-QualMin-r9						Q-QualMin-r9				OPTIONAL,		-- Need OP

threshServingLowQ-r9				ReselectionThresholdQ-r9	OPTIONAL		-- Need OP

]],

[[	q-QualMinWB-r11						Q-QualMin-r9				OPTIONAL	-- Cond WB-RSRQ

]],

[[	q-QualMinRSRQ-OnAllSymbols-r12			Q-QualMin-r9				OPTIONAL			-- Cond RSRQ

]],

[[	cellReselectionServingFreqInfo-v1310 CellReselectionServingFreqInfo-v1310 	OPTIONAL,		-- Need OP

redistributionServingInfo-r13			RedistributionServingInfo-r13 OPTIONAL,	--Need OR

cellSelectionInfoCE-r13					CellSelectionInfoCE-r13		OPTIONAL,		-- Need OP

t-ReselectionEUTRA-CE-r13				T-ReselectionEUTRA-CE-r13	OPTIONAL		-- Need OP

]],

[[	cellSelectionInfoCE1-r13				CellSelectionInfoCE1-r13	OPTIONAL	-- Need OP

]],

[[	cellSelectionInfoCE1-v1360			CellSelectionInfoCE1-v1360	OPTIONAL		-- Cond QrxlevminCE1

]]

}

RedistributionServingInfo-r13 ::=		SEQUENCE {

redistributionFactorServing-r13		INTEGER(0..10),

redistributionFactorCell-r13		ENUMERATED{true}				OPTIONAL,	--Need OP

t360-r13							ENUMERATED {min4, min8, min16, min32,infinity,

spare3,spare2,spare1},

redistrOnPagingOnly-r13				ENUMERATED {true}		OPTIONAL	--Need OP

}

CellReselectionServingFreqInfo-v1310 ::=	SEQUENCE {

cellReselectionSubPriority-r13				CellReselectionSubPriority-r13

}

-- Late non critical extensions

SystemInformationBlockType3-v10j0-IEs ::= SEQUENCE {

freqBandInfo-r10					NS-PmaxList-r10				OPTIONAL,	-- Need OR

multiBandInfoList-v10j0				MultiBandInfoList-v10j0		OPTIONAL,	-- Need OR

nonCriticalExtension				SystemInformationBlockType3-v10l0-IEs					OPTIONAL

}

SystemInformationBlockType3-v10l0-IEs ::= SEQUENCE {

freqBandInfo-v10l0					NS-PmaxList-v10l0			OPTIONAL,	-- Need OR

multiBandInfoList-v10l0				MultiBandInfoList-v10l0		OPTIONAL,	-- Need OR

nonCriticalExtension					SystemInformationBlockType3-r15-IEs		OPTIONAL

}

SystemInformationBlockType3-r15-IEs	::=		SEQUENCE {

mobilityStateParametersHSDN-r15				MobilityStateParametersHSDN-r15  OPTIONAL,

nonCriticalExtension							SEQUENCE {}                      OPTIONAL

}

-- ASN1STOP

| SystemInformationBlockType3 field descriptions |
| --- |
| allowedMeasBandwidth If absent, the value corresponding to the downlink bandwidth indicated by the dl-Bandwidth included in MasterInformationBlock applies. |
| cellSelectionInfoCE Parameters included in coverage enhancement S criteria. They may be used by the UE to select/reselect a cell in which it works in CE mode on the concerned non serving frequency. If absent, the UE acquires the information from the target cell on the concerned frequency. |
| cellSelectionInfoCE1 Parameters included in coverage enhancement S criteria. They may be used by the UE to select/reselect a cell in which it works in CE mode on the concerned non serving frequency. E-UTRAN includes this IE only if cellSelectionInfoCE in SIB3 is present. |  |
| cellReselectionInfoCommon Cell re-selection information common for cells. |
| cellReselectionServingFreqInfo Information common for Cell re-selection to inter-frequency and inter-RAT cells. |
| freqBandInfo A list of additionalPmax and additionalSpectrumEmission values, as defined in TS 36.101 [42, table 6.2.4-1] for UEs neither in CE nor BL UEs and TS 36.101 [42, table 6.2.4E-1] for UEs in CE or BL UEs, applicable for the intra-frequency neighouring E-UTRA cells if the UE selects the frequency band from freqBandIndicator in SystemInformationBlockType1. If E-UTRAN includes freqBandInfo-v10l0 it includes the same number of entries, and listed in the same order, as in freqBandInfo-r10. |
| intraFreqcellReselectionInfo Cell re-selection information common for intra-frequency cells. |
| mobilityStateParametersHSDN Parameters used to determine UE mobility state, the mobilityStateParametersHSDN-r15 only used for HSDN cell. |
| multiBandInfoList-v10j0 A list of additionalPmax and additionalSpectrumEmission values, as defined in TS 36.101 [42, table 6.2.4-1] for UEs neither in CE nor BL UEs and TS 36.101 [42, table 6.2.4E-1] for UEs in CE or BL UEs, applicable for the intra-frequency neighouring E-UTRA cells if the UE selects the frequency bands in multiBandInfoList (i.e. without suffix) or multiBandInfoList-v9e0. If E-UTRAN includes multiBandInfoList-v10j0, it includes the same number of entries, and listed in the same order, as in multiBandInfoList (i.e. without suffix). If E-UTRAN includes multiBandInfoList-v10l0 it includes the same number of entries, and listed in the same order, as in multiBandInfoList-v10j0. |
| p-Max Value applicable for the intra-frequency neighbouring E-UTRA cells. If absent the UE applies the maximum power according to the UE capability. |
| redistrOnPagingOnly If this field is present and the UE is redistribution capable, the UE shall only wait for the paging message to trigger E-UTRAN inter-frequency redistribution procedure as specified in 5.2.4.10 of TS 36.304 [4]. |
| q-Hyst Parameter Qhyst in TS 36.304 [4], Value in dB. Value dB1 corresponds to 1 dB, dB2 corresponds to 2 dB and so on. |
| q-HystSF Parameter “Speed dependent ScalingFactor for Qhyst” in TS 36.304 [4]. The sf-Medium and sf-High concern the additional hysteresis to be applied, in Medium and High Mobility state respectively, to Qhyst as defined in TS 36.304 [4]. In dB. Value dB-6 corresponds to -6dB, dB-4 corresponds to -4dB and so on. |
| q-QualMin Parameter “Qqualmin” in TS 36.304 [4], applicable for intra-frequency neighbour cells. If the field is not present, the UE applies the (default) value of negative infinity for Qqualmin. NOTE 1. |
| q-QualMinRSRQ-OnAllSymbols If this field is present and supported by the UE, the UE shall, when performing RSRQ measurements, perform RSRQ measurement on all OFDM symbols in accordance with TS 36.214 [48]. NOTE 1. |
| q-QualMinWB If this field is present and supported by the UE, the UE shall, when performing RSRQ measurements, use a wider bandwidth in accordance with TS 36.133 [16]. NOTE 1. |
| q-RxLevMin Parameter “Qrxlevmin” in TS 36.304 [4], applicable for intra-frequency neighbour cells. |
| redistributionFactorCell If redistributionFactorCell is present, redistributionFactorServing is only applicable for the serving cell otherwise it is applicable for serving frequency |
| redistributionFactorServing Parameter redistributionFactorServing in TS 36.304 [4]. |
| s-IntraSearch Parameter “SIntraSearchP” in TS 36.304 [4]. If the field s-IntraSearchP is present, the UE applies the value of s-IntraSearchP instead. Otherwise if neither s-IntraSearch nor s-IntraSearchP is present, the UE applies the (default) value of infinity for SIntraSearchP. |
| s-IntraSearchP Parameter “SIntraSearchP” in TS 36.304 [4]. See descriptions under s-IntraSearch. |
| s-IntraSearchQ Parameter “SIntraSearchQ” in TS 36.304 [4]. If the field is not present, the UE applies the (default) value of 0 dB for SIntraSearchQ. |
| s-NonIntraSearch Parameter “SnonIntraSearchP” in TS 36.304 [4]. If the field s-NonIntraSearchP is present, the UE applies the value of s-NonIntraSearchP instead. Otherwise if neither s-NonIntraSearch nor s-NonIntraSearchP is present, the UE applies the (default) value of infinity for SnonIntraSearchP. |
| s-NonIntraSearchP Parameter “SnonIntraSearchP” in TS 36.304 [4]. See descriptions under s-NonIntraSearch. |
| s-NonIntraSearchQ Parameter “SnonIntraSearchQ” in TS 36.304 [4]. If the field is not present, the UE applies the (default) value of 0 dB for SnonIntraSearchQ. |
| speedStateReselectionPars Speed dependent reselection parameters, see TS 36.304 [4]. If this field is absent, i.e, mobilityStateParameters is also not present, UE behaviour is specified in TS 36.304 [4]. |
| t360 Parameter “T360” in TS 36.304 [4]. Value min4 corresponds to 4 minutes, value min8 corresponds to 8 minutes, and so on. |
| threshServingLow Parameter “ThreshServing, LowP” in TS 36.304 [4]. |
| threshServingLowQ Parameter “ThreshServing, LowQ” in TS 36.304 [4]. |
| t-ReselectionEUTRA Parameter “TreselectionEUTRA” in TS 36.304 [4]. |
| t-ReselectionEUTRA-SF Parameter “Speed dependent ScalingFactor for TreselectionEUTRA” in TS 36.304 [4]. If the field is not present, the UE behaviour is specified in TS 36.304 [4]. |

- –	MobilityStateParameters

The IE MobilityStateParameters contains parameters to determine UE mobility state.

MobilityStateParameters information element

-- ASN1START

MobilityStateParameters ::=			SEQUENCE {

t-Evaluation						ENUMERATED {

s30, s60, s120, s180, s240, spare3, spare2, spare1},

t-HystNormal						ENUMERATED {

s30, s60, s120, s180, s240, spare3, spare2, spare1},

n-CellChangeMedium					INTEGER (1..16),

n-CellChangeHigh					INTEGER (1..16)

}

MobilityStateParametersHSDN-r15 ::=	SEQUENCE {

t-Evaluation-r15						ENUMERATED {

s180, s240, s300, s360, s420, s480, s540, s600, s660,

s720, s780, s840, s900, s960, spare2, spare1},

n-CellChangeMedium-r15				INTEGER (1..64),

n-CellChangeHigh-r15					INTEGER (1..64)

}

-- ASN1STOP

MobilityStateParameters field descriptions

n-CellChangeHigh The number of cell changes to enter high mobility state. Corresponds to NCR_H in TS 36.304 [4].

n-CellChangeMedium The number of cell changes to enter medium mobility state. Corresponds to NCR_M in TS 36.304 [4].

t-Evaluation The duration for evaluating criteria to enter mobility states. Corresponds to TCRmax in TS 36.304 [4]. Value in seconds, s30 corresponds to 30 s and so on.

t-HystNormal The additional duration for evaluating criteria to enter normal mobility state. Corresponds to TCRmaxHyst in TS 36.304 [4]. Value in seconds, s30 corresponds to 30 s and so on.