3GPP TSG-RAN WG2#102 	R2-1807001

# Busan, Korea, 21th-25th May 2018 											  Revision of R2-1804517

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0052 | rev | - | Current version: | 15.1.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on BSR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-05-10 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1) In Section 5.4.5 (BSR), the condition for triggering an SR upon BSR trigger while UL-SCH resources are available was agreed to be that “UL-SCH resources available for a new transmission do not meet the LCP mapping restriction”. Since the whole point of this condition is to address the latency aspects of a URLLC service, the “allowedServingCells” part of the LCP mapping restriction should not apply here, and on the contrary, it should not be required to trigger an SR if the BSR can be timely conveyed on the carrier of the twin duplicated LCH. Therefore the allowedServingCells mapping restriction should be excluded from the above condition. 2) In Section 5.4.5 (BSR), the procedure for reporting padding BSR captures that if the number of padding bits is equal to or larger than the size of the Long BSR plus its subheader, long BSR will be reported even if there is no any data in all LCGs: “report Long BSR for all LCGs which have data available for transmission.Since no LCG has any data for transmission, the Long BSR is reported without any BS field in that case. However, in Section 6.1.3.1 (BSR MAC CEs), there is a note stating: “NOTE: The number of the Buffer Size fields in the Long Truncated BSR format can be zero”. From the above, this note should be fixed to also include the Long BSR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Exclude the allowedServingCells mapping restriction from the LCP mapping restriction used for the condition for triggering an SR while UL-SCH is available for BSR transmission 2) Clarify in the Note about BSR with zero BS fields in Section 6.1.3.1 that it applies to Long BSR as well. Impact analysis Impacted functionality: BSR and SR. Inter-operability: - If the network is implemented according to the CR and the UE is not, the network will receive more SRs then expected from CA-duplicated DRBs. - If the UE is implemented according to the CR and the network is not, the network will receive less SRs then expected from CA-duplicated DRBs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1) SRs are triggered unnecessarily 2) The Note in Section 6.1.3.1 is not comprehensive and does not reflect the description in Section 5.4.5. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.5, 6.1.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | This CR is implemented on top of rapporteur’s latest running CR 57 rev2 in R2-1806229 |  |  |  |  |  |  |  |  |

Start of 1st change

### 5.4.5	Buffer Status Reporting

The Buffer Status reporting (BSR) procedure is used to provide the serving gNB with information about UL data volume in the MAC entity.

RRC configures the following parameters to control the BSR:

-	periodicBSR-Timer;

-	retxBSR-Timer;

-	logicalChannelSR-DelayTimerApplied;

-	logicalChannelSR-DelayTimer;

-	logicalChannelSR-Mask;

-	logicalChannelGroup.

Each logical channel may be allocated to an LCG using the logicalChannelGroup. The maximum number of LCGs is eight.

The MAC entity determines the amount of UL data available for a logical channel according to the data volume calculation procedure in TSs 38.322 and 38.323 [3] [4].

A BSR shall be triggered if any of the following events occur:

-	the MAC entity has new UL data available for a logical channel which belongs to an LCG; and either

-	the new UL data belongs to a logical channel with higher priority than the priority of any logical channel containing available UL data which belong to any LCG; or

-	none of the logical channels which belong to an LCG contains any available UL data.

in which case the BSR is referred below to as 'Regular BSR';

-	UL resources are allocated and number of padding bits is equal to or larger than the size of the Buffer Status Report MAC CE plus its subheader, in which case the BSR is referred below to as 'Padding BSR';

-	retxBSR-Timer expires, and at least one of the logical channels which belong to an LCG contains UL data, in which case the BSR is referred below to as 'Regular BSR';

-	periodicBSR-Timer expires, in which case the BSR is referred below to as 'Periodic BSR'.

For Regular BSR, the MAC entity shall:

1>	if the BSR is triggered for a logical channel for which logicalChannelSR-DelayTimerApplied is configured by upper layers:

2>	start or restart the logicalChannelSR-DelayTimer.

1>	else:

2>	if running, stop the logicalChannelSR-DelayTimer.

For Regular and Periodic BSR, the MAC entity shall:

1>	if more than one LCG has data available for transmission when the MAC PDU containing the BSR is to be built:

2>	report Long BSR for all LCGs which have data available for transmission.

1>	else:

2>	report Short BSR.

For Padding BSR:

1>	if the number of padding bits is equal to or larger than the size of the Short BSR plus its subheader but smaller than the size of the Long BSR plus its subheader:

2>	if more than one LCG has data available for transmission when the BSR is to be built:

3>	if the number of padding bits is equal to the size of the Short BSR plus its subheader:

4>	report Short Truncated BSR of the LCG with the highest priority logical channel with data available for transmission.

3>	else:

4>	report Long Truncated BSR of the LCG(s) with the logical channels having data available for transmission following a decreasing order of priority, and in case of equal priority, in increasing order of LCGID.

2>	else:

3>	report Short BSR.

1>	else if the number of padding bits is equal to or larger than the size of the Long BSR plus its subheader:

2>	report Long BSR for all LCGs which have data available for transmission.

For BSR triggered by retxBSR-Timer expiry, the MAC entity considers that the logical channel that triggered the BSR is the highest priority logical channel that has data available for transmission at the time the BSR is triggered.

The MAC entity shall:

1>	if the Buffer Status reporting procedure determines that at least one BSR has been triggered and not cancelled:

2>	if UL-SCH resources are available for a new transmission:

3>	instruct the Multiplexing and Assembly procedure to generate the BSR MAC CE(s);

3>	start or restart periodicBSR-Timer except when all the generated BSRs are long or short Truncated BSRs;

3>	start or restart retxBSR-Timer.

2>	if there is no UL-SCH resource available for a new transmission; or

2>	if the UL-SCH resources available for a new transmission do not meet the LCP mapping restrictions, excluding the allowedServingCells mapping restriction, (see subclause 5.4.3.1) configured for the logical channel(s) that triggered the BSR(s):

3>	if a Regular BSR has been triggered and logicalChannelSR-DelayTimer is not running:

4>	if the MAC entity is not configured with configured uplink grant(s); or

4>	if the Regular BSR was not triggered for a logical channel for which logical channel SR masking (logicalChannelSR-Mask) is setup by upper layers:

5>	trigger a Scheduling Request.

A MAC PDU shall contain at most one BSR MAC CE, even when multiple events have triggered a BSR. The Regular BSR and the Periodic BSR shall have precedence over the padding BSR.

The MAC entity shall restart retxBSR-Timer upon reception of a grant for transmission of new data on any UL-SCH.

All triggered BSRs may be cancelled when the UL grant(s) can accommodate all pending data available for transmission but is not sufficient to additionally accommodate the BSR MAC CE plus its subheader. All BSRs triggered prior to MAC PDU assembly shall be cancelled when a MAC PDU which includes a BSR MAC CE is transmitted.

Start of 2nd change

6.1.3	MAC Control Elements (CEs)

6.1.3.1	Buffer Status Report MAC CEs

Buffer Status Report (BSR) MAC CEs consist of either:

-	Short BSR format (fixed size); or

-	Long BSR format (variable size); or

-	Short Truncated BSR format (fixed size); or

-	Long Truncated BSR format (variable size).

The BSR formats are identified by MAC PDU subheaders with LCIDs as specified in Table 6.2.1-2.

The fields in the BSR MAC CE are defined as follows:

-	LCG ID: The Logical Channel Group ID field identifies the group of logical channel(s) whose buffer status is being reported. The length of the field is 3 bits;

-	LCGi: For the Long BSR format, this field indicates the presence of the Buffer Size field for the logical channel group i. The LCGi field set to "1" indicates that the Buffer Size field for the logical channel group i is reported. The LCGi field set to "0" indicates that the Buffer Size field for the logical channel group i is not reported. For the Long Truncated BSR format, this field indicates whether logical channel group i has data available. The LCGi field set to "1" indicates that logical channel group i has data available. The LCGi field set to "0" indicates that logical channel group i does not have data available;

-	Buffer Size: The Buffer Size field identifies the total amount of data available according to the data volume calculation procedure in TSs 38.322 and 38.323 [3] [4] across all logical channels of a logical channel group after the MAC PDU has been built (i.e. after the logical channel prioritization procedure, which may result the value of the Buffer Size field to zero). The amount of data is indicated in number of bytes. The size of the RLC and MAC headers are not considered in the buffer size computation. The length of this field for the Short BSR format and the Short Truncated BSR format is 5 bits. The length of this field for the Long BSR format and the Long Truncated BSR format is 8 bits. The values for the 5-bit and 8-bit Buffer Size fields are shown in Tables 6.1.3.1-1 and 6.1.3.1-2, respectively. For the Long BSR format and the Long Truncated BSR format, the Buffer Size fields are included in ascending order based on the LCGi. For the Long Truncated BSR format the number of Buffer Size fields included is maximised, while not exceeding the number of padding bits.

NOTE:	The number of the Buffer Size fields in the Long BSR and Long Truncated BSR format can be zero.

Figure 6.1.3.1-1: Short BSR and Short Truncated BSR MAC CE

Figure 6.1.3.1-2: Long BSR and Long Truncated BSR MAC CE

End of 2nd change