3GPP TSG-RAN WG2 Meeting #102	R2-1807156

Busan, South Korea, 21st - 25th May 2018

Agenda item:	10.4.1.3.9

Source:	Nokia, Nokia Shanghai Bell

Title:	Abortion of the RRC connection establishment

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

During the e-mail discussion “[101bis#51][NR] Connection control open issues” it was discussed whether abortion of RRC connection establishment triggered upper layers is needed in NR.

In this contribution we discuss further the abortion of the RRC connection establishment.

# 2	Discussion

In LTE the following is specified in 3GPP TS 36.331 [1]:

#### “5.3.3.9	Abortion of RRC connection establishment

If upper layers abort the RRC connection establishment procedure while the UE has not yet entered RRC_CONNECTED, the UE shall:

1>	stop timer T300, if running;

1>	reset MAC, release the MAC configuration and re-establish RLC for all RBs that are established;”

Abortion of the RRC connection establishment seems beneficial if upper layer is aware of such information that the connection is not needed anymore. This would decrease the amount of signalling, because the connection establishment followed by the connection release can be skipped. However, it should be noted that the abortion of the connection establishment may not happen very often. Specification effort for having the abortion functionality seems rather small and therefore we are open to have such functionality specified.

According to current draft TP on NR connection control the upper layers may ask establishment of an RRC connection while the UE is in RRC_IDLE or requests resume of an RRC connection while the UE is in RRC_INACTIVE:

#### “5.3.3.2	Initiation

The UE initiates the procedure when upper layers request establishment of an RRC connection while the UE is in RRC_IDLE.

…

#### 5.3.13.2	Initiation

The UE initiates the procedure when upper layers or AS requests resume of an RRC connection, when responding to NG-RAN paging or upon triggering RNA updates while the UE is in RRC_INACTIVE.”

Observation 1: The abortion of the connection may be needed for both RRC establishment and RRC Resume

Proposal 1: Send LS to CT1 asking whether abortion of the RRC connection establishment is supported in their specification

Proposal 2: Ask CT1 whether the abortion of the connection establishment is possible for both RRC establishment and RRC Resume

# 3	Conclusion

Based on the discussion and the observation the following is proposed:

Proposal 1: Send LS to CT1 asking whether abortion of the RRC connection establishment is supported in their specification

Proposal 2: Ask CT1 whether the abortion of the connection establishment is possible for both RRC establishment and RRC Resume

Draft LS is provided in [2]

# References

- 3GPP TS 36.331: "E-UTRA; Radio Resource Control (RRC) - Protocol Specification".

- R2-xxxx “Draft LS - Abortion of the RRC Connection Establishment", Nokia.