3GPP TSG-RAN WG2 Meeting #105                                     R2-1900267

Athens, Greece, 25th February – 1st March 2019

Source:	CATT

Title:	Text Proposal for Protocol Aspects in TR 38.855

Agenda Item:	11.8.2

Document for:	Discussion and Decision

- Introduction

In RAN2#104, the following agreements are achieved on protocol of NR positioning.

Agreements 1 LPP is reused and extended to support the new NR RAT dependent positioning methods (that are discussed in the scope of this SI) 2 RAN2 assume that NRPPa is reused to support new NR RAT dependent positioning methods (this is a RAN3 decision)

In this contribution, we discuss further details on LPP and NRPPa, and propose to capture corresponding results on LPP and NRPPa in TR 38.855.

- Discussion

  - LPP

- LPP protocol between UE and LMF

In order to support E-UTRAN-dependent positioning methods in NR, LPP has already been introduced in NR system [1]. The LPP PDU is carried in NAS PDU between the UE and the AMF as follows:

Figure 1 LPP protocol between UE and LMF

- LPP protocol between UE and NG RAN with location management

In the email discussion [3], location management in NG-RAN (here we can name it as RAN-LMF) was discussed. Most companies support location management functionality in NG-RAN. Furthermore, in our contribution [4], we propose RAN2 to transmit the location related information between NR-RAN with location management and UE in RRC message as transparent container, i.e. LPP can be carried over RRC. LPP protocol between UE and NR-RAN with location management is shown in figure 2.

Figure 2 LPP protocol between UE and NG RAN with location management

In order to support the new NR RAT dependent positioning methods, the following LPP procedures are re-used with some extensions:

- Exchange of positioning capabilities.

Some new NR RAT dependent positioning capabilities need to be introduced.

- Transfer of assistance data

In NR, some new NR specific assistance data need to be introduced to support new NR RAT depended positioning.

- Transfer of location information

The procedure is used to transfer of positioning measurements and/or position estimate between UE and LMF. Some new positioning measurements and/or position estimate may also be extended in order to support new NR RAT depended positioning.

  - NRPPa

- NRPPa protocol between NG RAN and LMF

In order to support E-UTRAN-dependent positioning methods in NR, NRPPa has already been introduced in NR system [1]. Figure 2 shows the protocol layering used to support transfer of NRPPa PDUs between an LMF and NG-RAN Node.

Figure 3 NRPPa protocol between NG RAN and LMF

- NRPPa protocol between NG RAN  and NG RAN with location management

Most companies support location management functionality in NG-RAN. Similar as location related information exchanged between NG-RAN and LMF, some location related information needs to be exchanged between NG-RAN and NG-RAN with location management, e.g. configurations of DL position, position measurements, and so on. In order to minimize standardization, we propose to NRPPa can be carried over Xn between NG RAN and NG RAN with location management. NRPPa protocol between NG RAN and NG RAN with location management is shown in figure 4.

Figure 4 NRPPa protocol between NG-RAN and NG RAN with location management

Two types of NRPPa procedures had already been introduced:

-	UE associated procedure, i.e. transfer of information for a particular UE (e.g. positioning measurements);

-	Non UE associated procedure, i.e. transfer of information applicable to the NG-RAN node and associated TPs (e.g. gNB/ng-eNB/TP timing information).

In order to support the new NR RAT dependent positioning methods, the above two types of NRPPa procedures needs to be extended, e.g. transfer new positioning measurements for uplink based solutions, or transfer new positioning assistance info for downlink based solutions.

- Conclusion

In section 2, we analyze existing agreements and possible extensions for LPP and NRPPa in NR. In addition, we further discuss impacts and protocols if location management functionality in NG-RAN is introduced. In section 5, we provide a TP in section 5 to capture these original agreements as an outcome of our studying on protocols.

Proposal: Capture TP in section 5 as an outcome of our studying on protocols.

- Reference

[1] TS 38.305 v15.2.0

[2] TR 38.855 v1.0.1 Study on NR positioning support

[3] R2-19xxxxx    Email discussion report on [104#35][NR] Positioning SI, Intel

[4] R2-1900266 Further Consideration on Supporting RAN-LMF in NR Positioning, CATT

- Text Proposals for TR 38.855

9.2	Procedure and protocol aspects

9.2.x Protocol aspect

- 9.2.x.1 Protocol between an LMF and UE

1) Protocol layering between Core Network LMF and UE

LPP is reused and extended to support the new NR RAT dependent positioning methods. The LPP PDU is carried in NAS PDU between the UE and the AMF as follows:

Figure x.x.x LPP protocol between UE and LMF

2) Protocol layering between RAN-LMF and UE

Location management functionality is supported in NG-RAN. In order to transmit the location related information between NR-RAN with location management (named as RAN-LMF) and UE, LPP can be carried over RRC. LPP protocol between UE and NR-RAN with location management is shown as follows:

Figure x.x.x LPP protocol between UE and NG-RAN with location management

- 9.2.x.2 Protocol between an LMF and NG-RAN node

1) Protocol layering between Core Network LMF and NG-RAN node

The following is the protocol layering used to support transfer of NRPPa PDUs between an LMF and NG-RAN Node.

Figure x.x.x NRPPa protocol between NG RAN and LMF

2) Protocol layering between RAN-LMF and NG-RAN node

Location management functionality is supported in NG-RAN. Similar as location related information exchanged between NG-RAN and LMF, some location related information needs to be exchanged between NG-RAN and NG-RAN with location management. NRPPa can be carried over Xn between NG-RAN and NG-RAN with location management. NRPPa protocol between NG-RAN and NG-RAN with location management is shown as follows:

Figure x.x.x NRPPa protocol between NG RAN and NG RAN with location management

9.2.y Procedure aspects

- 9.2.y.1 LPP Procedures

The following LPP procedures are re-used with some extensions:

- Exchange of positioning capabilities;

- Transfer of assistance data;

- Transfer of location information.

- 9.2.y.2 NRPPa Procedures

Two types of NRPPa procedures are reused with some extensions to support the new NR RAT dependent positioning methods, as follows:

- UE associated procedure;

- Non UE associated procedure.