3GPP TSG-RAN WG2 Meeting #107bis                                                            	R2-1912142

Chongqing, P.R.China, 14th - 18th Oct 2019

Source:	CATT

Title:	Issues for Log MDT Configuration in DC Scenario

Agenda Item:	6.12.2

Document for:	Discussion and Decision

Introduction

In RAN2#106 meeting, we made the following agreements for DC scenario [1]:

Agreements

1	Immediate MDT configurations are supported for DC scenario.

2	Logged MDT configurations can come from SN node in DC scenario.

3	The existing MDT framework is the baseline for the SCG cells related MDT configuration.

4	The triggers for MDT measurements associated to MCG and SCG are separate.

5	MN-SN coordination is required for MDT measurements’ configuration and reporting in DC framework.

6	If SRB3 is not configured, SN related measurements are transmitted to MN via SRB1/2 and then forwarded to SN.

7	If SRB3 is configured, MN related measurements are transmitted to MN via SRB1/2, SN related measurements are transmitted to SN via SRB3

In the contribution, we try to clarify the issues on log MDT configuration for DC scenario.

Discussion

  - Background for Log MDT in LTE

In LTE, we have the following rules for log MDT [2]:

“For Logged MDT, the configuration, measurement collection and reporting of the concerning measurement will always be done in cells of the same RAT type”.

“There is only one RAT-specific logged measurement configuration for Logged MDT in the UE. When the network provides a configuration, any previously configured logged measurement configuration will be entirely replaced by the new one. Moreover, logged measurements corresponding to the previous configuration will be cleared at the same time. It is left up to the network to retrieve any relevant data before providing a new configuration”.

In LTE, UE can only store one RAT-specific logged measurement configuration per time. Any previously configured logged measurement configuration will be entirely replaced by the new one.

Observation1: In LTE, UE can only store one RAT-specific logged measurement configuration per time. Any previously configured logged measurement configuration will be entirely replaced by the new one.

Log MDT feature is not extended for DC scenario in LTE as DC scenario is not deployed in LTE system. Companies have little interest to introduce log MDT for DC scenario.

Observation2: In LTE, Log MDT feature is not introduced for DC scenario.

  - Log MDT Configuration for DC Scenario in NR

In NR, DC scenario is introduced in the first release, so companies agreed to check the feasibility of introducing log MDT feature for DC scenario. In RAN2#106 meeting, we agreed logged MDT configurations can come from SN node in DC scenario. But this agreement is not clear enough, at least there has three different understandings:

Option1: For DC scenario, MN and SN can configure log MDT configuration to UE separately and no log MDT configuration coordination is needed between MN and SN. UE can only store one RAT-specific logged measurement configuration per time. Any previously configured logged measurement configuration will be entirely replaced by the new one.

Option2: For DC scenario, both MN and SN can configure log MDT configuration to UE but log MDT configuration coordination is needed between MN and SN, which means UE can only receive log MDT configuration from MN or SN. UE can only store one RAT-specific logged measurement configuration per time. Any previously configured logged measurement configuration will be entirely replaced by the new one.

Option3: For DC scenario, MN and SN can configure log MDT configuration to UE separately. UE is allowed to store more than one log MDT configuration and the details are FFS.

Based on the analysis above, we think further clarification is needed on log MDT configuration for DC scenario. So we propose the followings:

Proposal: RAN2 to discuss which option is more desirable:

Option1: For DC scenario, MN and SN can configure log MDT configuration to UE separately and no log MDT configuration coordination is needed between MN and SN. UE can only store one RAT-specific logged measurement configuration per time. Any previously configured logged measurement configuration will be entirely replaced by the new one.

Option2: For DC scenario, both MN and SN can configure log MDT configuration to UE but log MDT configuration coordination is needed between MN and SN, which means UE can only receive log MDT configuration from MN or SN. UE can only store one RAT-specific logged measurement configuration per time. Any previously configured logged measurement configuration will be entirely replaced by the new one.

Option3: For DC scenario, MN and SN can configure log MDT configuration to UE separately. UE is allowed to store more than one log MDT configuration and the details are FFS.

Conclusion

According to the analysis in section 2, we propose:

Proposal: RAN2 to discuss which option is more desirable:

Option1: For DC scenario, MN and SN can configure log MDT configuration to UE separately and no log MDT configuration coordination is needed between MN and SN. UE can only store one RAT-specific logged measurement configuration per time. Any previously configured logged measurement configuration will be entirely replaced by the new one.

Option2: For DC scenario, both MN and SN can configure log MDT configuration to UE but log MDT configuration coordination is needed between MN and SN, which means UE can only receive log MDT configuration from MN or SN. UE can only store one RAT-specific logged measurement configuration per time. Any previously configured logged measurement configuration will be entirely replaced by the new one.

Option3: For DC scenario, MN and SN can configure log MDT configuration to UE separately. UE is allowed to store more than one log MDT configuration and the details are FFS.

- 4 Reference

- RAN2#106 Chairman Notes

- TS37.320