3GPP TSG-RAN2 Meeting #107bis 	R2-1913268

# Chongqing, China,  14 – 18 October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1295 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Reconfiguration with sync for NE-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In NE-DC, a UE suspends all SCG DRBs due to SCG failure as specified in 36.331. ### 5.6.13 SCG failure information … The purpose of this procedure is to inform E-UTRAN about an SCG failure the UE has experienced i.e. SCG radio link failure, SCG change failure. #### 5.6.13.2 Initiation A UE initiates the procedure to report SCG failures when SCG transmission is not suspended and when one of the following conditions is met: 1> upon detecting radio link failure for the SCG, in accordance with 5.3.11; or 1> upon SCG change failure, in accordance with 5.3.5.7a; or 1> upon stopping uplink transmission towards the PSCell due to exceeding the maximum uplink transmission timing difference when powerControlMode is configured to 1, in accordance with clause 7.17.2 of TS 36.133 [29]. In case of DC, upon initiating the procedure, the UE shall: 1> suspend all SCG DRBs and suspend SCG transmission for split DRBs; 1> reset SCG-MAC; 1> stop T307; 1> if the UE is configured with NE-DC: 2> initiate transmission of the SCGFailureInformationEUTRA message via the NR MCG as specified in TS 38.331 [82], clause 5.7.3a; 1> else: 2> initiate transmission of the SCGFailureInformation message in accordance with 5.6.13.3;. The current text prevents the network from initiating handover for the UE with only SCG DRBs which are suspended due to SCG failure. - the reconfigurationWithSync is included in masterCellGroup only when AS security has been activated, and SRB2 with at least one DRB are setup and not suspended. If a handover condition is triggered for the UE with only SCG DRBs which are suspended, the network is not able to initiate handover for the UE. |
| Summary of change: |  | It is added that the reconfigurationWithSync can be included in masterCellGroup only when at least one DRB in SCG is setup and suspended. Impact analysis Impacted 5G architecture options: NE-DC Impacted functionality: Reconfiguration with sync in MCG Inter-operability: If the UE is implemented according to the CR but the network is not, there is no inter-operability issue. However, the network is not able to initiate handover for the UE which has a suspended DRB in SCG. If the network is implemented according to the CR but the UE is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The network is not able to initiate handover for the UE which has a suspended DRB in SCG. The UE with a suspended SCG cannot be handed over. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.3.5.2	Initiation

The Network may initiate the RRC reconfiguration procedure to a UE in RRC_CONNECTED. The Network applies the procedure as follows:

-	the establishment of RBs (other than SRB1, that is established during RRC connection establishment) is performed only when AS security has been activated;

-	the addition of Secondary Cell Group and SCells is performed only when AS security has been activated;

-	the reconfigurationWithSync is included in secondaryCellGroup only when at least one DRB is setup in SCG;

-	the reconfigurationWithSync is included in masterCellGroup only when at least one DRB is setup in SCG and suspended;

-	the reconfigurationWithSync is included in masterCellGroup only when AS security has been activated, and SRB2 with at least one DRB are setup and not suspended.