3GPP TSG-RAN WG2 #107bis	 TDoc R2-1913508

# Chongqing, China, 14 – 18 Oct 2019		      Revision of R2-1910786

Agenda Item:	6.2.2.3

Source:	Ericsson

Title:	2-step Random Access for NR-U

Document for:	Discussion, Decision

# Introduction

In NR-U, a listen before talk (LBT) operation may need be performed prior to any transmission. For the legacy four step random access this will result in four LBT occasions. As a result, RAN2 has concluded in the TR from the SI phase of NR-U [1] that the 2-step random access procedure is supported in NR-U.  In this paper, we discuss some potential issues with the 2-step random access procedure and propose corresponding solutions.

# Discussion

In the 2-step random access (RA) procedure, the UL messages (Msg1 and Msg3 in legacy 4-step procedure) are combined into MsgA and sent simultaneously (or consecutive in time to allow transmission without the need for a LBT between) and similarly the two DL messages (Msg2 and Msg4, combined to MsgB) are sent as a simultaneous response in the DL. The procedure is illustrated from a high level in Figure 1.

Figure 1 – Illustration of two-step procedure.

In the legacy four step procedure, one main usage of the first two messages is to obtain UL time alignment for the UE. In many situations, e.g. in small cells, for stationary UEs or UEs in connected mode, this may not be needed since either a TA equal to 0 will be sufficient (small cells) or a stored TA value could serve also for the current RA. In future radio networks and in NR-U deployments it can be expected that these situations are common, both due to dense deployments of small cells and a great number of e.g. stationary IoT devices. A possibility to skip the message exchange to obtain the TA value would lead to reduced RA latency and would be beneficial in several use cases, for example when transmitting infrequent small data packets.

In small cells or for stationary UEs, Random Access may not be needed to obtain UL time alignment.

Since every LBT faces the possibility to fail, the number of LBTs needed to perform a random access procedure is important for NR-U. A reduction of the needed number of LBTs reduces the risk of the UE or gNB not being able to transmit and thereby increasing the latency compared to the licenced case. The reduced number of needed LBTs needed for the 2-step procedure compared to the 4-step procedure is an important benefit for the 2-step procedure in NR-U.

For NR-U, the reduced number of LBTs needed for a 2-step Random access procedure compared to the legacy 4-step procedure, is an important benefit.

## The 2-step Random Access procedure

The details of the design have not yet been decided in RAN1 but may have great consequences for NR-U. One example is that for MsgA, where it has been agreed in the 2-step WID [2] that the preamble part and the Msg3 part are TDMed. How large a time difference that can be allowed between the preamble and the Msg3 part may have impacts on the number of LBTs that have to be performed. For NR-U, one of the main benefits of the 2-step RA is the reduced need of LBTs compared to the 4-step RA procedure. In case MsgA would need two LBTs much of the benefits of the 2-step RA would disappear. Therefore, it is of vital importance that MsgA can be transmitted using only one LBT, i.e. that both the preamble part and Msg3 part can be transmitted within the MCOT. We therefore propose

For NR-U, it should be possible to send the preamble part and PUSCH part of MsgA using only one LBT.

In the NR Rel. 15 PRACH configurations for short preamble formats there can be RO in different places of a slot, i.e. beginning, middle or end of a slot. Should the preamble transmission be done at a RO not falling in the end of the slot, a new LBT would be needed for the PUSCH transmission in the next slot. Methods to avoid this should be defined in RAN2. One example of how to do this would be to mask certain RO (in the same way as is done for 4-step CFRA by the PRACH mask) to indicate which ROs in the PRACH configuration that could be used for 2-step. We therefore propose:

A PRACH mask indicating which ROs are allowed for 2-step should be defined to ensure that the preamble and PUSCH in msgA can be transmitted without a gap even if the PRACH configuration has ROs which do not fall immediately before the PUSCH resource of msgA.

The ROs which are not usable for 2-step according to the new PRACH mask can still be used for 4-step, in case the 2-step and 4-step are configured with shared or overlapping ROs.

Possibilities to ensure there is no gap can be also be done by L1 methods, e.g. by prolonging the preamble transmission until the end of the slot. These methods should be considered by RAN1.

# Conclusion

In section 2 we made the following observations:

Observation 1	In small cells or for stationary UEs, Random Access may not be needed to obtain UL time alignment.

Observation 2	For NR-U, the reduced number of LBTs needed for a 2-step Random access procedure compared to the legacy 4-step procedure, is an important benefit.

Based on the discussion in section 2 we propose the following:

Proposal 1	For NR-U, it should be possible to send the preamble part and PUSCH part of MsgA using only one LBT.

Proposal 2	A PRACH mask indicating which ROs are allowed for 2-step should be defined to ensure that the preamble and PUSCH in msgA can be transmitted without a gap even if the PRACH configuration has ROs which do not fall immediately before the PUSCH resource of msgA.

# References

- 3GPP TR 38.889 v16.0.0 “Study on NR-based access to unlicensed spectrum (Release 16)

- RP-182894, New work item: 2-step RACH for NR, ZTE Corporation, Sanechips