3GPP TSG-RAN WG2 Meeting #108		R2-1914689

Reno, USA, 18th - 22th November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.304 | CR | 0142 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 38.304 Running CR on UE Power saving in NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav-Core |  |  |  |  |  | Date: |  |  | 2019-11-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To capture agreements for power saving in NR into TS 38.304. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of power saving feature in NR. This CR captures the idle/inactive aspects of power saving and it is based on the RAN2 agreements made so far. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Power saving feature in NR is missing in TS 38.304. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.4.2, 5.2.4.7.0, 5.2.4.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

5.2.4.2	Measurement rules for cell re-selection

Following rules are used by the UE to limit needed measurements:

-	If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE may choose not to perform intra-frequency measurements.

-	Otherwise, the UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for NR inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For a NR inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current NR frequency, the UE shall perform measurements of higher priority NR inter-frequency or inter-RAT frequencies according to TS 38.133 [8].

-	For a NR inter-frequency with an equal or lower reselection priority than the reselection priority of the current NR frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current NR frequency:

-	If the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE may choose not to perform measurements of NR inter-frequencies or inter-RAT frequency cells of equal or lower priority;

-	Otherwise, the UE shall perform measurements of NR inter-frequencies or inter-RAT frequency cells of equal or lower priority according to TS 38.133 [8].

-	If the UE supports relaxed measurement and relaxedMeasurementReselectionParts is present in SIB2, the UE may further limit the needed measurements, as specified in sub-clause 5.2.4.X.

Editor’s Note: The terminology of “relaxed measurement” is assumed.

Editor’s Note: The terminologies of relaxedMeasurementReselectionParts will be aligned with other specifications (e.g. TS 38.331).

Next change

5.2.4.7	Cell reselection parameters in system information broadcasts

5.2.4.7.0	General reselection parameters

Cell reselection parameters are broadcast in system information and are read from the serving cell as follows:

absThreshSS-BlocksConsolidation

This specifies minimum threshold of the beam which can be used for selection of the highest ranked cell, if rangeToBestCell is configured.

cellReselectionPriority

This specifies the absolute priority for NR frequency or E-UTRAN frequency.

cellReselectionSubPriority

This specifies the fractional priority value added to cellReselectionPriority for NR frequency or E-UTRAN frequency.

Qoffsets,n

This specifies the offset between the two cells.

Qoffsetfrequency

Frequency specific offset for equal priority NR frequencies.

Qhyst

This specifies the hysteresis value for ranking criteria.

Qoffsettemp

This specifies the additional offset to be used for cell selection and re-selection. It is temporarily used in case the RRC Connection Establishment fails on the cell as specified in TS 38.331 [3].

Qqualmin

This specifies the minimum required quality level in the cell in dB.

Qrxlevmin

This specifies the minimum required Rx level in the cell in dBm.

Qrxlevminoffsetcell

This specifies the cell specific Rx level offset in dB to Qrxlevmin.

Qqualminoffsetcell

This specifies the cell specific quality level offset in dB to Qqualmin.

rangeToBestCell

This specifies the R value range which the cells whose R value is within the range can be a candidate for the highest ranked cell. It is configured in SIB2 and used for intra-frequency and equal priority inter-frequency cell reselection and among the cells on the highest priority frequency(ies) for inter-frequency cell reselection within NR.

TreselectionRAT

This specifies the cell reselection timer value. For each target NR frequency and for each RAT other than NR, a specific value for the cell reselection timer is defined, which is applicable when evaluating reselection within NR or towards other RAT (i.e. TreselectionRAT for NR is TreselectionNR, for E-UTRAN TreselectionEUTRA).

NOTE:	TreselectionRAT is not broadcast in system information but used in reselection rules by the UE for each RAT.

TreselectionNR

This specifies the cell reselection timer value TreselectionRAT for NR. The parameter can be set per NR frequency as specified in TS 38.331 [3].

TreselectionEUTRA

This specifies the cell reselection timer value TreselectionRAT for E-UTRAN.

ThreshX, HighP

This specifies the Srxlev threshold (in dB) used by the UE when reselecting towards a higher priority RAT/ frequency than the current serving frequency. Each frequency of NR and E-UTRAN might have a specific threshold.

ThreshX, HighQ

This specifies the Squal threshold (in dB) used by the UE when reselecting towards a higher priority RAT/ frequency than the current serving frequency. Each frequency of NR and E-UTRAN might have a specific threshold.

ThreshX, LowP

This specifies the Srxlev threshold (in dB) used by the UE when reselecting towards a lower priority RAT/ frequency than the current serving frequency. Each frequency of NR and E-UTRAN might have a specific threshold.

ThreshX, LowQ

This specifies the Squal threshold (in dB) used by the UE when reselecting towards a lower priority RAT/ frequency than the current serving frequency. Each frequency of NR and E-UTRAN might have a specific threshold.

ThreshServing, LowP

This specifies the Srxlev threshold (in dB) used by the UE on the serving cell when reselecting towards a lower priority RAT/ frequency.

ThreshServing, LowQ

This specifies the Squal threshold (in dB) used by the UE on the serving cell when reselecting towards a lower priority RAT/ frequency.

SIntraSearchP

This specifies the Srxlev threshold (in dB) for intra-frequency measurements.

SIntraSearchQ

This specifies the Squal threshold (in dB) for intra-frequency measurements.

SnonIntraSearchP

This specifies the Srxlev threshold (in dB) for NR inter-frequency and inter-RAT measurements.

SnonIntraSearchQ

This specifies the Squal threshold (in dB) for NR inter-frequency and inter-RAT measurements.

SSearchDeltaP

This specifies the Srxlev delta threshold (in dB) for relaxed measurement.

SSearchThresholdP

This specifies the Qrxlevmeas absolute threshold (in dB) for relaxed measurement.

SSearchThresholdQ

This specifies the Qqualmeas absolute threshold (in dB) for relaxed measurement.

TSearchDeltaP [FFS]

This specifies the time period over which SSearchDeltaP is evaluated for relaxed measurement.

Editor’s Note: FFS whether the network is able to control the UE RRM measurement relaxation in dedicated manner.

Next change

5.2.4.X	Relaxed measurement

5.2.4.X.0	Relaxed measurement rules

When the UE is required to perform measurements of intra-frequency or NR inter-frequencies or inter-RAT frequency cells [of equal or lower priority] according to the measurement rules in sub-clause 5.2.4.2, the UE may choose to perform relaxed measurements [FFS according to TS 38.133 [8]] when:

-	The relaxed measurement criterion in sub-clause 5.2.4.12.1 is fulfilled.

Editor’s Note: FFS whether measurements on frequencies with higher priorities can be relaxed.

Editor’s Note: FFS whether detailed methods for relaxed measurements is captured in TS 38.133.

Editor’s Note: FFS whether to introduce other conditions for relaxed measurements, e.g. shorter value than 24H for cell reselection like in LTE.

5.2.4.X.1	Relaxed measurement criterion

The relaxed measurement criterion is fulfilled when:

-	If SSearchDeltaP is configured, (SrxlevRef – Srxlev) < SSearchDeltaP, [FFS for a period of TSearchDeltaP]; and,

-	If SSearchDeltaP is configured, Qrxlevmeas > SSearchThresholdP [FFS and if SSearchThresholdQ is configured, Qqualmeas > SSearchThresholdQ].

Where:

-	Srxlev = current Srxlev value of the serving cell (dB).

-	Squal = current Squal value of the serving cell (dB).

-	SrxlevRef = reference Srxlev value of the serving cell (dB), set as follows:

-	[FFS After selecting or reselecting a new cell, or]

-	[FFS If (Srxlev - SrxlevRef) > 0, or]

-	[FFS If the relaxed monitoring criterion has not been met for TSearchDeltaP:]:

-	[The UE shall set the value of SrxlevRef to the current Srxlev value of the serving cell];

-	TSearchDeltaP = FFS.

Editor’s Note: The terminologies of SSearchDeltaP, TSearchDeltaP and SSearchThresholdP/SSearchThresholdQ will be aligned with other specifications (e.g. TS 38.331).

Editor’s Note: FFS whether RSRQ needs to be considered for the measurement quantities for ‘low mobility criteria’.

Editor’s Note: FFS whether only RSRP or other qualities (e.g. RSRQ, SINR) need to be considered for the measurement quantities for ‘not at cell edge criteria’.

Editor’s Note: FFS how to determine the SrxlevRef, e.g. whether to reuse the mechanism in LTE or allow to “exit” relaxed measurement.

Editor’s Note: FFS whether TSearchDeltaP is fixed or configurable.

Editor’s Note: FFS whether beam-based conditions are considered separately.

End of change

# Annex – RAN2 agreements

RAN2#107 meeting:

Agreements - Measurement relaxation criteria can consider both low mobility and UE location in the cell (e.g. whether the UE is in cell-edge). - UE may activate relaxed measurement criteria if at least any of the following conditions are met: a) Serving Cell measurement does not change more than a relative threshold during a time period - LTE relaxed monitoring criteria in 36.304 is considered as a baseline. Additional enhancements to address aspects that are specific to NR can be considered. b) UE is not a cell edge, meaning that serving cell/beam RSRP/RSRQ/SINR is above a threshold FFS: Whether neighbour cell RSRP should also be considered.

RAN2#107bis meeting:

Agreements:

- Network can configure the triggering criteria independently (i.e. either cell-edge or low mobility or both)

- Cell-edge criteria will not consider neighbour cell measurements