3GPP TSG-RAN WG2 Meeting #108	R2-1914977

Reno, USA, 18 – 22 Nov. 2019

Agenda item:	6.8.2.1

Source: 	Huawei, HiSilicon

Title: 	Consideration on E-CID in NR

Document for:	 Discussion and Decision

Introduction

In the RAN1 #98 meeting, the following tables was agreed and a LS (R1-1909796) was sent to RAN2 with the following agreements on the UE/gNB measurements and their related positioning techniques

| DL/UL Reference Signals | UE Measurements | To facilitate support of the following positioning techniques |
| --- | --- | --- |
| Rel.16 DL PRS | DL RSTD | DL-TDOA |
| Rel.16 DL PRS | DL PRS RSRP | DL-TDOA, DL-AoD, Multi-RTT |
| Rel.16 DL PRS / Rel.16 SRS for positioning | UE Rx-Tx time difference | Multi-RTT |
| Rel. 15 SSB / CSI-RS for RRM | SS-RSRP(RSRP for RRM), SS-RSRQ(for RRM), CSI-RSRP (for RRM), CSI-RSRQ (for RRM), SS-RSRPB (for RRM) | E-CID |

| DL/UL Reference Signals | gNB Measurements | To facilitate support of the following positioning techniques |
| --- | --- | --- |
| Rel.16 SRS for positioning | UL RTOA | UL-TDOA |
| Rel.16 SRS for positioning | UL SRS-RSRP | UL-TDOA, UL-AoA, Multi-RTT |
| Rel.16 SRS for positioning, Rel.16 DL PRS | gNB Rx-Tx time difference | Multi-RTT |
| Rel.16 SRS for positioning, | AoA and ZoA | UL-AoA, Multi-RTT |

According to the first table, NR E-CID is only limited to Rel-15 RRM measurement, which makes NR E-CID less powerful than the LTE E-CID.

Using RTT and AoA of the serving cell showed good performance as evaluated in the SI [1]. Therefore, we think what was supported in LTE E-CID should also be considered in NR E-CID, which means the measurement to derive RTT and AoA should also be supported. Note that Rel-15 already supports LTE E-CID for ng-eNB reported to LMF, therefore it does not make much sense that UEs served by ng-eNB can have better E-CID performance than UEs served by gNB.

Supported scenarios

In LTE, for the E_CID positioning methods it requires the RTT and AOA only based on the PCell. For NR, the RTT measurement based on UE Rx – Tx time difference, and gNB Rx – Tx time difference, and AOA measurement at least with respect to the PCell should be supported.

Furthermore, at the time of discussion E-CID in LTE, there is no DC architecture, while under the current positioning architecture, dual connectivity within the NG-RAN is a possible option. For Rel-15 EN-DC architecture, current specification only supports RTT/AOA with respect to E-UTRA PCell; NR PSCell is not supported. Moreover, for NE-DC, the RTT/AOA measurement of either NR PCell or E-UTRA PSCell is not supported as well.

To exploit the benefit of wide-band transmission and massive MIMO of NR, RTT and AOA measurement based on both NR PCell/PSCell should be supported. E-UTRA PSCelll can be FFS.

Proposal 1: Include RTT/AOA measurements in the E-CID positioning for NR PCell and NR PSCell. FFS E-UTRA PSCell in case of NE-DC.

Measurement

As an implicit assumption that E-CID measurement would not require any assistance data from LMF, the measurement of RTT should be based on UE Rx – Tx time difference and gNB Rx – Tx time difference, and the reference signal can be any signal available in the cell.

Proposal 2: No dedicated assistance data from LMF is required for E-CID measurement.

- 3.1 UL part of E-CID

In LTE E-CID, both RTT and AoA are exploited along with the RRM measurements. Therefore, we think what was supported in LTE E-CID should also be considered in NR E-CID, which means the measurement to derive RTT and AoA should also be supported. Note that Rel-15 already supports LTE E-CID for ng-eNB reported to LMF, therefore it does not make much sense that UEs served by ng-eNB can have better E-CID performance than UEs served by gNB.

Similar to LTE, the UL part of E-CID is provided by gNB. The measurements include

- gNB Rx – Tx time difference

- AOA/ZOA

The measurement can be based on any uplink signals and channels. Details can be up to RAN1.

Proposal 3: For UL part of E-CID, support gNB Rx – Tx time difference and AOA/ZOA based on any uplink signals and channels, e.g., SRS or PRACH.

- 3.2 DL part of E-CID

In legacy LTE, the location server (E-SMLC) requests the UE to report UE Rx – Tx time difference and RRM measurement. For NR positioning, we think the same measurement should also be supported such that NR positioning can support DL E-CID.

Similar to LTE, the DL part of E-CID is provided by UE. The measurement includes

- UE Rx – Tx time difference

The measurement can be based on any downlink signals and channel. Details can be up to RAN1.

Proposal 4: For DL part of E-CID, support UE Rx – Tx time difference based on any downlink signals and channels, e.g., TRS or SS/PBCH block.

Measurement report

For NR E-CID, it is not clear how RTT is obtained at LMF, at least the following options can be considered

- Option 1. UE and gNB reporting its Rx – Tx time difference separately to the LMF.

- Option 2. UE reporting UE Rx – Tx time difference to the serving gNB in RRM and gNB reporting type-1 TA to the LMF.

For Option 1, it is like a single-cell RTT, which is simple to implement. For Option 2, additional impact on RRM can be foreseen, since there is no UE Rx – Tx time difference reporting in NR-RRM measurement. In our view, both options should be supported for deriving the RRT for E-CID.

Similar to multi-RTT, the measurement report is conveyed by UE and gNB individually to the LMF, since it is difficult to add additional RRM measurement report to convey the UE Rx – Tx time difference to the serving gNB. Hence, we think Option1 should be supported.

Proposal 5: Support the measurement reporting from UE and gNB to the LMF, respectively.

- From UE: UE Rx – Tx time difference.

- From gNB: gNB Rx – Tx time difference, AOA, and ZOA.

Conclusion

In this contribution, we considered E-CID in NR, and we get the following proposals:

Proposal 1: Include RTT/AOA measurements in the E-CID positioning for NR PCell and NR PSCell. FFS E-UTRA PSCell in case of NE-DC.

Proposal 2: No dedicated assistance data from LMF is required for E-CID measurement.

Proposal 3: For UL part of E-CID, support gNB Rx – Tx time difference and AOA/ZOA based on any uplink signals and channels, e.g., SRS or PRACH.

Proposal 4: For DL part of E-CID, support UE Rx – Tx time difference based on any downlink signals and channels, e.g., TRS or SS/PBCH block.

Proposal 5: Support the measurement reporting from UE and gNB to the LMF, respectively.

- From UE: UE Rx – Tx time difference.

- From gNB: gNB Rx – Tx time difference, AOA, and ZOA.