3GPP TSG-RAN WG2 Meeting #108	R2-1915499

Reno, USA, 18 – 22 November 2019

Agenda item:	6.9.3.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Failure Handling via CHO recovery

WID/SID:	NR_Mob_enh-Core - Release 16

Document for:	Discussion and Decision

# 1	Introduction

In RAN2#107bis, the following agreements were made:

Agreements

1.	Confirm the working assumption as an optional feature:

At RLF/HO failure/CHO failure, the UE performs cell selection and if the selected cell is a CHO candidate then the UE attempts CHO execution, otherwise re-establishment is performed.

If the CHO performed during failure handling procedure fails, the UE will perform re-establishment, i.e. we do not allow multiple attempts of CHO during failure case.

FFS on how to capture it in specification;

If UE doesn’t support this capability, it does re-establishment (just as now). Network can configure what UE does.

[1] provides a proposed realization of this agreement for TR36.331 which we will discuss in this paper.

# 2	Description of the proposed solution

Main aspect of the proposed realization can be read from section 5.3.7.3 which is already part of the re-establishment procedure.

Next change #### 5.3.7.3 Actions following cell selection while T311 is running Upon selecting a NR cell and the selected cell is one of the candidate cells in VarCHO-Config if attemptCHO is configured: 1> if this is the first cell selection procedure after failure: 2> stop timer T311; 2> apply the stored cho-RRCReconfig of the associated cell as specified in 5.3.5.3; Editor’s note: The definition of “first cell selection” needs to be clarified and we may define a variable to control it is the first time Upon selecting a suitable NR cell, the UE shall: 1> ensure having valid and up to date essential system information as specified in clause 5.2.2.2; 1> stop timer T311; 1> start timer T301; 1> if T390 is running: 2> stop timer T390 for all access categories; 2> perform the actions as specified in 5.3.14.4; 1> apply the default L1 parameter values as specified in corresponding physical layer specifications except for the parameters for which values are provided in SIB1; 1> apply the default MAC Cell Group configuration as specified in 9.2.2; 1> apply the CCCH configuration as specified in 9.1.1.2; 1> apply the timeAlignmentTimerCommon included in SIB1; 1> initiate transmission of the RRCReestablishmentRequest message in accordance with 5.3.7.4; NOTE: This procedure applies also if the UE returns to the source PCell. Upon selecting an inter-RAT cell, the UE shall: 1> perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure'.

For the sake of completeness, we also cite the initiation of the re-establishment procedure:

#### 5.3.7.2 Initiation The UE initiates the procedure when one of the following conditions is met: 1> upon detecting radio link failure of the MCG, in accordance with 5.3.10; or 1> upon re-configuration with sync failure of the MCG, in accordance with sub-clause 5.3.5.8.3; or 1> upon mobility from NR failure, in accordance with sub-clause 5.4.3.5; or 1> upon integrity check failure indication from lower layers concerning SRB1 or SRB2, except if the integrity check failure is detected on the RRCReestablishment message; or 1> upon an RRC connection reconfiguration failure, in accordance with sub-clause 5.3.5.8.2. Upon initiation of the procedure, the UE shall: 1> stop timer T310, if running; 1> stop timer T304, if running; 1> start timer T311; 1> suspend all RBs, except SRB0; 1> reset MAC; 1> release the MCG SCell(s), if configured; 1> release spCellConfig, if configured; 1> if MR-DC is configured: 2> perform MR-DC release, as specified in clause 5.3.5.10; 2> release p-NR-FR1, if configured; 2> release p-UE-FR1, if configured; 1> release delayBudgetReportingConfig, if configured, and stop timer T342, if running; 1> release overheatingAssistanceConfig, if configured, and stop timer T345, if running; 1> perform cell selection in accordance with the cell selection process as specified in TS 38.304 [20], clause 5.2.6.

We observe, that the proposed solution initiates re-establishment at RLF/HO failure/CHO failure just as today. Today’s re-establishment procedure is exit when the cell selection (which is already part of the re-establishment procedure) selects a CHO candidate. From our perspective this violates the agreement taken at RAN2#107bis.

Observation 1: The proposed running CR to 38.331 does not reflect the agreement on failure handling.

Whereas this might be considered just as a formal deviation, this also has a significant drawback. As we can read from the procedure above, all RBs are already suspended when cell selection is performed, which is unnecessary and removes a major part of the benefits of the feature. In our opinion, this is also not in line with the agreement, which proposes a pure cell selection first, and re-establishment (including RB suspension) only when the selected cell is not a CHO candidate.

Observation 2: Suspending the RBs before the cell selection removes major part of the benefits of executing CHO instead of full re-establishment.

# 3	Solution in line with RAN2#107bis agreement

A solution correctly reflecting the agreement would have to introduce an extra cell selection at RLF/HO failure/CHO failure, before re-establishment (see section 5.3.7) is initiated, i.e. re-establishment is no longer initiated directly at RLF/HO Failure/CHO Failure, but by the extra cell selection. This may lead to a slightly more elaborated selection, but it would rescue the benefit of keeping RBs alive. Cell selection does not have to be repeated during the re-establishment procedure, the results of the previous extra cell selection can be reused.

Proposal 1: The cell selection for the CHO failure handling is done before triggering re-establishment procedure.

Proposal 2:  RAN2 to specify the procedure illustrated in section 6 to correctly reflect the earlier RAN2 agreement.

More details can be found in the TP in section 6, where we show two options how to implement it.

# 4	Conclusion

This paper considered the issues associated with Stage 3 implementation of the CHO failure handling. The following observations and proposal have been made:

Observation 1: The proposed running CR to 38.331 does not reflect the agreement on failure handling.

Observation 2: Suspending the RBs before the cell selection removes major part of the benefits of executing CHO instead of full re-establishment.

Proposal 1: The cell selection for the CHO failure handling is done before triggering re-establishment procedure.

Proposal 2:  RAN2 to specify the procedure illustrated in section 6 to correctly reflect the earlier RAN2 agreement.

# 5	References

- R2-1914834 RRC running CR for NR mobility on CHO ( [107bis#54])3GPP TSG-RAN WG2 Meeting #108 Reno, USA, 18 – 22 November 2019

- R2-1915497 On RLF reporting for CHO and DAPS 3GPP TSG-RAN WG2 Meeting #108 Reno, USA, 18 – 22 November 2019

# 6	Text Proposals

## 6.1 Option 1

First change

### 5.3.7	RRC connection re-establishment

Text omitted

#### 5.3.7.2	Initiation

The UE initiates the procedure when one of the following conditions is met:

1>	upon detecting radio link failure of the MCG, in accordance with 5.3.10, and if conditions for CHO Recovery procedure as per 5.3.x are not met; or

1>	upon re-configuration with sync failure of the MCG, in accordance with sub-clause 5.3.5.8.3, and if conditions for CHO Recovery procedure as per 5.3.x are not met; or

1>	upon CHO recovery failure, in accordance with 5.3.x; or

1>	upon mobility from NR failure, in accordance with sub-clause 5.4.3.5; or

1>	upon integrity check failure indication from lower layers concerning SRB1 or SRB2, except if the integrity check failure is detected on the RRCReestablishment message; or

1>	upon an RRC connection reconfiguration failure, in accordance with sub-clause 5.3.5.8.2.

Upon initiation of the procedure, the UE shall:

1>	stop timer T310, if running;

1>	stop timer T304, if running;

1>	start timer T311;

1>	suspend all RBs, except SRB0;

1>	reset MAC;

1>	release the MCG SCell(s), if configured;

1>	release spCellConfig, if configured;

1>	if MR-DC is configured:

2>	perform MR-DC release, as specified in clause 5.3.5.10;

2>	release p-NR-FR1, if configured;

2>	release p-UE-FR1, if configured;

1>	release delayBudgetReportingConfig, if configured, and stop timer T342, if running;

1>	release overheatingAssistanceConfig, if configured, and stop timer T345, if running;

1>	perform cell selection in accordance with the cell selection process as specified in TS 38.304 [20], clause 5.2.6.

Next change

### 5.3.x	CHO recovery

#### 5.3.x.1	Initiation

The UE initiates the procedure when one of the following conditions is met:

1>	upon detecting radio link failure of the MCG, in accordance with 5.3.10, if  attemptCHO is configured and cho-ConfigList in VarCHO-Config is not empty; or

1>	upon re-configuration with sync failure of the MCG, in accordance with sub-clause 5.3.5.8.3, if attemptCHO is configured and cho-ConfigList in VarCHO-Config is not empty; or

Upon initiation of the procedure, the UE shall:

1> Start T311;

1> perform cell selection in accordance with the cell selection process as specified in TS 38.304 [20], clause 5.2.6.;

1> if the selected cell is one of the candidate cells included in VarCHO-Config;

2> Set cho-RecoveryFlag in VarCHO-Config to true;

2> Stop T311;

2> apply the stored cho-RRCReconfig associated to the selected cell and perform the actions as specified in 5.3.5.3;

1> else

2> Release cho-RecoveryFlag in VarCHO-Config;

2> Consider the CHO Recovery as failed;

2> perform RRC connection re-establishment, in accordance with sub-clause 5.3.7;

Next change

#### –	VarCHO-Config

The UE variable VarCHO-Config includes the accumulated configuration of the conditional handover configurations including the conditional handover execution condition and the stored target candidate cell RRCReconfiguration.

VarCHO-Config UE variable

-- ASN1START

-- TAG-VARCHO-CONFIG-START

VarCHO-Config ::=                    SEQUENCE {

cho-ConfigList			               CHO-ConfigToAddModList-r16		OPTIONAL,

cho-RecoveryFlag-r16				   ENUMERATED {true}				OPTIONAL

}

-- TAG-VARCHO-CONFIG-STOP

-- ASN1STOP

## 6.2 Option 2

First change

### 5.3.7	RRC connection re-establishment

Text omitted

#### 5.3.7.2	Initiation

The UE initiates the procedure when one of the following conditions is met:

1> upon triggering CHO recovery procedure, in accordance with 5.3.x; or

1>	upon integrity check failure indication from lower layers concerning SRB1 or SRB2, except if the integrity check failure is detected on the RRCReestablishment message; or

1>	upon an RRC connection reconfiguration failure, in accordance with sub-clause 5.3.5.8.2.

Upon initiation of the procedure, the UE shall:

1>	stop timer T310, if running;

1>	stop timer T304, if running;

1>	start timer T311;

1>	suspend all RBs, except SRB0;

1>	reset MAC;

1>	release the MCG SCell(s), if configured;

1>	release spCellConfig, if configured;

1>	if MR-DC is configured:

2>	perform MR-DC release, as specified in clause 5.3.5.10;

2>	release p-NR-FR1, if configured;

2>	release p-UE-FR1, if configured;

1>	release delayBudgetReportingConfig, if configured, and stop timer T342, if running;

1>	release overheatingAssistanceConfig, if configured, and stop timer T345, if running;

1>	perform cell selection in accordance with the cell selection process as specified in TS 38.304 [20], clause 5.2.6.

Next change

### 5.3.x	CHO recovery

#### 5.3.x.1	Initiation

The UE initiates the procedure when one of the following conditions is met:

1>	upon detecting radio link failure of the MCG, in accordance with 5.3.10; or

1>	upon re-configuration with sync failure of the MCG, in accordance with sub-clause 5.3.5.8.3; or

Upon initiation of the procedure, the UE shall:

1>	if UE does not support CHO recovery, or CHORecovery is not configured, or if VarCHO-Config is empty, or CHORecoveryFlag is set to 1;

2> Remove CHORecoveryFlag to 0;

2>	perform RRC connection re-establishment, in accordance with sub-clause 5.3.7;

1>	else

2>	Start T311;

2> perform cell selection in accordance with the cell selection process as specified in TS 38.304 [20], clause 5.2.6.;

2> if the selected cell is one of the candidate cells in VarCHO-Config:

3> Set CHORecoveryFlag to 1;

3> Stop T311;

3> apply the stored cho-RRCReconfig associated to the selected cell and perform the actions as specified in 5.3.5.3;

2> else

3> Remove CHORecoveryFlag to 0;

3> perform RRC connection re-establishment, in accordance with sub-clause 5.3.7;

Next change

#### –	VarCHO-Config

The UE variable VarCHO-Config includes the accumulated configuration of the conditional handover configurations including the conditional handover execution condition and the stored target candidate cell RRCReconfiguration.

VarCHO-Config UE variable

-- ASN1START

-- TAG-VARCHO-CONFIG-START

VarCHO-Config ::=                    SEQUENCE {

cho-ConfigList			               CHO-ConfigToAddModList-r16		OPTIONAL,

cho-RecoveryFlag-r16				   ENUMERATED {true}				OPTIONAL

}

-- TAG-VARCHO-CONFIG-STOP

-- ASN1STOP