3GPP TSG-RAN WG2 Meeting #109-e	R2-2000301

# Electronic meeting, 24th Feb - 6th Mar 2020

Source:	vivo

Title:	Fast recovery failure indication

Agenda item:	6.10.5

Document for:	Discussion and Decision

1	Introduction

At RAN2#105bis meeting, RAN2 has discussed use cases for MCG fast recovery and agreed that:

Agreements for MCG fast recovery:

0	MCG fast recovery targets all MRDC architecture options

1:	When MCG failure occurs, UE follows SCG failure-like procedure:

i.	UE does not trigger RRC connection re-establishment.

ii.	UE triggers an MCG failure procedure in which a failure information message is transmitted to the network via SCG.

2: 	MCG fast recovery targets the following use cases MCG leg RLF

FFS: Other uses cases. Can consider in future whether the mechanism can be also be applied in the case of other MCG failures.

3	MCG fast recovery can only be triggered after AS security has been activated and the SRB2 and at least one DRB have been setup

4	MCG failure indication should include:

i.	Available measurement results of MCG

ii.	MCG link failure cause

iii.	Available measurement results of SCG

iv.	Available measurement results of non-serving cells

5: 	For MCG failure indication, new RRC message in introduced, e.g. MCGFailureInformation.

6: 	SCG leg of the split SRB1 can be used for MCG fast recovery.

FFS: If configured, SRB3 can be used for MCG fast recovery. Priority is to complete the solution based on split SRB1

7:	New SRB is not introduced for MCG fast recovery.

At RAN2#106 meeting RAN2 agreed:

Agreements

FFS Whether a guard timer is needed for the MCG failure indication message

1	Once the MCG failure indication is triggered, the UE shall:

-	transmit the MCG failure indication;

−	suspend MCG transmission for all SRBs and DRBs;

−	reset MCG-MAC;

−	maintain the current measurement configurations from both the MN and the SN, and continue measurements based on configuration from the MN and the SN if possible.

FFS whether switch the primaryPath to SCG is needed

2	If SCG failure is detected while MCG is suspended then initiate RRC re-establishment procedure

3	Upon receiving the MCG failure indication, the MN sends reconfiguration with sync or RRC Release to the UE via SRB1.

4	Upon reception of reconfig with sync the UE resumes MCG transmission if suspended

At RAN2#107 meeting RAN2 agreed:

Agreements

1: Upon sending a MCG failure indication, UE starts a timer.

2: Upon resumption of MCG, UE stops the timer.

3: Upon expiry of the timer, UE initiates RRC connection re-establishment procedure.

4: Network can configure the timer value (no infinite value)

This contribution further discusses some remaining issues related to fast recovery, such as, UE behaviour if such fast recovery fails.

2	Discussion

As described in [1], fast recovery of MCG link in case of MR-DC is to utilize the SCG link and split SRBs for recovery during MCG failure while operating under MR-DC. For example as depicted in figure 1, there are basically 3 states for fast recovery:

- UE experiences MCG link failure

- UE has split SRB1 connection on SCG link, UE uses the SCG link to trigger MCG link recovery.

- If fast recovery procedure succeeds, the UE would recover MCG link connection.

Figure 1: Fast recovery of MCG link

For above basic states of fast recovery procedure of MCG link, it would be necessary to answer what should be UE behaviour if fast recovery fails, e.g. at guard timer expiry or SCG failure occurs?

After triggering MCG failure indication procedure, UE will expect a fast recovery RRC reconfiguration message from the network. If UE receives the fast recovery RRC reconfiguration message before the guard timer T expires, UE can send the fast recovery RRC reconfiguration complete and the fast recovery would succeed.

After triggering fast recovery procedure, if the procedure succeeds, UE will recover the MCG link. But if the fast recovery timer expires and UE did not receive fast recovery RRC reconfiguration message the fast recovery can be considered as failed.

The fast recovery failure may be due to the following potential reasons:

- Guard timer expiry

- During fast recovery procedure SCG failure happens

- Integrity check failure on SCG link

- UE could not apply the received fast recovery RRC reconfiguration

If fast recovery fails, for example due to SCG failure during fast recovery, UE would trigger re/establishment. The re/establishment may be done at one of the following nodes:

- Source MN: MN where MCG failure occurs

- Source SN: SN where fast recovery was triggered and failed

- New MN: different from source MN

- New SN: different from source SN

Observation 1: After fast recovery failure, UE may perform re/establishment to:

- Source MN: MN where MCG failure occurs

- Source SN: SN where fast recovery was triggered and failed

- New MN: different from source MN

- New SN: different from source SN

At re/establishment, UE would send a fast recovery failure information to network. This failure indication is SCG/MCGFailureInformation alike, but the cause cannot be considered as simple SCGFailureInformation or MCGFailureInformation.

For SCGFailureInformation a simple SN reestablishment or release or change is sufficient and the failure underlying cause can be restricted to only SN problem. As for MCGFailureInformation, in case of MCG RLF, the failure cause only involve MN. But for fast recovery failure the failure may be related to a joint MN and SN problem. For example, in case of fast recovery failure due to SCG failure, UE experiences consecutive failure at MN then at SN. Thus, fast recovery failure information should be different from SCGFailureInformation and MCGFailureInformation, The fast recovery failure information may include:

- MCGFailureInformation,

- SCGFailureInformation

- Re/establishment node related information, e.g., CGI, radio condition at re/establishment.

Therefore,

Proposal 1: If fast recovery failure occurs, after re-establishment UE should send a fast recovery failure message to network. The fast recovery failure message may include:

- MCGFailureInformation,

- SCGFailureInformation

- Re/establishment node related information, e.g., radio condition at re/establishment.

At reception of fast recovery failure message, after re/establishment from fast recovery failure, the receiving node should send a fast recovery failure indication to source MN/SN, like legacy RLF indication between two nodes.

Proposal 2: After re/establishment from fast recovery failure, the node to which UE re/establishes should send a fast recovery failure indication to source MN (where MCG failure occurs) and/or source SN (where fast recovery failure occurs).

Proposal 3: if proposal 3 is accepted, RAN2 to send an Ls to RAN3 to inform about RAN2 decision.

3	Conclusion

This contribution discusses remaining issues related to fast recovery. The contribution concludes with the following observation

Observation 1: After fast recovery failure, UE may perform re/establishment to:

- Source MN: MN where MCG failure occurs

- Source SN: SN where fast recovery was triggered and failed

- New MN: different from source MN

- New SN: different from source SN

The contribution also concludes with the following proposals:

Proposal 1: After fast recovery triggering, if UE receives for fast recovery RRC reconfiguration message before guard timer expires, the fast recovery is considered successful. Otherwise, the fast recovery is considered as failure.

Proposal 2: If fast recovery failure occurs, after re-establishment UE should send a fast recovery failure message to network. The fast recovery failure message may include:

- MCGFailureInformation,

- SCGFailureInformation

- Re/establishment node related information, e.g., radio condition at re/establishment.

Proposal 4: After re/establishment from fast recovery failure, the node to which UE re/establishes should send a fast recovery failure indication to source MN (where MCG failure occurs) and/or source SN (where fast recovery failure occurs).

Proposal 3: if proposal 3 is accepted, RAN2 to send an Ls to RAN3 to inform about RAN2 decision.

4	References

- RP-182076 “WID on Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements”, Ericsson, Nokia, Nokia Shanghai Bell, Huawei

- 37.340