3GPP TSG-RAN WG2 Meeting #109	electronic								    R2-2000386

E-Meeting, 24th Feb – 6th Mar, 2020

Source: 	vivo

Title:  	SR cancellation due to the truncated BFR MAC CE

Agenda Item:	6.16.4

Document for:	Discussion and Decision

Introduction

According to the email discussions on the running MAC CR [1] and the SCell BFR MAC CE [2], the SCell BFR MAC CE is of variable size (e.g. a truncated format) and the BFR due to the beam failure of each SCell can trigger an SR. In this contribution, we discuss how the UE cancel the pending SR triggered by the SCell BFR.

Discussion

SR cancellation

According to the SCell BFR MAC CE discussion [2], companies consider to introduce a new variable-size MAC CE for the SCell BFR report. When the number of configured SCells is more than 7, the size of the SCell BFR MAC can be from 7 bytes to 38 bytes. According to the MAC running CR for eMIMO, the SCell BFR report can use any type of uplink grant (e.g. including configured uplink grant, dynamic uplink grant and the grant provided in RAR). Then it is quite possible that an uplink grant with a slightly small TBS may not be able to include the beam failure information of all failed SCells. Then the SCell BFR MAC CE only only includes partial beam failure information of the failed SCells.

Observation 1: The size of the SCell BFR MAC can be from 7 bytes to 38 bytes.

Observation 2: An uplink grant with a slightly small TBS may not be able to include the beam failure information of all failed SCells in the SCell BFR MAC CE.

Figure 1: SR triggered by SCell BFR

According to the MAC running CR [1] of the eMIMO WI, when a beam failure of a SCell happens, the beam failure of this SCell triggers an independent BFR. And then the BFR of this SCell triggers a pending SR. However, all the pending SRs are cancelled when a SCell BFR MAC CE is transmitted. If the transmission of a SCell BFR MAC CE (which does not include the fail information of SCell 3) cancels all pending BFR SR(s), the transmission of the beam failure information of SCell 3 will be delayed until an uplink grant of new transmission is provided to the UE. Thus a reasonable way for the SR cancellation would be that the transmission of the beam failure information of a certain SCell only cancels the SR triggered by this SCell. Then the pending SR triggered by the pending SCell BFR information which is not reported can still initiate the SR transmission to require more uplink resource from the gNB.

Observation 3: If a SCell BFR MAC CE which does not include the failure information of all SCells cancels all pending BFR SR(s), the transmission of the not-reported failure information of the SCells will be delayed.

Proposal 1: The transmission of the beam failure information of a certain SCell only cancels the pending BFR SR triggered by this SCell.

Conclusions

Based on the analysis given above, we have the following observations proposals：

Observation 1: The size of the SCell BFR MAC can be from 7 bytes to 38 bytes.

Observation 2: An uplink grant with a slightly small TBS may not be able to include the beam failure information of all failed SCells in the SCell BFR MAC CE.

Observation 3: If a SCell BFR MAC CE which does not include the failure information of all SCells cancels all pending BFR SR(s), the transmission of the not-reported failure information of the SCells will be delayed.

Proposal 1: The transmission of the beam failure information of a certain SCell only cancels the pending BFR SR triggered by this SCell.

Reference

[1] R2-20xxxxx, Samsung, “MAC Running CR for NR eMIMO”

[2] R2-20xxxxx, Samsung, “Report of email discussion [108#70] [NR-eMIMO]: BFR MAC CE”