3GPP TSG-RAN WG2 Meeting #109-e	R2-2000483

Electronic meeting, 24th February - 6th March, 2020

Agenda item:       6.1.4

Source: 		Qualcomm Incorporated, Samsung

Title: 	(TP for NR_IAB BL CR to TS 38.321) MAC CE for guard symbols

Document for:	Discussion

# 1	Introduction

This paper discusses MAC CE to be used on IAB BH link for exchange of information about guard symbols based on RAN1 LS on physical layer configuration parameters [1][2].

# 2	Discussion

According to [1] and [2], the IAB-DU and IAB-donor DU should be able to send a DL MAC CE to the IAB-MT to inform the IAB-MT about the number of guard symbols provided at the slot edge (beginning and end of slot). The IAB-MT should further be able to send an UL MAC CE to the IAB-DU and/or IAB-donor DU to indicate the desired number of guard symbols to be provided at the slot edge.

In these MAC CEs, the number of guard symbols should be indicated for each of the following eight switching transitions:

-------------------------------------

MT to DU | DL Tx | UL Rx |

-------------------------------------

DL Rx         |          |            |

----------------------------------

UL Tx         |           |            |

-------------------------------------

-------------------------------------

DU to MT | DL Rx | UL Tx |

-------------------------------------

DL Tx         |           |           |

-------------------------------------

UL Rx         |           |           |

------------------------------------

The MAC CE should allocate 3bits to signal a range of [0, 4] guard symbols. Each MAC CE should further include the guard-symbol sub-carrier spacing as the reference with values of 15kHz, 30kHz, 60kHz, and 120kHz.

The TP for BL CR to TS 38.321 includes the specification of these MAC CEs.

Proposal 1: Include TP below into TS 38.321.

# 3	Conclusion

This paper discusses MAC CE to be used on the IAB BH link for exchange of information on guard symbols based on RAN1 LS on physical layer configuration parameters [1][2]. The following proposal has been made:

Proposal 1: Include TP below into TS 38.321.

# 4	References

[1]	R1-1913674: Updated consolidated parameter list for Rel-16 NR, 3GPP TSG RAN WG1 Meeting #99, Reno, USA, November 18th -22nd, 2019

[2]	R1-1913675: LS on updated Rel-16 LTE and NR parameter lists, 3GPP TSG RAN WG1 Meeting #99, Reno, USA, November 18th -22nd, 2019

# 5	TP for NR_IAB BL CR to TS 38.321

START OF CHANGE

# 5		MAC procedures

## 5.x		Guard symbols for IAB

For IAB operation, the MAC entity on the IAB-DU or IAB-donor DU should reserve a sufficient number of symbols at the beginning and the end of each slot to allow the child IAB-node to switch operation between its upstream and downstream links. The MAC entity on the IAB-DU or IAB-donor DU informs the child node about the number of guard symbols it provides via the DL Guard Symbol MAC CE. The IAB-MT on the child node can inform the IAB-DU or IAB-donor DU about the number of guard symbols desired via the UL Guard Symbol MAC CE.

A separate value for the number of guard symbols is specified for each of the following eight switching scenarios (see Table 5.x-1). Further details are provided in TS 38.213 [zz], clause 14.

Table 5.x-1: Switching scenarios and relevant guard symbols

| Switching scenario |  | Field for number of guard symbols in MAC CE |
| --- | --- | --- |
| IAB-MT operation to IAB-DU operation | DL Rx to DL Tx | NmbGS1 |
|  | DL Rx to UL Rx | NmbGS2 |
|  | UL Tx to DL Tx | NmbGS3 |
|  | UL Tx to UL Rx | NmbGS4 |
| IAB-DU operation to IAB-MT operation | DL Rx to DL Tx | NmbGS5 |
|  | DL Rx to UL Rx | NmbGS6 |
|  | UL Tx to DL Tx | NmbGS7 |
|  | UL Tx to UL Rx | NmbGS8 |

NEXT CHANGE

# 6	Protocol Data Units, formats and parameters

## 6.1		Protocol Data Units

### 6.1.1		General

Skip>>>>

### 6.1.2		MAC PDU (DL-SCH and UL-SCH except transparent MAC and Random Access Response)

Skip>>>>

### 6.1.3		MAC Control Elements (CEs)

Skip>>>>

#### 6.1.3.x	Guard Symbols MAC CE

The Guard Symbols MAC CE is identified by the MAC subheader LCIDs as specified in Table 6.2.1-1 for DL-SCH and in Table 6.2.1-2 for UL-SCH.

It has fixed size and consists of four octets defined as follows (Figure 6.1.3.x-1):

-     R: Reserved bit, set to 0;

-	Sub-carrier spacing (SCS): This field indicates the subcarrier spacing used as reference for the guard spacing. The length of this field is 2bits. The values for the SCS field are shown in Table 6.1.3.x-2.

-	Number of Guard Symbols (NmbGSi): This field indicates the number of guard symbols for the switching scenario shown in Table 5.x-1. The number of guard symbols can take values within the range of 0..4. Higher values 5-7 are reserved.

Figure 6.1.3.x-1: Guard Symbol MAC CE

Table 6.1.3.x-2: Subcarrier spacing for Guard Symbols MAC CE

| Subcarrier spacing | SCS value |
| --- | --- |
| 15kHz | 00 |
| 30kHz | 01 |
| 60kHz | 10 |
| 120kHz | 11 |

Skip>>>>

## 6.2		Formats and parameters

### 6.2.1		MAC subheader for DL-SCH and UL-SCH

Skip>>>>

Table 6.2.1-1 Values of LCID for DL-SCH

| Index | LCID values |
| --- | --- |
| 0 | CCCH |
| 1–32 | Identity of the logical channel |
| 33-46-y | Reserved |
| … | … |
| xx | Guard Symbols |
| … | … |
| 47 | Recommended bit rate |
| 48 | SP ZP CSI-RS Resource Set Activation/Deactivation |
| 49 | PUCCH spatial relation Activation/Deactivation |
| 50 | SP SRS Activation/Deactivation |
| 51 | SP CSI reporting on PUCCH Activation/Deactivation |
| 52 | TCI State Indication for UE-specific PDCCH |
| 53 | TCI States Activation/Deactivation for UE-specific PDSCH |
| 54 | Aperiodic CSI Trigger State Subselection |
| 55 | SP CSI-RS/CSI-IM Resource Set Activation/Deactivation |
| 56 | Duplication Activation/Deactivation |
| 57 | SCell Activation/Deactivation (four octets) |
| 58 | SCell Activation/Deactivation (one octet) |
| 59 | Long DRX Command |
| 60 | DRX Command |
| 61 | Timing Advance Command |
| 62 | UE Contention Resolution Identity |
| 63 | Padding |

Table 6.2.1-2 Values of LCID for UL-SCH

| Index | LCID values |
| --- | --- |
| 0 | CCCH of size 64 bits (referred to as "CCCH1" in TS 38.331 [5]) |
| 1–32 | Identity of the logical channel |
| 33–51-y | Reserved |
| … | … |
| xx | Guard Symbols |
| … | … |
| 52 | CCCH of size 48 bits (referred to as "CCCH" in TS 38.331 [5]) |
| 53 | Recommended bit rate query |
| 54 | Multiple Entry PHR (four octets Ci) |
| 55 | Configured Grant Confirmation |
| 56 | Multiple Entry PHR (one octet Ci) |
| 57 | Single Entry PHR |
| 58 | C-RNTI |
| 59 | Short Truncated BSR |
| 60 | Long Truncated BSR |
| 61 | Short BSR |
| 62 | Long BSR |
| 63 | Padding |

END OF CHANGE