3GPP TSG-RAN WG2 Meeting #109-e	R2-2000533

# Online, 24 February–6 March 2020

Agenda item:	6.0.3

Source:	Samsung

Title:	LCID extension for Rel-16

Document for:	Discussion and Agreement

# 1	Introduction

For Rel-15, RAN2 reserved quite many LCID (logical channel identifier) values for the future use (i.e. 14 values for DL (33–46) and 19 values for UL (33–51)). However, as Rel-16 introduces many features, each new feature requires several MAC CEs, which consumes the remaining LCID space.

The contribution provides the current status of LCID space, and discusses how to extend the LCID space for Rel-16 onwards.

# 2	Discussion

The following Tables 1 and 2 show the list of new MAC CEs to be introduced for Rel-16 across WIs.

Table 1 List of new DL MAC CEs

| WI | No | Purpose | Remarks |
| --- | --- | --- | --- |
| IAB | 1 | Extended logical channel ID field | R2-1915256 |
| IAB | 2 | Timing Delta MAC CE | R2-1915256; fixed (2B) |
| IAB | 3 | (ProvidedGuardSymbols from RAN1) | variable? |
| IIoT | 4 | Duplication RLC Activation/Deactivation MAC CE | [108#12]; fixed (1B) |
| 2-step RACH | 5 | Absolute Timing Advance Command MAC CE | [108#82]; fixed (2B) |
| eMIMO | 6 | Enhanced TCI States Activation/Deactivation for UE-specific PDSCH | [108#69]; variable |
| eMIMO | 7 | Extended PUCCH spatial relation Activation/Deactivation MAC CE | [108#68]; fixed (3B) |
| eMIMO | 8 | Group-based PUCCH spatial relation Activation/Deactivation MAC CE | [108#68]; fixed (2B) |
| eMIMO | 9 | AP SRS spatial relation Indication MAC CE | [108#68]; variable |
| eMIMO | 10 | SRS Pathloss Reference RS Activation/Deactivation MAC CE | [108#68]; fixed (3B) |
| eMIMO | 11 | PUSCH Pathloss Reference RS Activation/Deactivation MAC CE | [108#68]; fixed (3B) |
| eMIMO | 12 | CC list-based TCI States Activation/Deactivation for UE-specific PDSCH MAC CE | [108#68]; variable |
| eMIMO | 13 | CC list-based TCI State Indication for UE-specific PDCCH MAC CE | [108#68]; fixed (2B) |
| eMIMO | 14 | CC list-based SRS Activation/Deactivation MAC CE | [108#68]; TBD |
| eMIMO | 15 | (maybe one more according to RAN1 input) | - |
| Others | - | (Recommended bit rate from SA4) | R2-1916516; none or one LCID |
| NR-U |  | None | [108#75] |
| V2X |  | None | [108#100] |
| RACS |  | None | - |
| NTN |  | None | Study item |
| NR Pos |  | None | - |
| NR mob |  | None | - |
| DCCAe |  | None | [108#56] |
| Power saving |  | None | [108#78] |
| SON/MDT |  | None | [108#93] |
| SRVCC |  | None | - |
| CLI |  | None | - |
| PRN |  | None | - |
| URLLC |  | None | [108#111] |

Table 2 List of new UL MAC CEs

| WI | No | Purpose | Remarks |
| --- | --- | --- | --- |
| IAB | 1 | Extended logical channel ID field | R2-1915256 |
| IAB | 2 | Pre-emptive BSR MAC CE | R2-1915256; variable; Long format only (TBD) |
| IAB | 3 | (DesiredGuardSymbols from RAN1) | variable? |
| NR-U | 4 | LBT failure MAC CE | [108#75]; fixed (4B); no 1B format |
| V2X | 5 | Sidelink Configured Grant Confirmation | [108#100]; fixed (1B) |
| V2X | 6 | Truncated Sidelink BSR MAC CE | [108#100]; variable |
| V2X | 7 | Sidelink BSR MAC CE | [108#100]; variable |
| IIoT | 8 | Multiple Entry Configured Grant Confirmation | [108#12]; fixed (4B); no 1B format (TBD) |
| eMIMO | 9 | SCell BFR MAC CE (four octets Ci) | [108#70]; variable |
| eMIMO | 10 | SCell BFR MAC CE (one octet Ci) | [108#70]; variable |
| Others | - | (PHR enhancement from RAN4) | TBD; to include PMPRc; none or three |
| Others | - | (Recommended bit rate from SA4) | TBD; R2-1916516; none or one |
| RACS |  | None | - |
| NTN |  | None | Study item |
| NR Pos |  | None | - |
| NR mob |  | None | - |
| DCCAe |  | None | [108#56] |
| Power saving |  | None | [108#78] |
| SON/MDT |  | None | [108#93] |
| 2-step RACH |  | None | [108#82] |
| SRVCC |  | None | - |
| CLI |  | None | - |
| PRN |  | None | - |
| URLLC |  | None | [108#111] |

From the tables, it can be observed that at least 15 values for DL and 10 values for UL would be required for Rel-16 new MAC CEs. Note that 14 values for DL (33–46) and 19 values for UL (33–51) are reserved from Rel-15, so for the DL, the required number already exceeds the reserved values.

Observation 1: For DL, 15+ values are required for Rel-16 while 14 values are reserved (i.e. already exceeded).

Observation 2: For UL, 10+ values are required for Rel-16 while 19 values are reserved.

Hence, it is inevitable to extend the LCID space for MAC CEs from the observations above. Since the NR keeps evolving, it would be good to extend the LCID space for both DL and UL.

Proposal 1: To extend the LCID space for both DL and UL.

There might be several ways to extend the LCID space as proposed last November [1][2][3][4][5][6], but one of the simplest ways would be to re-use IAB running MAC CR [7]. That is, IAB has already defined the eLCID header format which extends maximum LCID value from 63 to 216 + 63 (= 65,599), as shown in Figure 6.1.2-1 and Table 6.2.1-1a/2a below. Then, from the 'Identity of the logical channel' values (which are 65,408 values) in Table 6.2.1-1a/2a, we may use 256 values from the end, which is shown in Table 3 below (highlighted in green).

Figure 6.1.2-1: R/F/LCID/(eLCID)/L MAC subheader with 8-bit L field [7]

Table 6.2.1-1a/2a Values of eLCID for DL/UL-SCH [7]

| Index | LCID values |
| --- | --- |
| 64 to (216 – 65) | Identity of the logical channel |
| (216 – 64) to (216 + 63) | Reserved |

Table 3 Possible changes of value range from eLCID space

| Index | LCID values |
| --- | --- |
| 64 to (216 – 321) | Identity of the logical channel |
| (216 – 320) to (216 – 65) | For additional MAC CEs (256 values) |
| (216 – 64) to (216 + 63) | Reserved |

Proposal 2-1: To re-use the IAB solution to extend the LCIDs for additional MAC CEs.

Proposal 2-2: To reserve 256 values from the value for 'Identity of the logical channel' for MAC CEs i.e. from (216 – 320) to (216 – 65).

If the proposals above are agreeable, there would be two reserved LCID spaces for MAC CEs i.e. legacy reserved value range (set1) and new reserved value range (set2), as shown in Table 4 below. It should be noted that set2 requires 2 more octets in the MAC subheader in the proposal. Therefore, RAN2 should also determine which spaces (i.e. set1 or set2) to be used for each new MAC CE listed in Tables 1 and 2 above. This can be determined by each WI rapporteur, and it is proposed to set an email discussion to gather input from each WI rapporteur for their MAC CE(s). After having them, actual LCID for each MAC CE can be assigned, and each running CR can be updated accordingly.

Table 4 LCID value range for MAC CEs

|  | DL | UL |
| --- | --- | --- |
| Set1 (legacy reserved values) | 34 to 46 (13 values; value 33 is used for eLCID) | (216 – 320) to (216 – 65) (256 values) |
| Set2 (new reserved values) | 34 to 51 (18 values; value 33 is used for eLCID) | (216 – 320) to (216 – 65) (256 values) |

Observation 3: Set2 requires 2 more octets in the MAC subheader.

Proposal 3: To set up an email discussion to gather input from each WI rapporteur for their MAC CE(s) (set1 or set2). Based on the input, we could assign LCID for each MAC CE.

# 3	Conclusion

Observation 1: For DL, 15+ values are required for Rel-16 while 14 values are reserved (i.e. already exceeded).

Observation 2: For UL, 10+ values are required for Rel-16 while 19 values are reserved.

Proposal 1: To extend the LCID space for both DL and UL.

Proposal 2-1: To re-use the IAB solution to extend the LCIDs for additional MAC CEs.

Proposal 2-2: To reserve 256 values from the value for 'Identity of the logical channel' for MAC CEs i.e. from (216 – 320) to (216 – 65).

Observation 3: Set2 requires 2 more octets in the MAC subheader.

Proposal 3: To set up an email discussion to gather input from each WI rapporteur for their MAC CE(s) (set1 or set2). Based on the input, we could assign LCID for each MAC CE.

# 4	References

[1]	R2-1916135	Consideration on remaining FFS of LCID extension	LG

[2]	R2-1915343	Remaining issues with extending LCID space on the backhaul	Samsung

[3]	R2-1915323	eLCID space and handling of IAB specific MAC CEs	Nokia

[4]	R2-1915118	Consideration on the Extended LCID in IAB	ZTE

[5]	R2-1915808	Extended MAC CE ID	Qualcomm

[6]	R2-1915914	Extension of the LCID	LG

[7]	R2-1915256	Running CR to 38.321 on Integrated Access and Backhaul for NR	Samsung