3GPP TSG-RAN WG2 Meeting #109e	DRAFT R2-2000572

Elbonia, 24 February – 6 March 2020

Title:	[DRAFT] Reply LS on Enhancements to QoS Handling for V2X Communication Over Uu Reference Point

Response to:	(S2-2001675)

Release:	Release 16

Work Item:	5G_V2X_NRSL-Core

Source:	Nokia [TSG RAN WG2]

To:	TSG SA WG2

Cc:	TSG RAN WG3

Contact Person:

#### Name:	Benoist Sébire

E-mail Address:	benoist.sebire@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 would like to thank SA2 for their reply LS on Enhancements to QoS Handling for V2X Communication Over Uu Reference Point. RAN2 has discussed the notification mechanism and noted the following: “Further, if the application in the network knows that the UE does not need the NAS signalling, then the PCF can disable this NAS signaling.”

To ensure that the network always makes the right decision, RAN2 believes that UE signalling cannot be avoided: the UE must tell the network whether the NAS notification is required. Without such an explicit indication, the network might either be too conservative and not always notify the UE when required, or signal more notifications than required and increase overhead unnecessarily. To limit the overall complexity, RAN2 would prefer that NAS signalling is used for such an indication.

2. Actions:

To SA2 group.

ACTION: 	RAN2 respectfully asks SA2 to take the above preference into account.

3. Date of Next TSG-RAN WG2 Meetings:

3GPP RAN2#109bis		04 – 20 April 2020			Sapporo, Japan

3GPP RAN2#110		25 – 30 May 2020			Athens, Greece