3GPP TSG-RAN WG2 Meeting #109-e	 R2-2000681

# eMeeting, 24th – 28th, February 2020

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1450 | rev | - | Current version: | 15.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on reporting of uplink TX direct current |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020/02/24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current text for reporting UL TX direct current is specified as following: 2> if the RRCReconfiguration includes the masterCellGroup containing the reportUplinkTxDirectCurrent; or 2> if the RRCReconfiguration includes the secondaryCellGroup containing the reportUplinkTxDirectCurrent: 3> include the uplinkTxDirectCurrentList for each serving cell with UL; 3> if UE is configured with SUL carrier: 4> include uplinkDirectCurrentBWP-SUL for each serving cell with SUL It seems that, in MR-DC, the UE shall report the TX current for all MCG and SCG serving cells no matter the enquiry is from MN or SN. This is incorrect in (NG)EN-DC and NE-DC, as the TX current reporting is for NR serving cell, not for LTE serving cell. It is also strange in NR-DC, the MN or SN may receive this reporting while it does not send the enquiry. Note that the above text is executed for both inner and outer RRCReconfigurationComplete in NR-DC. The UE will include all the information in both RRCReconfigurationComplete if either MN or SN send the enquiry. We believe that this is not the intended behavior. We propose to clarify that the UE including the TX current reporting only for the corresponding MCG or SCG serving cells. In addition, the description about “if UE is configured with SUL carrier” is unclear. The SUL configuration is per cell. We think that correct behaviour is that the UE including the SUL TX current reporting for a cell if the cell is configured with SUL. This also need to be clarified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In 5.3.5.3, for the content in RRC Reconfiguration Complete, clarify that the UE only reporting TX current for corresponding MCG or SCG serving cells depending on whether the enquiry (reportUplinkTxDirectCurrent) is included in master cell group or secondary cell group. In addition, clarify that the TX current reporting for SUL carrier is based on the per cell configuration. - In 5.3.5.3, for the content in RRC Resume Complete, clarify that the TX current reporting for SUL carrier is based on the per cell configuration. Impact analysis Impacted 5G architecture options: Standalone, EN-DC, NGEN-DC, NE-DC, NR-DC Impacted functionality: Uplink TX direct current reporting in RRC Reconfiguration Complete and Resume Complete Inter-operability: If the UE implements the CR but the network does not or vice-versa, the UE and NW may have different understanding on whehter the TX dirrect current reporting of a serving cell is included or not. The consequences if not approved remain. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In (NG)EN-DC or NE-DC, the UE is unable to report UL TX current for LTE serving cell but the specificaion requries UE to do this. In NR-DC, the NW may receive unexpected TX current reporting in the Reconfiguration Complete message. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.3, 5.3.13.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st change

#### 5.3.5.3	Reception of an RRCReconfiguration by the UE

<Skip unrelated Part>

1>	set the content of the RRCReconfigurationComplete message as follows:

2>	if the RRCReconfiguration includes the masterCellGroup containing the reportUplinkTxDirectCurrent:

3>	include the uplinkTxDirectCurrentList for each MCG serving cell with UL;

3>	include uplinkDirectCurrentBWP-SUL for each MCG serving cell configured with SUL carrier, if any, within the uplinkTxDirectCurrentList;

2>	if the RRCReconfiguration includes the secondaryCellGroup containing the reportUplinkTxDirectCurrent:

3>	include the uplinkTxDirectCurrentList for each SCG serving cell with UL;

3>	include uplinkDirectCurrentBWP-SUL for each SCG serving cell configured with SUL carrier, if any, within the uplinkTxDirectCurrentList;

2>	if the RRCReconfiguration message includes the mrdc-SecondaryCellGroupConfig with mrdc-SecondaryCellGroup set to eutra-SCG:

3>	include in the eutra-SCG-Response the E-UTRA RRCConnectionReconfigurationComplete message in accordance with TS 36.331 [10] clause 5.3.5.3;

2> if the RRCReconfiguration message includes the mrdc-SecondaryCellGroupConfig with mrdc-SecondaryCellGroup set to nr-SCG:

3>	include in the nr-SCG-Response the RRCReconfigurationComplete message;

<Skip unrelated Part>

2nd change

#### 5.3.13.4	Reception of the RRCResume by the UE

<Skip unrelated Part>

1>	set the content of the of RRCResumeComplete message as follows:

2>	if the upper layer provides NAS PDU, set the dedicatedNAS-Message to include the information received from upper layers;

2>	if the upper layer provides a PLMN, set the selectedPLMN-Identity to PLMN selected by upper layers (TS 24.501 [23]) from the PLMN(s) included in the plmn-IdentityList in SIB1;

2>	if the masterCellGroup contains the reportUplinkTxDirectCurrent:

3>	include the uplinkTxDirectCurrentList for each MCG serving cell with UL;

3>	include uplinkDirectCurrentBWP-SUL for each MCG serving cell configured with SUL carrier, if any, within the uplinkTxDirectCurrentList;

1>	submit the RRCResumeComplete message to lower layers for transmission;

1>	the procedure ends.