3GPP TSG-RAN WG2 #109e	R2-2000796

Electronic meeting, 24th February – 6th March, 2020

Title:		          [draft] LS on PHY and MAC interaction to support intra-UE prioritization

Release:	Rel-16

Work Item:	NR_IIOT

Source:	Ericsson [to be RAN2]

To:	RAN1

Contact Person:

#### Name:	Zhenhua Zou

Email Address:	zhenhua.zou@ericsson.com

1. Overall Description:

RAN2 would like RAN1 to consider the following information and RAN2 agreements in their work.

RAN2 has further discussed intra-UE data/data prioritization involving configured grants (CG) (i.e., CG versus dynamic grant (DG), and CG versus CG). Based on current RAN2 agreements (reflected in MAC Running CR [1]), MAC would in some cases generate two subsequent MAC PDUs, first for a DG, second for an overlapping CG and pass both of them to PHY. MAC decision is based on agreed “priority value of an uplink grant (UL-SCH resource) is the highest priority of the LCHs that is multiplexed or can be multiplexed in MAC PDU, taking into account LCH restrictions and data availability” – which may be independent of indicated PHY-priority for CG and DG. On the other hand, based on current TS 38.214, PHY would always prioritize the PDU corresponding to DG if both DG and CG has the same PHY-priority. Such PHY behavior would result in conflict with MAC decision. To solve this conflict, RAN2 has discussed and agreed that

- In the case when MAC generates two consecutive MAC PDUs, the second PDU has higher priority (LCH-based-priority) in MAC, and the first PDU is deprioritized.

- For overlapping grants, when MAC generates two MAC PDUs, the second PDU has higher priority from MAC point of view (LCH-based-priority), and thus shall be transmitted by PHY.

- From RAN2 point of view, PHY should consider MAC prioritization decision, i.e. when two MAC PDUs are created, the later MAC PDU is to be considered with higher priority.

2. Actions:

To RAN1:

ACTION: RAN2 respectfully ask RAN1 to consider the above information in their work and adopt their specifications accordingly, if required.

3. Date of Next TSG-RAN WG2 Meetings:

RAN-WG2 Meeting #109bis Sapporo, Japan		20-24 April, 2020

RAN-WG2 Meeting #110	Athens, Greece		25-29 May, 2020

RAN-WG2 Meeting #111	Toulouse, France		24-28 Aug, 2020

4. References

[1] R2-1916352_38321_Running_CR_IIOT_r3, Samsung, RAN2 #108

[2] R2-2000795, On PHY and MAC interaction to support intra-UE prioritization, Ericsson, RAN2#109-e