3GPP TSG-RAN WG2 Meeting #109-e	R2-2000850

Online, 24 February – 6 March 2020

Agenda item:	6.1.4

Source:	Nokia, Nokia Shanghai Bell

Title:	MAC CE for guard symbols indication

WID/SID:	NR_IAB-Core - Release 16

Document for:	Discussion and Decision

# 1	Introduction

RAN1 has made the following agreements in RAN1#98bis and RAN1#99, respectively:

Agreements: A parent IAB node can be made aware of the number of symbols Ng the child IAB node would like the parent IAB node not to use at the edge (beginning or end) of a slot when there is a transition between child MT and child DU. Separately or additionally, the child IAB node can be made aware of the number of guard symbols that the parent IAB node will provide. - Ng can be provided for each of the [8] possible transitions with potential overlap: MT to DU; DL Tx; UL Rx. DL Rx. UL Tx. DU to MT; DL Rx; UL Tx. DL Tx. UL Rx. - If Ng is not provided it is assumed to be 0 NOTE: this agreement does not introduce any performance requirement on IAB nodes.

Agreements: Desired Guard Symbols and Provided Guard Symbols are provided per cell and use 3 bits for each of the 8 transitions to indicate the number of guard symbols. - In Rel-16, a range of 0-4 symbols are supported for each transition. Additional entries are reserved for future use - A new parameter GuardSymbol-SCS is also provided which indicates the reference SCS (FR1: {15kHz, 30kHz, 60kHz}, FR2: {60kHz, 120kHz}) to be used for the guard symbols.

In this contribution, a MAC CE format is proposed to implement the RAN1 design.

# 2	Text proposal into TS 38.321

## 5.18 Handling of MAC CEs ### 5.18.1 General This clause specifies the requirements upon reception of the following MAC CEs: - SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE; - Aperiodic CSI Trigger State Subselection MAC CE; - TCI States Activation/Deactivation for UE-specific PDSCH MAC CE; - TCI State Indication for UE-specific PDCCH MAC CE; - SP CSI reporting on PUCCH Activation/Deactivation MAC CE; - SP SRS Activation/Deactivation MAC CE; - PUCCH spatial relation Activation/Deactivation MAC CE; - SP ZP CSI-RS Resource Set Activation/Deactivation MAC CE; - Recommended Bit Rate MAC CE; - Desired and Provided Guard Symbols MAC CE. (…) ### 5.18.XX Desired and Provided Guard Symbols The parent IAB-node may transmit the Provided Guard Symbols MAC CE to the MAC entity to indicate the provided number of guard symbols that will not be used for the IAB-node MT in slots where the IAB-node transitions between IAB-node MT and IAB-node DU. Upon reception of a Provided Guard Symbols MAC CE the MAC entity shall: - indicate to lower layers the number of provided guard symbols and the SCS configuration. The MAC entity may indicate the parent IAB-node the desired number of guard symbols the IAB-node would like the parent IAB-node not to use for the IAB-node MT at the edge (beginning or end) of a slot where the IAB-node transitions between IAB-node MT and IAB-node DU. The MAC entity may: 1> if a Desired Guard Symbols query has not been triggered: 2> trigger a Desired Guard Symbols query. If the MAC entity has UL resources allocated for new transmission the MAC entity shall: 1> for each Desired Guard Symbols query that the Desired Guard Symbols procedure determines has been triggered and not cancelled: 2> if the MAC entity has UL resources allocated for new transmission and the allocated UL resources can accommodate a Desired Guard Symbols MAC CE plus its subheader as a result of LCP as defined in clause 5.4.3.1: 3> instruct the Multiplexing and Assembly procedure to generate the Desired Guard Symbols MAC CE; 3> cancel this Desired Guard Symbols query. ### 6.1.3 MAC Control Elements (CEs) (…) #### 6.1.3.XX Desired and Provided Guard Symbols MAC CE The Desired and Provided Guard Symbols MAC CE is identified by MAC subheader with LCID as specified in Table 6.2.1-1 and 6.2.1-2, respectively. It has a fixed size and consists of four octets containing the following fields (Figure 6.1.3.XX-1): - SCS: This field indicates the GuardSymbol-SCS configuration which indicates the reference SCS to be used for the guard symbols. The length of the field is 2 bits and the indexes are encoded based on the following: Index 0: 15kHz; 1: 30kHz; 2: 60kHz; 3: 120kHz; - Ngi: This field indicates the desired or provided guard symbols for the given transition i as specified in Table 6.1.3.XX-1. Indices 0…4 can be used in this release of the specification. The length of the field is 3 bits; - R: Reserved bit, set to 0. Figure 6.1.3.XX-1: Timing Advance Command MAC CE Table 6.1.3.XX-1: Mapping between Ngi and transitions MT to DU; DL Tx; UL Rx. DL Rx; Ng0; Ng1. UL Tx; Ng2; Ng3. DU to MT; DL Rx; UL Tx. DL Tx; Ng4; Ng5. UL Rx; Ng6; Ng7. (…) ## 6.2 Formats and parameters ### 6.2.1 MAC subheader for DL-SCH and UL-SCH The MAC subheader consists of the following fields: - LCID: The Logical Channel ID field identifies the logical channel instance of the corresponding MAC SDU or the type of the corresponding MAC CE or padding as described in Tables 6.2.1-1 and 6.2.1-2 for the DL-SCH and UL-SCH respectively. There is one LCID field per MAC subheader. The LCID field size is 6 bits; - L: The Length field indicates the length of the corresponding MAC SDU or variable-sized MAC CE in bytes. There is one L field per MAC subheader except for subheaders corresponding to fixed-sized MAC CEs, padding, and MAC SDUs containing UL CCCH. The size of the L field is indicated by the F field; - F: The Format field indicates the size of the Length field. There is one F field per MAC subheader except for subheaders corresponding to fixed-sized MAC CEs, padding, and MAC SDUs containing UL CCCH. The size of the F field is 1 bit. The value 0 indicates 8 bits of the Length field. The value 1 indicates 16 bits of the Length field; - R: Reserved bit, set to 0. The MAC subheader is octet aligned. Table 6.2.1-1 Values of LCID for DL-SCH Index; LCID values. 0; CCCH. 1–32; Identity of the logical channel. 33-45; Reserved. 46; Provided Guard Symbols. 47; Recommended bit rate. 48; SP ZP CSI-RS Resource Set Activation/Deactivation. 49; PUCCH spatial relation Activation/Deactivation. 50; SP SRS Activation/Deactivation. 51; SP CSI reporting on PUCCH Activation/Deactivation. 52; TCI State Indication for UE-specific PDCCH. 53; TCI States Activation/Deactivation for UE-specific PDSCH. 54; Aperiodic CSI Trigger State Subselection. 55; SP CSI-RS/CSI-IM Resource Set Activation/Deactivation. 56; Duplication Activation/Deactivation. 57; SCell Activation/Deactivation (four octets). 58; SCell Activation/Deactivation (one octet). 59; Long DRX Command. 60; DRX Command. 61; Timing Advance Command. 62; UE Contention Resolution Identity. 63; Padding. Table 6.2.1-2 Values of LCID for UL-SCH Index; LCID values. 0; CCCH of size 64 bits (referred to as "CCCH1" in TS 38.331 [5]). 1–32; Identity of the logical channel. 33–50; Reserved. 51; Desired Guard Symbols. 52; CCCH of size 48 bits (referred to as "CCCH" in TS 38.331 [5]). 53; Recommended bit rate query. 54; Multiple Entry PHR (four octets Ci). 55; Configured Grant Confirmation. 56; Multiple Entry PHR (one octet Ci). 57; Single Entry PHR. 58; C-RNTI. 59; Short Truncated BSR. 60; Long Truncated BSR. 61; Short BSR. 62; Long BSR. 63; Padding.

# 3	Conclusion

Proposal: Agree on the TP in Section 2.