3GPP TSG-RAN WG2 Meeting #109 electronic	                      R2-2000927

24th Feb – 6th Mar 2020

Agenda Item:	6.0.2	 Feature List and UE capabilities

Source: 	CMCC

Title: 	UE Feature List for Rel-16 SON/MDT WI

Document for:	Discussion and decision

1 Introduction

This is to capture UE feature list for Rel-16 WI: SON/MDT support for NR.

2 UE feature list for SON/MDT support for NR

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x. Rel-16 SON/MDT support for NR | x-0 | Basic logged MDT | Logged measurements in RRC_IDLE and RRC_INACTIVE |  | loggedMeasurements | UE-NR-Capability | No | No |  | Optional with capability signalling |
|  | x-1 | Location reporting | Location reporting while performing MDT |  | n/a | n/a | n/a | n/a |  | Mandatory without capability signalling. Note: If location information is available, location information should be included while performing MDT. |
|  | x-2 | GNSS | Equipped with a standalone GNSS receiver |  | standaloneGNSS-Location | UE-NR-Capability | No | No |  | Optional with capability signalling |
|  | x-3 | Bluetooth and WLAN measurement collection | 1) Bluetooth measurements in RRC_CONNECTED state 2) WLAN measurements in RRC_CONNECTED state 3) Bluetooth measurements in RRC_IDLE and RRC_INACTIVE state 4) WLAN measurements in RRC_IDLE and RRC_INACTIVE state |  | 1) immMeasBT 2) immMeasWLAN 3) loggedMeasBT 4) loggedMeasWLAN | UE-NR-Capability | No | No |  | Optional with capability signalling |
|  | x-4 | Sensor data collection | 1) Uncompensated barometeric pressure measurement reporting 2) orientation information reporting 3) speed information reporting |  | 1) barometerMeasReport 2) orientationMeasReport 3) speedMeasReport | UE-NR-Capability | No | No |  | Optional with capability signalling |
|  | x-5 | Delay measurement | UL PDCP Packet Delay measurement and reporting in RRC_CONNECTED state |  | ulPDCP-Delay | UE-NR-Capability | No | No |  | Optional with capability signalling |
|  | x-6 | Mobility history information storage | Storage of mobility history information and the reporting in UEInformationResponse message |  | n/a | n/a | n/a | n/a |  | Optional without capability signalling |
|  | x-7 | SON | 1) CEF reporting 2) RLF reporting 3) RACH reporting |  | 1),2) n/a 3) rachReport | 1),2) n/a 3) UE-NR-Capability | 1),2) n/a 3) No | 1),2) n/a 3) No |  | 1), 2) Mandatory without capability signalling 3) Optional with capability signalling |

3 Reference

[1] R2-1915209	TS 38.306 CR on UE capabilities for SON MDT WI	CMCC	draftCR	Rel-16	38.306	15.7.0	NR_SON_MDT-Core

[2] RAN2 108#49 email discussion		TS 38.306 Running CR  vivo, CMCC