3GPP TSG-RAN WG2 eMeeting 109	R2-2000956

Electronic, 24th Feb- 6th Mar, 2020

Agenda Item:	6.13.4

Source:	Huawei, HiSilicon

Title:	Prioritized 2-step RACH

Document for:	Discussion, Decision

Introduction

We have already discussed about the prioritized 2-step RACH and achieved the following agreements [1] [2].

Agreements

- For 2 step CBRA, RA prioritisation is supported at least for handover and beam failure recovery.

- RA prioritisation for 2 step CBRA is controlled by networkRA prioritisation parameters are separately configured for 2 step CBRA and 4 step CBRA. -ra-Prioritization2Step is optionally added to BeamFailureRecoveryConfig IE and RACH-ConfigDedicated IE.

Agreements

1	RA prioritization for MCS and MPS should be applicable for both 2-step and 4-step RACH procedures.

2	RA prioritization parameters for MCS and MPS for both 2-step and 4-step RACH procedures are obtained from the SIB1 ra-PrioritizationForAccessIdentity field as defined in TS 38.321 Subclause 5.1.1, as agreed in main session for 4-step RA.

- Same mechanism as 4-step RA will be applied for 2-step RA

In this paper, we discuss some other aspects related to prioritized 2-step RACH and provide our considerations.

Prioritized 2-step RACH

According to the agreements, the parameters introduced in Rel-15 i.e., scalingFactorBI and powerRampingStepHighPriority are still applied to 2-step RACH. However, besides these two parameters, some other new parameters introduced in 2-step RACH can be considered as listed below.

| rsrp-Threshold-msgA | A RSRP-based criterion for RACH type selection, detailed UE behavior to be described by RAN2. |
| --- | --- |
| msgA-TransMax | The maximum number of MsgA transmission. The UE switches to 4-step RACH msg1 transmission if the number of MsgA transmission reach this value. |

rsrp-Threshold-msgA is the threshold for RACH type selection and a smaller value range can be configured for prioritization as in this case, it is more likely to perform a 2-step RACH procedure instead of 4-step RACH, which helps to reduce the access latency.

msgA-TransMax is the maximum number of MsgA transmission and the prioritized RACH can be configured with a larger value in order to have more 2-step RACH attempts instead of switching to 4-step RACH. And this also helps with the latency.

Therefore, we propose to introduce two new parameters for prioritized 2-step RACH, i.e., rsrp-Threshold-msgA-HighPriority and msgA-TransMaxHighPriority.

Proposal 1: Introduce rsrp-Threshold-msgA-HighPriority and msgA-TransMaxHighPriority for prioritized 2-step RACH.

Conclusion

In this paper, we discuss two new parameters for prioritized 2-step RACH and based on the discussion, we have the following proposal.

Proposal 1: Introduce rsrp-Threshold-msgA-HighPriority and msgA-TransMaxHighPriority for prioritized 2-step RACH.

References

- RAN2#107bis chairman note

- RAN2#108 chairman note