3GPP TSG-RAN WG2 #109-e	Tdoc R2-2000986

Electronic meeting, 24 February – 6 March 2020	Revision of R2-1915410

Agenda Item:	7.2.9

Source:	Ericsson

Title:	NB-IoT coexistence with NR

Document for:	Discussion, Decision

# 1	Introduction

So far coexistence between NB-IoT and NR has mostly been discussed in RAN1. Some of the agreements in RAN1#98 will require actions in RAN2. Agreements in RAN1meetings #98, #98bis and #99 [1] are:

RAN1#98:

R1-1909557 Feature lead summary of Coexistence of NB-IoT with NR Futurewei Agreement Specify NB-IoT resource reservation only for FDD/TDD NB-IoT non-anchor carriers. Agreement UL resource reservation for NB-IoT is supported. Agreement - FDD UL resource reservation is supported with subframe-level granularity - FFS: Slot-level, symbol-level - DL resource reservation is supported with slot-level and symbol(s)-level granularity. Agreement The reserved resource in NB-IoT non-anchor carrier is semi-statically configured by higher layer signalling. - FFS whether the resource in an invalid and/or valid subframe indicated by legacy downlink bitmap can be configured as the reserved resource - FFS signalling Agreement NB-IoT transmission can be postponed or dropped depending on the granularity of the reserved resources. - NB-IoT transmission is dropped for symbol-level reserved resources. - NB-IoT transmission is dropped for slot-level reserved resources. - NB-IoT transmission is postponed for subframe-level reserved resources.

RAN1#98bis:

R1-1910884 Feature lead summary of Coexistence of NB-IoT with NR Futurewei R1-1911383 Further topics in coexistence of NB-IoT with NR Futurewei Agreement NB-IoT symbols that carry NRS are not reserved Agreement For resource reservation for NB-IoT in Rel-16, the configuration is independent from legacy subframe level resource reservation. Agreement For unicast, NPDCCH and NPDSCH scrambled by C-RNTI that would fall into the reserved resource are dropped for symbol-level and slot-level reserved resources. - Dropped means punctured Agreement For unicast, NPUSCH scrambled by C-RNTI or SPS-C-RNTI that would fall into the reserved resource is postponed for subframe-level reserved resources. Agreement UL resource reservation for NB-IoT with slot-level and symbol(s)-level granularity in addition to subframe-level granularity is supported. - FFS: whether the DMRS symbols can be configured as reserved resources Agreement NB-IoT reserved resource can be separately configured per NB-IoT non-anchor carrier. Agreement For unicast transmission, dynamic DCI signalling can be used to [indicate or override] which reserved resources are used for the scheduled NB-IoT transmission.

RAN1#99:

R1-1912633 Feature lead summary of Coexistence of NB-IoT with NR Futurewei R1-1913398 Feature lead summary of Coexistence of NB-IoT with NR Futurewei Agreement For unicast, NPUSCH scrambled by C-RNTI and SPS-C-RNTI that would fall into the reserved resource is dropped for symbol and slot-level reserved resources. Agreement - For slot-level reservation in UL NB-IoT, DMRS transmission in a reserved slot is dropped. (for 15kHz) - For symbol-level reservation in UL NB-IoT, DMRS symbols can be reserved Agreement Reserved resource configuration is configured by cell-specific higher-layer signalling. R1-1913545 Feature lead summary of Coexistence of NB-IoT with NR Futurewei Agreement For 10msec periodicity, if only subframe level granularity: a bitmap with length 10 bits is used for indicating reserved subframes - It is configured per non-anchor carrier, and it can be different from the anchor-carrier For 10msec periodicity, if finer granularity: a bitmap with length 20 bits, each subframe has 2 bits per subframe - 00 both slots can be used - 01 2nd slot reserved (if a second bitmap is configured, it is for symbol-level granularity) - The length of the second bitmap is 5 bits for downlink, 7 bits for uplink - 10 1st slot reserved (if a third bitmap is configured, it is for symbol-level granularity) - The length of the third bitmap is 5 bits for downlink, 7 bits for uplink - 11 both slots are reserved Agreement (RRC impact) The first bitmap is 20 or 80 bits. eNB can configure periodicity and start position. - The periodicity T is {10ms, 20ms, 40ms, 80ms, 160ms} - The start position is in a granularity of 10ms Agreement For unicast transmission, dynamic DCI signalling can be used to indicate whether the resource reservation or continuous transmission applies to the scheduled transmission - Note: the resource reservation is the Rel-16 resource reservation.

# 2	Discussion

RAN1 agreements that require action from RAN2 are to add UE capabilities and configuration for resource reservation [2]; a feature intended to be used for improving coexistence with NR.

Add UE capabilities for resource reservation in UL and DL to enable improved coexistence between NB-IoT and NR, see text proposal.

Add configuration for resource reservation in UL and DL to enable improved coexistence between NB-IoT and NR, see text proposal.

# Text Proposal

[36.306]

#### 4.3.4.xy	resourceReservation-DL-r16

This field indicates that the UE supports DL resource reservation resource with subframe-level, slot-level and symbol-level granularity of NB-IoT non-anchor carriers for improved NR coexistence performance.

#### 4.3.4.xz	resourceReservation-UL-r16

This field indicates that the UE supports UL resource reservation with subframe-level, slot-level and symbol-level granularity of NB-IoT non-anchor carriers for improved NR coexistence performance.

[36.331]

#### –	SystemInformationBlockType2-NB

The IE SystemInformationBlockType2-NB contains radio resource configuration information that is common for all UEs.

NOTE:	UE timers and constants related to functionality for which parameters are provided in another SIB are included in the corresponding SIB.

SystemInformationBlockType2-NB information element

-- ASN1START

SystemInformationBlockType2-NB-r13 ::=	SEQUENCE {

radioResourceConfigCommon-r13			RadioResourceConfigCommonSIB-NB-r13,

ue-TimersAndConstants-r13				UE-TimersAndConstants-NB-r13,

freqInfo-r13							SEQUENCE {

ul-CarrierFreq-r13						CarrierFreq-NB-r13			OPTIONAL,	-- Need OP

additionalSpectrumEmission-r13			AdditionalSpectrumEmission

},

timeAlignmentTimerCommon-r13			TimeAlignmentTimer,

multiBandInfoList-r13	SEQUENCE (SIZE (1..maxMultiBands)) OF AdditionalSpectrumEmission		OPTIONAL,	-- Need OR

lateNonCriticalExtension				OCTET STRING					OPTIONAL,

...,

[[	cp-Reestablishment-r14				ENUMERATED {true}				OPTIONAL		-- Need OP

]],

[[	servingCellMeasInfo-r14				ENUMERATED {true} 				OPTIONAL,		-- Need OR

cqi-Reporting-r14					ENUMERATED {true} 				OPTIONAL		-- Need OR

]],

[[	enhancedPHR-r15						ENUMERATED {true}		OPTIONAL,	-- Need OR

freqInfo-v1530						SEQUENCE {

tdd-UL-DL-AlignmentOffset-r15		TDD-UL-DL-AlignmentOffset-NB-r15

}	OPTIONAL,		-- Cond TDD

cp-EDT-r15							ENUMERATED {true}		OPTIONAL,	-- Need OR

up-EDT-r15							ENUMERATED {true}		OPTIONAL	-- Need OR

]],

[[ nr-CoexistenceImprParams-r16			NR-CoexistenceImprParams-r16	OPTIONAL	-- Need OR

]]

}

NR-CoexistenceImprParams-r16 ::=	SEQUENCE {

subframeConfig-DL-r16				ENUMERATED (10ms, 40ms)			OPTIONAL,		-- Need OR

resourceReservationSlot-DL-r16		ENUMERATED (20bits, 80bits) 	OPTIONAL,		-- Need OR

resourceReservationSymbol-DL2-r16	BIT STRING (SIZE(5))	OPTIONAL,	-- Cond resourceReservationSlot-DL

resourceReservationSymbol-DL1-r16	BIT STRING (SIZE(5))	OPTIONAL,	-- Cond resourceReservationSlot-DL

resourceReservationPeriod-DL-r16	ENUMERATED (10ms,20ms,40ms,80ms,160ms)	OPTIONAL,	--Need OR

resourceReservationStartPos-DL-r16	ENUMERATED (10ms,20ms,40ms,80ms,160ms)	OPTIONAL,	--Need OR

subframeConfig-FDD-UL-r16			ENUMERATED (10ms, 40ms)				OPTIONAL,		-- Need OR

resourceReservationSlot-UL-r16		ENUMERATED (20bits, 80bits) 		OPTIONAL,		-- Need OR

resourceReservationSymbol-UL2-r16	BIT STRING (SIZE(7))	OPTIONAL,	-- Cond resourceReservationSlot-UL

resourceReservationSymbol-UL1-r16	BIT STRING (SIZE(7))	OPTIONAL,	-- Cond resourceReservationSlot-UL

resourceReservationPeriod-UL-r16		ENUMERATED (10ms,20ms,40ms,80ms,160ms)  OPTIONAL,		-- Need OR

resourceReservationStartPos-DL-r16	ENUMERATED (10ms,20ms,40ms,80ms,160ms) OPTIONAL		-- Need OR

}

-- ASN1STOP

SystemInformationBlockType2-NB field descriptions

additionalSpectrumEmission The UE requirements related to IE AdditionalSpectrumEmission are defined in TS 36.101 [42], clause 6.2.4F.

cp-EDT For FDD: This field indicates whether the UE is allowed to initiate CP-EDT, see 5.3.3.1b.

cp-Reestablishment This field indicates if the NB-IoT UE is allowed to trigger RRC connection re-establishment when AS security has not been activated.

cqi-Reporting For FDD: This field indicates if downlink channel quality reporting in RRCConnectionReestablishmentRequest-NB, RRCConnectionRequest-NB and RRCConnectionResumeRequest-NB message is allowed.

enhancedPHR For FDD: This field indicates if the NB-IoT UE is allowed to report enhanced PHR in MSG3 as specified in TS 36.321 [6].

multiBandInfoList A list of additionalSpectrumEmission i.e. one for each additional frequency band included in multiBandInfoList in SystemInformationBlockType1-NB, listed in the same order.

resourceReservationPeriod-DL Periodicity in milliseconds for downlink time-domain resource reservation for unicast transmission.

resourceReservationStartPos-DL Start position in milliseconds for downlink time-domain resource reservation for unicast transmission. For 10ms periodicity, {0}. For 20ms periodicity, {0, 10}. For 40ms periodicity, {0, 10, 20, 30}. For 80ms periodicity, {0, 10, 20, 30, 40, 50, 60, 70}. For 160ms periodicity, {0, 10, 20, 30, 40, 50, 60, 70, 80, 90, 100, 110, 120, 130, 140, 150}.

resourceReservationPeriod-UL Periodicity in milliseconds for uplink time-domain resource reservation for unicast transmission.

resourceReservationSlot-DL Slot- and symbol(s)-level downlink time-domain resource reservation for non-anchor carrier.

resourceReservationSlot-UL Configuration of slot- and symbol(s)-level uplink time-domain resource reservation for non-anchor carrier.

resourceReservationSymbol-DL1 Symbol-level downlink time-domain resource reservation for non-anchor carrier (for the first slot in a subframe) This parameter can be configured only if resourceReservationSlot-DL is configured

resourceReservationSymbol-DL2 Symbol-level downlink time-domain resource reservation for non-anchor carrier (for the second slot in a subframe) This parameter can be configured only if resourceReservationSlot-DL is configured

resourceReservationSymbol-UL1 Symbol-level uplink time-domain resource reservation for non-anchor carrier (for the first slot in a subframe). This parameter can be configured only if resourceReservationSlot-UL is configured

resourceReservationSymbol-UL2 Symbol-level uplink time-domain resource reservation for non-anchor carrier (for the second slot in a subframe). This parameter can be configured only if resourceReservationSlot-UL is configured.

servingCellMeasInfo This field indicates if serving cell idle mode measurement reporting in RRCConnectionReestablishmentComplete-NB, RRCConnectionResumeComplete-NB and RRCConnectionSetupComplete-NB is allowed.

subframeConfig-DL Subframe-level dowllink time-domain resource reservation for non-anchor carrier.

subframeConfig-FDD-UL Subframe-level uplink time-domain resource reservation for non-anchor carrier in FDD.

tdd-UL-DL-AlignmentOffset Indicates the offset between the UL carrier frequency center with respect to DL carrier frequency center for the anchor carrier.

ul-CarrierFreq For FDD: Uplink carrier frequency as defined in TS 36.101 [42], clause 5.7.3F. If operationModeInfo in the MIB-NB is set to standalone and the field is absent, the value of the carrier frequency is determined by the TX-RX frequency separation defined in TS 36.101 [42], table 5.7.4-1, and the value of the carrier frequency offset is 0. If operationModeInfo in the MIB-NB is not set to standalone, the field is mandatory present. For TDD: This field is absent and the uplink carrier frequency is same as the downlink frequency.

up-EDT For FDD: This field indicates whether the UE is allowed to initiate UP-EDT, see 5.3.3.1b.

| Conditional presence | Explanation |
| --- | --- |
| TDD | The field is mandatory present for TDD; otherwise the field is not present and the UE shall delete any existing value for this field. |
| resourceReservationSlot-DL | The field is optionally present, Need OR, if resourceReservationSlot-UL is configured. Otherwise the field is not present. |
| resourceReservationSlot-UL | The field is optionally present, Need OR, if resourceReservationSlot-UL is configured. Otherwise the field is not present. |

[text omitted]

#### –	UE-Capability-NB

The IE UE-Capability-NB is used to convey the NB-IoT UE Radio Access Capability Parameters, see TS 36.306 [5]. The IE UE-Capability-NB is transferred in NB-IoT only.

UE-Capability-NB information element

-- ASN1START

UE-Capability-NB-r13 ::=		SEQUENCE {

accessStratumRelease-r13		AccessStratumRelease-NB-r13,

ue-Category-NB-r13				ENUMERATED {nb1}					OPTIONAL,

multipleDRB-r13					ENUMERATED {supported}				OPTIONAL,

pdcp-Parameters-r13				PDCP-Parameters-NB-r13				OPTIONAL,

phyLayerParameters-r13			PhyLayerParameters-NB-r13,

rf-Parameters-r13				RF-Parameters-NB-r13,

dummy							SEQUENCE {}							OPTIONAL

}

UE-Capability-NB-Ext-r14-IEs ::=		SEQUENCE {

ue-Category-NB-r14					ENUMERATED {nb2}				OPTIONAL,

mac-Parameters-r14					MAC-Parameters-NB-r14			OPTIONAL,

phyLayerParameters-v1430			PhyLayerParameters-NB-v1430		OPTIONAL,

rf-Parameters-v1430					RF-Parameters-NB-v1430,

nonCriticalExtension				UE-Capability-NB-v1440-IEs		OPTIONAL

}

UE-Capability-NB-v1440-IEs ::=		SEQUENCE {

phyLayerParameters-v1440			PhyLayerParameters-NB-v1440		OPTIONAL,

nonCriticalExtension				UE-Capability-NB-v14x0-IEs		OPTIONAL

}

UE-Capability-NB-v14x0-IEs ::=		SEQUENCE {

-- Following field is only to be used for late REL-14 extensions

lateNonCriticalExtension			OCTET STRING					OPTIONAL,

nonCriticalExtension				UE-Capability-NB-v1530-IEs		OPTIONAL

}

UE-Capability-NB-v1530-IEs ::=		SEQUENCE {

earlyData-UP-r15					ENUMERATED {supported}			OPTIONAL,

rlc-Parameters-r15					RLC-Parameters-NB-r15,

mac-Parameters-v1530				MAC-Parameters-NB-v1530,

phyLayerParameters-v1530			PhyLayerParameters-NB-v1530		OPTIONAL,

tdd-UE-Capability-r15				TDD-UE-Capability-NB-r15		OPTIONAL,	nonCriticalExtension				UE-Capability-NB-v16xy-IEs		OPTIONAL

}

UE-Capability-NB-v16xy-IEs ::=		SEQUENCE {

phyLayerParameters-v16xy			PhyLayerParameters-NB-v16xy		OPTIONAL,

nonCriticalExtension				SEQUENCE {}						OPTIONAL

}

TDD-UE-Capability-NB-r15 ::=		SEQUENCE {

ue-Category-NB-r15					ENUMERATED {nb2}				OPTIONAL,

phyLayerParametersRel13-r15			PhyLayerParameters-NB-r13		OPTIONAL,

phyLayerParametersRel14-r15			PhyLayerParameters-NB-v1430		OPTIONAL,

phyLayerParameters-v1530			PhyLayerParameters-NB-v1530		OPTIONAL,

...

}

AccessStratumRelease-NB-r13 ::=		ENUMERATED {rel13, rel14, rel15, spare5, spare4, spare3, spare2, spare1, ...}

PDCP-Parameters-NB-r13		::= SEQUENCE {

supportedROHC-Profiles-r13			SEQUENCE {

profile0x0002						BOOLEAN,

profile0x0003						BOOLEAN,

profile0x0004						BOOLEAN,

profile0x0006						BOOLEAN,

profile0x0102						BOOLEAN,

profile0x0103						BOOLEAN,

profile0x0104						BOOLEAN

},

maxNumberROHC-ContextSessions-r13	ENUMERATED {cs2, cs4, cs8, cs12}	DEFAULT cs2,

...

}

RLC-Parameters-NB-r15		::=		SEQUENCE {

rlc-UM-r15							ENUMERATED {supported}				OPTIONAL

}

MAC-Parameters-NB-r14		::=		SEQUENCE {

dataInactMon-r14					ENUMERATED {supported}					OPTIONAL,

rai-Support-r14						ENUMERATED {supported}				OPTIONAL

}

MAC-Parameters-NB-v1530		::=		SEQUENCE {

sr-SPS-BSR-r15						ENUMERATED {supported}			OPTIONAL

}

PhyLayerParameters-NB-r13	::=		SEQUENCE {

multiTone-r13						ENUMERATED {supported}			OPTIONAL,

multiCarrier-r13						ENUMERATED {supported}			OPTIONAL

}

PhyLayerParameters-NB-v1430	::=		SEQUENCE {

multiCarrier-NPRACH-r14				ENUMERATED {supported}			OPTIONAL,

twoHARQ-Processes-r14				ENUMERATED {supported}			OPTIONAL

}

PhyLayerParameters-NB-v1440	::=		SEQUENCE {

interferenceRandomisation-r14		ENUMERATED {supported}			OPTIONAL

}

PhyLayerParameters-NB-v1530	::=		SEQUENCE {

mixedOperationMode-r15				ENUMERATED {supported}			OPTIONAL,

sr-WithHARQ-ACK-r15					ENUMERATED {supported}			OPTIONAL,

sr-WithoutHARQ-ACK-r15				ENUMERATED {supported}			OPTIONAL,

nprach-Format2-r15					ENUMERATED {supported}			OPTIONAL,

additionalTransmissionSIB1-r15		ENUMERATED {supported}			OPTIONAL,

npusch-3dot75kHz-SCS-TDD-r15		ENUMERATED {supported}			OPTIONAL

}

PhyLayerParameters-NB-v16xy	::=		SEQUENCE {

resourceReservation-DL-r16				ENUMERATED {supported}			OPTIONAL,

resourceReservation-UL-r16				ENUMERATED {supported}			OPTIONAL

}

[text omitted]

| UE-Capability-NB field descriptions | FDD/TDD appl | FDD/TDD diff |
| --- | --- | --- |
| accessStratumRelease Set to rel15 in this version of the specification. | FDD/TDD | No |
| additionalTransmissionSIB1 Indicates whether the UE supports additional SIB1 transmission as specified in TS 36.213 [23]. | FDD | - |
| dataInactMon Indicates whether the UE supports the data inactivity monitoring as specified in TS 36.321 [6]. | FDD/TDD | No |
| dummy This field is not used in the specification. It shall not be sent by the UE. | NA | NA |
| earlyData-UP Indicates whether the UE supports EDT for User plane CIoT EPS optimisations, as defined in TS 24.301 [35]. | FDD | - |
| interferenceRandomisation For FDD: Indicates whether the UE supports interference randomisation in connected mode as defined in TS.36.211 [21]. | FDD | - |
| maxNumberROHC-ContextSessions Set to the maximum number of concurrently active ROHC contexts supported by the UE, excluding context sessions that leave all headers uncompressed. cs2 corresponds with 2 (context sessions), cs4 corresponds with 4 and so on. The network ignores this field if the UE supports none of the ROHC profiles in supportedROHC-Profiles. | FDD/TDD | No |
| mixedOperationMode Defines whether the UE supports multi-carrier operation with mixed operation mode, standalone or inband/guardband, between the anchor carrier and the non-anchor carrier for unicast, paging, and random access as specified in TS 36.300 [9]. | FDD | - |
| multiCarrier Defines whether the UE supports multi -carrier operation. | FDD/TDD | Yes |
| multicarrier-NPRACH Defines whether the UE supports NPRACH on non-anchor carrier as specified in TS 36.321 [6]. | FDD/TDD | Yes |
| multipleDRB Defines whether the UE supports multiple DRBs. | FDD/TDD | No |
| multiNS-Pmax Defines whether the UE supports the mechanisms defined for NB-IoT cells broadcasting NS-PmaxList-NB. | FDD/TDD | No |
| multiTone Defines whether the UE supports UL multi-tone transmissions on NPUSCH. | FDD/TDD | Yes |
| nprach-Format2 Defines whether the UE supports NPRACH resources using preamble format 2. | FDD | - |
| npusch-3dot75kHz-SCS-TDD Indicates whether the UE supports NPUSCH with 3.75kHz SCS for TDD. | TDD | - |
| powerClassNB-14dBm Defines whether the UE supports power class 14dBm in all the bands supported by the UE as specified in TS 36.101 [42]. If powerClassNB-20dBm is included, the UE shall not include the field powerClassNB-14dBm. | FDD/TDD | No |
| powerClassNB-20dBm Defines whether the UE supports power class 20dBm in NB-IoT for the band, as specified in TS 36.101 [42]. If neither powerClassNB-14dBm nor powerClassNB-20dBm is included, UE supports power class 23 dBm in the NB-IoT band. | FDD/TDD | No |
| rai-Support Defines whether the UE supports release assistance indication (RAI) as specified in TS 36.321 [6]. | FDD/TDD | No |
| resourceReservation-DL, resourceReservation-UL DL resource reservation with slot-level and symbol-level granularity of NB-IoT non-anchor carriers. UL resource reservation with subframe-level, slot-level and symbol-level granularity of NB-IoT non-anchor carriers. | FDD/TDD | Yes |

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Add UE capabilities for resource reservation in UL and DL to enable improved coexistence between NB-IoT and NR, see text proposal.

Proposal 2	Add configuration for resource reservation in UL and DL to enable improved coexistence between NB-IoT and NR, see text proposal.

# References

R1-1913595 RAN1 agreement list (meetings #98, #98bis, and #99)

R1-1913673 R16 L1 parameter list NB_IOTenh3