3GPP TSG-RAN WG2 Meeting #109	R2-2001046

Athens, Greece, 24 – 28 February 2020

Agenda item:	6.7.1

Source:	Nokia (rapporteur)

Title:	Summary of open issues for IIOT WI

WID/SID:	NR_IIOT - Release 16

Document for:	Discussion and Decision

# 1	Introduction

This document lists the remaining issues related to NR IIOT WI. It is based on the latest versions of running CRs and summaries from CRs rapporteurs and aims on gathering all the main aspects to be resolved before WI completion in one place, as a guidance for contributions preparations by companies.

Companies are requested to focus their contributions on the remaining issues identified in the running CRs, on the issues which were identified during the e-mail discussions and on the issues for which there was no clear conclusion in the e-mail discussion.

Since this is a very last stage of the WI, it is recommended that papers provide not only the discussion and the proposals, but also the related Text Proposals, if applicable. The latest running CRs should be used as a baseline for TPs and are copied here for convenience:

Running 38.300 CR

Running 38.321 CR

Running 38.331 CR

Running PDCP 38.323 CR

# 2	Remaining issues list

## 2.1	TS 38.300

| Editor’s note / open issue | How to handle |
| --- | --- |
| FFS whether padding removal is supported. | Discussion based on contributions, should consider a reply LS from SA1 |
| FFS whether there are other restrictions of how many SPS configurations are supported, e.g. per cell / per UE. FFS whether there are other restrictions of how many CG configurations are supported, e.g. per cell / per UE. | Decide based on the discussion on UE feature list discussion (RAN1) |
| FFS whether autonomous retransmission can utilize CG configuration different from the one, which was intended for original transmission. | Discussion based on contributions |
| FFS whether the LCP restriction based on indication of priority applies also to CG. | Discussion based on contributions |
| For NR-DC, it is FFS how the nodes can coordinate RLC entities activation/deactivation between each other (pending RAN3 discussions). | Wait for RAN3 discussion and discuss during RAN2#109bis |
| FFS how the need for reference time information can be determined for any given connected UE. | Discussion based on contributions |
| FFS how exactly propagation delay compensation should be captured considering the following FFS from RAN2#108: “R2 assume that UE may perform propagation delay compensation; We don’t specify how the UE perform propagation delay compensation; For unicast and broadcast, the network can indicate to the UE to not do delay compensation.” | Discussion based on contributions |
| FFS (in SA2) whether burst size is also included in TSCAI. | TSCAI does not include burst size (MDBV is used), FFS can be removed. |

## 2.2	TS 38.331

| Editor’s note / open issue | How to handle |
| --- | --- |
| Editor’s note FFS: whether and how to capture propagation delay compensation. -- FFS whether for unicast and broadcast, the network can indicate to the UE to not do delay compensation | Discussion based on contributions |
| Editor’s note: FFS: WHETHER we follow the legacy restriction that the configured grant type1 can only be configured for either or SUL, OR the configured grant type1 can be configured for both UL and SUL. Editor’s note: In this implementation, it is assumed that the LCH configured with allowedCG-List is allowed to be mapped to dynamic grant. This requires a confirmation from RAN2. Editor’s note: FFS the maximum length of the allowedList, i.e., the maximum number of configured grant configurations per MAC entity. | Discussion based on contributions |
| Editor’s Note: The name phy-PriorityIndex needs to be confirmed and what name to use needs to be aligned also with TS 38.300 and TS 38.321. Editor’s note: The name allowedPHY-PriorityIndex needs to be confirmed and what name to use needs to be aligned also with TS 38.300 and TS 38.321. Editor’s Note: The name phy-PriorityIndex needs to be confirmed and what name to use needs to be aligned also with TS 38.300 and TS 38.321. Editor’s note: In this implementation, it is assumed that the LCH configured with allowedPHY-PriorityIndex is allowed to be mapped to dynamic grant without any priority indication. FFS: The mapping restriction between a LCH configured with allowedPHY-PriorityIndex and a grant without any priority indication. Editor’s note: FFS whether allowedPHY-PriorityIndex applies for configured grant. Editor’s Note: It is FFS whether SR/data prioritization can be a separate configurable parameter from data/data prioritization. | Discussion based on contributions |
| Editor’s note: Configuration of autonomousReTx per MAC entity needs to be confirmed. | Discussion based on contributions |
| Editor’s note: There is support to have Ethernet Padding Removal and there is a tentative but postponed agreement that ”padding removal is an optional and configurable feature”. Whether/how to configure this feature is FFS and will be updated after RAN2#109. In the configuration of the Ethenet header compression, it is FFS, to include configurable parameters discussed in email discussion RAN2#108#53, e.g., drb-ContinueEHC, maxCID_EHC | Discussion based on contributions |
| Editor’s note: FFS: Whether the initial PDCP duplication state of the associated RLC entity is always activated for SRB, as in legacy Rel-15. Editor’s note: Further updates may be needed, after the clarification of the pdcp-Duplication field that will be discussed in RAN2#109. Editor’s note: The name ”secondary RLC entity” will be updated according to the discussion related with TS 38.323. | Discussion based on contributions |

## 2.3	TS. 38.323

| Editor’s note / open issue | How to handle |
| --- | --- |
| Whether to reset EHC protocol at PDCP re-establishment is FFS. | Discussion based on contributions |
| The text needs to be updated after the roles of Rel-15 Duplication MAC CE and Rel-16 Duplication MAC CE are decided. | Discussion based on contributions |
| The need for configuration parameters is FFS. (Protocol Parameters for EHC) | Discussion based on contributions |
| Formats of EHC full header packet, EHC compressed header packet, and EHC feedback packet will be specified. | Discussion based on contributions |
| Whether and how to capture processing order of ROHC and EHC when both are configured. | Discussion based on contributions |
| Whether to discard duplicated PDUs when an RLC entity is indicated to deactivate PDCP duplication (but PDCP duplication is still activated). | Discussion based on contributions |

## 2.4	TS. 38.321

| Editor’s note / open issue | How to handle |
| --- | --- |
| The text above may need to be updated after confirmation of RAN1 working assumption: In case of collision only between more than one SPS PDSCHs each without a corresponding PDCCH, a UE is not required to decode SPS PDSCHs other than the SPS PDSCH with the lowest SPS configuration index among collided SPS PDSCHs. - The UE shall report HARQ-ACK feedback only for the SPS PDSCH with the lowest SPS configuration index among collided SPS PDSCHs | Discussion based on contributions |
| The texts in this version of the running CR assume that lch-basedPrioritization, prioritization of resource conflict based on priority as a new feature of IIOT WI, is configurable for backward compatibility and separation from exisitng texts for UEs not supporting this feature. This feature requires a confirmation of RAN2. Thus, whether and how to configure it is FFS. This terminology may be changed after the discussion on MAC CE priority. | Discussion based on contributions |
| It is FFS whether SR/data prioritization can be a separate configurable parameter from data/data prioritization. | Discussion based on contributions |
| Priority determination considering MAC CE and configuredGrantTimer is FFS. | Discussion based on contributions |
| It is FFS whether an uplink grant addressed to CS-RNTI with NDI=1 (i.e. retransmission of a configured grant) is a configured grant or not. In this version of running CR, it is assumed that an uplink grant addressed to CS-RNTI with NDI=1 is considered as a dynamic grant. | Discussion based on contributions |
| It is FFS whether an uplink grant addressed to CS-RNTI with NDI=0 (i.e. (re-)activation of type 2 CG) is a configured grant or not. In this version of running CR, it is not clearly captured. | Discussion based on contributions |
| It is FFS how UE handles the case that at least two uplink grants with different MAC PDUs overlap with an SR transmission. | Discussion based on contributions |
| UE autonomous retransmission using the same HARQ process for the different CG configuration is FFS. | Discussion based on contributions |
| In case that retransmission grant for a deprioritized configured grant is deprioritized again and the MAC entity is configured with autonomousReTx, whether UE performs the autonomos retransmission in the subsequent configured grant is FFS. This running CR assumes that UE does not perform the autonomous retransmission in this case. | Discussion based on contributions |
| Whether this MAC CR needs to capture something to reflect a RAN2#108 agreement “The case when the next CG resource cannot be used for a retransmission because of UE processing time limitation can occur (no consensus on whether this is a corner case or a mainstream case). Leave the timeline restriction to UE implementation (we don’t specify a new number, can specify something)” is FFS. | Discussion based on contributions |
| It is FFS whether allowedPriorityLevels applies for configured grant. | Discussion based on contributions |
| The names, allowedCG-List and allowedPriorityLevels, should be aligned with RRC. | Wait for conclusion in RRC running CR |
| It is assumed that Multiple Entry Configured Grant Confirmation MAC CE has the same priority with Confirmation Grant Confirmation MAC CE in this version. The confirmation of this assumption may be needed. | Discussion based on contributions |
| When Multiple Entry Configured Grant Confirmation MAC CE is generated is FFS. | Discussion based on contributions |
| It is an FFS whether and how Rel-15 MAC CE turns on and off PDCP duplication with more than 2 RLC entities. | Discussion based on contributions |
| In the current version of the running CR, fixed size MAC CE of four octets is assumed as an example. The format in detail should be discussed and updated later. | Discussion based on contributions |
| It is an FFS whether this MAC CE has a fixed size or not. | Discussion based on contributions |
| ConfiguredGrantIndex is a CG ID unique per MAC entity and configured by RRC in addition to CG ID introduced by RAN1. The name may be changed to align with the RRC specification. | Wait for conclusion in RRC running CR |
| In the current version of the running CR, it is assumed that this MAC CE reports confirmation of type 2 configured grants. | Discussion based on contributions |
| It is assumed that index i for RLCi field is determined by ascending order of logical channel ID of secondary RLC entities in MCG and SCG. But it may need a confirmation. | Discussion based on contributions |
| How the ON/OFF of PDCP duplication is signalled is FFS. It may be specified in PDCP specification. | Discussion based on contributions |
| It is FFS whether full DRB ID is included in the MAC CE, instead of DRBdup Index. | Discussion based on contributions |

## 2.5	UE feature list (TS 38.822, 38.306)

| Editor’s note / open issue | How to handle |
| --- | --- |
| The need for Ethernet padding removal and Ethernet padding addition features is to be confirmed during RAN2#109. | Discussion based on contributions |
| It is FFS whether Ethernet padding removal capability is needed (if its support is confirmed). | Discussion based on contributions |
| It is FFS whether LCH priority-based prioritization covers both data/data prioritization and SR/data prioritization or if separate features are needed. | Discussion based on contributions |
| It is FFS whether “Additional DL SPS periodicities” and “Additional CG periodicities” features require the signalling of maximum value of N supported by the UE. | Discussion based on contributions |
| It is FFS whether “PDCP duplication with more than two RLC entities” feature needs to be separate for UM and AM RLC modes. | Discussion based on contributions |
| It is FFS whether a capability for EHC context continuation and maxNumberEHC-ContextSessions-r16 are needed (pending the discussion on remaining aspects of EHC). | Discussion based on contributions |
| It is FFS whether a UE supporting [multipleCG-Configs-r16] shall also support lch-ToConfiguredGrantMapping-r16. | Discussion based on contributions |