3GPP TSG-RAN WG2 Meeting #109e	R2-2001140

# Online, 24 February – 6 March 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1737 | rev | - | Current version: | 13.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on codebook-HARQ-ACK-r13 capability for CA with more than 5CCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_enh_b5C-Core |  |  |  |  |  | Date: |  |  | 2020-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To indicate a method which UE supports to determine HARQ-ACK codebook size, the UE signals the capability: codebook-HARQ-ACK-r13 (see: TS36.331) as follows: - first bit of the capability set to "1" implies the UE supports the DAI-based codebook size determination - second bit is set to "1" if the UE supports the codebook determination based on the number of configured CCs. To support Rel-13 CA aggregation enhancements, RAN1#82bis agreed (see: report): - eNodeB can configure by RRC signaling an eCA UE to determine HARQ-ACK codebook size according to either (a) or (b) as follows: a) DAI based solution (…) b) Number of configured CCs based solution (…) - Both solution (a) and solution (b) are mandatory feature as UE capability from RAN1 recommendation point of views for UEs supporting more than 5 CCs The requirement added on codebook-HARQ-ACK-r13 in this specification, that was supposed to follow the recommendation, does not explicitly reflect the RAN1 agreement: “For both solutions, it is mandatory for UEs of this release of the specification if carrier aggregation with more than 5 DL component carriers is supported. “ |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following change is made: - codebook-HARQ-ACK-r13 capability is updated to reflect better RAN1#82bis agreement: - Both solution (a) and solution (b) are mandatory feature as UE capability from RAN1 recommendation point of views for UEs supporting more than 5 CCs Impact analysis Impacted functionality: functionality impacted. Inter-operability: - If the network is implemented according to the CR and the UE is not the UE may use single HARQ-ACK codebook size determination method that won’t allow more than 5 CCs configuration. - If the UE is implemented according to the CR and the network is not there is no interoperability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Different interpretations regarding support of HARQ-ACK codebook size determination methods exist. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.4.42 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Modified Subclause

#### 4.3.4.42	codebook-HARQ-ACK-r13

This field defines whether HARQ ACK codebook size based on the DAI-based solution and/or the number of configured CCs as specified in TS 36.213 [22] is supported by the UE. If the UE supports carrier aggregation with more than 5 DL component carriers, it is mandatory to support both solutions (i.e. DAI-based solution and the number of configured CCs) for this UE .

End of Modified Subclause