3GPP TSG-RAN WG2 #109-e	R2-2001186

# Electronic meeting, 24 Feb - 6 Mar 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1415 | rev | 1 | Current version: | 15.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on SCell release |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-02-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to the current version of 38.331, in 5.3.5.5.8, the SCell configuration is released but it is not mentioned what happens with parts of the UE configuration that have a parameter of type ServCellIndex referring to the deleted serving cell. For example, the CSI configuration of another (not deleted) serving cell of the same cell group can include a report to be sent on that serving cells: - CellGroupConfig-> sCellToAddModList-> SCellConfig-> sCellConfigDedicated-> ServingCellConfig->csi-MeasConfig-> csi-ReportConfigToAddModList-> CSI-ReportConfig-> carrier-> ServCellIndex - CellGroupConfig-> SpCellConfig-> spCellConfigDedicated-> ServingCellConfig-> csi-MeasConfig-> csi-ReportConfigToAddModList-> CSI-ReportConfig-> carrier-> ServCellIndex If the network does not explicitly remove any reference to a non-existing serving cell, it is unclear whether the UE will autonomously delete the corresponding items (as they cannot be used) or suspend them for later reuse and another serving cell is later configured with the same value of ServCellIndex. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add a clarification in “release the SCell”, to show that this section is only about the entry for this SCell index in sCellToAddModList. - Add a note to clarify that any reference to a deleted and not re-added SCell should be removed explicitly. Impact analysis Impacted 5G architecture options: Standalone, EN-DC, NE-DC, NR-DC Impacted functionality: SCell release. Inter-operability: 1. If the network is implemented according to the CR and the UE is not, there is no inter-operability issue because the network will avoid ambiguous situations. 2. If the UE is implemented according to the CR and the network is not, the consequences if not approved remain. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Configuration mismatch between the UE and the network can occur if an RRC message deletes an SCell while not explictly removing references to the deleted SCell in other parts of the UE configuration (unclear if the UE deletes or keeps them until another SCell is configured with the same SCell index). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.5.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE START

##### 5.3.5.5.8	SCell Release

The UE shall:

1>	if the release is triggered by reception of the sCellToReleaseList:

2>	for each sCellIndex value included in the sCellToReleaseList:

3>	if the current UE configuration includes an SCell with value sCellIndex:

4>	release any parameter configured through an entry in sCellToAddModList identified by this value of sCellIndex.

NOTE:	If a reference to a deleted and not re-added sCellIndex value is not removed explicity, either according to this procedure or according to another procedure triggered by the same RRC message, the UE behaviour is unspecified.

FIRST CHANGE END