3GPP TSG-RAN WG2 #109-e	R2-2001278

Electronic meeting, 24 February – 6 March 2020

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.355 | CR | draftCR | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Single positioning method approach in LPP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  | Date: |  |  | 2020-02-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Develop a detailed proposal for a single positioning method to compare with the multiple methods in the running CR |  |  |  |  |  |  |  |  |
| Summary of change: |  | RAN2-108 (based on R2-1914728): 1 Updated the impacted spec to TS37.355 2 Updated RAN1 parameters based on DRAFT R1-1913674 and endorsed R1-1913536 3 Updated measurement reporting structure based on R2-1915652; 4 Captured following agreements: - For Multi-RTT positioning, the DL-PRS information for the candidate TRPs are provided by an LMF to the UE in an LPP Provide Assistance Data message. - The time/frequency occupancy of the DL-PRS required in the UL-PRS (SRS) information is provided as part of the DL-PRS assistance data for Multi-RTT positioning. UL-PRS (SRS) information includes an index/pointer to the relevant information in the DL-PRS assistance data (e.g., DL-PRS Resource Set ID/Resource ID). - The time/frequency occupancy of the SSBs required in both, DL-PRS and UL-PRS is grouped in a single IE, and a pointer/index is used to reference the required information. RAN2-108: NR dependent positioning: To capture RAN1 parameters (agreed in R1-1911564) into stage 3 specification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR Positioning respresentation in LPP will be inefficient and the benefits of gathering information elements at one logical place are not realized. The length of the specification will be significantly longer. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Based on TS37.355 15.0.0 |  |  |  |  |  |  |  |  |

- First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.305: "Stage 2 functional specification of User Equipment (UE) positioning in E-UTRAN".

[3]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[4]	IS-GPS-200, Revision D, Navstar GPS Space Segment/Navigation User Interfaces, March 7th, 2006.

[5]	IS-GPS-705, Navstar GPS Space Segment/User Segment L5 Interfaces, September 22, 2005.

[6]	IS-GPS-800, Navstar GPS Space Segment/User Segment L1C Interfaces, September 4, 2008.

[7]	IS-QZSS, Quasi Zenith Satellite System Navigation Service Interface Specifications for QZSS, Ver.1.1, July 31, 2009.

[8]	Galileo OS Signal in Space ICD (OS SIS ICD), Issue 1.2, February 2014, European Union.

[9]	Global Navigation Satellite System GLONASS Interface Control Document, Version 5.1, 2008.

[10]	Specification for the Wide Area Augmentation System (WAAS), US Department of Transportation, Federal Aviation Administration, DTFA01-96-C-00025, 2001.

[11]	RTCM-SC104, RTCM Recommended Standards for Differential GNSS Service (v.2.3), August 20, 2001.

[12]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); "Radio Resource Control (RRC); Protocol specification".

[13]	3GPP TS 25.331: "Radio Resource Control (RRC); Protocol Specification".

[14]	3GPP TS 44.031: "Location Services (LCS); Mobile Station (MS) - Serving Mobile Location Centre (SMLC) Radio Resource LCS Protocol (RRLP)".

[15]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[16]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical Channels and Modulation".

[17]	3GPP TS 36.214: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer – Measurements".

[18]	3GPP TS 36.133: "Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management".

[19]	3GPP TS 23.003: "Numbering, addressing and identification".

[20]	OMA-TS-LPPe-V1_0, LPP Extensions Specification, Open Mobile Alliance.

[21]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[22]	ITU-T Recommendation X.691 (07/2002) "Information technology - ASN.1 encoding rules: Specification of Packed Encoding Rules (PER)" (Same as the ISO/IEC International Standard 8825-2).

[23]	BDS-SIS-ICD-2.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal (Version 2.0)", December 2013.

[24]	ATIS-0500027: "Recommendations for Establishing Wide Scale Indoor Location Performance", May 2015.

[25]	Bluetooth Special Interest Group: "Bluetooth Core Specification v4.2", December 2014.

[26]	IEEE 802.11, Part 11: "Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

[27]	IETF RFC 6225, "Dynamic Host Configuration Protocol Options for Coordinate-Based Location Configuration Information", July 2011.

[28]	3GPP TS 36.213: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures".

[29]	"Earth Gravitational Model 96 (EGM96)", National Geospatial-Intelligence Agency, NASA.

[30]	RTCM Standard 10403.3: "Differential GNSS (Global Navigation Satellite Systems) Services" – Version 3, October 7, 2016.

[31]	IGS ANTEX: "The Antenna Exchanged Format" – version 1.4, September 15, 2010.

[32]	Federal Information Processing Standards Publication 197, "Specification for the ADVANCED ENCRYPTION STANDARD (AES)", November 26, 2001.

[33]	NIST Special Publication 800-38A, "Recommendation for Block Cipher Modes of Operation Methods and Techniques", 2001.

[34]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[35]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[36]	3GPP TS 38.215: "NR; Physical layer measurements".

[37]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[x1]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[x2]	3GPP TS 38.211: "3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Physical channels and modulation".

[x3]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

- Next change

# 3	Definitions and Abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], TS 36.305 [2] and TS 23.271 [3] apply. Other definitions are provided below.

Anchor carrier: In NB-IoT, a carrier where the UE assumes that NPSS/NSSS/NPBCH/SIB-NB for FDD or NPSS/NSSS/NPBCH for TDD are transmitted.

Location Server: a physical or logical entity (e.g., E-SMLC or SUPL SLP) that manages positioning for a target device by obtaining measurements and other location information from one or more positioning units and providing assistance data to positioning units to help determine this. A Location Server may also compute or verify the final location estimate.

NB-IoT: NB-IoT allows access to network services via E-UTRA with a channel bandwidth limited to 200 kHz.

Reference Source: a physical entity or part of a physical entity that provides signals (e.g., RF, acoustic, infra-red) that can be measured (e.g., by a Target Device) in order to obtain the location of a Target Device.

Target Device: the device that is being positioned (e.g., UE or SUPL SET).

Transmission Point (TP): A set of geographically co-located transmit antennas for one cell, part of one cell or one PRS-only TP. Transmission Points can include base station (eNodeB) antennas, remote radio heads, a remote antenna of a base station, an antenna of a PRS-only TP, etc. One cell can be formed by one or multiple transmission points. For a homogeneous deployment, each transmission point may correspond to one cell.

Observed Time Difference Of Arrival (OTDOA): The time interval that is observed by a target device between the reception of downlink signals from two different TPs. If a signal from TP 1 is received at the moment t1, and a signal from TP 2 is received at the moment t2, the OTDOA is t2 – t1.

PRS-only TP: A TP which only transmits PRS signals for PRS-based TBS positioning and is not associated with a cell.

Transmission-Reception Point (TRP): A set of geographically co-located antennas for transmission and reception functionalities associated to positioning

Relative Time Difference (RTD): The relative time difference between a TRP i and a TRP j, is defined as tj – ti, where ti and tj are defined as the time when TRP i and j transmit the start of one subframe respectively.

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply.

ADR	Accumulated Delta-Range

A-GNSS	Assisted-GNSS

AP	Access Point

ARFCN	Absolute Radio Frequency Channel Number

ARP	Antenna Reference Point

BDS	BeiDou Navigation Satellite System

BSSID	Basic Service Set Identifier

BTS	Base Transceiver Station (GERAN)

CID	Cell-ID (positioning method)

CNAV	Civil Navigation

CRS	Cell-specific Reference Signals

DL-AOD	Downlink Azimuth Angle Of Departure

DL-TDOA	Downlink Time Difference Of Arrival

ECEF	Earth-Centered, Earth-Fixed

ECGI	Evolved Cell Global Identifier

ECI	Earth-Centered-Inertial

E-CID	Enhanced Cell-ID (positioning method)

EGNOS	European Geostationary Navigation Overlay Service

E-SMLC	Enhanced Serving Mobile Location Centre

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

EOP	Earth Orientation Parameters

EPDU	External Protocol Data Unit

FDMA	Frequency Division Multiple Access

FEC	Forward Error Correction

FKP	(German) Flächen-Korrektur-Parameter (area correction parameter)

FTA	Fine Time Assistance

GAGAN	GPS Aided Geo Augmented Navigation

GLONASS	GLObal'naya NAvigatsionnaya Sputnikovaya Sistema (Engl.: Global Navigation Satellite System)

GNSS	Global Navigation Satellite System

GPS	Global Positioning System

HA GNSS	High-Accuracy GNSS (RTK, PPP)

ICD	Interface Control Document

IGS	International GNSS Service

IOD	Issue of Data

IS	Interface Specification

LLA	Latitude Longitude Altitude

LMF	Location Management Function

LPP	LTE Positioning Protocol

LPPa	LTE Positioning Protocol Annex

LSB	Least Significant Bit

MAC	Master Auxiliary Concept

MBS	Metropolitan Beacon System

MO-LR	Mobile Originated Location Request

MSAS	Multi-functional Satellite Augmentation System

MSB	Most Significant Bit

msd	mean solar day

MT-LR	Mobile Terminated Location Request

Multi-RTT	Mulitple-Round Trip Time

NAV	Navigation

NB-IoT	NarrowBand Internet of Things

NCGI	NR Cell Global Identifier

NICT	National Institute of Information and Communications Technology

NI-LR	Network Induced Location Request

NPRS	Narrowband Positioning Reference Signals

NR	NR Radio Access

NRSRP	Narrowband Reference Signal Received Power

NRSRQ	Narrowband Reference Signal Received Quality

NTSC	National Time Service Center of Chinese Academy of Sciences

OSR	Observation Space Representation

OTDOA	Observed Time Difference Of Arrival

PDU	Protocol Data Unit

PPP	Precise Point Positioning

PRB	Physical Resource Block

PRC	Pseudo-Range Correction

PRS	Positioning Reference Signals

posSIB	Positioning System Information Block

PZ-90	Parametry Zemli 1990 Goda – Parameters of the Earth Year 1990

QZS	Quasi Zenith Satellite

QZSS	Quasi-Zenith Satellite System

QZST	Quasi-Zenith System Time

RF	Radio Frequency

RRC	Range-Rate Correction

Radio Resource Control

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

RSTD	Reference Signal Time Difference

RTK	Real-Time Kinematic

RTT	Round Trip Time

RU	Russia

SBAS	Space Based Augmentation System

SET	SUPL Enabled Terminal

SFN	System Frame Number

SLP	SUPL Location Platform

SSID	Service Set Identifier

SSR	State Space Representation

SUPL	Secure User Plane Location

SV	Space Vehicle

TB	Terrestrial Beacon

TBS	Terrestrial Beacon System

TLM	Telemetry

TOA	Time Of Arrival

TOD	Time Of Day

TOW	Time Of Week

TP	Transmission Point

TRP	Transmission-Reception Point

UDRE	User Differential Range Error

ULP	User Plane Location Protocol

USNO	US Naval Observatory

UT1	Universal Time No.1

UTC	Coordinated Universal Time

WAAS	Wide Area Augmentation System

WGS-84	World Geodetic System 1984

WLAN	Wireless Local Area Network

# 4	Functionality of Protocol

## 4.1	General

### 4.1.1	LPP Configuration

LPP is used point-to-point between a location server (E-SMLC. LMF or SLP) and a target device (UE or SET) in order to position the target device using position-related measurements obtained by one or more reference sources. Figure 4.1.1-1 shows the configuration as applied to the control- and user-plane location solutions for E-UTRAN and NG-RAN (as defined in TS 36.305 [2], TS 38.305 [x1] , TS 23.273 [x3]  and TS 23.271 [3]).

NB-IoT is a non-backward compatible variant of E-UTRAN supporting a reduced set of functionalities. In this specification, procedures and messages specified for the UE equally apply to the UE in NB-IoT.

# Figure 4.1.1-1: LPP Configuration for Control- and User-Plane Positioning in E-UTRAN or NG-RAN

### 4.1.2	LPP Sessions and Transactions

An LPP session is used between a Location Server and the target device in order to obtain location related measurements or a location estimate or to transfer assistance data. A single LPP session is used to support a single location request (e.g., for a single MT-LR, MO-LR or NI-LR). Multiple LPP sessions can be used between the same endpoints to support multiple different location requests (as required by TS 23.271 [3]). Each LPP session comprises one or more LPP transactions, with each LPP transaction performing a single operation (capability exchange, assistance data transfer, or location information transfer). In E-UTRAN and NG-RAN, the LPP transactions are realized as LPP procedures. The instigator of an LPP session will always instigate the first LPP transaction, but subsequent transactions may be instigated by either end. LPP transactions within a session may occur serially or in parallel. LPP transactions are indicated at the LPP protocol level with a transaction ID in order to associate messages with one another (e.g., request and response).

Messages within a transaction are linked by a common transaction identifier.

### 4.1.3	LPP Position Methods

Internal LPP positioning methods and associated signalling content are defined in this specification.

This version of the specification defines OTDOA (based on LTE signals), A-GNSS, E-CID (based on LTE signals), Sensor, TBS, WLAN, Bluetooth, NR E-CID, NR DL-TDOA, NR DL-AOD and NR Multi-RTT positioning methods.

- Next change

# 5	LPP Procedures

## 5.1	Procedures related to capability transfer

The purpose of the procedures that are grouped together in this clause is to enable the transfer of capabilities from the target device to the server. Capabilities in this context refer to positioning and protocol capabilities related to LPP and the positioning methods supported by LPP.

These procedures instantiate the Capability Transfer transaction from TS 36.305 [2] and TS 38.305 [x1].

### 5.1.1	Capability Transfer procedure

The Capability Transfer procedure is shown in Figure 5.1.1-1.

# Figure 5.1.1-1: LPP Capability Transfer procedure

1.	The server sends a RequestCapabilities message to the target. The server may indicate the types of capability needed.

2.	The target responds with a ProvideCapabilities message to the server. The capabilities shall correspond to any capability types specified in step 1. This message shall include the endTransaction IE set to TRUE.

### 5.1.2	Capability Indication procedure

The Capability Indication procedure allows the target to provide unsolicited capabilities to the server and is shown in Figure 5.1.2-1.

# Figure 5.1.2-1: LPP Capability Indication procedure

1.	The target sends a ProvideCapabilities message to the server. This message shall include the endTransaction IE set to TRUE.

### 5.1.3	Reception of LPP Request Capabilities

Upon receiving a RequestCapabilities message, the target device shall generate a ProvideCapabilities message as a response.

The target device shall:

1>	for each positioning method for which a request for capabilities is included in the message:

2>	if the target device supports this positioning method:

3>	include the capabilities of the device for that supported positioning method in the response message;

1>	set the IE LPP-TransactionID in the response message to the same value as the IE LPP-TransactionID in the received message;

1>	deliver the response message to lower layers for transmission.

### 5.1.4	Transmission of LPP Provide Capabilities

When triggered to transmit a ProvideCapabilities message, the target device shall:

1>	for each positioning method whose capabilities are to be indicated:

2>	set the corresponding IE to include the device's capabilities;

2>	if OTDOA capabilities are to be indicated:

3>	include the IE supportedBandListEUTRA;

1>	deliver the response to lower layers for transmission.

Editor’s Note: FFS on whether supportedBandListNR is needed.

## 5.2	Procedures related to Assistance Data Transfer

The purpose of the procedures in this clause is to enable the target to request assistance data from the server to assist in positioning, and to enable the server to transfer assistance data to the target in the absence of a request.

These procedures instantiate the Assistance Data Transfer transaction from TS 36.305 [2] and TS 38.305 [x1].

- Next change

## 5.3	Procedures related to Location Information Transfer

The purpose of the procedures in this clause is to enable the server to request location measurement data and/or a location estimate from the target, and to enable the target to transfer location measurement data and/or a location estimate to a server in the absence of a request.

These procedures instantiate the Location Information Transfer transaction in TS 36.305 [2] and TS 38.305 [x1].

NOTE:	The service layer (e.g. NAS or OMA SUPL ULP) would be used to transfer information associated with a location request from a target to a server (MO-LR).

- Next change

## 6.3	Message Body IEs

Editor’s Note: FFS on whether use use critical extension for Rel-16 to branch out message body for LTE and combined LTE and NR.

#### –	RequestCapabilities

The RequestCapabilities message body in a LPP message is used by the location server to request the target device capability information for LPP and the supported individual positioning methods.

-- ASN1START

# RequestCapabilities ::= SEQUENCE {

criticalExtensions		CHOICE {

c1						CHOICE {

requestCapabilities-r9		RequestCapabilities-r9-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture	SEQUENCE {}

}

}

# RequestCapabilities-r9-IEs ::= SEQUENCE {

commonIEsRequestCapabilities		CommonIEsRequestCapabilities		OPTIONAL,	-- Need ON

a-gnss-RequestCapabilities			A-GNSS-RequestCapabilities			OPTIONAL,	-- Need ON

otdoa-RequestCapabilities			OTDOA-RequestCapabilities			OPTIONAL,	-- Need ON

ecid-RequestCapabilities			ECID-RequestCapabilities			OPTIONAL,	-- Need ON

# epdu-RequestCapabilities			EPDU-Sequence						OPTIONAL,	-- Need ON

# ...,

# [[	sensor-RequestCapabilities-r13	Sensor-RequestCapabilities-r13		OPTIONAL,	-- Need ON

# tbs-RequestCapabilities-r13		TBS-RequestCapabilities-r13			OPTIONAL,	-- Need ON

# wlan-RequestCapabilities-r13	WLAN-RequestCapabilities-r13		OPTIONAL,	-- Need ON

# bt-RequestCapabilities-r13		BT-RequestCapabilities-r13			OPTIONAL	-- Need ON

# ]] ,

# [[	nr-ecid-RequestCapabilities-r16	NR-ECID-RequestCapabilities-r16		OPTIONAL,	-- Need ON

# nr-pos-RequestCapabilities-r16	NR-Pos-RequestCapabilities-r16		OPTIONAL,	-- Need ON

# ]]

}

-- ASN1STOP

#### –	ProvideCapabilities

The ProvideCapabilities message body in a LPP message indicates the LPP capabilities of the target device to the location server.

-- ASN1START

# ProvideCapabilities ::= SEQUENCE {

criticalExtensions		CHOICE {

c1						CHOICE {

provideCapabilities-r9		ProvideCapabilities-r9-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture	SEQUENCE {}

}

}

# ProvideCapabilities-r9-IEs ::= SEQUENCE {

commonIEsProvideCapabilities		CommonIEsProvideCapabilities		OPTIONAL,

a-gnss-ProvideCapabilities			A-GNSS-ProvideCapabilities			OPTIONAL,

otdoa-ProvideCapabilities			OTDOA-ProvideCapabilities			OPTIONAL,

ecid-ProvideCapabilities			ECID-ProvideCapabilities			OPTIONAL,

# epdu-ProvideCapabilities			EPDU-Sequence						OPTIONAL,

# ...,

# [[	sensor-ProvideCapabilities-r13	Sensor-ProvideCapabilities-r13		OPTIONAL,

# tbs-ProvideCapabilities-r13		TBS-ProvideCapabilities-r13			OPTIONAL,

# wlan-ProvideCapabilities-r13	WLAN-ProvideCapabilities-r13		OPTIONAL,

# bt-ProvideCapabilities-r13		BT-ProvideCapabilities-r13			OPTIONAL

# ]],

# [[	nr-ecid-ProvideCapabilities-r16	NR-ECID-ProvideCapabilities-r16		OPTIONAL,

# nr-pos-ProvideCapabilities-r16	NR-Pos-ProvideCapabilities-r16		OPTIONAL

# ]]

}

-- ASN1STOP

#### –	RequestAssistanceData

The RequestAssistanceData message body in a LPP message is used by the target device to request assistance data from the location server.

-- ASN1START

# RequestAssistanceData ::= SEQUENCE {

criticalExtensions		CHOICE {

c1						CHOICE {

requestAssistanceData-r9	RequestAssistanceData-r9-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture	SEQUENCE {}

}

}

# RequestAssistanceData-r9-IEs ::= SEQUENCE {

commonIEsRequestAssistanceData		CommonIEsRequestAssistanceData		OPTIONAL,

a-gnss-RequestAssistanceData		A-GNSS-RequestAssistanceData		OPTIONAL,

otdoa-RequestAssistanceData			OTDOA-RequestAssistanceData			OPTIONAL,

epdu-RequestAssistanceData			EPDU-Sequence						OPTIONAL,

...,

# [[	sensor-RequestAssistanceData-r14

# Sensor-RequestAssistanceData-r14	OPTIONAL,

# tbs-RequestAssistanceData-r14	TBS-RequestAssistanceData-r14		OPTIONAL,

# wlan-RequestAssistanceData-r14	WLAN-RequestAssistanceData-r14		OPTIONAL

# ]],

# [[

# nr-ecid-RequestAssistanceData-r16	NR-ECID-RequestAssistanceData-r16	OPTIONAL,

# nr-pos-RequestAssistanceData-r16	NR-Pos-RequestAssistanceData-r16	OPTIONAL

# ]]

}

-- ASN1STOP

#### –	ProvideAssistanceData

The ProvideAssistanceData message body in a LPP message is used by the location server to provide assistance data to the target device either in response to a request from the target device or in an unsolicited manner.

-- ASN1START

# ProvideAssistanceData ::= SEQUENCE {

criticalExtensions		CHOICE {

c1						CHOICE {

provideAssistanceData-r9	ProvideAssistanceData-r9-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture	SEQUENCE {}

}

}

# ProvideAssistanceData-r9-IEs ::= SEQUENCE {

commonIEsProvideAssistanceData		CommonIEsProvideAssistanceData		OPTIONAL,	-- Need ON

a-gnss-ProvideAssistanceData		A-GNSS-ProvideAssistanceData		OPTIONAL,	-- Need ON

otdoa-ProvideAssistanceData			OTDOA-ProvideAssistanceData			OPTIONAL,	-- Need ON

epdu-Provide-Assistance-Data		EPDU-Sequence						OPTIONAL,	-- Need ON

...,

# [[

# sensor-ProvideAssistanceData-r14	Sensor-ProvideAssistanceData-r14	OPTIONAL,	-- Need ON

# tbs-ProvideAssistanceData-r14		TBS-ProvideAssistanceData-r14		OPTIONAL,	-- Need ON

# wlan-ProvideAssistanceData-r14		WLAN-ProvideAssistanceData-r14		OPTIONAL	-- Need ON

# ]],

# [[

# nr-ecid-ProvideAssistanceData-r16	NR-ECID-ProvideAssistanceData-r16	OPTIONAL,	-- Need ON

# nr-pos-ProvideAssistanceData-r16	NR-Pos-ProvideAssistanceData-r16	OPTIONAL	-- Need ON

# ]]

}

-- ASN1STOP

ProvideAssistanceData field descriptions

commonIEsProvideAssistanceData This IE is provided for future extensibility and should not be included in this version of the protocol.

#### –	RequestLocationInformation

The RequestLocationInformation message body in a LPP message is used by the location server to request positioning measurements or a position estimate from the target device.

-- ASN1START

# RequestLocationInformation ::= SEQUENCE {

criticalExtensions		CHOICE {

c1						CHOICE {

requestLocationInformation-r9	RequestLocationInformation-r9-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture	SEQUENCE {}

}

}

# RequestLocationInformation-r9-IEs ::= SEQUENCE {

commonIEsRequestLocationInformation

CommonIEsRequestLocationInformation	OPTIONAL,	-- Need ON

a-gnss-RequestLocationInformation	A-GNSS-RequestLocationInformation	OPTIONAL,	-- Need ON

otdoa-RequestLocationInformation	OTDOA-RequestLocationInformation	OPTIONAL,	-- Need ON

ecid-RequestLocationInformation		ECID-RequestLocationInformation		OPTIONAL,	-- Need ON

# epdu-RequestLocationInformation		EPDU-Sequence						OPTIONAL,	-- Need ON

# ...,

# [[

# sensor-RequestLocationInformation-r13

# Sensor-RequestLocationInformation-r13

# OPTIONAL,	-- Need ON

# tbs-RequestLocationInformation-r13	TBS-RequestLocationInformation-r13	OPTIONAL,	-- Need ON

# wlan-RequestLocationInformation-r13	WLAN-RequestLocationInformation-r13	OPTIONAL,	-- Need ON

# bt-RequestLocationInformation-r13	BT-RequestLocationInformation-r13	OPTIONAL	-- Need ON

# ]],

# [[

# nr-ecid-RequestLocationInformation-r16	NR-ECID-RequestLocationInformation-r16	OPTIONAL,	-- Need ON

# nr-pos-RequestLocationInformation-r16 	NR-Pos-RequestLocationInformation-r16	OPTIONAL 	-- Need ON

# ]]

}

-- ASN1STOP

RequestLocationInformation field descriptions

commonIEsRequestLocationInformation This field specifies the location information type requested by the location server and optionally other configuration information associated with the requested location information. This field should always be included in this version of the protocol.

#### –	ProvideLocationInformation

The ProvideLocationInformation message body in a LPP message is used by the target device to provide positioning measurements or position estimates to the location server.

-- ASN1START

# ProvideLocationInformation ::= SEQUENCE {

criticalExtensions		CHOICE {

c1						CHOICE {

provideLocationInformation-r9	ProvideLocationInformation-r9-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture	SEQUENCE {}

}

}

# ProvideLocationInformation-r9-IEs ::= SEQUENCE {

commonIEsProvideLocationInformation

CommonIEsProvideLocationInformation	OPTIONAL,

a-gnss-ProvideLocationInformation	A-GNSS-ProvideLocationInformation	OPTIONAL,

otdoa-ProvideLocationInformation	OTDOA-ProvideLocationInformation	OPTIONAL,

ecid-ProvideLocationInformation		ECID-ProvideLocationInformation		OPTIONAL,

# epdu-ProvideLocationInformation		EPDU-Sequence						OPTIONAL,

# ...,

# [[

# sensor-ProvideLocationInformation-r13

# Sensor-ProvideLocationInformation-r13

# OPTIONAL,

# tbs-ProvideLocationInformation-r13	TBS-ProvideLocationInformation-r13	OPTIONAL,

# wlan-ProvideLocationInformation-r13	WLAN-ProvideLocationInformation-r13	OPTIONAL,

# bt-ProvideLocationInformation-r13	BT-ProvideLocationInformation-r13	OPTIONAL

# ]],

# [[

# nr-ecid-ProvideLocationInformation-r16	NR-ECID-ProvideLocationInformation-r16	OPTIONAL,

# nr-pos-ProvideLocationInformation-r16	NR-Pos-ProvideLocationInformation-r16	OPTIONAL,

# ]]

}

-- ASN1STOP

Editor’s Note: FFS on how to capture NR E-CID, Multi-RTT, DL-AoD, DL-TDOA for NR, introduce highlevel IE NR-Positioning and common subsection NR positioning measurements or define them as separate IEs and separate subsections.

- Next change

## 6.4	Common IEs

Common IEs comprise IEs that are applicable to more than one LPP positioning method.

### 6.4.1	Common Lower-Level IEs

#### –	AccessTypes

The IE AccessTypes is used to indicate several cellular access types using a bit map.

-- ASN1START

# AccessTypes ::= SEQUENCE {

accessTypes		BIT STRING {	eutra		(0),

utra		(1),

gsm			(2),

nb-iot		(3),

nr-v1510	(4) } (SIZE (1..8)),

...

}

-- ASN1STOP

AccessTypes field descriptions

accessTypes This field specifies the cellular access type(s). This is represented by a bit string, with a one-value at the bit position means the particular access type is addressed; a zero-value means not addressed.

#### –	ARFCN-ValueEUTRA

The IEs ARFCN-ValueEUTRA and ARFCN-ValueEUTRA-v9a0 are used to indicate the ARFCN of the E-UTRA carrier frequency, as defined in TS 36.331 [12].

-- ASN1START

# ARFCN-ValueEUTRA ::= INTEGER (0..maxEARFCN)

ARFCN-ValueEUTRA-v9a0 ::=	INTEGER (maxEARFCN-Plus1..maxEARFCN2)

ARFCN-ValueEUTRA-r14 ::=	INTEGER (0..maxEARFCN2)

maxEARFCN					INTEGER ::= 65535	-- Maximum value of EUTRA carrier frequency

maxEARFCN-Plus1				INTEGER ::= 65536	-- Lowest value extended EARFCN range

maxEARFCN2					INTEGER ::= 262143	-- Highest value extended EARFCN range

-- ASN1STOP

NOTE:	For fields using the original value range, as defined by IE ARFCN-ValueEUTRA i.e. without suffix, value maxEARFCN indicates that the E-UTRA carrier frequency is indicated by means of an extension.

#### –	ARFCN-ValueNR

The IE ARFCN-ValueNR is used to indicate the ARFCN applicable for a downlink, uplink or bi-directional (TDD) NR global frequency raster, as defined in TS 38.101-2 [34] and TS 38.101-1 [37].

-- ASN1START

ARFCN-ValueNR-r15 ::= INTEGER (0..3279165)

-- ASN1STOP

#### –	ARFCN-ValueUTRA

The IE ARFCN-ValueUTRA is used to indicate the ARFCN of the UTRA carrier frequency, as defined in TS 25.331 [13].

-- ASN1START

ARFCN-ValueUTRA ::=	INTEGER (0..16383)

-- ASN1STOP

#### –	CarrierFreq-NB

The IE CarrierFreq-NB is used to provide the NB-IoT carrier frequency, as defined in TS 36.101 [21].

-- ASN1START

CarrierFreq-NB-r14 ::=		SEQUENCE {

carrierFreq-r14				ARFCN-ValueEUTRA-r14,

carrierFreqOffset-r14		CarrierFreqOffsetNB-r14				OPTIONAL,

...

}

-- ASN1STOP

CarrierFreq-NB field descriptions

carrierFreq This field specifies the ARFCN applicable for the NB-IoT carrier frequency as defined in TS 36.101 [21, Table 5.7.3-1].

carrierFreqOffset This field specifies the offset of the NB-IoT channel number to EARFCN as defined in TS 36.101 [21].

#### –	CarrierFreqOffsetNB

The IE CarrierFreqOffsetNB is used to provide the offset of the NB-IoT channel number to EARFCN of a NB-IoT carrier.

-- ASN1START

CarrierFreqOffsetNB-r14 ::=		ENUMERATED {

v-10, v-9, v-8,	v-7, v-6, v-5, v-4, v-3, v-2, v-1, v-0dot5,

v0, v1, v2, v3, v4, v5, v6, v7, v8, v9

}

-- ASN1STOP

CarrierFreqOffsetNB field descriptions

CarrierFreqOffsetNB This field specifies the offset of the NB-IoT channel number to EARFCN as defined in TS 36.101 [21]. Value v-10 means -10, v-9 means -9, and so on.

#### –	CellGlobalIdEUTRA-AndUTRA

The IE CellGlobalIdEUTRA-AndUTRA specifies the global Cell Identifier for E-UTRA or UTRA, the globally unique identity of a cell in E-UTRA or UTRA.

-- ASN1START

# CellGlobalIdEUTRA-AndUTRA ::= SEQUENCE {

plmn-Identity		SEQUENCE {

mcc		SEQUENCE (SIZE (3))	OF INTEGER (0..9),

mnc		SEQUENCE (SIZE (2..3))	OF INTEGER (0..9)

},

cellIdentity		CHOICE {

eutra	BIT STRING (SIZE (28)),

utra	BIT STRING (SIZE (32))

},

...

}

-- ASN1STOP

CellGlobalIdEUTRA-AndUTRA field descriptions

plmn-Identity This field identifies the PLMN of the cell as defined in TS 36.331 [12].

cellIdentity This field defines the identity of the cell within the context of the PLMN as defined in TS 36.331 [12] and TS 25.331 [13]. The size of the bit string allows for the 32-bit extended UTRAN cell ID; in case the cell ID is shorter, the first bits of the string are set to 0.

#### –	CellGlobalIdGERAN

The IE CellGlobalIdGERAN specifies the global Cell Identifier for GERAN, the globally unique identity of a cell in GERAN.

-- ASN1START

# CellGlobalIdGERAN ::= SEQUENCE {

plmn-Identity		SEQUENCE {

mcc		SEQUENCE (SIZE (3))	OF INTEGER (0..9),

mnc		SEQUENCE (SIZE (2..3))	OF INTEGER (0..9)

},

locationAreaCode		BIT STRING (SIZE (16)),

cellIdentity			BIT STRING (SIZE (16)),

...

}

-- ASN1STOP

CellGlobalIdGERAN field descriptions

plmn-Identity This field identifies the PLMN of the cell.

locationAreaCode This field is a fixed length code identifying the location area within a PLMN.

cellIdentity This field specifies the cell Identifier which is unique within the context of the GERAN location area.

#### –	ECGI

The IE ECGI specifies the Evolved Cell Global Identifier (ECGI), the globally unique identity of a cell in E-UTRA (TS 36.331 [12]).

NOTE:	The IE ECGI is also used for NB-IoT access.

-- ASN1START

# ECGI ::= SEQUENCE {

mcc				SEQUENCE (SIZE (3))	OF INTEGER (0..9),

mnc				SEQUENCE (SIZE (2..3))	OF INTEGER (0..9),

cellidentity	BIT STRING (SIZE (28))

}

-- ASN1STOP

#### –	Ellipsoid-Point

The IE Ellipsoid-Point is used to describe a geographic shape as defined in TS 23.032 [15].

-- ASN1START

# Ellipsoid-Point ::= SEQUENCE {

latitudeSign				ENUMERATED {north, south},

degreesLatitude				INTEGER (0..8388607),			-- 23 bit field

degreesLongitude			INTEGER (-8388608..8388607)		-- 24 bit field

}

-- ASN1STOP

#### –	Ellipsoid-PointWithUncertaintyCircle

The IE Ellipsoid-PointWithUncertaintyCircle is used to describe a geographic shape as defined in TS 23.032 [15].

-- ASN1START

# Ellipsoid-PointWithUncertaintyCircle ::= SEQUENCE {

latitudeSign				ENUMERATED {north, south},

degreesLatitude				INTEGER (0..8388607),			-- 23 bit field

degreesLongitude			INTEGER (-8388608..8388607),	-- 24 bit field

uncertainty					INTEGER (0..127)

}

-- ASN1STOP

#### –	EllipsoidPointWithUncertaintyEllipse

The IE EllipsoidPointWithUncertaintyEllipse is used to describe a geographic shape as defined in TS 23.032 [15].

-- ASN1START

# EllipsoidPointWithUncertaintyEllipse ::= SEQUENCE {

latitudeSign				ENUMERATED {north, south},

degreesLatitude				INTEGER (0..8388607),			-- 23 bit field

degreesLongitude			INTEGER (-8388608..8388607),	-- 24 bit field

uncertaintySemiMajor		INTEGER (0..127),

uncertaintySemiMinor		INTEGER (0..127),

orientationMajorAxis		INTEGER (0..179),

confidence					INTEGER (0..100)

}

-- ASN1STOP

#### –	EllipsoidPointWithAltitude

The IE EllipsoidPointWithAltitude is used to describe a geographic shape as defined in TS 23.032 [15].

-- ASN1START

# EllipsoidPointWithAltitude ::= SEQUENCE {

latitudeSign				ENUMERATED {north, south},

degreesLatitude				INTEGER (0..8388607),			-- 23 bit field

degreesLongitude			INTEGER (-8388608..8388607),	-- 24 bit field

altitudeDirection			ENUMERATED {height, depth},

altitude					INTEGER (0..32767)				-- 15 bit field

}

-- ASN1STOP

#### –	EllipsoidPointWithAltitudeAndUncertaintyEllipsoid

The IE EllipsoidPointWithAltitudeAndUncertaintyEllipsoid is used to describe a geographic shape as defined in TS 23.032 [15].

-- ASN1START

# EllipsoidPointWithAltitudeAndUncertaintyEllipsoid ::= SEQUENCE {

latitudeSign				ENUMERATED {north, south},

degreesLatitude				INTEGER (0..8388607),			-- 23 bit field

degreesLongitude			INTEGER (-8388608..8388607),	-- 24 bit field

altitudeDirection			ENUMERATED {height, depth},

altitude					INTEGER (0..32767),				-- 15 bit field

uncertaintySemiMajor		INTEGER (0..127),

uncertaintySemiMinor		INTEGER (0..127),

orientationMajorAxis		INTEGER (0..179),

uncertaintyAltitude			INTEGER (0..127),

confidence					INTEGER (0..100)

}

-- ASN1STOP

#### –	EllipsoidArc

The IE EllipsoidArc is used to describe a geographic shape as defined in TS 23.032 [15].

-- ASN1START

# EllipsoidArc ::= SEQUENCE {

latitudeSign				ENUMERATED {north, south},

degreesLatitude				INTEGER (0..8388607),			-- 23 bit field

degreesLongitude			INTEGER (-8388608..8388607),	-- 24 bit field

innerRadius					INTEGER (0..65535),				-- 16 bit field,

uncertaintyRadius			INTEGER (0..127),

offsetAngle					INTEGER (0..179),

includedAngle				INTEGER (0..179),

confidence					INTEGER (0..100)

}

-- ASN1STOP

#### –	EPDU-Sequence

The EPDU-Sequence contains IEs that are defined externally to LPP by other organizations.

-- ASN1START

# EPDU-Sequence ::= SEQUENCE (SIZE (1..maxEPDU)) OF EPDU

maxEPDU INTEGER ::= 16

# EPDU ::= SEQUENCE {

ePDU-Identifier			EPDU-Identifier,

ePDU-Body				EPDU-Body

}

# EPDU-Identifier ::= SEQUENCE {

ePDU-ID					EPDU-ID,

ePDU-Name				EPDU-Name		OPTIONAL,

...

}

# EPDU-ID ::= INTEGER (1..256)

# EPDU-Name ::= VisibleString (SIZE (1..32))

# EPDU-Body ::= OCTET STRING

-- ASN1STOP

EPDU-Sequence field descriptions

EPDU-ID This field provides a unique integer ID for the externally defined positioning method. Its value is assigned to the external entity that defines the EPDU. See table External PDU Identifier Definition for a list of external PDU identifiers defined in this version of the specification.

EPDU-Name This field provides an optional character encoding which can be used to provide a quasi-unique name for an external PDU – e.g., by containing the name of the defining organization and/or the name of the associated public or proprietary standard for the EPDU.

EPDU-Body The content and encoding of this field are defined externally to LPP.

# External PDU Identifier Definition

| EPDU-ID | EPDU Defining entity | Method name | Reference |
| --- | --- | --- | --- |
| 1 | OMA LOC | OMA LPP extensions (LPPe) | OMA-TS-LPPe-V1_0 [20] |

#### –	HighAccuracyEllipsoidPointWithUncertaintyEllipse

The IE HighAccuracyEllipsoidPointWithUncertaintyEllipse is used to describe a geographic shape as defined in TS 23.032 [15].

-- ASN1START

HighAccuracyEllipsoidPointWithUncertaintyEllipse-r15 ::= SEQUENCE {

degreesLatitude-r15				INTEGER(-2147483648..2147483647),

degreesLongitude-r15			INTEGER(-2147483648..2147483647),

uncertaintySemiMajor-r15		INTEGER (0..255),

uncertaintySemiMinor-r15		INTEGER (0..255),

orientationMajorAxis-r15		INTEGER (0..179),

confidence-r15					INTEGER (0..100)

}

-- ASN1STOP

#### –	HighAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid

The IE HighAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid is used to describe a geographic shape as defined in TS 23.032 [15].

-- ASN1START

HighAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid-r15 ::= SEQUENCE {

degreesLatitude-r15				INTEGER(-2147483648..2147483647),

degreesLongitude-r15			INTEGER(-2147483648..2147483647),

altitude-r15					INTEGER(-64000..1280000),

uncertaintySemiMajor-r15		INTEGER (0..255),

uncertaintySemiMinor-r15		INTEGER (0..255),

orientationMajorAxis-r15		INTEGER (0..179),

horizontalConfidence-r15		INTEGER (0..100),

uncertaintyAltitude-r15			INTEGER (0..255),

verticalConfidence-r15			INTEGER (0..100)

}

-- ASN1STOP

#### –	HorizontalVelocity

The IE HorizontalVelocity is used to describe a velocity shape as defined in TS 23.032 [15].

-- ASN1START

# HorizontalVelocity ::= SEQUENCE {

bearing						INTEGER(0..359),

horizontalSpeed				INTEGER(0..2047)

}

-- ASN1STOP

#### –	HorizontalWithVerticalVelocity

The IE HorizontalWithVerticalVelocity is used to describe a velocity shape as defined in TS 23.032 [15].

-- ASN1START

# HorizontalWithVerticalVelocity ::= SEQUENCE {

bearing						INTEGER(0..359),

horizontalSpeed				INTEGER(0..2047),

verticalDirection			ENUMERATED{upward, downward},

verticalSpeed				INTEGER(0..255)

}

-- ASN1STOP

#### –	HorizontalVelocityWithUncertainty

The IE HorizontalVelocityWithUncertainty is used to describe a velocity shape as defined in TS 23.032 [15].

-- ASN1START

# HorizontalVelocityWithUncertainty ::= SEQUENCE {

bearing						INTEGER(0..359),

horizontalSpeed				INTEGER(0..2047),

uncertaintySpeed			INTEGER(0..255)

}

-- ASN1STOP

#### –	HorizontalWithVerticalVelocityAndUncertainty

The IE HorizontalWithVerticalVelocityAndUncertainty is used to describe a velocity shape as defined in TS 23.032 [15].

-- ASN1START

# HorizontalWithVerticalVelocityAndUncertainty ::= SEQUENCE {

bearing						INTEGER(0..359),

horizontalSpeed				INTEGER(0..2047),

verticalDirection			ENUMERATED{upward, downward},

verticalSpeed				INTEGER(0..255),

horizontalUncertaintySpeed	INTEGER(0..255),

verticalUncertaintySpeed	INTEGER(0..255)

}

-- ASN1STOP

#### –	LocationCoordinateTypes

The IE LocationCoordinateTypes defines a list of possible geographic shapes as defined in TS 23.032 [15].

-- ASN1START

# LocationCoordinateTypes ::= SEQUENCE {

ellipsoidPoint											BOOLEAN,

ellipsoidPointWithUncertaintyCircle						BOOLEAN,

ellipsoidPointWithUncertaintyEllipse					BOOLEAN,

polygon													BOOLEAN,

ellipsoidPointWithAltitude								BOOLEAN,

ellipsoidPointWithAltitudeAndUncertaintyEllipsoid		BOOLEAN,

ellipsoidArc											BOOLEAN,

...,

[[

highAccuracyEllipsoidPointWithUncertaintyEllipse-r15				BOOLEAN		OPTIONAL,

highAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid-r15	BOOLEAN		OPTIONAL

]]

}

-- ASN1STOP

#### –	NCGI

The IE NCGI specifies the NR Cell Global Identifier (NCGI) which is used to identify NR cells globally (TS 38.331 [35]).

-- ASN1START

# NCGI-r15 ::= SEQUENCE {

mcc-r15					SEQUENCE (SIZE (3)) 	OF INTEGER (0..9),

mnc-r15					SEQUENCE (SIZE (2..3)) 	OF INTEGER (0..9),

nr-cellidentity-r15		BIT STRING (SIZE (36))

}

-- ASN1STOP

#### –	PeriodicAssistanceDataControlParameters

The IE PeriodicAssistanceDataControlParameters is used in a periodic assistance data delivery procedure as described in clauses 5.2.1a and 5.2.2a.

-- ASN1START

PeriodicAssistanceDataControlParameters-r15 ::= SEQUENCE {

periodicSessionID-r15			PeriodicSessionID-r15,

...,

[[

updateCapabilities-r15		UpdateCapabilities-r15		OPTIONAL

]]

}

PeriodicSessionID-r15 ::= SEQUENCE {

periodicSessionInitiator-r15	ENUMERATED { locationServer, targetDevice, ... },

periodicSessionNumber-r15		INTEGER (0..255),

...

}

UpdateCapabilities-r15 ::= BIT STRING {primaryCellID-r15	(0)} (SIZE(1..8))

-- ASN1STOP

PeriodicAssistanceDataControlParameters field descriptions

periodicSessionID This field identifies a particular periodic assistance data delivery session and the initiator of the session.

updateCapabilities This field identifies the capabilities of the sending entity to support an update of periodic assistance data. A bit value set to one indicates a capability is supported and a bit value set to zero indicates a capability is not supported.

#### –	Polygon

The IE Polygon is used to describe a geographic shape as defined in TS 23.032 [15].

-- ASN1START

# Polygon ::= SEQUENCE (SIZE (3..15)) OF PolygonPoints

# PolygonPoints ::= SEQUENCE {

latitudeSign				ENUMERATED {north, south},

degreesLatitude				INTEGER (0..8388607),			-- 23 bit field

degreesLongitude			INTEGER (-8388608..8388607)		-- 24 bit field

}

-- ASN1STOP

#### –	PositioningModes

The IE PositioningModes is used to indicate several positioning modes using a bit map.

-- ASN1START

# PositioningModes ::= SEQUENCE {

posModes		BIT STRING {	standalone	(0),

ue-based	(1),

ue-assisted	(2)

} (SIZE (1..8)),

...

}

-- ASN1STOP

PositioningModes field descriptions

posModes This field specifies the positioning mode(s). This is represented by a bit string, with a one-value at the bit position means the particular positioning mode is addressed; a zero-value means not addressed.

#### –	SegmentationInfo

The IE SegmentationInfo is used by a sender to indicate that LPP message segmentation is used, as specified in clause 4.3.5.

-- ASN1START

# SegmentationInfo-r14 ::= ENUMERATED { noMoreMessages, moreMessagesOnTheWay }

-- ASN1STOP

SegmentationInfo field descriptions

SegmentationInfo noMoreMessages indicates that this is the only or last LPP message segment used to deliver the entire message body. moreMessagesOnTheWay indicates that this is one of multiple LPP message segments used to deliver the entire message body.

#### –	VelocityTypes

The IE VelocityTypes defines a list of possible velocity shapes as defined in TS 23.032 [15].

-- ASN1START

# VelocityTypes ::= SEQUENCE {

horizontalVelocity										BOOLEAN,

horizontalWithVerticalVelocity							BOOLEAN,

horizontalVelocityWithUncertainty						BOOLEAN,

horizontalWithVerticalVelocityAndUncertainty			BOOLEAN,

...

}

-- ASN1STOP

- Next change

### 6.5.a	NR Enhanced Cell ID Positioning

This clause defines the information elements for NR ECID positioning (TS 38.305 [x1]).

#### 6.5.a.1	NR E-CID Location Information

#### –	NR-ECID-ProvideLocationInformation

The IE NR-ECID-ProvideLocationInformation is used by the target device to provide NR ECID location measurements to the location server. It may also be used to provide NR ECID positioning specific error reason.

-- ASN1START

# NR-ECID-ProvideLocationInformation-r16 ::= SEQUENCE {

nr-ECID-SignalMeasurementInformation-r16	NR-ECID-SignalMeasurementInformation-r16 OPTIONAL,

nr-ECID-Error-r16							NR-ECID-Error-r16						 OPTIONAL,

...

}

-- ASN1STOP

#### 6.a.1.2	NR-ECID Location Information Elements

#### –	NR-ECID-SignalMeasurementInformation

The IE NR-ECID-SignalMeasurementInformation is used by the target device to provide NR ECID measurements to the location server.

-- ASN1START

# NR-ECID-SignalMeasurementInformation-r16 ::= SEQUENCE {

primaryCellNRMeasuredResults-r16	MeasuredResultsNRElement-r16			OPTIONAL,

measuredResultsNRList-r16			MeasuredResultsNRList-r16,

measResultCellListSFTD-NR-r16		MeasResultCellListSFTD-NR-r16			OPTIONAL

...

# }

# NR-MeasuredResultsList-r16 ::= SEQUENCE (SIZE(1..32)) OF MeasuredResultsElement-r16

# NR-MeasuredResultsElement-r16 ::= SEQUENCE {

systemFrameNumber			BIT STRING (SIZE (10)),

nr-PhysCellId-r16			INTEGER (0..1007) 			OPTIONAL,

nr-CellGlobalId-r16			NCGI-r15					OPTIONAL,

nr-arfcn-r16				ARFCN-ValueNR-r15 			OPTIONAL,

measResultNR-r16				SEQUENCE {

cellResults-r16					SEQUENCE{

resultsSSB-Cell-r16				MeasQuantityResults-r16				OPTIONAL,

resultsCSI-RS-Cell-r16			MeasQuantityResults-r16				OPTIONAL

},

rsIndexResults-r16				SEQUENCE{

resultsSSB-Indexes-r16			ResultsPerSSB-IndexList-r16			OPTIONAL,

resultsCSI-RS-Indexes-r16		ResultsPerCSI-RS-IndexList-r16		OPTIONAL

}																		OPTIONAL

},

MeasQuantityResults-r16 ::= SEQUENCE {

nr-RSRP-r16	INTEGER (0..127)		OPTIONAL,

nr-RSRQ-r16	INTEGER (0..127)		OPTIONAL

}

ResultsPerSSB-IndexList-r16::= SEQUENCE (SIZE (1..64)) OF ResultsPerSSB-Index-r16

ResultsPerSSB-Index-r16 ::= SEQUENCE {

ssb-Index-r16					INTEGER (0..63),

ssb-Results-r16					MeasQuantityResults-r16					OPTIONAL

}

ResultsPerCSI-RS-IndexList-r16::= SEQUENCE (SIZE (1..64)) OF ResultsPerCSI-RS-Index-r16

ResultsPerCSI-RS-Index-r16 ::= SEQUENCE {

csi-RS-Index-r16				INTEGER (0..95),

csi-RS-Results-r16				MeasQuantityResults-r16					OPTIONAL

}

-- ASN1STOP

NR-ECID-SignalMeasurementInformation field descriptions

primaryCellMeasuredResults This field contains measurements for the primary cell when the target device reports measurements for both primary cell and neighbour cells. This field shall be omitted when the target device reports measurements for the primary cell only, in which case the measurements for the primary cell is reported in the measuredResultsList.

systemFrameNumber This field specifies the system frame number of the measured cell during which the measurements have been performed. The target device shall include this field if it was able to determine the SFN of the cell at the time of measurement.

nr-PhysCellId This field specifies the physical cell identity of the measured cell.

nr-CellGlobalId This field specifies cell global ID of the measured cell. The target device shall provide this field if it was able to determine the NCGI of the measured cell at the time of measurement.

Nr-arfcn This field specifies the ARFCN of the measured NR carrier frequency, as defined in TS 38.331 [35].

resultsSSB-Cell This attribute specifies the SS reference signal received power (SS-RSRP) and quality (SS-RSRQ) measurement aggregated at cell level, as defined in TS 38.331 [35].

resultsCSI-RS-Cell This attribute specifies the CSI-RS reference signal received power (CSI-RSRP) and quality (CSI-RSRQ) measurement aggregated at cell level, as defined in TS 38.331 [35].

ssb-Results This attribute specifies the SS reference signal received power (SS-RSRP) and quality (SS-RSRQ) measurement per SSB resource, as defined in TS 38.331 [35].

csi-RS-Results This attribute specifies the CSI-RS reference signal received power (CSI-RSRP) and quality (CSI-RSRQ) per CSI-RS resource, as defined in TS 38.331 [35].

nr-SS-RSRPB-Result This field specifies the SS reference signal received power per branch (SS-RSRPB) measurement, as defined in TS 38.331 [35].

#### 6.a.1.3	NR-ECID Location Information Request

#### –	NR-ECID-RequestLocationInformation

The IE NR-ECID-RequestLocationInformation is used by the location server to request NR-ECID location measurements from a target device.

-- ASN1START

# NR-ECID-RequestLocationInformation-r16 ::= SEQUENCE {

requestedMeasurements-r16		BIT STRING {	ssrsrpReq		(0),

ssrsrqReq		(1),

csirsrpReq		(2),

csirsrqReq		(3),

sftdReq			(4)} (SIZE(1..8)),

...

}

-- ASN1STOP

NR-ECID-RequestLocationInformation field descriptions

requestedMeasurements This field specifies the NR-ECID measurements requested. This is represented by a bit string, with a one-value at the bit position means the particular measurement is requested; a zero-value means not requested.

#### 6.a.1.4	NR-ECID Capability Information

#### –	NR-ECID-ProvideCapabilities

The IE NR-ECID-ProvideCapabilities is used by the target device to indicate its capability to support NR-ECID and to provide its NR-ECID positioning capabilities to the location server.

-- ASN1START

# NR-ECID-ProvideCapabilities-r16 ::= SEQUENCE {

nr-ECID-MeasSupported -r16		BIT STRING {	ssrsrpSup		(0),

ssrsrqSup		(1),

csirsrpSup		(2),

csirsrqSup		(3),

sftdSup			(4) } (SIZE(1..8)),

periodicalReporting-r16				ENUMERATED { supported }		OPTIONAL,

triggeredReporting-r16				ENUMERATED { supported }		OPTIONAL,

...

}

-- ASN1STOP

NR-ECID-Provide-Capabilities field descriptions

nr-ECID-MeasSupported This field specifies the NR ECID measurements supported by the target device. This is represented by a bit string, with a one-value at the bit position means the particular measurement is supported; a zero-value means not supported. A zero-value in all bit positions in the bit string means only the basic Cell ID positioning method is supported by the target device.

periodicalReporting This field, if present, indicates that the target device supports periodicalReporting of NR ECID measurements. If this field is absent, the location server may assume that the target device does not support periodicalReporting in CommonIEsRequestLocationInformation.

triggeredReporting This field, if present, indicates that the target device supports triggeredReporting for the cellChange event. If this field is absent, the location server may assume that the target device does not support triggeredReporting in CommonIEsRequestLocationInformation.

#### 6.a.1.5	NR-ECID Capability Information Request

#### –	NR-ECID-RequestCapabilities

The IE NR-ECID-RequestCapabilities is used by the location server to request the capability of the target device to support NR-ECID and to request NR-ECID positioning capabilities from a target device.

-- ASN1START

# NR-ECID-RequestCapabilities ::= SEQUENCE {

...

}

-- ASN1STOP

#### 6.a.1.6	NR-ECID Error Elements

#### –	NR-ECID-Error

The IE NR-ECID-Error is used by the location server or target device to provide NR-ECID error reasons to the target device or location server, respectively.

-- ASN1START

# NR-ECID-Error-r16 ::= CHOICE {

locationServerErrorCauses-r16		NR-ECID-LocationServerErrorCauses-r16,

targetDeviceErrorCauses-r16			NR-ECID-TargetDeviceErrorCauses-r16,

...

}

-- ASN1STOP

#### –	NR-ECID-LocationServerErrorCauses

The IE NR-ECID-LocationServerErrorCauses is used by the location server to provide NR-ECID error reasons to the target device.

-- ASN1START

# NR-ECID-LocationServerErrorCauses-r16 ::= SEQUENCE {

Cause-r16		ENUMERATED	{	undefined,

...

},

...

}

-- ASN1STOP

#### –	NR-ECID-TargetDeviceErrorCauses

The IE NR-ECID-TargetDeviceErrorCauses is used by the target device to provide NR-ECID error reasons to the location server.

-- ASN1START

# NR-ECID-TargetDeviceErrorCauses-r16 ::= SEQUENCE {

Cause-r16		ENUMERATED {	undefined,

requestedMeasurementNotAvailable,

notAllrequestedMeasurementsPossible,

...

},

ss-RSRPMeasurementNotPossible				NULL		OPTIONAL,

ss-RSRQMeasurementNotPossible				NULL		OPTIONAL,

csi-RSRPMeasurementNotPossible				NULL		OPTIONAL,

csi-RSRQMeasurementNotPossible				NULL		OPTIONAL,

sftdMeasurementNotPossible					NULL		OPTIONAL,

...

}

-- ASN1STOP

### 6.5.b	NR Positioning

This subclause defines the information elements for downlink NR positioning measurements based on DL-PRS signals (TS 38.305 [38]).

#### 6.5.b.1	NR Positioning Assistance Data

#### –	NR-Pos-ProvideAssistanceData

The IE NR-Pos-ProvideAssistanceData is used by the location server to provide assistance data to enable UE-assisted and UE-based downlink NR positioning and NR positioning specific error reason.

Throughout clause 6.5.b, "assistance data reference TRP" refers to the first TRP in the TRP list in the IE NR-TRP-Info.

NOTE 1:	The location server should include at least one TRP for which the SFN can be obtained by the target device, either as the assistance data reference TRP or any other TRP in the TRP list. Otherwise the target device will be unable to perform the NR positioning measurement and the positioning operation will fail.

-- ASN1START

# NR-Pos-ProvideAssistanceData-r16 ::= SEQUENCE {

# nr-TRP-PRS-InfoList-r16		NR-TRP-PRS-InfoList-r16,

# nr-TRP-StaticInfo-r16		NR-TRP-StaticInfo-r16		OPTIONAL,	-- Cond UEB

# nr-TRP-DynamicInfo-r16		NR-TRP-DynamicInfo-r16		OPTIONAL,	-- Cond UEB

nr-Error-r16				NR-Error-r16				OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| UEB | This field is present if NR positioning UEB AD is configured; otherwise it is not present |

#### 6.5.b.2	NR Positioning Assistance Data Elements

#### –	NR-TRP-PRS-InfoList

The IE NR-TRS-PRS-InfoList is used to provide configuration information for NR TRPs including DL-PRS.

NOTE: 	This IE provides DL-PRS configuration information for a list of TRPs. The first TRP in the list corresponding to the first positioning frequency layer is defined as the reference TRP.

-- ASN1START

# NR-TRP-PRS-InfoList-r16 ::= SEQUENCE (SIZE (1..nrMaxFreqLayers)) OF NR-TRP-PRS-FreqLayer-InfoElement-r16

# NR-TRP-PRS-FreqLayer-InfoElement-r16 ::= SEQUENCE {

# nr-TRP-PRS-FreqLayer-InfoList-r16	NR-TRP-PRS-FreqLayer-InfoList-r16,

nr-PRS-SubcarrierSpacing-r16	ENUMERATED {kHz15, kHz30, kHz60, kHz120, ...},

OPTIONAL,	-- Cond NotSameAsFL1-0

nr-PRS-ResourceBandwidth-r16	INTEGER (1..63), 			OPTIONAL,	-- Cond NotSameAsFL1-1

nr-PRS-StartPRB-r16				INTEGER(0..2176),			OPTIONAL,	-- Cond NotSameAsFL1-2

nr-PRS-PointA-r16				ARFCN-ValueNR-r15,			OPTIONAL,	-- Cond NotSameAsFL1-3

nr-PRS-CombSizeN-r16			ENUMERATED {n2, n4, n6, n12, ...},

OPTIONAL,	-- Cond NotSameAsFL1-4

nr-PRS-CyclicPrefix-r16			ENUMERATED {normal, extended, ...},

OPTIONAL,	-- Cond NotSameAsFL1-5

...

}

NR-TRP-PRS-FreqLayer-InfoList-r16 ::= SEQUENCE (SIZE (1..nrMaxTRPperPFL)) OF NR-TRP-PRS-InfoElement-r16

# NR-TRP-PRS-InfoElement-r16 ::= SEQUENCE {

nr-PRS-ResourceSetList-r16		SEQUENCE (SIZE (1..nrMaxResourceSets)) NR-PRS-ResourceSet-r16,

nr-DL-PRS-SFN0-Offset-r16		SEQUENCE {

sfn-Offset-r16					INTEGER (0..1023),

integerSubframeOffset-r16		INTEGER (0..9)					OPTIONAL	-- Need OP

}	OPTIONAL,

nr-PRS-expectedRSTD-r16			INTEGER (-3841..3841)		OPTIONAL,	-- Cond NotRefTRP

nr-PRS-expectedRSTD-uncerainty-r16

INTEGER (-246..246) 		OPTIONAL,	-- Cond NotRefTRP

# nrPhysCellId-r16				INTEGER (0..1007)			OPTIONAL

nrCellGlobalID-r16				NCGI-r15					OPTIONAL,

nrARFCN-r16						ARFCN-ValueNR				OPTIONAL,

nr-TRP-ID-r16					INTEGER (0..nrMaxTRP-1)		OPTIONAL,

nr-TRP-UEB-refIndices-r16		NR-TRP-UEB-refIndices-r16	OPTIONAL,	-- Cond UEB

# ...

# }

nrMaxFreqLayers		INTEGER ::=   4

nrMaxTRPperPFL  	INTEGER ::=  64

nrMaxResourceSets	INTEGER ::=   2

nrMaxTRP-1			INTEGER ::= 255

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| NotSameAsFL1-0 | The field is absent if the NR PRS subcarrier spacing is the same as in the first frequency layer in the list; otherwise it is present. |
| NotSameAsFL1-1 | The field is absent if the NR PRS resource bandwidth is the same as the in the first frequency layer in the list; otherwise it is present. |
| NotSameAsFL1-2 | The field is absent if the NR PRS Point A is the same as the in the first frequency layer in the list; otherwise it is present. |
| NotSameAsFL1-3 | The field is absent if the NR PRS Start PRB is the same as in the first frequency layer in the list; otherwise it is present. |
| NotSameAsFL1-4 | The field is absent if the NR PRS comb size is the same as in the first frequency layer in the list; otherwise it is present. |
| NotSameAsFL1-5 | The field is absent if the NR PRS cyclic prefix is the same as in the first frequency layer in the list; otherwise it is present. |
| NotRefTRP | These fields are present for all TRPs except the reference TRP (the first TRP of the first frequency layer) if RSTD measurements are configured. Otherwise, they are not present. |
| UEB | This field is present if NR positioning UEB AD is configured; otherwise it is not present |

NR-TRP-InfoList field descriptions

nr-PRS-CombSizeN This parameter indicates Resource element (RE) spacing in each symbol of DL PRS Resource. All DL PRS Resource Sets belonging to the same Positioning Frequency Layer have the same value of combSize.

nr-PRS-SubcarrierSpacing This parameter indicates Subcarrier Spacing for DL PRS Resource. 15, 30, 60 kHz for FR1; 60, 120 kHz for FR2.

nr-PRS-ResourceBandwidth This parameter indicates the number of PRBs allocated for DL PRS Resource (allocated DL PRS bandwidth). All DL PRS Resources of the DL PRS Resource Set have the same bandwidth. All DL PRS Resource Sets belonging to the same Positioning Frequency Layer have the same value of DL PRS Bandwidth and Start PRB. Value 1 equals 24, value 2 equals to 28, value 3 equals to 32 and so on.

nr-PRS-StartPRB This parameter indicates start PRB index defined as offset with respect to reference DL PRS Point A configured for positioning frequency layer.

nr-PRS-PointA This parameter indicates absolute frequency of the reference resource block for DL PRS. Its lowest subcarrier is also known as DL PRS Point A. A single Point A for DL PRS resource allocation is provided per positioning frequency layer. All DL PRS Resources belonging to the same DL PRS Resource Set have common Point A.

nr-PRS-CyclicPrefix This parameter indicates Cyclic Prefix Type for DL PRS Resource.

nr-TRP-ID This field indicates the ID that to be used along with a DL PRS Resource Set ID and a DL PRS Resources ID to uniquely identify a DL PRS Resource. Each TRP is only associated with one TRP-ID.

nr-TRP-UEB-refIndices This field provides cross-referencing indices into the lists representing UE-based assistance data for locationInfo, beamInfo and rtdInfo. that to be used along with a DL PRS Resource Set ID and a DL PRS Resources ID to uniquely identify a DL PRS Resource. Each TRP is only associated with one trp-ID.

nr-PRS-SFN0-Offset Defines time offset of the SFN0 slot 0 for given TRP with respect to SFN0 slot 0 of reference TRP.

nr-PhysCellId This field specifies the physical cell identity of the associated TRP, as defined in TS 38.331 [35].

nr-PRS-ExpectedRSTD This field indicates the RSTD value that the target device is expected to measure between this TRP and the assistance data reference TRP. The nr-PRS-ExpectedRSTD field takes into account the expected propagation time difference as well as transmit time difference of PRS positioning occasions between the two TRPs. The RSTD value can be negative and is calculated as (nr-PRS-ExpectedRSTD-8192). The resolution is 3Ts, with Ts=1/(15000*2048) seconds.

nr-PRS-ExpectedRSTD-Uncertainty This field indicates the uncertainty in nr-PRS-ExpectedRSTD value. The uncertainty is related to the location server′s a-priori estimation of the target device location. The nr-PRS-ExpectedRSTD and nr-PRS-ExpectedRSTD-Uncertainty together define the search window for the target device. The scale factor of the nr-PRS-ExpectedRSTD-Uncertainty field is 3Ts, with Ts=1/(15000*2048) seconds. The target device may assume that the beginning of the PRS occasion group of the PRS configuration with the longest PRS occasion group periodicity (NOTE) of the neighbour TRP is received within the search window of size [ nr-PRS-ExpectedRSTD-Uncertainty3Ts, nr-PRS-ExpectedRSTD-Uncertainty3Ts] centered at TREF + 1 millisecondN + (nr-PRS-ExpectedRSTD8192) 3Ts, where TREF is the reception time of the beginning of the first PRS occasion group of the first PRS configuration of the assistance data reference cell at the target device antenna connector, N = 0 when the EARFCN of the neighbour TRP is equal to that of the assistance data reference TRP, and N = prs-SubframeOffset otherwise.

nr-CellGlobalId This field specifies the NCGI, the globally unique identity of a cell in NR, of the associated TRP, as defined in TS 38.331 [35]. The server should include this field if it considers that it is needed to resolve ambiguity in the TRP indicated by nr-PhysCellId.

nrARFCNRef This field specifies the NRARFCN of the TRP.

#### –	NR-PRS-ResourceSetList

The IE NR-PRS-ResourceSetList provides the information related to the configuration of NR DL-PRS of a TRP.

-- ASN1START

# NR-PRS-ResourceSetList-r16 ::= SEQUENCE {

nr-PRS-ResourceSetList-r16		SEQUENCE (SIZE (1.. nrMaxSetsPerTrpPerPFL)) NR-PRS-ResourceSet-r16,

...

# }

NR-PRS-ResourceSet-r16 ::= SEQUENCE {

nr-PRS-ResourceSetId-r16			NR-PRS-ResourceSetId-r16,

nr-PRS-Periodicity-and-ResourceSetSlotOffset-r16-r16	NR-PRS-Periodicity-and-ResourceSetSlotOffset-r16,

nr-PRS-ResourceRepetitionFactor-r16	ENUMERATED {n1, n2, n4, n6, n8, n16, n32, ...},

nr-PRS-ResourceTimeGap-r16			ENUMERATED {s1, s2, s4, s8, s16, s32, ...},

nr-PRS-ResourceList-r16				SEQUENCE (SIZE (1..nrMaxResourcesPerSet)) OF NR-PRS-Resource-r16,

nr-PRS-NumSymbols-r16				ENUMERATED {n2, n4, n6, n12, ...},

lnr-PRS-MutingPatternList-r16		SEQUENCE {

mutingOption1-r16					SEQUENCE {

mutingPattern-r16					MutingPattern-r16,

nr-PRS-MutingBitRepetitionFactor-r16	ENUMERATED {n1, n2, n4, n8, ...}	OPTIONAL	--Need OR

},

mutingOption2-r16					SEQUENCE {

mutingPattern-r16					MutingPattern-r16

}

},

nr-PRS-ResourcePower-r16			INTEGER (-60..50),

...

}

NR-PRS-Resource-r16 ::= SEQUENCE {

nr-PRS-ResourceId-r16				NR-PRS-ResourceID-r16,

nr-PRS-SequenceId-r16				INTEGER {0.. 4095},

nr-PRS-ReOffset-r16					CHOICE {

n2-r16							INTEGER (0..1),

n4-r16							INTEGER (0..3),

n6-r16							INTEGER (0..5),

n12-r16							INTEGER (0..11)

},

nr-PRS-ResourceSlotOffset-r16		INTEGER (0..nrMaxResourceOffsetValue),

nr-PRS-ResourceSymbolOffset-r16		INTEGER (0..12),

nr-PRS-QCL-Info-r16					NR-PRS-QCL-Info-r16,

...

}

MutingPattern-r16 ::= CHOICE {

po2-r16								BIT STRING (SIZE(2)),

po4-r16								BIT STRING (SIZE(4)),

po6-r16								BIT STRING (SIZE(6)),

po8-r16								BIT STRING (SIZE(8)),

po16-r16							BIT STRING (SIZE(16)),

po32-r16							BIT STRING (SIZE(32)),

...

}

NR-PRS-QCL-Info-r16 ::= CHOICE {

ssb-r16                          SEQUENCE {

pci-r16                              INTEGER (0..1007),

ssb-Index-r16                        INTEGER (0..63),

rs-Type-r16                          ENUMERATED {typeC, typeC-plus-typeD}

},

dl-PRS-r16                       SEQUENCE {

nr-PRS-ResourceSetId-r16	   		NR-PRS-ResourceSetId-r16,

nr-PRS-ResourceId-r16	   			NR-PRS-ResourceID-r16,

rs-Type-r16                         ENUMERATED {typeD}

}

}

NR-PRS-Periodicity-and-ResourceSetSlotOffset-r16 ::= CHOICE {

scs15-r16		CHOICE {

n4-r16					INTEGER (0..3),

n5-r16					INTEGER (0..4),

n8-r16					INTEGER (0..7),

n10-r16					INTEGER (0..9),

n16-r16					INTEGER (0..15),

n20-r16					INTEGER (0..19),

n32-r16					INTEGER (0..31),

n40-r16					INTEGER (0..39),

n64-r16					INTEGER (0..63),

n80-r16					INTEGER (0..79),

n160-r16				INTEGER (0..159),

n320-r16				INTEGER (0..319),

n640-r16				INTEGER (0..639),

n1280-r16				INTEGER (0..1279),

n2560-r16				INTEGER (0..2559),

n5120-r16				INTEGER (0..5119),

n10240-r16				INTEGER (0..10239),

...},

scs30-r16		CHOICE {

n8-r16					INTEGER (0..7),

n10-r16					INTEGER (0..9),

n16-r16					INTEGER (0..15),

n20-r16					INTEGER (0..19),

n32-r16					INTEGER (0..31),

n40-r16					INTEGER (0..39),

n64-r16					INTEGER (0..63),

n80-r16					INTEGER (0..79),

n128-r16				INTEGER (0..127),

n160-r16				INTEGER (0..159),

n320-r16				INTEGER (0..319),

n640-r16				INTEGER (0..639),

n1280-r16				INTEGER (0..1279),

n2560-r16				INTEGER (0..2559),

n5120-r16				INTEGER (0..5119),

n10240-r16				INTEGER (0..10239),

n20480-r16				INTEGER (0..20479),

...},

scs60-r16		CHOICE {

n16-r16					INTEGER (0..15),

n20-r16					INTEGER (0..19),

n32-r16					INTEGER (0..31),

n40-r16					INTEGER (0..39),

n64-r16					INTEGER (0..63),

n80-r16					INTEGER (0..79),

n128-r16				INTEGER (0..127),

n160-r16				INTEGER (0..159),

n256-r16				INTEGER (0..255),

n320-r16				INTEGER (0..319),

n640-r16				INTEGER (0..639),

n1280-r16				INTEGER (0..1279),

n2560-r16				INTEGER (0..2559),

n5120-r16				INTEGER (0..5119),

n10240-r16				INTEGER (0..10239),

n20480-r16				INTEGER (0..20479),

n40960-r16				INTEGER (0..40959),

...},

scs120-r16		CHOICE {

n32-r16					INTEGER (0..31),

n40-r16					INTEGER (0..39),

n64-r16					INTEGER (0..63),

n80-r16					INTEGER (0..79),

n128-r16				INTEGER (0..127),

n160-r16				INTEGER (0..159),

n256-r16				INTEGER (0..255),

n320-r16				INTEGER (0..319),

n512-r16				INTEGER (0..511),

n640-r16				INTEGER (0..639),

n1280-r16				INTEGER (0..1279),

n2560-r16				INTEGER (0..2559),

n5120-r16				INTEGER (0..5119),

n10240-r16				INTEGER (0..10239),

n20480-r16				INTEGER (0..20479),

n40960-r16				INTEGER (0..40959),

n81920-r16				INTEGER (0..81919),

...},

...

}

NR-PRS-ResourceID-r16 ::= INTEGER (0.. nrMaxResourcesPerSet-1)

NR-PRS-ResourceSetID-r16 ::= INTEGER (0.. nrMaxResourceSetsPerTRP-1)

nrMaxResourcesPerSet		INTEGER ::=    64	-- Max PRS resources configurable for one set

nrMaxResourcesPerSet-1		INTEGER ::=    63	-- Max PRS resources configurable for one set - 1

nrMaxSetsPerTrpPerPFL		INTEGER ::= 	2	-- Max resource sets per TRP per PFL

nrMaxResourceSetsPerTRP-1	INTEGER ::= 	7	-- Max PRS resource sets per TRP - 1

nrMaxResourceOffsetValue 	INTEGER ::=   511	-- Max resource offset value

-- ASN1STOP

NR-PRS-ResourceSetList field descriptions

nr-PRS-Periodicity-and-ResourceSetSlotOffset This field specifies the periodicity of DL PRS allocation in slots configured per DL PRS Resource Set and the slot offset with respect to SFN slot 0 for a TRP where DL PRS Resource Set is configured (i.e. slot where the first DL PRS Resource of DL PRS Resource Set occurs).

nr-PRS-ResourceRepetitionFactor This parameter controls how many times each DL-PRS Resource is repeated for a single instance of the DL-PRS Resource Set. It is applied to all resources of DL PRS Resource Set.

nr-PRS-ResourceTimeGap This parameter indicates offset in units of slots between two repeated instances of a DL PRS Resource corresponding to the same DL-PRS Resource ID within a single instance of the DL PRS Resource Set. DL-PRS-ResourceTimeGap is provided only if DL-PRS-ResourceRepetitionFactor is configured and is greater than 1. The time duration spanned by one DL PRS Resource set containing repeated DL PRS Resources should not exceed DL-PRS-Periodicity.

nr-PRS-MutingPatternList List of nr-PRS-MutingPattern, first entry is for Option 1 and second entry is for Option 2. The following options are supported for the applicability of the bitmap. • Option 1: Each bit in the bitmap corresponds to a configurable number of consecutive instances (in a periodic transmission of DL-PRS resource sets) of a DL-PRS Resource set o All DL-PRS Resources within a DL-PRS Resource Set instance are muted for a DL-PRS Resource Set instance that is indicated to be muted by the bitmap • Option 2: Each bit in the bitmap corresponds to a single repetition index for each of the DL-PRS Resources within an instance of a DL-PRS Resource Set (The length of the bitmap is equal to NR-PRS-ResourceRepetitionFactor) o The above applies to all instances of the DL-PRS Resource Set that the above DL-PRS Resources are part of. • Bitmap size values: 2, 4, 6, 8, 16, 32 bits Bit value “0” indicates a muted DL PRS transmission, and the value “1” indicates DL PRS transmission UE can be configured with any of the following combinations of DL PRS muting options: Option 1 only Option 2 only Option 1 and Option 2

nr-PRS-MutingBitRepetitionFactor This parameter indicates the configurable number of consecutive instances (in a periodic transmission of DL-PRS resource sets) of a DL-PRS Resource Set applicable to single bit of Option 1 Muting bitmap.

nr-PRS-ReOffset This parameter indicates Resource element offset in frequency domain for the first symbol in a DL PRS Resource. The relative RE offsets of following symbols are defined relative to the RE Offset in frequency domain of the first symbol in the DL PRS resource.

nr-PRS-ResourceSlotOffset This parameter indicates points to starting slot of DL PRS Resource with respect to corresponding DL-PRS-ResourceSetSlotOffset.

nr-PRS-QCL-Info This parameter indicates QCL indication with other DL reference signals for serving and neighboring cells.

nr-PRS-NumSymbol This parameter indicates the number of symbols per DL PRS Resource within a slot.

nr-PRS-SequenceId This parameter indicates the sequence Id used to initialize cinit value used in pseudo random generator TS38.211 [x2, 5.2.1] for generation of DL PRS sequence for transmission on a given DL PRS Resource.

#### 6.5.b.3	NR-Pos Assistance Data Request

#### –	NR-Pos-RequestAssistanceData

The IE NR-Pos-RequestAssistanceData is used by the target device to request assistance data from a location server.

-- ASN1START

# NR-Pos-RequestAssistanceData-r16 ::= SEQUENCE {

nrPhysCellId-r16		INTEGER (0..1007)											OPTIONAL,

adType-r16				BIT STRING {dl-prs   (0),

trp-loc  (1),

trp-rtd  (2),

trp-beam (3) } (SIZE (1..8)),

...

}

-- ASN1STOP

NR-Pos-RequestAssistanceData field descriptions

nrPhysCellId This field specifies the NR physical cell identity of the current primary cell of the target device.

adType This field specifies the assistance data requested. This is represented by a bit string, with a one-value at the bit position means the particular assistance data is requested; a zero-value means not requested. Bit 0 indicates that NR DL-PRS assistance data is requested. Bit 1 indicates TRP location information to support UE-based positioning is requested. Bit 2 indicates TRP relative time difference information to support UE-based positioning is requested. Bit 3 indicates TRP beam information to support UE-based positioning is requested.

#### 6.5.8.4	NR Positioning Location Information

#### –	NR-Pos-ProvideLocationInformation

The IE NR-Pos-ProvideLocationInformation is used by the target device to provide NR positioning measurements or location estimate to the location server. It may also be used to provide NR positioning measurements specific error reason.

-- ASN1START

# NR-Pos-ProvideLocationInformation-r16 ::= SEQUENCE {

nr-pos-SignalMeasurementInformation-r16 NR-Pos-SignalMeasurementInformation-r16	OPTIONAL,

# nr-pos-LocationInformation-r16			NR-Pos-LocationInformation-r16			OPTIONAL, -- Cond UEB

nr-posError-r16							NR-Pos-Error-r16						OPTIONAL,

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| UEB | The field is mandatory present for the UE based NR positioning; otherwise it is not present. |

#### 6.5.8.5	NR Positioning Location Information Elements

#### –	NR-Pos-SignalMeasurementInformation

The IE NR-Pos-SignalMeasurementInformation is used by the target device to provide NR positioning measurements to the location server. The measurements are provided as a list of TRPs, where the first TRP in the list is used as reference TRP. The first TRP in the list may or may not be the same as the assistance data reference TRP provided by the location server. Furthermore, the target device selects a reference resource per TRP, and compiles the measurements per TRP based on the selected reference resource. If configured, additional measurements of RSTD and/or of RSRP within the same TRP, are also provided.

-- ASN1START

# NR-Pos-SignalMeasurementInformation-r16 ::= SEQUENCE {

nr-posList-r16				NR-PosList-r16,

...

}

nr-TRP-ID-r16 					INTEGER (0..nrMaxTRP-1)		OPTIONAL,

# NR-PosList-r16 ::= SEQUENCE (SIZE(0..nrMaxTRP-1)) OF

# NR-PosListElement-r16

# NR-PosListElement-r16 ::= SEQUENCE {

nr-TRP-ID-r16 						INTEGER (0..nrMaxTRP-1),

nr-PRS-ResourceId-r16				NR-PRS-ResourceId -r16	  		OPTIONAL,

nr-PRS-ResourceSetId-r16			NR-PRS-ResourceSetId-r16 		OPTIONAL,

nr-rstd-r16							INTEGER (FFS)					OPTIONAL,	-- Cond NotFirstElement

nr-additionalPaths-r16				NR-AdditionalPathList-r16		OPTIONAL,

nr-toaQuality-r16					NR-TOAMeasQuality-r16			OPTIONAL,	-- Cond TOAmeas

nr-ueRxTxTD-r16						INTEGER (FFS)					OPTIONAL,	-- Cond RXTXmeas

nr-RxTxSync-Quality-r16				NR-RxTxSyncQuality-r16 			OPTIONAL,	-- Cond RXTXmeas

nr-rsrp-r16							INTEGER (0..127)				OPTIONAL,	-- Cond RSRPmeas

nr-rsrp-Quality-r16					NR-RSRPMeasQuality-r16			OPTIONAL,	-- Cond RSRPmeas

nr-sameTRP-RSTD-List-r16			NR-SameTRP-RSTD-List-r16,		OPTIONAL,	-- Cond ReportRelRSTDRes

nr-sameTRP-RSRP-List-r16			NR-SameTRP-RSRP-List-r16,		OPTIONAL,	-- Cond ReportRSRPListRes

...

}

NR-AdditionalPathList-r16 ::= SEQUENCE (SIZE(1..nrMaxAddPaths-r16)) OF NR-AdditionalPath-r14

NR-SameTRP-RSTD-List-r16 ::= SEQUENCE (SIZE(1..nrMax-Rel-RSTD)) OF

NR-SameTRP-RSTD-ListElement-r16

NR-SameTRP-Additional-RSRP-List-r16 ::= SEQUENCE (SIZE(1..nrMax-Additional-RSRP)) OF

NR-SameTRP-Additional-RSRP-ListElement-r16

NR-SameTRP-RSTD-ListElement-r16 ::= SEQUENCE {

nr-PRS-ResourceId-r16				NR-PRS-ResourceId-r16	  		OPTIONAL,

nr-PRS-ResourceSetId-r16			NR-PRS-ResourceSetId-r16 		OPTIONAL,

nr-sameTRP-rstd-r16					INTEGER (FFS)					OPTIONAL,

nr-additionalPaths-r16				NR-AdditionalPathList-r16		OPTIONAL,

nr-toaQuality-r16					NR-TOAMeasQuality-r16			OPTIONAL,

...

}

NR-SameTRP-Additional-RSRP-ListElement-r16 ::= SEQUENCE {

nr-PRS-ResourceId-r16				NR-PRS-ResourceId-r16	  		OPTIONAL,

nr-PRS-ResourceSetId-r16			NR-PRS-ResourceSetId-r16 		OPTIONAL,

nr-rsrp-r16							INTEGER (0..127)				OPTIONAL,	-- Cond RSRPmeas

nr-rsrp-Quality-r16					NR-RSRPMeasQuality-r16			OPTIONAL,	-- Cond RSRPmeas

...

}

nrMax-Rel-RSTD		  INTEGER ::= 3	-- Max relative RSTD within same TRP

nrMax-Additional-RSRP INTEGER ::= 7	-- Max additional RSRP within same TRP

nrMaxAddPaths		  INTEGER ::= 3	-- Max additional paths

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| NotFirstElement | This field, except for the first element in the list, is mandatory present when RSTD measurements are configured, otherwise it is not present |
| TOAmeas | The field is mandatory present when measurements based on TOA measurements are configured; otherwise it is not present (Need ON). |
| RXTXmeas | The field is mandatory present when UE Rx-Tx measurements are configured; otherwise it is not present (Need ON). |
| ReportRelRes | The field is mandatory present when UE Rx-Tx measurements are configured; otherwise it is not present (Need ON). |

NR-Pos-SignalMeasurementInformation field descriptions

#### –	NR-Pos-LocationInformation

The IE NR-Pos-LocationInformation is included by the target device when location information derived using NR positioning is provided to the location server.

-- ASN1START

# NR-Pos-LocationInformation-r16 ::= SEQUENCE {

# nr-Pos-TimeStamp-r16		NR-Pos-TimeStamp-r16		OPTIONAL,

...

}

-- ASN1STOP

NR-Pos-LocationInformation field descriptions

nr-Pos-TimeStamp This field specifies the time for which the measurement of the reference resource of the reference TRP was obtained.

#### –	NR-Pos-TimeStamp

The IE NR-Pos-TimeStamp provides the NR positioning time stamp associated to the measurements of the reference resource of a TRP.

-- ASN1START

# NR-Pos-TimeStamp-r16 ::= SEQUENCE {

systemFrameNumber-r16		BIT STRING (SIZE (10)),

utcTime-r16					UTCTime						OPTIONAL,	-- Cond UTCmeas

utcTime-ms-r16				INTEGER (0..999)			OPTIONAL,	-- Cond UTCmeas

utcTime-us-r16				INTEGER (0..999)			OPTIONAL,	-- Cond UTCmeas-fine

utcTime-ns-r16				INTEGER (0..999)			OPTIONAL,	-- Cond UTCmeas-fine

utcTime-subns-r16			INTEGER (0..9)				OPTIONAL,	-- Cond UTCmeas-fine

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| UTCmeas | The field is mandatory present if the positioning measurement is requested to be associated with an UTC time at ms level or a UTC fine level; otherwise it is not present. |
| UTCmeas-fine | The field is mandatory present if the positioning measurement is requested to be associated with a fine UTC time at sub ns level; otherwise it is not present. |

NR-Pos-TimeStamp field descriptions

systemFrameNumber This field specifies the NR System Frame Number (SFN).

utcTime This field provides the UTC time in the form of YYMMDDhhmmssZ.

utcTime-ms This field provides the fractional part of the UTC time in ms resolution

utcTime-us This field provides the fractional part of the UTC time in us resolution

utcTime-ns This field provides the fractional part of the UTC time in ns resolution

utcTime-subns This field provides the fractional part of the UTC time in sub-ns resolution (0.1 ns resolution

#### –	NR-AdditionalPath

The IE NR-AdditionalPath is used by the target device to provide information about additional paths in association to the TOA measurements associated to NR positioning in the form of a relative time difference and a quality value. The additional path nr-relativeTimeDifference is the detected path timing relative to the detected path timing used for the TOA value, and each additional path can be associated with a quality value nr-path-Quality.

-- ASN1START

# NR-AdditionalPath-r16 ::= SEQUENCE {

nr-relativeTimeDifference-r16	FFS,

nr-path-Quality-r16				NR-TOAMeasQuality-r16				OPTIONAL,

...

}

-- ASN1STOP

| NR-AdditionalPath field descriptions |  |
| --- | --- |
| nr-relativeTimeDifference This field specifies the additional detected path timing relative to the detected path timing of the reference resource. A positive value indicates that the particular path is later in time than the detected path of the reference; a negative value indicates that the particular path is earlier in time than the detected path of the reference |  |
| nr-path-Quality This field specifies the target device′s best estimate of the quality of the detected timing of the additional path. |  |

#### –	NR-TOAMeasQuality

#### –	NR-RSRPMeasQuality

#### 6.5.8.6 	NR Positioning Location Information Request

#### –	NR-Pos-RequestLocationInformation

The IE NR-Pos-RequestLocationInformation is used by the location server to request NR positioning from a target device. Details of the required measurements (details of TRP configurations) are conveyed in the NR-Pos-ProvideAssistanceData IE in a separate Provide Assistance Data message.

-- ASN1START

# NR-Pos-RequestLocationInformation ::= SEQUENCE {

assistanceAvailability-r16		BOOLEAN,

rstdMeasurements-r16			ENUMERATED { requested }		OPTIONAL,		-- Need ON

rstdRelMeasurements-r16			ENUMERATED { requested }		OPTIONAL,		-- Need ON

ueRxTxMeasurements-r16			ENUMERATED { requested }		OPTIONAL,		-- Need ON

rsrpAddMeasurements-r16			ENUMERATED { requested }		OPTIONAL,		-- Need ON

fineTimeStamp-r16				ENUMERATED { requested }		OPTIONAL,		-- Need ON

toaAggreg-r16					ENUMERATED { first, strongest, ... }	OPTIONAL,	-- Need ON

multipathTOA-r16				ENUMERATED { requested }		OPTIONAL,		-- Need ON

motionMeasurements-r16			ENUMERATED { requested }		OPTIONAL,		-- Need ON

...

}

-- ASN1STOP

NR-Pos-RequestLocationInformation field descriptions

assistanceAvailability This field indicates whether the target device may request additional NR positioning measurements assistance data from the server. TRUE means allowed and FALSE means not allowed.

rstdMeasurements This field, if present, indicates that the target device is requested to report DL PRS RSTD per TRP in relation to a reference TRP

rstdRelMeasurements This field, if present, indicates that the target device is requested to report DL PRS RSTD for different resources within the same TRP in relation to a reference resource of the TRP

ueRxTxMeasurements This field, if present, indicates that the target device is requested to report UE Rx-Tx per DL PRS resource in relatiojn to a configured uplink time reference and , in consideration of per TRP measurement aggregation if configured

rsrpMeasurements This field, if present, indicates that the target device is requested to report DL PRS RSRP per resource.

rsrpAddMeasurements This field, if present, indicates that the target device is requested to report Additional DL PRS RSRP per resource within the same TRP.

fineTimeStamp This field, if present, indicates that the target device is requested to report a fine time stamp of the reference TRP, in consideration of per TRP measurement aggregation if configured.

toaAggreg This field, if present, indicates that the target device is requested to report timing aggregated per TRP, and the aggregation strategy. The value ‘first’ means that the DL-PRS TOA per TRP is determined a the DL-PRS TOA of the resource corresponding to the shortest time of flight. The value ‘strongest’ means that the DL-PRS TOA per TRP is determined a the DL-PRS TOA of the resource corresponding to the shortest DL PRS RSRP.

multipathTOA This field, if present, indicates that the target device is requested to report additional detected path timing information.

motionMeasurements This field, if present, indicates that the target device is requested to report the motion measurements (deltaSFN and motionTimeSource) in NR-Pos-SignalMeasurementInformation as well as the IE Sensor-MotionInformation in IE Sensor-ProvideLocationInformation.

#### 6.5.8.7	NR Positioning Capability Information

#### –	NR-Pos-ProvideCapabilities

The IE NR-Pos-ProvideCapabilities is used by the target device to indicate its capability to support NR positioning and to provide its NR positioning capabilities to the location server.

-- ASN1START

# NR-Pos-ProvideCapabilities-r16 ::= SEQUENCE {

# rstdReporting-r16 				ENUMERATED { supported }		OPTIONAL,

# rstdRelReporting-r16 			ENUMERATED { supported }		OPTIONAL,

# ueRxTxReporting-r16 			ENUMERATED { supported }		OPTIONAL,

# rsrpReporting-r16 				ENUMERATED { supported }		OPTIONAL,

# rsrpAddReporting-r16 			ENUMERATED { supported }		OPTIONAL,

# fineTimeStamp-r16				ENUMERATED { supported }		OPTIONAL,

# additionalPathsReport-r16 		ENUMERATED { supported }		OPTIONAL,

ulSRStransmission-r16			ENUMERATED { supported }		OPTIONAL,

periodicalReporting-r16			PositioningModes				OPTIONAL,

idleStateForMeasurements-r16	ENUMERATED { required }			OPTIONAL,

motionMeasurements-r16			ENUMERATED { supported }		OPTIONAL,

# uebLocRtd-r16	 				ENUMERATED { supported }		OPTIONAL,

# uebLocBeam-r16	 				ENUMERATED { supported }		OPTIONAL,

...

}

-- ASN1STOP

NR-Pos-ProvideCapabilities field descriptions

rsrpReporting This field, if present, indicates that the target device supports NR DL PRS RSRP measurement reporting

rstdReporting This field, if present, indicates that the target device supports NR DL PRS RSTD measurement reporting

ueRxTxReporting This field, if present, indicates that the target device supports NR UE RxTx measurement reporting

fineTimeStamp This field, if present, indicates that the target device supports fine time stamp reporting

additionalPathsReport This field, if present, indicates that the target device supports reporting of timing information for additional detected paths for reported TOA-based measurements

ulSRStransmission This field, if present, indicates that the target device supports configuration and tramsission of UL-SRS

periodicalReporting This field, if present, specifies the positioning modes for which the target device supports periodicalReporting. This is represented by a bit string, with a one-value at the bit position means periodicalReporting for the positioning mode is supported; a zero-value means not supported. If this field is absent, the target device does not support periodicalReporting in CommonIEsRequestLocationInformation.

idleStateForMeasurements This field, if present, indicates that the target device requires idle state to perform positioning measurements.

motionMeasurements This field, if present, indicates that the target device supports reporting of motion measurements (deltaSFN and motionTimeSource) in NR-SignalMeasurementInformation. The presence of this field implies presence of sensor-MotionInformationSup in IE Sensor-ProvideCapabilities.

uebLocRtd This field, if present, indicates that the target device supports NR UE-based positioning exploiting TRP location and relative time difference information

uebLocBeam This field, if present, indicates that the target device supports NR UE-based positioning exploiting TRP location and beam information

#### 6.5.8.8	NR Positioning Capability Information Request

#### –	NR-Pos-RequestCapabilities

The IE NR-Pos-RequestCapabilities is used by the location server to request the capability of the target device to support NR positioning and to request NR positioning capabilities from a target device.

-- ASN1START

# NR-Pos-RequestCapabilities ::= SEQUENCE {

...

}

-- ASN1STOP

#### 6.5.8.9	NR Positioning Error Elements

#### –	NR-Pos-Error

The IE NR-Pos-Error is used by the location server or target device to provide NR positioning error reasons to the target device or location server, respectively.

-- ASN1START

# NR-Pos-Error-r16 ::= CHOICE {

locationServerErrorCauses-r16		NR-Pos-LocationServerErrorCauses-r16,

targetDeviceErrorCauses-r16			NR-Pos-TargetDeviceErrorCauses-r16,

...

}

-- ASN1STOP

#### –	NR-Pos-LocationServerErrorCauses

The IE NR-Pos-LocationServerErrorCauses is used by the location server to provide NR positioning error reasons to the target device.

-- ASN1START

# NR-Pos-LocationServerErrorCauses-r16 ::= SEQUENCE {

cause-r16		ENUMERATED	{	undefined,

assistanceDataNotSupportedByServer,

notProvidedAssistanceDataNotSupportedByServer,

assistanceDataSupportedButCurrentlyNotAvailableByServer,

...

},

...

}

-- ASN1STOP

#### –	NR-Pos-TargetDeviceErrorCauses

The IE NR-Pos-TargetDeviceErrorCauses is used by the target device to provide NR positioning error reasons to the location server.

-- ASN1START

# NR-Pos-TargetDeviceErrorCauses-r16 ::= SEQUENCE {

cause-r16		ENUMERATED {	undefined,

assistanceDataMissing,

unableToMeasureAnyTRP,

noUplinkSrsConfigured,

thereWereNotEnoughSignalsReceivedForUeBasedPos,

UebAssistanceDataMissing,

...

},

...

}

-- ASN1STOP

End Text Proposal Change