3GPP TSG RAN WG2#109-e	R2-2001394

Electronic meeting, 24th February – 6th March, 2020

Agenda Item	: 7.1.13

Source	: LG Electronics Inc.

Title	: Clarification for the condition of PUR configuration request procedure

Document for	: Discussion and Decision

1.	Introduction

According to RAN2 agreements [1], a UE may send the PUR configuration request message to receive PUR configuration from the network.

In this contribution, we suggest to discuss the condition of the PUR configuration request procedure.

2.	Discussion

According to the current running CR [2], the conditions to initiate the PUR configuration request procedure are listed as follows. The UE initiates the procedure if the serving cell supports PUR and the UL data size to be transmitted using PUR is smaller than or equal to the supported TBS based on the UE category.

Condition of the PUR configuration request procedure

1>	for CP transmission using PUR, SystemInformationBlockType2 (SystemInformationBlockType2-NB in NB-IoT) includes cp-PUR; or

1>	for UP transmission using PUR, SystemInformationBlockType2 (SystemInformationBlockType2-NB in NB-IoT) includes up-PUR;

1>	the size of the resulting MAC PDU including the total UL data size of the traffic is smaller than or equal to the maximum supported TBS based on the UE category.

However, the highlighted text is confusing because the UE does not have the MAC PDU including the UL data at present. Therefore, we suggest to update the sentence for the expected MAC PDU to be transmitted using PUR. In this way, the presentation is also aligned with the condition for initiating EDT.

1>	for mobile originating calls the size of the resulting MAC PDU including the total UL data is expected to be smaller than or equal to the TBS signalled in edt-TBS as specified in TS 36.321 [6], clause 5.1.1;

Proposal. Running CR on 36.331 should be updated as proposed in Annex regarding the condition of the PUR configuration request procedure.

Annex shows a TP capturing the change for the condition of the PUR configuration request procedure.

3.	Conclusion

Based on the discussion, we propose the following:

Proposal. Running CR on 36.331 should be updated as proposed in Annex regarding the condition of the PUR configuration request procedure.

4.	References

[1] R2-1916424 “RAN2 RAN2 agreements for Rel-16 additional enhancements for NB-IoT and MTC”, November 2019, Reno, USA

[2] Email discussion [108#28][R16 RRC] RRC Merge, “Draft_36331-g00”

5.	Annex: TP to 36.331

START OF CHANGES

#### 5.6.X.2	Initiation

A UE capable of performing UL transmissions using PUR// A NB-IoT UE in RRC_CONNECTED may initiate the procedure when all of the following conditions are met:

Editor's Note:	 Conflict between eMTC and NB-IoT CRs (SpecRap)

1>	for CP transmission using PUR, SystemInformationBlockType2 (SystemInformationBlockType2-NB in NB-IoT) includes cp-PUR; or

1>	for UP transmission using PUR, SystemInformationBlockType2 (SystemInformationBlockType2-NB in NB-IoT) includes up-PUR;

1>	the size of the resulting MAC PDU including the total UL data size of the traffic is expected to be smaller than or equal to the maximum supported TBS based on the UE category.

NOTE 1:	It is up to UE implementation how the UE determines whether the size of UL data is suitable for transmission using PUR.

Upon initiating the procedure, the UE shall:

1>	initiate transmission of the PURConfigurationRequest message in accordance with 5.6.X.3;

END OF CHANGES