3GPP TSG-RAN WG2 Meeting #109_e	R2-2001886

24 February – 06 March 2020

Source: 	Document Rapporteur (BlackBerry)

Title:	RAN2 agreements for Rel-16 additional enhancements for NB-IoT and MTC

Agenda Item:	7.1.1 and 7.2.1

Document for:	Email endorsement

# Introduction

This document lists RAN2 agreements made for Rel-16 Additional enhancements for NB-IoT (WI code NB_IOTenh3-Core; WID in RP-193224) and Rel-16 Additional MTC enhancements for LTE (WI code LTE_eMTC5-Core, WID in RP-192875), until and including the RAN2#109_e, 24 Feb – 06 March 2020.

NOTE:	Email agreements for LSs/CRs/specific topics are not listed in this document.

# 1	Additional MTC enhancements for LTE

## 1.1	Organisational

RAN2#103bis agreements: To capture the agreements from all Rel-16 IoT meetings in the same document as agreed in Rel-16 NB-IoT session. The draft can be provided in R2-1815744. [103bis#xx][ eMTC/NB-IoT R16] To capture the agreements from the meeting in a document for information. (BlackBerry) Intended outcome: Endorsed report Deadline: Thursday 2018-10-19 RAN2#104 to RAN2#107 agreements: None (except for this document, see clause 5). RAN2#107bis agreements: On R2-1912002 Reply LS on NAS Aspects of Mobile-terminated Early Data Transmission: RAN2 will wait for the response from SA3 to decide on whether MT-EDT is supported for 5GC. For both eMTC and NB-IoT, there is no need to capture procedure to indicate absence of up-CIoT-5GS-Optimisation/ cp-CIoT-5GS-Optimisation to upper layers. Four candidate values are defined for establishmentCause-r16 including spares. RAN2#108 agreements: On R2-1916292 CT1 LS on Extended NAS timers for Coverage Enhancement in 5GS: Send a LS reply which can be formulated in an email discussion after the meeting. RAN2#109_e agreements: None (except for this document, see clause 5).

## 1.2	Mobile-terminated (MT) early data transmission (EDT)

Treated jointly with 2.2 under this subclause.

RAN2#103bis agreements: RAN2 intends to support MT-initiated EDT for both CP and UP solutions. The intention to use MT-EDT is for user data, i.e. not for NAS signalling. RAN2#104 agreements: MT EDT are evaluated at least based on battery life, network resource efficiency, security, reliability and potential impact on core network. MT-EDT is intended for DL data which can be transmitted in one transport block. Use cases that require DL data transmission with or without UL data transmission as a response should be supported for MT-EDT. RAN2#105 agreements: DL data in paging message is excluded (Opt A). RNTI in paging message to schedule the DL data is excluded (Opt B). Working assumption: DL data scheduled, i.e. DL grant, in paging message is excluded (Opt C). Working assumption: DL data scheduled in paging occasion is excluded (Opt D). RAN2#105bis agreements: For both UP and CP solutions, an MT-EDT indication is needed in the S1 paging message to eNB. It is up to RAN3 to decide how such indication is provided to the eNB. RAN2 assumes that MME initiates MT-EDT. It is up to eNB to use MT-EDT based on e.g., UE capability. RAN2 assumes that DL data information is needed from S-GW/SCEF to the MME to assist MME to initiate MT-EDT. For the UP solution, the DL data are ciphered and sent over DTCH. For both UP and CP solutions eNB sends MT EDT indication to the UE via paging. For Msg-2 based solution (if agreed): A CF RACH resource is provided in the paging message to page the UE for MT-EDT. FFS whether/how security related concerns are addressed and how number of repetitions required and RNTI are provided. For Msg-4 based solution (if agreed): For UP solution, RRCConnectionResume is used in Msg4 in case UL transmission is expected in response. R2-1905276 LS on Mobile-terminated Early Data Transmission RAN2 LS out Rel-16 To:RAN3, SA2, CT1, and CT4. RAN2#106 agreements: DL data scheduled, i.e. DL grant, in paging message is excluded (Opt C). DL data scheduled in paging occasion is excluded (Opt D). DL data transmitted in Msg4 option B is excluded (Msg4-B). For Msg2-based solution (if agreed): No need to cipher the dedicated RACH resource provided in the paging message. RAN2 intends to introduce a mechanism to acknowledge that Msg2 was received by the intended UE. At least one dedicated RACH resource is provided in the paging message for MT-EDT. FFS what other parameters, e.g., CE level, subcarrier index, are needed to be provided. FFS: whether RNTI is provided in the paging message. For Msg4-based solution (if agreed): Working assumption: UE should be able to indicate in Msg3 if it intends to send data in the UL. RAN2#107 agreements: Msg2 based solution is not pursued in this release. Working assumptions: - For CP solution UE triggers RA using a Rel-15 EDT preamble. - For UP solution, UE may trigger RA using a non-EDT preamble. - For CP and UP solutions no new TB size for Msg3 is introduced. No indication is sent in Msg3 regarding whether an UL grant needs to be provided in Msg4, i.e. up to network implementation. In Msg4, same as MO-EDT, the network may move the UE to connected mode or release the UE to idle mode. FFS how to include user data in Msg4 for the CP solution if the UE is moved to connected mode. Fallback is supported in the case there is more downlink data to be sent, i.e. network can move the UE to connected mode in Msg4. From RAN2 stand point indicating the DL data size in the S1 paging message would be beneficial and sufficient, i.e. separate indication is not required R2-1911603 Reply LS on Mobile-terminated Early Data Transmission NB_IOTenh3-Core, LTE_eMTC5-Core To:SA2, RAN3 Cc:SA3, CT1, CT4. RAN2#107bis agreements: Non-EDT Msg1 and Msg2 are used in MT UP-EDT solution. Introduce a new resume cause for MT UP-EDT in RRCConnectionResumeRequest. Same as in MO-EDT; MT UP-EDT can be initiated only if NCC was provided in the previous connection. Same as in MO-EDT; RRCConnectionResumeRequest and RRCConnectionResume or RRCConnectionRelease are used in Msg3 and Msg4 for MT UP-EDT, respectively. Same as in MO-EDT; DL data in Msg4 in MT UP-EDT can be segmented. UL data can be multiplexed with RRCConnectionResumeComplete. For MT CP-EDT, UE transmits Rel-15 EDT preamble and the EDT UL grant is provided in Msg2. Same as MO-EDT; RRCEarlyDataRequest is used in Msg3 for MT CP-EDT. Add the MT-Data establishment cause to RRCEarlyDataRequest message for MT CP-EDT. For MT CP-EDT, confirm the WA that no new TBS for Msg3 is introduced. Same as in MO-EDT; RRCConnectionSetup or RRCEarlyDataComplete can be transmitted in Msg4 for MT CP-EDT. For MT CP-EDT, RRCConnectionSetup is extended to optionally include dedicatedInfoNAS. RAN2#108 agreements: FFS if UE category information, i.e., Cat-M2, Cat-NB2, is provided in the UE Radio Paging information container. If the UE supports MT-EDT (optional) for the CP solution, it shall also support MO-EDT for the CP solution. If the UE supports MT-EDT (optional) for the UP solution, it shall also support MO-EDT for the UP solution. Support of MT-EDT is optional at the UE without AS capability. It should be possible for the UE to indicate separate capability for CP MT-EDT and UP MT-EDT at NAS level. Send a reply LS to SA2 and CT1 to inform about the agreement above. R2-1916368 Reply LS on Mobile-terminated Early Data Transmission LS out Rel-16 LTE_eMTC5-Core, NB_IOTenh3-Core, 5G_CIoT To:SA2, CT1 Cc:RAN3, RAN, SA RAN2#109_e agreements: UE category information, i.e., Cat-M2 (Cat-NB2 for NB-IoT), is provided in the UE Radio Paging information container. FFS how the use of UE category information is captured in the specifications. Capture in stage 2 that the eNB uses the UE category included in the S1 paging message to trigger MT-EDT for message size beyond Cat M1/NB1. In RRC section 5.3.3.3a, for UP MT-EDT, lower layers are not configured for EDT. In RRC section 5.3.2.3, clarify that the mt-EDT is the one included in the UE’s paging record.

## UE-group wake-up signal (WUS)

See subclause 2.3 where this topic is treated jointly.

## Transmission in preconfigured resources

See subclause 2.4 where this topic is treated jointly.

## Scheduling multiple DL/UL transport blocks

Treated jointly with 2.5 under this subclause.

RAN2#103bis agreements: Working assumption: For MTCH in SC-PTM, configuration for multiple scheduling is transmitted in MCCH. Backwards compatibility is FFS. SPS is not supported for NB-IoT in Release 16. Enhancements for “SR with SPS for BSR” can be considered. RAN2#104 agreements: RAN2 intends to support separate/shared SC-MTCH transmission. Multiple TB scheduling is supported for UEs in connected mode. It is FFS if it is supported for EDT. UEs in connected mode are configured with multiple TB scheduling via dedicated RRC signalling assuming that a new DCI format is introduced. This is pending RAN1 agreement. UEs report capability to indicate support of multiple TB scheduling in connected mode. RAN2#105 to RAN2#105bis agreements: None. RAN2#106 agreements: Multi-TB scheduling for Msg4 during EDT (and MT-EDT) is not considered in Rel-16. For multi-TB scheduling, when HARQ ack bundling is configured, HARQ RTT timer is not started until the last TB has been received. FFS: For multi-TB scheduling for both LTE-M and NB-IoT, how to handle drx-InactivityTimer when HARQ Ack bundling is configured. UE capability for multiple TB is indicated separately for CE Mode A and CE mode B. UE capability for multiple TB is indicated separately for uplink and downlink. Wait for RAN1 before further discussion on multiple TB support for SC-PTM. RAN2#107 agreements: None. RAN2#107bis agreements: For NB-IoT, when multiple TBs are scheduled, drx-InactivityTimer is only (re-)started when both (UL) HARQ RTT Timers expire. For NB-IoT, drx-InactivityTimer is stopped when NPDCCH indication for transmission of multiple TBs is received. For NB-IoT, (UL) HARQ RTT timers for both HARQ processes are started in the subframe containing the last repetition of the (PUSCH transmission) PDSCH reception of the last TB. Working assumption: For NB-IoT, the length of HARQ RTT timer is set to k+2*N+1+delta. FFS if delta is deltaPDCCH Working assumption: For NB-IoT, the length of UL HARQ RTT timer is set to 1+delta. FFS if delta is deltaPDCCH For eMTC, (UL) HARQ RTT timers for all scheduled HARQ processes are started in the subframe containing the last repetition of the (PUSCH transmission) PDSCH reception of the last TB. RAN2#108 agreements: For non-interleaved NB-IoT case (clarified at RAN2#109_e): The length of HARQ RTT timer is set to k+2*N+1+delta. The length of UL HARQ RTT timer is set to 1+delta. Use deltaPDCCH concept (same as in legacy timer length) for both HARQ RTT timers. For NB-IoT; introduce a new IE, i.e., sc-mtch-InfoListMultiTB-v16xy in SCPTMConfiguration-NB message to signal scheduling information for SC-MTCH using multiple TBs transmission. For eMTC; the length of HARQ RTT timer is set to 7+l*N for unbundled HARQ ACK. For NB-IoT, for interleaved transmission, timer length of UL HARQ RTT timer is set to 1+delta. For NB-IoT, for interleaved transmission: - Without bundled HARQ, timer length of HARQ RTT timer is set to k+2*N+1+delta. - With bundled HARQ, timer length of HARQ RTT timer is set to k+N+3+delta. RAN2#109_e agreements: For LTE-M, the length of HARQ RTT timer is set to 7+k*N for bundled HARQ ACK, where k is equal to the number of HARQ ACK bundles. Remove Editor's note from the document that captures RAN2 agreements and clarify that those agreements are for non-interleaved NB-IoT case. Capture the following RAN1 agreement in RAN2 specifications: “For NB-IoT, support of multiTB-UL-r16 and multiTB-DL-r16 is conditional on support of two HARQ processes.”’ For LTE-M and NB-IoT, multiple TBs scheduling is enabled separately for uplink and downlink for unicast. Capture TP for MultiTB-Config-NB in R2-2000644 in the running RRC CR for NB-IoT. For LTE-M and NB-IoT, multiple TBs scheduling in multicast is optional without capability reporting. For LTE-M and NB-IoT, the configuration for scheduling gap is in SCPTMConfiguration(-NB) (SC-MCCH).

## 1.6	Quality report in Msg3

RAN2#103bis agreements: Quality report in Msg3 is introduced for EDT. FFS for non-EDT. RAN2#104 agreements: RAN2 waits for progress on the discussion of MT-EDT before deciding on whether channel quality report in Msg3 is introduced for EDT. Channel quality report in Msg3 is introduced for non-EDT. RAN2#105 agreements: UE reports at most one DL quality measurement in Msg3 transmission. This is pending RAN1 agreement. For EDT, new MAC CE will be defined to report the channel quality in Msg3. FFS whether an LCID (lowest priority) or eLCID is used. Channel quality in Msg3 is reported with no explicit differentiation on whether the measurement was made in T1 or T2. RAN2#105bis agreements: eLCID based solution is not supported. A new LCID is used for eMTC pending approval in the main room. IoT informs the main room regarding the pending agreement above and the alternatives, e.g. to use one of LCID values reserved for NB-IoT or sidelink. Report from main session: Agreement made in IoT/MTC breakout session to use one of the reserved LCID values is confirmed. R2-1905277 LS on quality report in Msg3 for LTE-M RAN2 LS out Rel-16 To:RAN1 Cc:RAN4 RAN2#106 agreements: None. RAN2#107 agreements: Explicit signaling is introduced to trigger the aperiodic DL quality report in connected mode. FFS whether DCI or MAC CE. MAC CE is used to provide the aperiodic DL quality report in connected mode. DL quality report in Msg3 in idle mode is enabled via system information broadcast. UE capability reporting is not needed for the DL quality report in Msg3 in idle mode. UE capability reporting is introduced for the aperiodic DL quality report in connected mode. RAN2#107bis agreements: For 8-bit DL quality report, same MAC CE is used for reporting in Msg3 and connected mode. For 8-bit DL quality report; LCID value for the quality report is transmitted in addition to the LCID value for UL-CCCH. Codepoint/index of “10001” is used for 8-bit DL quality report for eMTC. For EDT, 8-bit DL quality report has lower priority than MO data from UL-CCCH. For non-EDT, 8-bit DL quality report has higher priority than MO data not from UL-CCCH, i.e., one level above “data from any Logical Channel, except data from UL-CCCH”. For non-EDT, R+F2+E MAC subheader is used for 2-bit DL quality report. 2 separate indicators are introduced in SIB to enable 8-bit and 2-bit DL quality report, i.e., FFS if it is possible to indicate 2-bit only. RAN2#108 agreements: DL quality in Msg3 in connected mode is not supported. RAN2#109_e agreements: RAN2 confirms that 2-bit CQI report in MSG3 is supported. Quality Report trigger in Connected Mode for eMTC is the same MAC CE as agreed for NB-IoT. When 2-bit CQI reporting enabled in MSG3 then 8-bit reporting shall also be enabled. Separate flags are not required to enable/disable CQI reporting per CE level.

## 1.7	MPDCCH performance improvement using CRS

RAN2#103bis agreements: eNB enables CRS for improving channel estimation on MPDCCH via broadcasted RRC signalling. RAN2#104 agreements: CRS for improving channel estimation on MPDCCH is configured via SIB2. This agreement can be revisited if RAN2 identifies that it would be more reasonable to configure it via SIB1-BR. It is FFS whether configuration would implicitly mean to enable the feature. UEs report capability to indicate support of using CRS for improving MPDCCH channel estimation. It is FFS if enabling/disabling is needed for the UE. RAN2#105 to RAN2#105bis agreements: None. RAN2#106 agreements: No need to introduce a separate indication to enable the feature. Configuration of CRS for MPDCCH performance improvement enables the feature implicitly for UEs in Idle mode. Working assumption: If dedicated configuration is not provided in connected mode, UE can use the broadcasted configuration if it exists. RAN2#107 agreements: None. RAN2#107bis agreements: UE shall consider the feature disabled in idle mode when configuration in idle mode is not broadcasted. The dedicated configuration is released when UE is released to idle mode. FFS when UE is suspended. RAN2#108 agreements: If a dedicated configuration is not provided, the UE uses the broadcasted configuration. When UE is suspended, dedicated configuration is released. RAN2#109_e agreements: Power ratio can be included in dedicated signalling (in addition to SIB2). Localized mapping type is possible to be included only in dedicated signalling. Value “predefined” is DEFAULT. It should be possible to enable the feature for IDLE mode UEs but disable in CONNECTED mode. TP1 in R2-2001879 (with separate IEs CRS-ChEstMPDCCH-ConfigCommon-r16 and CRS-ChEstMPDCCH-ConfigDedicated-r16) is baseline for running CR.

## 1.8	Improvements for non-BL UEs

RAN2#103bis agreements: RAN2 intends to support CMAS/ETWS for non-BL UEs in CE mode in connected mode. RAN2#104 agreements: None. RAN2#105 agreements: SIBs for ETWS and CMAS are not provided via dedicated signaling. RAN2#105bis agreements: Working assumption: In connected mode non-BL UEs in CE monitor MPDCCH to receive ETWS indication and/or CMAS indication using CSS in the same narrowband where unicast transmission can be received. R2-1905278 LS on ETWS/CMAS in connected mode narrowband RAN2 LS out Rel-16 To:RAN1 RAN2#106 agreements: None. RAN2#107 agreements: In connected mode, non-BL UEs in CE monitor MPDCCH to receive ETWS and/or CMAS notification using Type 0 CSS in the same narrowband where unicast transmission can be received. UE stays in RRC_CONNECTED after receiving the indication for ETWS/CMAS notification. It is up to UE implementation how ETWS/CMAS notification is acquired. RAN2 does not intend to introduce any signalling optimization regarding MIB, SIB1-BR and SIB10/11/12 acquisition. RAN2 understands that UEs in CE Mode B may not be able to acquire ETWS notification within 4 seconds requirement. RAN2#107bis agreements: Non-BL UE supporting ETWS/CMAS in CE mode is required to monitor the same subframes as in legacy, i.e. non-BL UEs in CE, to receive ETWS/CMAS notification when in connected mode. Introduce capability for support of ETWS/CMAS indication reception in RRC CONNECTED by UE supporting CE Mode A. Introduce capability for support of ETWS/CMAS indication reception in RRC CONNECTED by UE supporting CE Mode B. Introduce in dedicated signaling an indication whether ETWS/CMAS notification is enabled for UE in connected mode. RAN2#108 agreements: Use SI-RNTI for ETWS/CMAS notification. UE is enabled to monitor for ETWS/CMAS notification via dedicated signalling. The reception types for SI-RNTI for ETWS/CMAS notification is captured in TS 36.302. For feedback based on CSI-RS, capability bit is introduced only for CE Mode A. RAN2#109_e agreements: None.

## 1.9	Stand-alone deployment

RAN2#103bis agreements: The standalone deployment supports legacy operations for legacy BL UEs or UEs in CE mode. LTE control channel region for DL transmission is supported for Rel-16 BL UEs and UEs in CE mode. RAN2#104 agreements: None. RAN2#105 agreements: Rel-16 non-BL UEs supporting CE mode can operate in CE mode in both idle and connected mode in standalone deployment regardless of whether criterion S for normal coverage is fulfilled. FFS if legacy non-BL UEs supporting CE mode can operate in CE mode in both idle and connected mode in standalone deployment regardless of whether criterion S for normal coverage is fulfilled. Introduce signalling in SIB1-BR to enable using the LTE control channel region for broadcast transmission assuming that only an indicator is needed. FFS: Introduce signalling in a dedicated message to enable using the LTE control channel region for DL unicast transmission. Introduce UE capability to indicate that UE supports unicast PDSCH reception in LTE control channel region. RAN2#105bis agreements: RAN2 agrees with the intention that non-BL UEs should be able to camp in a standalone cell when it is not possible to acquire SIB1. RAN2#106 agreements: None. RAN2#107 agreements: LTE Control Channel Region for DL transmission: FFS if, from Rel-16, it should be possible for a non-BL UE that fullfills S criteria for normal coverage to camp in a “normal” cell, i.e. not standalone, in enhanced coverage. This discussion will continue as part of the AI 12.1.8 Improvements for non-BL UEs. RAN2#107bis to RAN2#108 agreements: None. RAN2#109_e agreements: In standalone deployment, if a UE considers itself to be in enhanced coverage with S criteria of normal coverage fulfilled, absolute priorities for cell reselection are used (i.e. UE does not switch to ranking as it would when in enhanced coverage due to S-criteria).

## Mobility Enhancements

RAN2#103bis agreements: None. RAN2#104 agreements: An LS is sent to inform RAN1 that the overhead introduced by signalling agreed parameters per cell is excessive, and ask which parameters are carrier specific for potential optimization. Relaxation of RRM measurements for UEs using WUS: RAN2 will wait for RAN4 to progress. R2-1818662 LS on RSS based measurements signalling LS out Rel-16 To: RAN1. LTE_eMTC5-Core. RAN2#105 to RAN2#105bis agreements: None. RAN2#106 agreements: Add num-DRX-CyclesRelaxed to RadioResourceConfigCommon for eMTC. Capture the following in the corresponding running 36.331 CR: - “numDRX-CyclesRelaxed: Maximum number of consecutive DRX cycles during which the UE can use WUS for synchronisation and skip serving cell measurements, see TS 36.213 [23]. Value n1 corresponds to 1 DRX cycle, value n2 corresponds to 2 DRX cycles and so on. The actual duration during which the UE can skip serving cell measurements is: Min (signalled value x paging cycle, 2.56s), where paging cycle is the value of defaultPagingCycle parameter provided in SystemInformationBlockType2.” FFS if the value in Min() is 2.56sec. RAN2#107 to RAN2#109_e agreements: None.

## Coexistence with NR

RAN2#103bis to RAN2#108 agreements: None. RAN2#109_e agreements: For NB-IoT: Resource reservation configuration for NR coexistence is provided via dedicated RRC signaling. Resource reservation configuration for NR coexistence is provided in PhysicalConfigDedicated-NB. New Rel-16 IE (s) can be introduced in PhysicalConfigDedicated-NB for providing resource reservation configuration for NR coexistence. For FDD, two independent parameters in UL and DL are needed for providing UL and DL resource reservation configuration separately. For TDD, two independent parameters in UL and DL are needed for providing UL and DL resource reservation configuration separately. One new IE structure for resource reservation configuration for both DL and UL, and for both TDD and FDD are introduced. Independent definition for periodicity and start position parameters are introduced and the dependency between them can be clarified in the field description. Working assumption: Introduce two UE capabilities for handling resources reservation on DL or UL in PhyLayerParameters-NB-v16xy. Working assumption: Two UE capabilities for handling resources reservation on DL or UL in PhyLayerParameters-NB-v16xy can be applied to both FDD and TDD, e.g., with separate values for FDD or TDD. Working assumption: Two UE capabilities for handling resources reservation on DL or UL can be introduced into TS 36.306, e.g., section 4.3.4. For eMTC: Configurations related to resource reservation and DL subcarrier puncturing for NR coexistence are provided via dedicated RRC signaling. The configurations related to resource reservation and DL subcarrier puncturing for NR coexistence can be provided in PhysicalConfigDedicated. For FDD, two independent parameters in UL and DL are needed for providing UL and DL resource reservation configuration separately. For TDD, two independent parameters in UL and DL are needed for providing UL and DL resource reservation configuration separately. One new IE structure for resource reservation configuration for both DL and UL, and for both TDD and FDD are introduced. A parameter for providing DL subcarrier puncturing configuration can be introduced in PhysicalConfigDedicated, which is for both FDD and TDD. Working assumption: Introduce four UE capabilities for handling resources reservation on UL and DL, and for CE mode A and CE mode B separately, in PhyLayerParameters-v16xy. Working assumption: Introduce two UE capabilities for handling DL subcarrier puncturing for CE mode A and CE mode B separately, in PhyLayerParameters-v16xy. Working assumption: Six UE capabilities mentioned in Proposal 2-7 and Proposal 2-8 in R2-2001883 for handling resources reservation or DL subcarrier puncturing can be applied to both FDD and TDD, e.g., with separate values for FDD or TDD. Working assumption: Six UE capabilities for handling resources reservation or DL subcarrier puncturing can be introduced into TS 36.306. Send a LS to RAN1 to check whether it should be possible to have a common configuration for all NB-IoT carriers for resource reservation for NR coexistence. For NB-IoT and eMTC whether it is possible to optimize signalling.

## Connection to 5GC

RAN2#105 agreements: R2-1902436 Response LS on Completion of Study on Cellular IoT support and evolution for the 5G System LS out Rel-16 NB_IOTenh3-Core, LTE_eMTC5-Core To:SA2. RAN, RAN3. Cc:SA, CT. R2-1902431 Response LS on on eDRX cycles for CM-CONNECTED with RRC inactive LS out Rel-16 NB_IOTenh3-Core, LTE_eMTC5-Core To:SA2, CT1, CT4, RAN3. RAN2#105bis agreements: Support of eDRX in CM-IDLE and EDT: Working assumption: Introduce a new IE “cp-EDT-5GC-r16” in SIB2-BR/SIB2-NB to indicate ng-eNB connected to 5GC supports CP-EDT optimization. Rel-15 eMTC/NB-IoT EDT PRACH Configuration parameters can be commonly used for both EPC and 5GC. Use critical extension for RRCEarlyDataRequest/RRCEarlyDataRequest-NB messages for CP-EDT eMTC/NB-IoT UEs connecting to 5GC. Introduce 48-bit 5G S-TMSI in critical extension of RRCEarlyDataRequest /RRCEarlyDataRequest-NB messages. For eMTC, in critical extension of RRCEarlyDataRequest message, add establishment cause of mo-Data. For NB-IoT, in critical extension of RRCEarlyDataRequest-NB message, add establishment cause of mo-Data and mo-ExceptionData. Working assumption: For eMTC/NB-IoT, delayTolerantAccess is not applicable for 5GC. Update stage 2 spec to capture that idle mode eDRX feature is supported for eMTC and NB-IoT connected to 5GC. For eMTC, hyperSFN in SIB1-BR is commonly used for both EPC and 5GC. For NB-IoT, hyperSFN-LSB in MIB-NB and hyperSFN-MSB in SIB1-NB are commonly used for both EPC and 5GC. For eMTC connected to 5GC, introduce a flag in SIB1-BR to indicate if idle mode eDRX is allowed in the cell. Working assumption: No indication of support for idle mode eDRX is needed for NB-IoT connected to 5GC. For eMTC and NB-IoT, systemInfoModification-eDRX is commonly used in paging messages for both EPC and 5GC. Use 5G S-TMSI as input for Hash ID to calculate PH and PTW_start. Support of RRC_INACTIVE and eDRX in CM-CONNECTED: RRC_CONNECTED Mode extended long DRX cycle, e.g. 5.12 and 10.24 sec, is supported for eMTC connected to 5GC. FFS whether new optional UE capability is required. RRC_CONNECTED Mode long DRX cycle, e.g., 5.12 and 10.24 sec, is supported for NB-IoT connected to 5GC. Mandatory to support without new UE capability. For eMTC, eDRX-Config-CycleStartOffset and drx-StartOffset are commonly used for both EPC and 5GC. For NB-IoT, DRX-Cycle and drx-StartOffset are commonly used for both EPC and 5GC. RAN2#106 agreements: Support of RRC_INACTIVE and eDRX in CM-CONNECTED: RRC_INACTIVE state in NB-IoT connected to 5GC is not supported. RRC_INACTIVE state with short eDRX cycles is optionally supported for eMTC connected to 5GC with capability signalling. FFS if EDT in RRC_INACTIVE state is not supported in Rel-16. UP optimization solution is supported for both eMTC and NB-IoT connected to 5GC with capability signalling. eLTE is assumed to be the baseline for RRC_INACTIVE state for eMTC UEs connected to 5G. Working assumption: Same cell selection/reselection criteria and measurement rules for BL UEs and non-BL UEs in CE in RRC_IDLE when attached to EPC are applied to BL UEs and non-BL UEs in CE in RRC_INACTIVE or RRC_IDLE when attached to 5GC. For transmission over DRB, NB-IoT/eMTC UE connected to 5GC uses the same AS security algorithms as eLTE regardless of PDCP type. KeNB root key is derived from Kamf as specified in TS 33.501. For NB-IoT/eMTC UE connected to 5GC, AS security algorithms are indicated by using LTE code points, e.g., EIA, EEA. R2-1908202 Reply LS on Conclusion on 5G CIoT user plane small data solution LS out Rel-16 FS_CIoT_5G To:SA2, RAN3 Cc:RAN, SA, CT RAN2#107 agreements: Support of eDRX in CM-IDLE and EDT: Support of User Plane CIoT 5GS optimization is optional for both eMTC and NB-IoT devices connected to 5GC without capability signaling. Indication for support is provided in Msg5, i.e. RRCConnectionSetupComplete. For NB-IoT and eMTC, introduce a new parameter, up-CIoT-5GS-Optimisation-r16, in SIB1-BR/SIB1-NB to indicate per PLMN support of User plane CIoT 5GS optimisations. For eMTC, broadcast signaling is not required to indicate support of N3 data transfer since it is mandatory for both the UE and the network. For NB-IoT, introduce a new parameter, n3-5GS-DataTransfer-r16, in SIB1-NB to indicate per PLMN support of N3 user plane data transfer. For eMTC, introduce new parameters, up-CIoT-5GS-Optimisation-r16 and cp-CIoT-5GS-Optimisation-r16 in RRCConnectionSetupComplete when accessing 5GC. For NB-IoT, introduce new parameters, up-CIoT-5GS-Optimisation-r16 and n3-5GS-DataTransfer-r16 in RRCConnectionSetupComplete-NB when accessing 5GC. For NB-IoT and eMTC, when the UE in RRC_IDLE using the UP CIoT optimisation reselects to another CN type, it discards the UE AS context and the UE identity. For NB-IoT and eMTC, UE goes to RRC_IDLE and initiates NAS recovery procedure when the UE switches between CN types at RRC Connection re-establishment. Introduce a new IE up-EDT-5GC-r16 in SIB2-BR/SIB2-NB to indicate ng-eNB connected to 5GC supports UP MO-EDT. Introduce a new UE capability earlyData-UP-5GC-r16 in UE-EUTRA-Capability, UE-Capability-NB to indicate support of UP MO-EDT in 5GC. Other: From RAN2 standpoint it is feasible to introduce LTE-M indication in RRCConnectionSetupComplete message if UE selected 5GC. R2-1911601 Reply LS on LTE-M identification in 5GC LS out Rel-16 5G_CIOT To:SA2, RAN3 Cc:RAN RAN2#107bis agreements: Support of eDRX in CM-IDLE and EDT: For NB-IoT and eMTC connected to 5GC: - NCC is always provided during suspension. - adopt early security activation for resumption from RRC_IDLE. RAN2 intends to have the same UE identity, i.e. I-RNTI or resumeID, for UP CIoT 5GS Optimisation for both eMTC and NB-IoT. For NB-IoT and eMTC connected to 5GC, full configuration in RRCConnectionResume(-NB) is supported. For NB-IoT and eMTC connected to 5GC, the EPS UP-EDT procedure is used. For NB-IoT and eMTC connected to 5GC, security is not deactivated and SRB1 is not suspended at EDT fallback for UP-EDT. R2-1914045 LS on RAN2 agreements for 5GC CIoT LSout Rel-16 LTE_eMTC5-Core, NB_IOTenh3-Core To:RAN3. Support of RRC_INACTIVE and eDRX in CM-CONNECTED: - For eMTC when connected to 5GC: The Rel-15 suspension/ resumption procedure to/from RRC_INACTIVE applies. eMTC UEs in RRC_INACTIVE monitor RAN initiated paging and CN initiated paging via I-RNTI and 5G-S-TMSI respectively. Upon reception of CN paging in RRC_INACTIVE, eMTC UE moves to RRC_IDLE without suspension. For eMTC, ran-PagingCycle is extended with values rf512 and rf1024. The RNA configuration and RNA update procedure in Rel-15 RRC_INACTIVE apply to eMTC. The resume cause ‘ran-Update’ is applicable to eMTC UEs connected to 5GC that support RRC_INACTIVE. IDLE mode cell selection/reselection criteria and measurement rules also apply in RRC_INACTIVE state for eMTC UEs connected to 5GC. Neighbouring cell relaxed monitoring is also applicable in RRC_INACTIVE state for eMTC UEs connected to 5GC. Indication in SIB2-BR to indicate that the eNB supports RRC_INACTIVE can be implicit. If a UE in RRC_INACTIVE moves to an eNB that does not support RRC_INACTIVE, the UE enters RRC_IDLE, i.e. not suspended state. No additional capability is needed to indicate support for short extended DRX operation in RRC_INACTIVE state. EDT is not supported in RRC_INACTIVE in Rel-16. RAN2#108 agreements: PTW is not introduced for RRC_INACTIVE state with short eDRX. FFS: If configured with eDRX cycle in CM-IDLE, eMTC UEs in RRC_INACTIVE monitor paging occasions (POs) during CM-IDLE PTW according to the cell paging cycle or UE specific DRX if configured. FFS: if “short eDRX for RRC-INACTIVE” is configured, eMTC UEs in RRC_INACTIVE monitor paging occasions (POs) according to the RAN paging cycle, i.e. 5.12sec, 10.24sec, for RRC_INACTIVE. FFS: if “short eDRX for RRC-INACTIVE”, i.e. 5.12sec, 10.24sec, is not configured, eMTC UEs in RRC_INACTIVE monitor paging occasions (POs) according to the min {UE specific DRX cycle, default DRX cycle, RAN paging cycle}. Rel-15 WUS and Rel-16 Group WUS are not supported for eMTC UEs in RRC_INACTIVE. I-RNTI is used as the UE identifier for UP CIoT 5GS Optimization for both eMTC and NB-IoT. Send a LS to RAN3 to inform them about the agreements in RAN2. UE signaling category M1 shall set the LTE-M indication in RRCConnectionSetupComplete message. Introduce ‘LTE-M Indication enumerated {TRUE} optional’ in RRCConnectionSetupComplete message. LTE-M indication in RRCConnectionSetupComplete message is only sent when UE selects 5GC. RAN2 understands that a UE signalling Cat M1 and optionally Cat M2 shall not signal any other optional UE category. R2-1916369 LS on UE identifier for UP CIoT 5GS Optimisation LS out Rel-16 To:RAN3 LTE_eMTC5-Core, NB_IOTenh3-Core RAN2#109_e agreements: The three proposals below are describing the legacy behaviour and companies agree that they are applicable to 5GC: - When idle mode eDRX is configured and TeDRX is larger than 5.12 s, eMTC UEs in RRC_IDLE only need to monitor the paging occasions during the PTW. The UE monitors the paging occasions according to the shortest of the cell default paging cycle and the UE specific DRX (if configured). - When idle mode eDRX is configured and TeDRX is equal to 5.12 s, eMTC UEs in RRC_IDLE monitor the paging occasions according to TeDRX. - When idle mode eDRX is not configured, eMTC UEs in RRC_IDLE monitor the paging occasions according to the shortest of the cell default paging cycle and the UE specific DRX (if configured). DRBs are resumed upon receiving RRCConnectionResume in UP optimization when connected to 5GC. When idle mode eDRX is not configured, eMTC UEs in RRC_INACTIVE monitor the paging occasions according to the shortest of the cell default paging cycle, the UE specific DRX (if configured), and the RAN paging cycle (if configured). When idle mode eDRX is not configured, eMTC UEs in RRC_INACTIVE cannot be configured with values 5.12 sec and 10.24 sec DRB resumption for EDT for eMTC UEs connected to 5GC follows the same principle as in EPC, i.e.: - drb-ContinueROHC is provided in RRCConnectionRelease message triggering the suspension in RRC_IDLE. The flag applies to all DRBs. - When resuming the DRBs for EDT, RRC procedure text triggers PDCP re-establishment and provides NR PDCP with the drb-ContinueROHC indication received in RRCConnectionRelease message. When idle mode eDRX is configured, eMTC UEs in RRC_INACTIVE monitor the paging occasions (POs) during CM-IDLE PTW according to the min {UE specific DRX cycle, default DRX cycle, RAN paging cycle} and monitor paging occasions (POs) outside CM-IDLE PTW according to RAN paging cycle. PDCP Suspend is triggered at the time of suspension to RRC_IDLE for eMTC UEs connected to 5GC. DRB resumption for non-EDT for eMTC UEs connected to 5GC follows the same principle as in RRC_INACTIVE, i.e.: - When resuming the DRBs for non-EDT, RRC procedure text does not trigger PDCP re-establishment. - PDCP re-establishment and ROHC continuation for each DRB are triggered by the presence of the respective flags in RRCConnectionResume message as specified in TS 38.331 [82], clause 5.3.5.6. When resuming the RRC connection, the default RLC configuration and default (NR) PDCP configuration is applied to SRB1 for eMTC and NB-IoT UEs connected to 5GC.

## Other

RAN2#103bis to RAN2#108 agreements: None.

# Additional enhancements for NB-IoT

## Organisational

RAN2#103bis agreements: To create document agreed after each meeting capturing agreements from all meetings. Copy/paste of chair notes, not intended to be a further discussion of agreements. One week email agreement after each meeting BlackBerry will be rapporteur of the document. RAN2#104 to RAN2#109_e agreements: None (except for this document, see clause 5).

## Mobile-terminated (MT) early data transmission (EDT)

See subclause 1.2 where this topic is treated jointly.

## UE-group wake-up signal (WUS)

Treated jointly with 1.3 under this subclause.

RAN2#103bis agreements: The aim of UE grouping for WUS is reducing the false alarm probability. At least UE_ID based grouping is supported for UE-Group based WUS. This doesn’t exclude other options. Further discuss whether the following are supported: - Service based grouping - DRX/eDRX based grouping - Gap based grouping. - Coverage based grouping RAN2#104 agreements: Further discuss the benefit and feasibility of using service based parameters for grouping in addition to UE-ID. Can discuss group distribution further, including Rel-15/16 mechanism interaction, once we know more about number of groups and more about the grouping solution (e.g. service based parameters). RAN2 will decide on the UE to WUS group mapping. RAN2#105 agreements: Feasibility of the solution based on the following attributes for deriving the service-type for GWUS can be studied further: - Paging Probability - Mobility. RAN2#105bis agreements: Additional grouping based on DRX/eDRX is not supported. Coverage based grouping is not supported. Additional grouping based on gap is not supported. - FFS whether number of groups can depend on gap duration. RAN2#106 agreements: WUS grouping based on paging probability is beneficial for improving false wake-up probability for UEs which are not frequently paged, but may increase false wake-up probability for UEs which are frequently paged. Send an LS to SA2, cc RAN3, CT1, to ask about the feasibility and impact for the MME to provide information on UE paging probability. R2-1908260 LS on assistance indication for WUS grouping LS out Rel-16 LTE_eMTC5-Core, NB_IOTenh3-Core To:SA2 Cc: CT1, R3 RAN2#107 agreements: Paging probability information is negotiated between the UE and MME via NAS signalling. This paging probability information needs to be provided by S1 paging. eNB configures, via broadcast, the relation between this paging probability information and WUS group on Uu interface. R2-1911589 Reply LS on assistance indication for WUS LS out Rel-16 LTE_eMTC5-Core, NB_IOTenh3-Core To: SA2, CT1, RAN3. RAN2#107bis agreements: Rel-16 WUS can be configured without Rel-15 WUS. If both R15 and R16 WUS are configured, then maximum duration, time offset for different gap types, and transmit power for WUS sequence need to be configured with the same values for both R15 and R16. - FFS: numPOs The time locations of R16 WUS resources need to be known by the UE. Signalling details can be discussed as part of stage 3 CR. FFS: A WUS group cannot be mapped to both UEs using UE-ID based only and UEs using UE-ID + paging probability based grouping. If both Release 15 WUS and Release 16 WUS are configured then values configured for parameters in Release 15 WUS also apply to Release 16 WUS. If only Release 16 WUS is configured then values for the following parameters shall be configured: MaxDurationFactor, numPOs, FrequencyLocation (eMTC only), timeOffsetDRX, timeOffset-eDRX-short, timeOffset-eDRX-Long, numDRX-CyclesRelaxed. FFS: A different number of WUS resources can be configured for each gap type (DRX, eDRX short, eDRX long). - FFS: When one or more R16 WUS resources are configured, the WUS Paging probability mapping to WUS group can be provided separately for each gap type (i.e. timeoffset-DRX, timeoffset-eDRX-short, timeoffset-eDRX-long). Same or different number of WUS groups can be configured for each R16 WUS resource. UE WUS group hopping between WUS resources is configured per cell. FFS how to configure the following agreed cases: - When R15 WUS is configured then all R16 WUS resources are adjacent to R15 WUS in the time domain (for eMTC and NB-IoT) and in the frequency domain (for eMTC only). - When only R16 WUS resources are configured, frequency location provided for the first R16 resource (i.e. FrequencyLocation). (for eMTC only). - When only R16 WUS resources are configured, location in time provided for the first R16 resource for each gap type (i.e. timeOffset-DRX, timeOffset-eDRX-short, timeoffset-eDRX-Long). (for eMTC and NB-IoT). RAN2 assumes one or more UEs may not be configured at NAS with paging probability. If R16 WUS is configured, at least one WUS group will be assigned to UEs that are not configured with paging probability. FFS Mapping of paging probabilities to WUS group. R16 WUS can be supported on all or subset of the paging carriers/paging narrowbands. RAN2#108 agreements: Inform SA2 that RAN2 recognise the issue created by a mobile UE using WUS but RAN2 has not yet concluded on a solution Ask SA2 to consider introducing WUS awareness in MME from Release 15 numPOs is the same for all WUS resources. Allow configuration of different number of WUS resources for each gap type. The signalling allows configuration of any of the following: - eNB configures all WUS groups for UE ID based grouping. - eNB configures all WUS groups for paging probability based grouping. In this case WUS group(s) corresponding to the highest probability are used for UEs without paging probability configured. - Combination of above two configurations; some WUS groups for paging probability based grouping and some WUS groups for UE ID based grouping. - If more than one WUS group corresponds to the UE’s configured probability then UE ID is used to determine a WUS group. Number and value of paging probability thresholds be the same for all gap types. Each gap type can have different number of WUS groups for each probability threshold. For each paging probability threshold there shall be at least 1 WUS group. WA: Maximum number probability thresholds is 3 giving 4 group. R2-1916440 Reply LS on assistance indication for WUS LS out Rel-16 NB_IOTenh3-Core, LTE_eMTC5-Core To:SA2, CT1, RAN3 Cc: SA RAN2#109_e agreements: For NB-IoT, RAN2 agree signaling changes proposed in Table 5 from R2-2000306 as the baseline. For NB-IoT, RAN2 assume the changes proposed in Table 7, 8 and 9 from R2-2000306 as the baseline for signalling group WUS information. For NB-IoT, RAN2 use the changes proposed in Table 10 from R2-2000306 as the baseline. For eMTC, RAN2 agree to use the changes proposed in Table 12 from R2-2000306 as the baseline. For eMTC, RAN2 assume the changes proposed in Table 15, 16 and 17 from R2-2000306 as the baseline for signalling group WUS information. For eMTC and NB-IoT support the same paging probability range and granularity. No special handling of WUS resource overlap is specified and UE use the WUS resource corresponding to its gap capability. Update stage 2 to explain group WUS in more detail using text proposed in R2-2000639 as starting point. From RAN2 point of view paging escalation does not need to be mandated. TP in R2-2001790 is endorsed. Working assumption: For NB-IoT, if only one R16 WUS resource is configured and no Release 15 WUS resource is configured then R16 WUS resource is always in primary location. Support of Release 16 WUS is independent to support of Release 15 WUS. Define WUS group selection based on the formula defined in R2-2001472. FFS: Code points for paging probability thresholds. Mechanism to minimize false wake-up.

## Transmission in preconfigured resources

Treated jointly with 1.4 under this subclause.

RAN2#103bis agreements: Transmission in dedicated preconfigured uplink resources in IDLE mode is supported for UEs with a valid timing advance. Initially we will focus on dedicated preconfigured uplink resources in idle mode - Shared resources can also be discussed RAN2#104 agreements: The eNB configures the dedicated preconfigured uplink resources via RRC dedicated signaling. Methods for eNB to obtain information used to help configuring the dedicated preconfigured uplink resource to the UE is FFS. Periodic D-PUR with duration is supported FFS if one shot D-PUR is supported. Release of the dedicated preconfigured resources are supported, details for NW triggered and UE triggered are FFS. RAN2#105 agreements: Multi-shot D-PUR is supported with the possibility to configure as a single shot. UE may perform a D-PUR request/information, if D-PUR is indicated as enabled in the cell. Network makes the decision on the D-PUR configuration. Request/information can include: - Requested TBS - Requested periodicity - Other information FFS. The eNB can (re)configure and release D-PUR by dedicated RRC signalling. D-PUR configuration is released when the eNB doesn’t detect “m” consecutive UE transmissions. The UE must release the D-PUR when it does a RA procedure on a new cell. D-PUR configuration can be set up without a pre-defined end (infinite). RAN2#105bis agreements: From RAN2 point of view it is possible to configure TBS for D-PUR for both NB-IoT and eMTC up to the maximum supported based on the UE category and TBS capability. For UP the UE may transmit D-PUR release request/(re)configuration request when transmitting using D-PUR. FFS For CP. The following are FFS: Whether a UE can have more than one parallel D-PUR configurations Ability of the UE to request (indicate) a time offset The range of values and limits of preconfigured D-PUR allocations. Whether mechanisms for a UE to reject a D-PUR configuration including by explicitly or implicitly releasing are needed. RAN2#106 agreements: RRC response message needs to be supported by the UE and could be used in all cases. For some cases L1 signalling is sufficient to acknowledge, i.e. RRC response message is not needed. RAN2 assumes the L1 signalling for acknowledgement is sent only after the eNB determines there is no pending downlink data or signalling. RAN2 assumes that the configuration for D-PUR provided by RRC signalling is not updated via L1 signalling, and should check with RAN1 what parameters are planned to be (re)configured using DCI. It is feasible to provide the UE with a UE-specific RNTI for D-PUR. Common or shared RNTI is also feasible. The RNTI used for D-PUR is signalled together with other D-PUR configuration. R2-1908196 Reply LS on PUR Working Assumptions for NB-IoT and LTE-M LS out Rel-16 LTE_eMTC5-Core, NB_IOTenh3-Core To: RAN1. RAN2#107 agreements: D-PUR procedural steps: Valid TA is a requirement in order to initiate D-PUR transmission. The UE may use the D-PUR resource to send RRCConnectionRequest or RRCConnectionResumeRequest to establish or resume RRC connection. - FFS: whether the UE can send part of the data using the padding in this case. FFS: whether the UE can segment and send part of the data using the D-PUR resource. For the CP solution, the uplink data are encapsulated as a NAS PDU in an uplink RRC message transmitted in CCCH. For the UP solution, the uplink data are transmitted in DTCH. After the uplink D-PUR transmission, the UE monitors PDCCH under the control of a timer: - The timer starts after D-PUR transmission. - The timer restarts if a scheduling for D-PUR retransmission is received. - The UE considers that the D-PUR transmission has failed if the timer expires. - The timer is stopped when D-PUR procedure ends/succeeds. The downlink RRC response message, if needed, for the CP solution may include the following optional information: - downlink data encapsulated as a NAS PDU (downlink application layer response or pending data in MME). - redirection information. - D-PUR (re-)configuration and release. - FFS extendedWaitTime. The downlink RRC response message for the UP solution may include the following optional information: - Resume ID. - NCC (mandatory) - the downlink RRC response message for the UP solution is always provided. - redirection information. - D-PUR (re-)configuration and release. - FFS extendedWaitTime. The MAC CE for TA update can be sent along with the RRC transmission of the downlink RRC response message for the CP solution and UP solution. - FFS for CP solution if MAC CE for TA update can be sent without a downlink RRC response message. After reception of D-PUR transmission, the eNB can move the UE to RRC connection by RRCConnectionSetup message or RRCConnectionResume message. Fallback after D-PUR transmission is not successful is not specified i.e. it is up to UE implementation to initiate legacy RA, MO-EDT or wait for next D-PUR occasion. - FFS how to handle the skip in case of failure (UL or DL). D-PUR TA validation criteria: TA validation criterion “Serving cell changes” is implicitly always enabled, which means that TA is considered invalid when the UE initiates RA procedure in a different cell than where TA was last validated. Configuration for TA validation criteria is provided in dedicated RRC signaling. - It should be possible to disable each or all of the optional TA validation criteria (i.e., TA timer, (N)RSRP change) via RRC signaling. UE keeps the PUR configuration while TA is considered invalid, but PUR cannot be used until eNB validates the existing TA/provides a new TA. Working assumption: Counter for D-PUR occasions, i.e., “n”, is not introduced and “indefinite” or “one-shot” are the only possible configurations. A new TA timer is defined for UEs configured with D-PUR in idle mode. - The (re)starting times for TA timer need to be aligned between UE and eNB. The details of the mechanism are FFS. - TA timer is restarted after TA is updated. - The value range for the TA timer is FFS. Value of “infinity” is possible. D-PUR Request, (re)configuration and release mechanism: D-PUR request can be sent only by BL UE, UE in CE or NB-IoT UE; and which are capable of D-PUR. D-PUR request can be sent when the UE is in RRC_CONNECTED. D-PUR request includes number of PUR grant occasions requested with possibility to request infinite. FFS other values. UE can request D-PUR release. FFS how. A new RRC message is introduced for transmission of PUR request when UE is in RRC_CONNECTED (i.e., not for the cases of sending PUR request during EDT and during PUR). UE-specific PUR (re)configuration can be provided while UE is in RRC_CONNECTED. PUR (re)configuration can be included in RRC Connection Release. At least the following information can be included in PUR (re)configurations: - “m” consecutive missed allocations before release, FFS values. - Time Alignment Timer for idle mode. - RSRP change threshold for Serving cell. For UP solution, when PUR request is being piggybacked in the PUR transmission, same RRC message used for PUR transmission is used to include PUR request. PUR (re)configuration can be provided in DL RRC response message (message FFS) of the D-PUR procedure. Explicit reject message (NW-> UE) in response to PUR request is not introduced. Delta configuration is supported for PUR reconfiguration. If the UE performs EDT or moves to RRC_CONNECTED and comes back to RRC_IDLE in the same cell, PUR configuration remains valid unless specifically released or reconfigured by network or other triggers. PUR can be released explicitly by RRCConnectionRelease message and DL RRC response message (FFS message) of the D-PUR procedure. - FFS: RRCEarlyDataComplete. FFS: When UE initiates RACH/EDT, whether it has D-PUR configuration(s) is not explicitly notified to the network. EDT cannot be initiated solely for the purpose of sending PUR request in EDT Msg3. UE is not restricted from initiating RRC Connection for the purpose of sending PUR request (i.e. this agreement has no impact to legacy RRC Connection Establishment / Resume procedures). L1 ACK: RAN2 confirm the intention of the previous agreement as follows: - If RRC response message is not needed, eNB may send L1 ACK to acknowledge the PUR transmission in UL. The L1 ACK concludes the PUR procedure and UE moves to Idle. RAN2#107bis agreements: For CP and UP solutions unless otherwise stated: For PUR transmission and associated response, no new RRC messages are introduced and the existing RRC messages are re-used (S-TMSI and establishment cause for CP, and Resume ID and establishment cause for UP, are mandatory IEs). - Existing establishment causes are re-used. PUR update request is performed only in RRC_CONNECTED (i.e. not included with PUR transmission). This replaces the previous agreement for UP regarding piggyback of PUR request with the PUR transmission. L2/L3 expectation indication and downlink channel quality report are not included with PUR transmission. For CP solution AS RAI, BSR are not included with PUR transmission. FFS for UP solution whether AS RAI, BSR can be included with PUR transmission. For the CP solution, MAC CE for TA update can be sent without downlink RRC response message. (For UP RRC response message is always required). In case the UE moves to RRC-CONNECTED, a new C-RNTI can be provided in RRC. If absent the UE maintains the PUR-RNTI as C-RNTI. Working assumption: PUR is indicated as enabled in the cell using a flag in SIB2. Upon detecting that SIB indication of PUR support is turned off in the cell, UE shall release all PUR configurations. Existing SIB update mechanism is used to update the indication. PUR configuration can be provided without PUR Configuration Request from the UE, therefore optional radio access capabilities (separate for UP and CP) to indicate UE is capable of performing UL transmissions using PUR are introduced. When UE initiates RACH/EDT, whether it has PUR configuration(s) is not explicitly notified to the network. PUR (re)configuration is not supported in RRCEarlyDataComplete. In case PUR (re)configuration is provided in RRC Connection release, no additional explicit success/failure indication is introduced, i.e., existing methods are sufficient. No further UE behaviour is specified in case successful PUR reconfiguration is not confirmed using the existing methods (i.e., the PUR configuration that the UE considers valid depends on whether UE received the reconfiguration). It is up to the network implementation how to handle this scenario. NW releases PUR only upon successful confirmation that UE received the release message using the existing methods. PUR request includes optional indication that L1 ACK is sufficient. NW has the final decision whether to use L1 ACK or not. No new RRC message for PUR release request is needed (i.e. PUR configuration request is used for requesting PUR release). “UE is stationary/quasi-stationary” is not a precondition before sending PUR configuration request. “UL data size is limited to maximum supported TBS based on the UE category/capability” is a precondition before sending PUR configuration request. UE cannot be configured with more than one PUR configuration - Therefore, PUR config identity/index is not needed in PUR configuration Information on TBS size is provided in PUR configuration. - Exact details also depend on RAN1 agreements. For CP, similar to EDT, “the size of the resulting MAC PDU including the total UL data is expected to be smaller than or equal to the TBS configured for PUR” is a precondition before initiating UL transmission using PUR. FFS for UP. - Sending RRC Connection request without the CP data is not excluded due to this. For UP, refer to PUR messages as “RRCConnectionResumeRequest for PUR” and “RRCConnectionResumeRequest for EDT or PUR” etc. in the specifications. PUR TA timer is configurable up to hour(s) level, disabled/infinity is possible. - Exact values FFS. Configurable value of m. Not configured means release by “m” skip mechanism is disabled. - Exact values FFS. - Further details on “m” operation are FFS: - - UE shall increase ‘m’ when (1) PUR occasion is not used while UE is in RRC_IDLE and (2) PUR occasion is used in RRC_IDLE but no ACK is received. - - Network shall increase ‘m’ when no ACK is sent by the network. - - ‘m’ is not increased (neither by UE nor eNB) while UE is in a dedicated RRC connection. - - Value of ‘m’ is reset to zero after successful communication between UE and eNB (both in RRC_IDLE or RRC_CONNECTED). - - ‘m’ is not increased (neither by UE nor eNB) while barring timer is running. RAN2#108 agreements: “m” operation for PUR: UE shall increase ‘m’ when (1) PUR occasion is not used while UE is in RRC_IDLE and (2) PUR occasion is used in RRC_IDLE but no response (none of explicit HARQ ACK/NACK, L1 ACK or L2/L3 response) is received. RAN2 notes RAN1#96 agreement “After data transmission on PUR, if nothing is received by the UE in a time period, the UE shall fallback to legacy RACH/EDT procedure.” contradicts with RAN2#107 agreement. RAN2 reconfirms the RAN2#107 agreement “Fallback after D-PUR transmission is not successful is not specified i.e. it is up to UE implementation to initiate legacy RA, MO-EDT or wait for next D-PUR occasion”. Network shall increase ‘m’ when no response corresponding to a PUR occasion (none of explicit HARQ ACK/NACK, L1 ACK or L2/L3 response) is sent by the network. ‘m’ is not increased (neither by UE nor eNB) while UE is in RRC_CONNECTED. Counter ‘m’ is reset to zero after successful communication between UE and eNB using PUR. Counter ‘m’ is not reset to zero after successful communication between UE (with a valid PUR configuration) and eNB in RRC_CONNECTED. Existing access barring methods referenced from 5.3.3.2 in TS 36.331, except per-RSRP barring, are applicable for PUR. ‘m’ is increased if PUR is skipped due to access barring (i.e., no special handling). ‘m’ is increased if PUR is skipped due to UE being in extendedWaitTime (i.e., no special handling). Configurable value of m = {2, 4, 8, spare}. Others: After L1 ACK the PUR procedure is terminated and UE is not required to continue monitoring PDCCH after this (as per previous agreements). L1 ACK is not intended for application layer response. BSR for eMTC and DPR for NB-IoT can be included in PUR transmission for UP solution. - FFS CP solution. - FFS AS RAI. resumeID is set to the stored resumeIdentity when transmitting the RRCConnectionResumeRequest message using PUR. RRCEarlyDataComplete message is not used to release D-PUR configuration. PUR configuration request may contain a time offset request, i.e. requested time of the first PUR transmission. Details FFS. PUR configuration may contain a time offset, i.e. time of the first PUR transmission. Details FFS. Timer for PUR response is configurable, i.e. the length of the window during which the UE monitors for PUR response. Details FFS. TA timer for PUR is maintained in MAC. TA timer is restarted every time TA is received or explicitly re-validated (i.e. successful PUR transmission alone does not restart the timer). For initiating transmission using PUR, the interaction with NAS is up to UE implementation. For PURConfigurationRequest, in A.6, all entries (P, A-I, A-C) are '-'. RAN2 assumes Periodic TAU timer is restarted upon successful PUR transmission. (same as EDT). For UP solution, UE can segment and transmit part of the data in initial PUR transmission. Security is re-activated for PUR even when RRC Connection Resume is sent without any UL data, no separate indication is needed to separate legacy and PUR cases. For CP solution, eNB stores part of the PUR configuration needed to receive the PUR transmission. FFS whether full configuration is kept in eNB or part of it in MME. PUR configuration request doesn't differentiate between whether the request is for CP or UP. No PUR blacklisting is introduced (from RAN2 point of view). Confirm the working assumption from RAN2#107bis on flag in SIB2. Will have separate pur-Enabled flag in SIB2(-NB) for UP and CP. It is up to UE implementation how the UE determines whether the UL data is suitable for transmission using PUR. Capture as NOTE in 5.3.3.1x. If paging and PUR transmission opportunity collide, PUR transmission is prioritized. No mechanism to prevent UEs from sending configuration requests in close succession. R2-1916437 Feedback on RAN1 agreements on PUR LS out NB_IOTenh3-Core, LTE_eMTC5-Core To:RAN1 R2-1916435 LS reply PUR transmission for NB-IoT/eMTC LS out NB_IOTenh3-Core, LTE_eMTC5-Core To:RAN1 Cc:RAN4 RAN2#109_e agreements: PUR TA timer configuration is provided to MAC when RRC receives PUR configuration from eNB. When TA validation fails due to other than expiration of TA timer, the PUR TA timer is not stopped (i.e. keeps running until expiry). MAC entity starts the PUR TA timer when the MAC entity is configured with the PUR TA timer. TA adjustment by DCI is captured in MAC specification 5.4.x.2 to include the condition “when a Timing Advance Command MAC control element is received or PDCCH indicates timing advance adjustment as specified in TS 36.212 [5]”. RAN2 confirms TA validation procedure is captured/kept in RRC spec. When "PUR fallback indication" is received, MAC stops monitoring PDCCH in PUR response window. (Already captured in MAC CR) Upon L1 ACK indication received from lower layers, MAC indicated PUR success to the RRC. In RRC CR 5.3.3.3x, add “NOTE: UE actions upon reception of fallback/failure indication from lower layers (see TS 36.213 [23]) is left up to implementation.” Remove Editor’s Notes. Upon PUR fallback indication from lower layers, MAC indicates PUR fallback and PUR failure separately to the RRC. Upon reception of RRC message indicating successful PUR transmission, RRC does not need to indicate this to MAC layer. Similar to EDT, upon transmission using PUR, RRC configures PHY to use PUR. EDT value for timer t300 applies when UL data is included in transmission using PUR. When UL data is not included (i.e. only RRC message is included) in transmission using PUR, non-EDT value applies to t300. PUR periodicity includes at least values of several minutes, tens of minutes, ~hour, several hours, ~one day. FFS exact minimum and maximum values and total number of values. TA validation criterion “Serving cell changes” applies also when handover and RRC Connection Re-establishment results in RA in a new cell. TA timer range is multiple of PUR periodicities, e.g. 1,…, 8. - FFS on exact values and whether offset is applied so that e.g. retransmissions are covered. For NB-IoT: The value range for PUR response timer is same as in EDT (FDD): {pp1, pp2, pp3, pp4, pp8, pp16, pp32, pp64} with upper boundary 10.24 s. For eMTC: The value range for PUR response timer is same as in EDT: {sf240, sf480, sf960, sf1920, sf3840, sf5760, sf7680, sf10240}. Number of PUR grant occasions requested can be one or infinity. Working assumptions: (Can be used as baseline for CR and revisit if there is a problem): - RRC provides PUR configuration to MAC once and MAC calculates the PUR grant for each PUR occasion. - “m” counter is maintained in MAC. When the counter value reaches the configured max value, MAC sends indication to RRC to release PUR configuration. PUR periodicity configuration granularity is based on counts of binary multiples of HSFN, i.e. full SFN cycles (= 10.24 s). PUR periodicity is {hsf8, hsf16, hsf32, hsf64, hsf128, hsf256, hsf512, hsf1024, hsf2048, hsf4096, hsf8192, spareX, [FFS]}. Maximum PUR time offset range should be the same as maximum PUR periodicity. FFS further details e.g. how exact PUR start time is configured. - FFS: How storing of PUR parameters would be split between eNB and MME and other details before agreeing on where PUR configuration is stored for CP solution. If and how eNB links CP-PUR configuration to each UE in RRC_IDLE. Where to capture PUR release due to RACH initiation on a new cell. - FFS (to ask RAN1): RAN2 to confirm L1 update on repetition number is not intended to update the RRC configuration (i.e. higher layer configuration) but adjust the configuration provided by higher layers. Whether L1 adjustment applies only to retransmissions or also future PUR UL transmissions and where it is stored.

## 2.5	Scheduling multiple DL/UL transport blocks

See subclause 1.5 where this topic is treated jointly.

## 2.6	Network management tool enhancement

RAN2#103bis agreements: ANR reporting for NB-IoT only uses idle-mode measurements (i.e. we won’t introduce connected mode measurements) Support RACH report for NB-IoT Will consider whether information in LTE RACH report is extended RAN2#104 agreements: SON-ANR: RAN2 understanding is that the purpose of SON/ANR reporting in NB-IoT is network optimisation rather than immediately updating neighbour relations like with LTE ANR, and is therefore not time critical. SON reporting does not trigger RRC connection establishment/resume - FFS whether this includes EDT. SON information can be reported along with EDT, FFS what and how. - RACH report: In addition to legacy parameters for RACH reporting, the first selected resource pool (E.g. CE level, EDT) is included in the RACH report. This agreement is also applicable for eMTC. RAN2#105 agreements: - RLF report: The UE informs network about having an RLF report available via rlf-InfoAvailable indication during the initial RRC connection re-establishment process after RLF. The rlf-InfoAvailable parameter is included in Msg5 of the RRC connection re-establishment process. It is up to the network to decide when to request for the RLF report, using UEInformationRequest/UEInformationResponse procedure. In case of no/failed RRC connection re-establishment, the UE may store the RLF report for indicating rlf-InfoAvailable in Msg5 of later connection establishment. The following legacy parameters are supported for RLF reporting in NB-IoT: - Failed cell ECGI - Last Serving Cell RSRP/RSRQ (Whether OK for CP-solution is FFS) - Absolute Time Stamp (optional, if available) - Location Info (optional, if available) - FFS: further information. RLF reporting does not trigger RRC connection establishment/resume, including EDT. Neither rlf-InfoAvailable indication nor RLF report is included in the Msg3 of the EDT procedure to indicate to the network about an available RLF report. - RACH report: For NB-IoT RACH report, range of numberOfPreamblesSent is INTEGER (1..64). contentionDetected is Boolean. Initial CEL is included in NB-IoT RACH report. Working assumption: A Boolean flag indicating EDT fallback (i.e., the UE started with EDT NPRACH resources and went through a fallback to non-EDT NPRACH resources) is included in NB-IoT RACH report. Take the ASN.1 structure provided in the report (proposal 5) as baseline for NB-IoT running CR. Initial CEL is included in eMTC RACH report. Working assumption: A Boolean flag indicating EDT fallback (i.e., the UE started with EDT PRACH resources and went through a fallback to non-EDT PRACH resources) is included in eMTC RACH report. Take the ASN.1 structure provided in the report (proposal 8) as baseline for eMTC running CR. RAN2 aims on standardizing a single method of sending NB-IoT RACH report, i.e., avoid multiple solutions. Proposed solutions should be discussed considering signalling overhead, UE power consumption, security aspects and other reports being considered in other email discussions. NB-IoT RACH report includes the information from the latest successful RACH procedure. NB-IoT RACH report information is discarded when the UE goes to IDLE. FFS how to report. - SON-ANR: Solution direction based on option a: - Single set of measurements only. - No new measurement requirements. - ANR measurement reporting using the UE Information Request / Response framework is supported. Other methods FFS. - ANR reporting for the CP solution is not supported in Rel-16. RAN2#105bis agreements: A Boolean flag indicating EDT fallback (i.e., the UE started with EDT NPRACH resources and went through a fallback to non-EDT NPRACH resources) is included in NB-IoT RACH report. A Boolean flag indicating EDT fallback (i.e., the UE started with EDT PRACH resources and went through a fallback to non-EDT PRACH resources) is included in eMTC RACH report. At least RACH report for NB-IoT based on UE information request/response procedure is supported. It is up to the network to decide when to request for the RACH report. RAN2#106 agreements: - SON-ANR: ANR report does not trigger EDT. ANR report is not carried in EDT Msg3. anr-InfoAvailable indication can be reported in EDT Msg3. ANR procedure steps: - 1) UE receives “measurement” configuration in dedicated signalling. - 2) When the UE is released to idle: The UE performs the instructed measurements according to existing RAN4 cell reselection measurement performance requirements. CGI reading is limited to cells being above a certain threshold. Threshold details FFS. 3) Reporting takes place, upon network request, next time UE is connected mode again. - RACH report: Not support EDT RACH report in EDT Msg3. RAN2#107 agreements: - SON-ANR: - Configuration: Dedicated ANR measurement configuration is provided in RRCConnectionRelease message only. The list of frequencies where to perform the measurements can be optionally provided in dedicated signalling, as a subset of the frequencies signalled in system information. Per default, the list frequencies signalled in system information is used. Maximum of 8 carriers can be configured. A blacklist of cells can be optionally signalled per carrier. - Measurements: ANR measurements are only performed while UE remains camped on the same cell from which the ANR measurement configuration was received. (i.e. When the UE changes cell the ANR measurements are stopped). Neighbour cell relaxed monitoring criteria and Ssearch criteria don’t apply while performing an ANR measurement. ANR measurement doesn’t affect DRX / eDRX operation. CGI-reading applies to strongest cell on each frequency, if above the RSRP threshold. The RSRP threshold is an absolute threshold that is common to all frequencies. - Reporting: anr-InfoAvailable indication is reported in and only in RRCConnectionSetupComplete, RRCConnectionResumeComplete, RRCConnectionReestablishmentComplete and RRCConnectionResumeRequest for EDT. The ANR report might be empty for a given carrier, if a measurement is performed and no strong cell is found. UE only indicates the availability of the report if 1) a report is available and 2) if the RPLMN is included in the list of EPLMNs stored by the UE when the ANR measurement was configured. The ANR report is discarded after: - Timer expiry. Value FFS. - Power off or detach. - Reporting anr-InfoAvailable and returning to idle. UE Information Request / Response is the only reporting procedure for ANR measurement. For a measured carrier the following information is included: - Carrier frequency. - Strongest cell PCI, RSRP/RSRQ. - Strongest cell CGI-info if RSRP above the threshold. Serving cell RSRQ/RSRP and CGI are included in the ANR report. ANR reporting is optional for NB-IoT UEs with capability signaling. FFS: ANR applicability to 5GC. RAN2#107bis agreements: UE is not required to measure more than 2 carriers. FFS whether an indication of when the UE performed the ANR measurement is needed. RAN2#108 agreements: None. RAN2#109_e agreements: - ANR: The NOTE about the ANR measurement requirements is sufficient and the Editor’s Note can be removed. Nothing additional needs to be specified on when UE follows DRX or eDRX requirements for ANR measurements. No need to specify additional requirements for UE using PSM. The validity timer is fixed. Working assumption: 96 hours. ANR measurements is not applicable to 5GC in Rel-16. Can be considered in Rel-17. The blackcell list size is 16 and maxCellBlack is used as the maximum. A maximum of two frequencies can be configured and reported for ANR measurements. FFS: Whether a time indication of when the ANR measurements were performed is included in the report, and whether it is a time stamp or a simple indication “immediately after going to IDLE, immediately before going to CONNECTED, in between”. - RACH/RLF: The processing delay of the UE information procedure in Table 11-2-2 is set to 45 ms. The UE information procedure can only be used when AS security has been activated. RACH report is not applicable to 5GC. RLF report is not applicable to 5GC. Support of RACH report is optional with capability reporting. Support of RLF report is optional at the UE without capability reporting. RLF report is discarded after 48 hours if not fetched. RLF report availability and RLF report is only provided if the current RPLMN is a PLMN that was present in the UE's EPLMN List or the RPLMN at the time of RLF detection. FFS: The re-establishment cell id is also included in the RLF report. FFS: RLF report is kept during state transitions and RAT changes.

## 2.7	Improved multi-carrier operation

RAN2#103bis agreements: None. RAN2#104 agreements: Re-use the code points defined in Rel-14. Study the impact of re-using the Rel-14 RRC reporting mechanism and consider whether a MAC mechanism should be used instead. RAN2 further study how to support the use case of enabling measurements in non-anchor carrier while reducing measurement on anchor carrier. RAN2#105 agreements: None. RAN2#105bis agreements: - Channel Quality Reporting: Existing Msg3 signalling is used for reporting downlink channel quality when UE makes access on non-anchor carrier. Existing SIB2 signalling is used to enable UE to report downlink channel quality on non-anchor carrier. The DL channel quality reported in connected mode corresponds to the carrier used for the unicast transmission (i.e. configured by MSG4 or by a subsequent reconfiguration procedure). eNB enables the reporting of the DL Channel quality in connected mode. - FFS whether dedicated or broadcast signaling is used for enabling. Periodic reporting or on-demand reporting are not supported. DL channel quality reported in connected mode is optional for the UE. Working assumption that the same code points as Msg3 reporting is used. FFS whether MAC or RRC is used for reporting. R2-1905263 LS on non-anchor carrier CQI reporting in MSG3 Qualcomm Incorporated LS out Rel-16 NB_IOTenh3-Core To:RAN1 Cc: RAN4 - Non-anchor carrier measurements for RRM: If RAN4 confirms feasibility, and NRS is present in non-anchor carrier, RAN2 can make use of that for RRM measurements in non-anchor carrier. R2-1905264 LS on non-anchor carrier idle mode measurements for RRM Qualcomm Incorporated LS out Rel-16 NB_IOTenh3-Core To:RAN4 Cc: RAN1 RAN2#106 agreements: - Channel Quality Reporting: The DL channel quality of the configured carrier in RRC connected mode is reported by MAC CE. Use the same LCID as agreed for eMTC. We need to wait for RAN4 before deciding when to report. FFS whether broadcast or dedicated signalling is used for NW to enable the reporting. R2-1908262 LS on DL channel quality report for configured carrier in RRC connected mode LS out Rel-16 NB_IOTenh3-Core To:RAN4 Cc:RAN1 RAN2#107 agreements: - Channel Quality Reporting: Quality report in connected mode is transmitted on demand. No implicit trigger is supported. The trigger command is defined as a MAC CE with empty payload and one of the reserved LCID is used. No feature enable indication is defined in RRC signalling. UE capability signalling for quality reporting in connected mode is defined. RAN2 confirm that DL Channel quality in MSG3 applies to initial access, i.e. when MAC MSG3 contain a CCCH SDU, and that the report corresponds to the carrier where the UE performs the random access procedure. From RAN2 point of view, there is no need for a physical layer trigger for DL channel quality in connected mode. From RAN2 point of view, there is no need for DL channel quality reporting (for RAR or unicast carrier) in MSG3 in connected mode. CCCH SDU in the MSG3 buffer needs to be updated after receiving RAR. Support of DL channel quality in MSG3 for non-anchor carrier is optional without capability reporting and is a separate capability from support of DL channel quality in MSG3 for the anchor carrier. Use the codepoint/index of “10001” for the MAC CE DL channel quality report. Introduce a one-byte MAC CE DL channel quality report to support a maximum of 8 bits report. FFS: The new MAC CE DL channel quality report has the next priority after MAC CE for BSR. When the measurement is available, the MAC CE DL channel quality report is sent at the first opportunity according to the MAC logical channel prioritisation rules. Support of DL channel quality in MSG3 for non-anchor carrier and support of DL channel quality in connected mode also apply when the UE is accessing via 5GC. R2-1911587 Reply LS on channel quality report in connected mode for NB-IoT LS out Rel-16 NB_IOTenh3-Core To:RAN1, RAN4 RAN2#107bis agreements: - Channel Quality Reporting: Update the notes in sections 5.3.3.3, 5.3.3.3a, 5.3.3.3b and 5.3.7.4 according to RAN4 agreements and remove the Editor’s note. The codepoint/index of “10001” is used for MAC CE DL quality report trigger. Design the MAC CE DL Channel quality report with 4 reserved bits and one 4 bit field CQI-NPDCCH-NB. The coding of CQI-NPDDCH-NB follows the same definition as in RRC, and MAC spec refers to the table in RRC. The UE starts to perform DL channel quality measurements upon reception of the MAC CE DL quality report trigger. The reporting of DL channel quality measurements occurs when the UE receives a grant. The measurement procedure is defined in RAN4. For NB-IoT, the new MAC CE DL channel quality report has the next priority above “data from any Logical Channel, except data from UL-CCCH”. - Non-anchor carrier measurements for RRM: Introduce support for idle mode serving cell measurements on non-anchor paging carrier in Rel-16. Idle mode serving cell measurements on non-anchor paging carriers is implicitly enabled based on presence of the NRS configuration. Support of idle mode RRM measurements on non-anchor paging carriers is optional at the UE without capability reporting. The UE may use only NRSRP measurements on a non-anchor carrier while the neighbour cell relaxed monitoring criterion is met, otherwise UE uses NRSRP measurement on anchor carrier. When the UE performs NRSRP measurements on a non-anchor carrier, it translates the NRSRP measurements on a non-anchor paging carrier to the equivalent NRSRP measurements of the anchor carrier using the parameter nrs-PowerOffsetNonAnchor. - Update Qrxlevmeas in the table in 36.304 to capture this. When the UE is only performing NRSRP measurements on a non-anchor carrier, the Squal criteria is not applicable. RAN2#108 agreements: None. RAN2#109_e agreements: Introduce parameter nrs-NonAnchorConfig in systemInformationBlockType2-NB based on TP in R2-2000624 with rewording of the field description. - nrs-NonAnchorConfig For FDD: Indicates whether NRS is present on non-anchor paging carriers even when no paging NPDCCH is transmitted, see TS 36.211 [21], clause 10.2.6. Include the Text Proposal in section 5.2 of R2-2000624 for TS 36.304. Details can be revisited next meeting.

## 2.8	Inter-RAT cell selection

RAN2#103bis agreements: RAN2 understands that cell reselection between NB-IoT and any other RAT is not within the scope of this WI NB-IoT network may indicate eMTC/LTE/GERAN assistance information for inter-RAT cell selection eMTC/LTE network may indicate NB-IoT assistance information for inter-RAT cell selection Intention is not to provide cell reselection parameters It is up to UE implementation how and when to perform measurements for cell selection of the target RAT RAN2#104 agreements: NB-IoT network may indicate frequency identifiers of neighbouring eMTC/LTE/GERAN carriers to assist inter-RAT selection. eMTC/LTE network may indicate frequency identifiers of neighbouring NB-IoT carriers to assist inter-RAT selection. Indicate whether each LTE neighbour frequency supports eMTC/LTE/both. Indicate whether each GERAN neighbour frequency supports EC-GSM/PEO. RAN2#105 agreements: Priorities of eMTC/LTE/GERAN frequencies are not provided by NB-IoT network as assistance information for inter-RAT cell selection. Priorities of NB-IoT frequencies are not provided by eMTC/LTE network as assistance information for inter-RAT cell selection. Suitability criteria of eMTC/LTE/GERAN frequencies are not provided by NB-IoT network as assistance information for inter-RAT cell selection. Suitability criteria of NB-IoT frequencies are not provided by eMTC/LTE network as assistance information for inter-RAT cell selection. RAN2#105bis to RAN2#106 agreements: None. RAN2#107 agreements: In NB-IoT and in LTE, a new System Information block should be defined for IRAT cell selection assistance information containing the parameters agreed previously. - The normal SI update procedures apply. R2-1911583 IRAT Cell Selection Assistance information LS out Rel-16 NB_IOTenh3-Core To:RAN6 RAN2#107bis to RAN2#109_e agreements: None.

## 2.9	Coexistence with NR

RAN2#103bis to RAN2#109_e agreements: None.

## 2.10	Connection to 5GC

RAN2#105 agreements: See subclause 1.12 RAN2#105bis agreements: For NB-IoT: SIB1-NB extended to include 5GC PLMN list, per PLMN indication of Cell Reserved for Operator Use, common 5GC Tracking Area Code and 5GC Cell Identity across all PLMNs, common Cell Barring for 5GC connectivity across all PLMNs. For NB-IoT/eMTC: SIB-NB/SIB1-BR extended to include per PLMN indication of the supported CIoT 5GS Optimisations. For NB-IoT: Update Paging-NB message to include 5G S-TMSI as UE Identity for core network paging. For NB-IoT: Adopt critical extension of RRCConnectionRequest-NB message for 5GC connectivity and include 48 bit 5G S-TMSI and random value as Initial UE identity along with specific RRC establishment cause. For NB-IoT: Update RRCConnectionSetupComplete-NB to include RegisteredAMF-r15, full 5G S-TMSI (48 bit long). For NB-IoT/eMTC: FFS :Whether s-NSSAI-list-r15 is applicable. For NB-IoT/eMTC: FFS: Applicability of NR PDCP for SRB1, DRBs For eMTC: UAC feature is supported. For NB-IoT: FFS how to support access control. For NB-IoT: FFS how to support slicing. For NB-IoT: FFS whether to adopt SDAP as user plane protocol, and whether AS reflective QoS is applicable. For eMTC: Adopt SDAP as user plane protocol, and AS reflective QoS is optional. For NB-IoT/eMTC: Working assumption that CN type is not used in RRCConnectionRelease/ RRCConnectionRelease-NB. For NB-IoT: How to support RRC Connection Re-establishment for the UP and CP optimization. For NB-IoT/eMTC: FFS whether for data transfer through DRBs, use AS security algorithms same as LTE AS security algorithms, and KeNB root key is derived from Kamf as specified in TS 33.501. For NB-IoT/eMTC: FFS whether AS security algorithms are identified by using LTE code points. For eMTC: mt-Access, mo-Signaling, mo-Data, emergency, highPriorityAccess and mo-VoiceCall are applicable establishment causes. For NB-IoT/eMTC: Release cause loadBalancingTAURequired at RRC connection release is not applicable. RAN2#106 agreements: Indication of supported CIoT features and other common aspects: RAN2 agrees to send a LS to SA2, CC: CT1,RAN3 - ask how many slices NB-IoT/eMTC UEs can support simultaneously and whether S-NSSAI will be used by the ng-eNB in addition to the indications of the supported 5G CIoT optimisations for AMF selection. - ask about applicability 5GC QoS frame work for NB-IoT/5GC - ask how QoS mapping is handled during mobility between NB-IoT and other RATs. NB-IoT UE AS forwards all CIoT feature support indications broadcast in the cell to NAS as assistance info for CN type selection by NAS. eMTC UE AS forwards all CIoT feature support indications broadcast in the cell to NAS as assistance info for CN type selection by NAS. For NB-IoT, CN Type is not used in RRCConnectionRelease-NB message as NAS based redirection between EPC and 5GC is used for CIoT. eMTC re-uses the existing CN type in RRCConnectionRelease message For eMTC intra RAT connected mode mobility (for intra 5GC case), adopt R15 LTE connected to 5GC introduced changes for RRCConnectionReconfiguration (including Mobility Control Info) including SecurityConfigHO-v1530 -> intra5GC-r15 IE. For eMTC intra RAT connected mode mobility (for inter system mobility between EPC and 5GC), adopt R15 LTE connected to 5GC introduced MobilityFromEUTRACommand carrying RRCConnectionReconfiguration (including Mobility Control Info) including SecurityConfigHO-v1530 -> fivegc-ToEPC-r15 and epc-To5GC-r15 IEs. For eMTC, re-use SIB25 and the procedure in subclause 5.3.16 of 36.331 as a baseline. Ask SA1: - whether UAC needs to be supported for NB-IoT. - whether a new UAC access category corresponding to mo-ExceptionData can be defined as one of the reserved standardised access categories in the range 9-31. - Whether up to 64 access categories need to be defined for support of UAC, given the currently supported establishment causes and necessity for operator defined categories and potential future extension - Whether access classes 0-9 (mapped to access identity 0) are applicable or not - Whether barring for access identities 1, 2, 11 to 15, is common for all access categories or independently controlled for different access categories. Use the same Enhanced Coverage authorisation mechanism of UEs connected to EPC to the NB-IoT/eMTC UEs connected to 5GC and no impacts on RAN2 specification are identified. R2-1908190 LS on support for flow based QoS & slicing for NB-IoT & eMTC connected to 5GC LS out Rel-16 LTE_eMTC5-Core, NB_IOTenh3-Core To:SA2 Cc:CT1, RAN3. R2-1908263 LS on UAC for NB-IoT LS out Rel-16 NB_IOTenh3-Core, 5G_CIoT To:SA1, CT1 Cc:SA2, RAN3 Other: RRC Connection re-establishment for the control plane for NB-IoT connected to 5GC is supported, assuming feasibility is confirmed. Send a LS to SA3, SA2 CC: RAN3, CT1, CT4 - Ask whether they can introduce RRC Connection re-establishment for the Control plane for NB-IoT connected to 5GC. - Ask the feasibility that a truncated 5G S TMSI (e.g. 40 bits) needs to be defined. R2-1908264 LS on RRC Connection Re-Establishment for CP for NB-IoT connected to 5GC LS out Rel-16 NB_IOTenh3-Core, 5G_CIoT To:SA3, SA2 Cc:CT1, RAN3, CT4 RAN2#107 agreements: Indication of supported CIoT features and other common aspects: SDAP is not supported for NB-IoT. Maximum 8 slices are supported by both NB-IoT and eMTC. In Msg5, both NB-IoT and eMTC UEs provide S-NSSAI to indicate network slice that UE wants to connect. AS reflective QoS is not supported for NB-IoT connected to 5GC. NB-IoT UEs connected to 5GC support a maximum of 2 DRBs. Each NB-IoT DRB is mapped to a default QoS flow of that PDU session. Control Plane optimization feature is optional for eMTC devices connecting to 5GC and mandatory for NB-IoT devices connected to 5GC. Introduce a new IE “cp-CIoT-5GC-Optimisation-r16” in SIB1-BR to indicate ng-eNB connected to 5GC is supporting control plane optimisation and no need of any indicator in SIB1-NB to indicate support for CP optimisation. When eMTC UE is initiating RRC connection setup procedure for connecting to 5GC, use R15 critical extension of RRCConnectionRequest message including 5GC UE Identity and 5GC establishment cause values. In R15, RRC Connection Setup Complete Message is enhanced to include registeredAMF-r15, s-NSSAI-list-r15, ng-5G-S-TMSI-Bits-r15, gummei-Type-v1540, guami-Type-r15 and all these changes are applicable for eMTC 5GC UEs as well. In R16 NB-IoT, RRCConnectionSetupComplete-NB message needs also to include gummei-Type-v16, guami-Type-r16. In RRCConnectionSetupComplete message, introduce a new IE “cp-CIoT-5GC-Optimisation-r16” to assist ng-eNB for the selection of proper AMF supporting control plane optimization. Working assumption: We will introduce a AS Release Assistance Information (RAI) that conveys at least the following information: - No further UL/DL higher layer PDUs. - No further UL, 1 single DL higher layer PDU expected. - Further details including CP/UP applicability can be discussed in the next meeting. R2-1911865 Reply LS on UAC for NB-IoT LS out Rel-16 5G_CIOT, NB_IOTenh3-Core To:SA1; Cc:CT1, SA2, RAN3 RAN2#107bis agreements: Indication of supported CIoT features and other common aspects: Use NR PDCP for LTE-M devices connected to 5GC. NR PDCP support is not specified for NB-IoT devices connected to 5GC. NB-IoT UE supports a maximum of two DRBs when connected to 5GC. - FFS whether this is indicated by a new capability or the existing UE capability, multipleDRB-r13. Introduce a MAC mechanism with 2 bits for RAI. For NB-IoT, up to 64 access categories are defined for support of UAC, same as for eLTE. For NB-IoT, will support either barring factor as in eLTE or 10 bit bitmap bitmap as in NB-IoT to distribute UEs. FFS which method. For NB-IoT, uac-BarringForAccessIdentity using a 7 bit map barring corresponding to access identities 1, 2, 11 to 15 is re-used. For NB-IoT, the parameter uac-AC1-SelectAssistInfo is used to signal whether Access Category 1 applies to the UE. FFS: For NB-IoT, do not use UAC-BarringInfoSetList. FFS: For NB-IoT, uac-BarringBitmap and uac-BarringForAccessIdentity per access category as in eLTE. Access barring per RSRP is supported for eMTC and NB-IoT. RAN2#108 agreements: Introduce barring factor for Access identity 0 for UAC in NB-IoT. Introduce barring timer with barring factor for Access identity 0 for UAC in NB-IoT. Access barring bitmap for access identities is signalled common to access categories for UAC in NB-IoT. Access barring parameters are provided directly per access category. Barring info sets are not introduced for UAC in NB-IoT. Access barring parameters are provided directly per PLMN for UAC in NB-IoT. NB-IoT UEs in RRC_CONNECTED mode performs access barring check based on the latest UAC parameters acquired prior to entering RRC_CONNECTED. Reuse SIB14-NB for 5GC and introduce a new indication in MIB for 5GC barring. Introduce a MAC mechanism with 2 bits for RAI when connected to EPC, including CP and UP optimisations (same mechanism as for 5GC). FFS whether any feature is excluded (e.g. PUR, etc). The AS RAI informs the network whether a) no subsequent DL and UL data transmission is expected; or, b) a single subsequent DL transmission is expected; For EPS it is optional for a UE to support AS RAI, with capability reporting. For EPS, introduce indication in SIB2 (SIB2-NB) if AS RAI is configured in the cell. For 5GS, AS RAI is always enabled for UEs (NB-IoT or LTE-M) connected to 5GC. RRC Connection re-establishment for the control plane for NB-IoT connected to 5GC is supported. RRC Connection re-establishment for the control plane for NB-IoT UEs connected to 5GC is optional, without capability reporting. Working assumption: The values ‘n’ and ‘m’ for the truncation of the 5G-S-TMSI are signalled per PLMN in SystemInformationBlockType2-NB. They are an implicit indication of eNB and AMF support. Create a critical version of RRCConnectionResumeRequest-NB message for CP CIoT 5GS optimisation, including the ue-Identity, re-establishmentCause, and cqi-NPDCCH (short). Define a new IE ReestabUE-Identity-CP-5GC-NB in RRCConnectionReestablishmentRequest-NB message, including truncated5G-S-TMSI (40 bits), ul-NAS-MAC (16 bits) and ul-NAS-Count (5 bits). The UE goes to RRC_IDLE and initiates NAS recovery procedure when the UE switches between CN types at RRC Connection re-establishment for the control plane. 5G-S-TMSI is used for PO determination for eMTC and NB-IoT UE connection to 5GC. 5G-S-TMSI mod 16384 is used as the UE_ID for PO calculation for eMTC and NB-IoT UE connection to 5GC. RAN2 will consider introducing a mechanism for minimising ping-pong between CN types in RRC_IDLE/RRC_INACTIVE. RAN2#109_e agreements: AS RAI can be used when connected to EPC or 5GC, including when in RRC connected mode and using CP/UP optimisations, EDT, or PUR. AS RAI can be provided with any higher layer PDU transmission in the UL including the last one or with no higher layer PDU transmission in the UL. AS RAI is provided in the same MAC CE as the DL channel quality report. One of the codepoints for AS RAI implies “no indication”. AS RAI has higher priority than data when AS RAI and DL channel quality report are provided in the same MAC CE. No other mechanisms are introduced to provide R16 AS RAI. Codepoints for AS RAI are allocated as follows: - Code Point 00: No RAI information - Code Point 01: no subsequent DL and UL data transmission is expected - Code Point 10: a single subsequent DL transmission is expected - Code Point 11: Reserved. AS RAI, when triggered, should have higher priority than data if including AS RAI would not lead to data segmentation. For EDT and PUR: When AS RAI is triggered by upper layers but cannot be sent along with the associated MAC SDU due to MAC prioritisation, AS RAI is cancelled. - FFS non-EDT/non-PUR case. Similar as UP CIoT EPS Optimization, rrc-SuspendIndication in RRCConnectionReject can be supported for UP CIoT 5GS Optimization. No change for specification is needed. DL channel quality report can be supported for both NB-IoT and eMTC connected to 5GC. Confirm the working assumption that cause delayTolerantAccess it not applicable to 5GC. Confirm the working assumption that there is no need for an indication of extended Idle mode DRX support in system information for NB-IoT. Confirm the working assumption that there is a new IE cp-EDT-5GC-r16 in SIB2-BR/SIB2-NB to indicate ng-eNB connected to 5GC supports CP MO-EDT. Revert the working assumption that the values ‘n’ and ‘m’ for the truncation of the 5G-S-TMSI are signalled per PLMN in SystemInformationBlockType2-NB. Remove the IE cp-ReestablishmentPLMNList-5GC-r16 in SystemInformationBlockType2-NB. The existing capability multipleDRB-r13 is also applicable to 5GC. PUR is supported in EPC and 5GC. Introduce separate indications up-PUR-5GC-r16 and cp-PUR-5GC-r16 in SIB2-BR/SIB2-NB. Introduce separate UE capabilities pur-UP-5GC-r16 and pur-CP-5GC-r16. Add ab-PerRSRP-r16 parameter (same definition as SIB14-BR) in SIB25-BR. BL UEs or UEs in CE in RRC_CONNECTED mode performs access barring check based on the latest UAC parameters acquired prior to entering RRC_CONNECTED. For 5GC, CP re-establishment is always enabled and there is no need for an indication in system information. SystemInformationBlockType25-BR follows the same system information update mechanism as SIB14-BR and does not affect the value tag. A new parameter uac-ParamModification (similar to eab-ParamModification) is introduced in the Paging message and in the Direct Indication Information to indicate SIB25-BR modification and scheduling.

## 2.11	UE specific DRX

RAN2#108 agreements: None. RAN2#109_e agreements: For both option 1 and option 2, a SIB indication is needed at least for enabling/disabling the use of UE specific DRX for NB-IoT accessing EPC. Indicate in the Reply LS to SA2 that both options are feasible from RAN2 point of view. Indicate in the Reply LS to SA2 that RAN2 has a preference for Option 2. FFS value range for NB-IoT. FFS: 320ms, 640ms, 1280ms, 2560ms, 5120ms and 10240ms are supported for UE specific DRX cycle in NB-IoT cell, for both EPS and 5GS. Reply CT1 LS to inform the values for UE specific DRX cycle in NB-IoT cell are FFS. R2-2001815 Reply LS on Rel-16 NB-IoT enhancements LS out Rel-16 NB_IOTenh3-Core To: SA2, CT1, RAN3, SA Cc: RAN, CT

## 2.12	Other

RAN2#103bis to RAN2#109_e agreements: None.

# References: List of approved outgoing Rel-16 LSs

| Tdoc | Meeting | Title | WI | To | Cc |
| --- | --- | --- | --- | --- | --- |
| R2-1818662 | RAN2-104 | LS on RSS based measurements signalling | LTE_eMTC5-Core | R1 |  |
| R2-1902431 | RAN2-105 | Response LS on on eDRX cycles for CM-CONNECTED with RRC inactive | FS_CIoT_5G | SA2, CT1, CT4, RAN3 |  |
| R2-1902436 | RAN2-105 | Response LS on Completion of Study on Cellular IoT support and evolution for the 5G System | FS_CIoT_5G | SA2, RAN, R3 | SA, CT |
| R2-1905263 | RAN2-105bis | LS on non-anchor carrier CQI reporting in MSG3 | NB_IOTenh3-Core | R1 | R4 |
| R2-1905264 | RAN2-105bis | LS on non-anchor carrier idle mode measurements for RRM | NB_IOTenh3-Core | R4 | R1 |
| R2-1905276 | RAN2-105bis | LS on Mobile-terminated Early Data Transmission | LTE_eMTC5, NB_IOTenh3 | R3, SA2, CT1, CT4 |  |
| R2-1905277 | RAN2-105bis | LS on quality report in Msg3 for LTE-M | LTE_eMTC5-Core | R1 | R4 |
| R2-1905278 | RAN2-105bis | LS on ETWS/CMAS in connected mode narrowband | LTE_eMTC5-Core | R1 |  |
| R2-1908190 | RAN2-106 | LS on support for flow based QoS & slicing for NB-IoT & eMTC connected to 5GC | LTE_eMTC5-Core, NB_IOTenh3-Core | SA2 | CT1, R3 |
| R2-1908196 | RAN2-106 | Reply LS on PUR Working Assumptions for NB-IoT and LTE-M | LTE_eMTC5-Core, NB_IOTenh3-Core | R1 |  |
| R2-1908202 | RAN2-106 | Reply LS on Conclusion on 5G CIoT user plane small data solution | FS_CIoT_5G | SA2, R3 | RAN, SA, CT |
| R2-1908260 | RAN2-106 | LS on assistance indication for WUS grouping | LTE_eMTC5-Core, NB_IOTenh3-Core | SA2 | CT1, R3 |
| R2-1908262 | RAN2-106 | LS on DL channel quality report for configured carrier in RRC connected mode | NB_IOTenh3-Core | R4 | R1 |
| R2-1908263 | RAN2-106 | LS on Access Categories for NB-IoT | NB_IOTenh3-Core, 5G_CIoT | SA1, CT1 | SA2, R3 |
| R2-1908264 | RAN2-106 | LS on RRC Connection Re-Establishment for CP for NB-IoT connected to 5GC | NB_IOTenh3-Core, 5G_CIoT | SA3, SA2 | CT1, R3, CT4 |
| R2-1911583 | RAN2-107 | LS to RAN6 on IRAT Cell Selection Assistance information | NB_IOTenh3-Core | R6 |  |
| R2-1911587 | RAN2-107 | Reply LS on channel quality report in connected mode for NB-IoT | NB_IOTenh3-Core | R1, R4 |  |
| R2-1911589 | RAN2-107 | Reply LS on assistance indication for WUS | LTE_eMTC5-Core, NB_IOTenh3-Core | SA2, CT1, R3 |  |
| R2-1911601 | RAN2-107 | Reply LS on LTE-M identification in 5GC | 5G_CIOT | SA2, RAN3 | RAN |
| R2-1911603 | RAN2-107 | Reply LS on Mobile-terminated Early Data Transmission | NB_IOTenh3-Core, LTE_eMTC5-Core | SA2, R3 | SA3, CT1, CT4 |
| R2-1911865 | RAN2-107 | Reply LS on UAC for NB-IoT | 5G_CIOT, NB_IOTenh3-Core | SA1 | CT1, SA2, R3 |
| R2-1914045 | RAN2-107bis | LS on RAN2 agreements for 5GC CIoT | LTE_eMTC5-Core, NB_IOTenh3-Core | R3 |  |
| R2-1916368 | RAN2-108 | Reply LS on Mobile-terminated Early Data Transmission | LTE_eMTC5-Core, NB_IOTenh3-Core | SA2, CT1 | R3, RAN, SA |
| R2-1916369 | RAN2-108 | LS on UE identifier for UP CIoT 5GS Optimisation | LTE_eMTC5-Core, NB_IOTenh3-Core | R3 |  |
| R2-1916435 | RAN2-108 | LS reply PUR transmission for NB-IoT/eMTC | NB_IOTenh3-Core, LTE_eMTC5-Core | R1 | R4 |
| R2-1916437 | RAN2-108 | Feedback on RAN1 agreements on PUR | NB_IOTenh3-Core, LTE_eMTC5-Core | R1 |  |
| R2-1916440 | RAN2-108 | Reply LS on assistance indication for WUS | NB_IOTenh3-Core, LTE_eMTC5-Core | SA2, CT1, R3 | SA |
| R2-2001815 | RAN2-109_e | Reply LS on Rel-16 NB-IoT enhancements | NB_IOTenh3-Core | SA2, CT1, R3, SA | RAN, CT |

# References: Reports from breakout sessions on NB-IoT and MTC

| Meeting | Date | Title | Tdoc |
| --- | --- | --- | --- |
| RAN2-103bis, Chengdu, China | 08-12 October 2018 | Report NB-IoT breakout session | R2-1815603 |
| RAN2-103bis, Chengdu, China | 08-12 October 2018 | Report eMTC breakout session | R2-1815604 |
| RAN2-104, Spokane, USA | 12-16 November 2018 | Report NB-IoT breakout session | R2-1818523 |
| RAN2-104, Spokane, USA | 12-16 November 2018 | Report eMTC breakout session | R2-1818524 |
| RAN2-105, Athens, Greece | 25 Feb - 01 March 2019 | Report NB-IoT breakout session | R2-1902233 |
| RAN2-105, Athens, Greece | 25 Feb - 01 March 2019 | Report eMTC breakout session | R2-1902234 |
| RAN2-105bis, Xi’an, China | 08-12 April 2019 | Report NB-IoT breakout session | R2-1905183 |
| RAN2-105bis, Xi’an, China | 08-12 April 2019 | Report eMTC breakout session | R2-1905184 |
| RAN2-106, Reno, USA | 13-17 May 2019 | Report NB-IoT breakout session | R2-1908103 |
| RAN2-106, Reno, USA | 13-17 May 2019 | Report eMTC breakout session | R2-1908104 |
| RAN2-107, Prague | 26-30 August 2019 | Report NB-IoT breakout session | R2-1911513 |
| RAN2-107, Prague | 26-30 August 2019 | Report eMTC breakout session | R2-1911514 |
| RAN2-107bis, Chongqing, China | 14-18 October 2019 | Report NB-IoT breakout session | R2-1914137 |
| RAN2-107bis, Chongqing, China | 14-18 October 2019 | Report eMTC breakout session | R2-1914133 |
| RAN2-108, Reno, USA | 18-22 November 2019 | Report NB-IoT breakout session | R2-1916287 |
| RAN2-108, Reno, USA | 18-22 November 2019 | Report eMTC breakout session | R2-1916283 |
| RAN2-109_e | 24Feb – 06March 2020 | Report NB-IoT breakout session | R2-2001814 |
| RAN2-109_e | 24Feb – 06March 2020 | Report eMTC breakout session | R2-2001663 |

# Change history

| Tdoc | Title | Comment |
| --- | --- | --- |
| R2-1816063 | RAN2 agreements for Rel-16 additional enhancements for NB-IoT and MTC | Post RAN2-103bis |
| R2-1818633 | RAN2 agreements for Rel-16 additional enhancements for NB-IoT and MTC | Post RAN2-104 |
| R2-1902405 | RAN2 agreements for Rel-16 additional enhancements for NB-IoT and MTC | Post RAN2-105 |
| R2-1905262 | RAN2 agreements for Rel-16 additional enhancements for NB-IoT and MTC | Post RAN2-105bis |
| R2-1908188 | RAN2 agreements for Rel-16 additional enhancements for NB-IoT and MTC | Post RAN2-106 |
| R2-1911575 | RAN2 agreements for Rel-16 additional enhancements for NB-IoT and MTC | Post RAN2-107 |
| R2-1914102 | RAN2 agreements for Rel-16 additional enhancements for NB-IoT and MTC | Post RAN2-107bis |
| R2-1916424 | RAN2 agreements for Rel-16 additional enhancements for NB-IoT and MTC | Post RAN2-108 |
| R2-2001886 | RAN2 agreements for Rel-16 additional enhancements for NB-IoT and MTC | Post RAN2-109_e |