3GPP TSG-RAN WG2 #109-e	Tdoc R2-2002063

Electronic meeting, 24 February – 6 March 2020	Revision of R2-2000986

Agenda Item:	7.2.9

Source:	Ericsson

Title:	NB-IoT coexistence with NR

Document for:	Discussion, Decision

# 1	Introduction

So far coexistence between NB-IoT and NR has mostly been discussed in RAN1. Some of the agreements in RAN1#98 will require actions in RAN2. Agreements in RAN1meetings #98, #98bis and #99 [1] are:

RAN1#98:

R1-1909557 Feature lead summary of Coexistence of NB-IoT with NR Futurewei Agreement Specify NB-IoT resource reservation only for FDD/TDD NB-IoT non-anchor carriers. Agreement UL resource reservation for NB-IoT is supported. Agreement - FDD UL resource reservation is supported with subframe-level granularity - FFS: Slot-level, symbol-level - DL resource reservation is supported with slot-level and symbol(s)-level granularity. Agreement The reserved resource in NB-IoT non-anchor carrier is semi-statically configured by higher layer signalling. - FFS whether the resource in an invalid and/or valid subframe indicated by legacy downlink bitmap can be configured as the reserved resource - FFS signalling Agreement NB-IoT transmission can be postponed or dropped depending on the granularity of the reserved resources. - NB-IoT transmission is dropped for symbol-level reserved resources. - NB-IoT transmission is dropped for slot-level reserved resources. - NB-IoT transmission is postponed for subframe-level reserved resources.

RAN1#98bis:

R1-1910884 Feature lead summary of Coexistence of NB-IoT with NR Futurewei R1-1911383 Further topics in coexistence of NB-IoT with NR Futurewei Agreement NB-IoT symbols that carry NRS are not reserved Agreement For resource reservation for NB-IoT in Rel-16, the configuration is independent from legacy subframe level resource reservation. Agreement For unicast, NPDCCH and NPDSCH scrambled by C-RNTI that would fall into the reserved resource are dropped for symbol-level and slot-level reserved resources. - Dropped means punctured Agreement For unicast, NPUSCH scrambled by C-RNTI or SPS-C-RNTI that would fall into the reserved resource is postponed for subframe-level reserved resources. Agreement UL resource reservation for NB-IoT with slot-level and symbol(s)-level granularity in addition to subframe-level granularity is supported. - FFS: whether the DMRS symbols can be configured as reserved resources Agreement NB-IoT reserved resource can be separately configured per NB-IoT non-anchor carrier. Agreement For unicast transmission, dynamic DCI signalling can be used to [indicate or override] which reserved resources are used for the scheduled NB-IoT transmission.

RAN1#99:

R1-1912633 Feature lead summary of Coexistence of NB-IoT with NR Futurewei R1-1913398 Feature lead summary of Coexistence of NB-IoT with NR Futurewei Agreement For unicast, NPUSCH scrambled by C-RNTI and SPS-C-RNTI that would fall into the reserved resource is dropped for symbol and slot-level reserved resources. Agreement - For slot-level reservation in UL NB-IoT, DMRS transmission in a reserved slot is dropped. (for 15kHz) - For symbol-level reservation in UL NB-IoT, DMRS symbols can be reserved Agreement Reserved resource configuration is configured by cell-specific higher-layer signalling. R1-1913545 Feature lead summary of Coexistence of NB-IoT with NR Futurewei Agreement For 10msec periodicity, if only subframe level granularity: a bitmap with length 10 bits is used for indicating reserved subframes - It is configured per non-anchor carrier, and it can be different from the anchor-carrier For 10msec periodicity, if finer granularity: a bitmap with length 20 bits, each subframe has 2 bits per subframe - 00 both slots can be used - 01 2nd slot reserved (if a second bitmap is configured, it is for symbol-level granularity) - The length of the second bitmap is 5 bits for downlink, 7 bits for uplink - 10 1st slot reserved (if a third bitmap is configured, it is for symbol-level granularity) - The length of the third bitmap is 5 bits for downlink, 7 bits for uplink - 11 both slots are reserved Agreement (RRC impact) The first bitmap is 20 or 80 bits. eNB can configure periodicity and start position. - The periodicity T is {10ms, 20ms, 40ms, 80ms, 160ms} - The start position is in a granularity of 10ms Agreement For unicast transmission, dynamic DCI signalling can be used to indicate whether the resource reservation or continuous transmission applies to the scheduled transmission - Note: the resource reservation is the Rel-16 resource reservation.

This version is a revision of R2-2000986 which was initially submitted to RAN2#109-e. The discussion text has been revised and Proposal 2 has been changed. Text proposal on configuration has been removed and table of configuration from L1 parameter list has been added for reference.

# 2	Discussion

RAN1 agreements that require action from RAN2 are to add UE capabilities and configuration for resource reservation [2]; a feature intended to be used for improving coexistence with NR.

Add UE capabilities for resource reservation in UL and DL to enable improved coexistence between NB-IoT and NR, see text proposal.

The parameters that RAN1 has provided in [2] are cell-specific (parameters listed in table below). Hence, using system information (SI) for configuration could make sense. However, for this particular scenario such an approach has a few drawbacks, such as there are many parameters which may result in very large SI size, since SI messages are repeated this may unnecessarily increase UE power consumption and delay. Also, resource reservation is only supported starting form Rel-16. Finally, there are some configurations that could benefit from being initiated with dedicated signalling and the possibility to enable/disable the configuration. Therefore, parameters related to resource reservation are instead proposed to be provided in UE-specific configuration.

Parameters related to resource reservation are configured in dedicated RRC signalling.

| Parameter name in RAN1 spec | Parameter name in text | Description | Value range | Comment |
| --- | --- | --- | --- | --- |
| valid-subframe-config-DL | Valid subframe configuration in DL | Configuration of subframe-level dowllink time-domain resource reservation for non-anchor carrier. | {10ms, 40ms} | Agreement For 10msec periodicity, if only subframe level granularity: a bitmap with length 10 bits is used for indicating reserved subframes • It is configured per non-anchor carrier, and it can be different from the anchor-carrier Agreement For unicast transmission, dynamic DCI signalling can be used to indicate whether the resource reservation or continuous transmission applies to the scheduled transmission - Note: the resource reservation is the Rel-16 resource reservation. |
| slot-reserved-resource-config-DL | Configuration of DL time-domain resource reservation for slot level granularity | Configuration of slot- and symbol(s)-level downlink time-domain resource reservation for non-anchor carrier. | {20 bits, 80bits} | Note that FDD and TDD may require different signaling. For resource reservation for NB-IoT in Rel-16, the configuration is independent from legacy subframe level resource reservation. NB-IoT symbols that carry NRS are not reserved. Agreement NB-IoT reserved resource can be separately configured per NB-IoT non-anchor carrier. Agreement Reserved resource configuration is configured by cell-specific higher-layer signalling. Agreement For 10msec periodicity, if only subframe level granularity: a bitmap with length 10 bits is used for indicating reserved subframes • It is configured per non-anchor carrier, and it can be different from the anchor-carrier For 10msec periodicity, if finer granularity: a bitmap with length 20 bits, each subframe has 2 bits per subframe • 00 both slots can be used • 01 2nd slot reserved (if a second bitmap is configured, it is for symbol-level granularity) o The length of the second bitmap is 5 bits for downlink, 7 bits for uplink • 10 1st slot reserved (if a third bitmap is configured, it is for symbol-level granularity) o The length of the third bitmap is 5 bits for downlink, 7 bits for uplink • 11 both slots are reserved |
| symbol- reserved-resource-config-DL-second-slot | Configuration of DL time-domain resource reservation for symbol-level granularity for second slot | Configuration of symbol-level downlink time-domain resource reservation for non-anchor carrier (for the second slot in a subframe) This parameter can be configured only if "slot-reserved-resource-config-DL" is configured | {5 bits} | Note that FDD and TDD may require different signaling. Same agreement as in row 36 |
| symbol- reserved-resource-config-DL-first-slot | Configuration of DL time-domain resource reservation for symbol-level granularity for first slot | Configuration of symbol-level downlink time-domain resource reservation for non-anchor carrier (for the first slot in a subframe) This parameter can be configured only if "slot-reserved-resource-config-DL" is configured | {5 bits} | Note that FDD and TDD may require different signaling. Same agreement as in row 36 |
| reserved-resource-DL-time-periodicity | Configuration of DL time-domain resource reservation periodicity | Configuration of periodicity in milliseconds for downlink time-domain resource reservation for unicast transmission. | 10, 20, 40, 80, 160 |  |
| reserved-resource-DL-time-start-position | Configuration of DL time-domain resource reservation start position | Configuration of start position in milliseconds for downlink time-domain resource reservation for unicast transmission. | For 10ms periodicity, {0}. For 20ms periodicity, {0, 10}. For 40ms periodicity, {0, 10, 20, 30}. For 80ms periodicity, {0, 10, 20, 30, 40, 50, 60, 70}. For 160ms periodicity, {0, 10, 20, 30, 40, 50, 60, 70, 80, 90, 100, 110, 120, 130, 140, 150}. |  |
| valid-subframe-config-FDD-UL | Valid subframe configuration in UL | Configuration of subframe-level uplink time-domain resource reservation for non-anchor carrier in FDD. | {10ms, 40ms} | Note that valid subframe configuration is already supported for FDD DL, TDD DL, and TDD UL. Agreement NB-IoT reserved resource can be separately configured per NB-IoT non-anchor carrier. Agreement Reserved resource configuration is configured by cell-specific higher-layer signalling. Agreement For unicast transmission, dynamic DCI signalling can be used to indicate whether the resource reservation or continuous transmission applies to the scheduled transmission - Note: the resource reservation is the Rel-16 resource reservation. |
| slot-reserved-resource-config-UL | Configuration of UL time-domain resource reservation for slot-level granularity | Configuration of slot- and symbol(s)-level uplink time-domain resource reservation for non-anchor carrier. | {20 bits, 80bits} | Note that FDD and TDD may require different signaling. For resource reservation for NB-IoT in Rel-16, the configuration is independent from legacy subframe level resource reservation. Agreement NB-IoT reserved resource can be separately configured per NB-IoT non-anchor carrier. Agreement Reserved resource configuration is configured by cell-specific higher-layer signalling. |
| symbol- reserved-resource-config-UL-second-slot | Configuration of UL time-domain resource reservation for symbol-level granularity for second slot | Configuration of symbol-level uplink time-domain resource reservation for non-anchor carrier (for the second slot in a subframe). This parameter can be configured only if "slot-reserved-resource-config-UL" is configured. | {7 bits} | Note that FDD and TDD may require different signaling. Same agreement as in row 36 |
| symbol- reserved-resource-config-UL-first-slot | Configuration of UL time-domain resource reservation for symbol-level granularity for first slot | Configuration of symbol-level uplink time-domain resource reservation for non-anchor carrier (for the first slot in a subframe). This parameter can be configured only if "slot-reserved-resource-config-UL" is configured | {7 bits} | Note that FDD and TDD may require different signaling. Same agreement as in row 36 |
| reserved-resource-UL-time-periodicity | Configuration of UL time-domain resource reservation periodicity | Configuration of periodicity in milliseconds for uplink time-domain resource reservation for unicast transmission. | 10, 20, 40, 80, 160 |  |
| reserved-resource-UL-time-start-position | Configuration of UL time-domain resource reservation start position | Configuration of start position in milliseconds for uplink time-domain resource reservation for unicast transmission. | For 10ms periodicity, {0}. For 20ms periodicity, {0, 10}. For 40ms periodicity, {0, 10, 20, 30}. For 80ms periodicity, {0, 10, 20, 30, 40, 50, 60, 70}. For 160ms periodicity, {0, 10, 20, 30, 40, 50, 60, 70, 80, 90, 100, 110, 120, 130, 140, 150}. |  |

# Text Proposal

[36.306]

#### 4.3.4.xy	resourceReservation-DL-r16

This field indicates that the UE supports DL resource reservation resource with subframe-level, slot-level and symbol-level granularity of NB-IoT non-anchor carriers for improved NR coexistence performance.

#### 4.3.4.xz	resourceReservation-UL-r16

This field indicates that the UE supports UL resource reservation with subframe-level, slot-level and symbol-level granularity of NB-IoT non-anchor carriers for improved NR coexistence performance.

[36.331]

#### –	UE-Capability-NB

The IE UE-Capability-NB is used to convey the NB-IoT UE Radio Access Capability Parameters, see TS 36.306 [5]. The IE UE-Capability-NB is transferred in NB-IoT only.

UE-Capability-NB information element

-- ASN1START

UE-Capability-NB-r13 ::=		SEQUENCE {

accessStratumRelease-r13		AccessStratumRelease-NB-r13,

ue-Category-NB-r13				ENUMERATED {nb1}					OPTIONAL,

multipleDRB-r13					ENUMERATED {supported}				OPTIONAL,

pdcp-Parameters-r13				PDCP-Parameters-NB-r13				OPTIONAL,

phyLayerParameters-r13			PhyLayerParameters-NB-r13,

rf-Parameters-r13				RF-Parameters-NB-r13,

dummy							SEQUENCE {}							OPTIONAL

}

UE-Capability-NB-Ext-r14-IEs ::=		SEQUENCE {

ue-Category-NB-r14					ENUMERATED {nb2}				OPTIONAL,

mac-Parameters-r14					MAC-Parameters-NB-r14			OPTIONAL,

phyLayerParameters-v1430			PhyLayerParameters-NB-v1430		OPTIONAL,

rf-Parameters-v1430					RF-Parameters-NB-v1430,

nonCriticalExtension				UE-Capability-NB-v1440-IEs		OPTIONAL

}

UE-Capability-NB-v1440-IEs ::=		SEQUENCE {

phyLayerParameters-v1440			PhyLayerParameters-NB-v1440		OPTIONAL,

nonCriticalExtension				UE-Capability-NB-v14x0-IEs		OPTIONAL

}

UE-Capability-NB-v14x0-IEs ::=		SEQUENCE {

-- Following field is only to be used for late REL-14 extensions

lateNonCriticalExtension			OCTET STRING					OPTIONAL,

nonCriticalExtension				UE-Capability-NB-v1530-IEs		OPTIONAL

}

UE-Capability-NB-v1530-IEs ::=		SEQUENCE {

earlyData-UP-r15					ENUMERATED {supported}			OPTIONAL,

rlc-Parameters-r15					RLC-Parameters-NB-r15,

mac-Parameters-v1530				MAC-Parameters-NB-v1530,

phyLayerParameters-v1530			PhyLayerParameters-NB-v1530		OPTIONAL,

tdd-UE-Capability-r15				TDD-UE-Capability-NB-r15		OPTIONAL,	nonCriticalExtension				UE-Capability-NB-v16xy-IEs		OPTIONAL

}

UE-Capability-NB-v16xy-IEs ::=		SEQUENCE {

phyLayerParameters-v16xy			PhyLayerParameters-NB-v16xy		OPTIONAL,

nonCriticalExtension				SEQUENCE {}						OPTIONAL

}

TDD-UE-Capability-NB-r15 ::=		SEQUENCE {

ue-Category-NB-r15					ENUMERATED {nb2}				OPTIONAL,

phyLayerParametersRel13-r15			PhyLayerParameters-NB-r13		OPTIONAL,

phyLayerParametersRel14-r15			PhyLayerParameters-NB-v1430		OPTIONAL,

phyLayerParameters-v1530			PhyLayerParameters-NB-v1530		OPTIONAL,

...

}

AccessStratumRelease-NB-r13 ::=		ENUMERATED {rel13, rel14, rel15, spare5, spare4, spare3, spare2, spare1, ...}

PDCP-Parameters-NB-r13		::= SEQUENCE {

supportedROHC-Profiles-r13			SEQUENCE {

profile0x0002						BOOLEAN,

profile0x0003						BOOLEAN,

profile0x0004						BOOLEAN,

profile0x0006						BOOLEAN,

profile0x0102						BOOLEAN,

profile0x0103						BOOLEAN,

profile0x0104						BOOLEAN

},

maxNumberROHC-ContextSessions-r13	ENUMERATED {cs2, cs4, cs8, cs12}	DEFAULT cs2,

...

}

RLC-Parameters-NB-r15		::=		SEQUENCE {

rlc-UM-r15							ENUMERATED {supported}				OPTIONAL

}

MAC-Parameters-NB-r14		::=		SEQUENCE {

dataInactMon-r14					ENUMERATED {supported}					OPTIONAL,

rai-Support-r14						ENUMERATED {supported}				OPTIONAL

}

MAC-Parameters-NB-v1530		::=		SEQUENCE {

sr-SPS-BSR-r15						ENUMERATED {supported}			OPTIONAL

}

PhyLayerParameters-NB-r13	::=		SEQUENCE {

multiTone-r13						ENUMERATED {supported}			OPTIONAL,

multiCarrier-r13						ENUMERATED {supported}			OPTIONAL

}

PhyLayerParameters-NB-v1430	::=		SEQUENCE {

multiCarrier-NPRACH-r14				ENUMERATED {supported}			OPTIONAL,

twoHARQ-Processes-r14				ENUMERATED {supported}			OPTIONAL

}

PhyLayerParameters-NB-v1440	::=		SEQUENCE {

interferenceRandomisation-r14		ENUMERATED {supported}			OPTIONAL

}

PhyLayerParameters-NB-v1530	::=		SEQUENCE {

mixedOperationMode-r15				ENUMERATED {supported}			OPTIONAL,

sr-WithHARQ-ACK-r15					ENUMERATED {supported}			OPTIONAL,

sr-WithoutHARQ-ACK-r15				ENUMERATED {supported}			OPTIONAL,

nprach-Format2-r15					ENUMERATED {supported}			OPTIONAL,

additionalTransmissionSIB1-r15		ENUMERATED {supported}			OPTIONAL,

npusch-3dot75kHz-SCS-TDD-r15		ENUMERATED {supported}			OPTIONAL

}

PhyLayerParameters-NB-v16xy	::=		SEQUENCE {

resourceReservation-DL-r16				ENUMERATED {supported}			OPTIONAL,

resourceReservation-UL-r16				ENUMERATED {supported}			OPTIONAL

}

[text omitted]

| UE-Capability-NB field descriptions | FDD/TDD appl | FDD/TDD diff |
| --- | --- | --- |
| accessStratumRelease Set to rel15 in this version of the specification. | FDD/TDD | No |
| additionalTransmissionSIB1 Indicates whether the UE supports additional SIB1 transmission as specified in TS 36.213 [23]. | FDD | - |
| dataInactMon Indicates whether the UE supports the data inactivity monitoring as specified in TS 36.321 [6]. | FDD/TDD | No |
| dummy This field is not used in the specification. It shall not be sent by the UE. | NA | NA |
| earlyData-UP Indicates whether the UE supports EDT for User plane CIoT EPS optimisations, as defined in TS 24.301 [35]. | FDD | - |
| interferenceRandomisation For FDD: Indicates whether the UE supports interference randomisation in connected mode as defined in TS.36.211 [21]. | FDD | - |
| maxNumberROHC-ContextSessions Set to the maximum number of concurrently active ROHC contexts supported by the UE, excluding context sessions that leave all headers uncompressed. cs2 corresponds with 2 (context sessions), cs4 corresponds with 4 and so on. The network ignores this field if the UE supports none of the ROHC profiles in supportedROHC-Profiles. | FDD/TDD | No |
| mixedOperationMode Defines whether the UE supports multi-carrier operation with mixed operation mode, standalone or inband/guardband, between the anchor carrier and the non-anchor carrier for unicast, paging, and random access as specified in TS 36.300 [9]. | FDD | - |
| multiCarrier Defines whether the UE supports multi -carrier operation. | FDD/TDD | Yes |
| multicarrier-NPRACH Defines whether the UE supports NPRACH on non-anchor carrier as specified in TS 36.321 [6]. | FDD/TDD | Yes |
| multipleDRB Defines whether the UE supports multiple DRBs. | FDD/TDD | No |
| multiNS-Pmax Defines whether the UE supports the mechanisms defined for NB-IoT cells broadcasting NS-PmaxList-NB. | FDD/TDD | No |
| multiTone Defines whether the UE supports UL multi-tone transmissions on NPUSCH. | FDD/TDD | Yes |
| nprach-Format2 Defines whether the UE supports NPRACH resources using preamble format 2. | FDD | - |
| npusch-3dot75kHz-SCS-TDD Indicates whether the UE supports NPUSCH with 3.75kHz SCS for TDD. | TDD | - |
| powerClassNB-14dBm Defines whether the UE supports power class 14dBm in all the bands supported by the UE as specified in TS 36.101 [42]. If powerClassNB-20dBm is included, the UE shall not include the field powerClassNB-14dBm. | FDD/TDD | No |
| powerClassNB-20dBm Defines whether the UE supports power class 20dBm in NB-IoT for the band, as specified in TS 36.101 [42]. If neither powerClassNB-14dBm nor powerClassNB-20dBm is included, UE supports power class 23 dBm in the NB-IoT band. | FDD/TDD | No |
| rai-Support Defines whether the UE supports release assistance indication (RAI) as specified in TS 36.321 [6]. | FDD/TDD | No |
| resourceReservation-DL, resourceReservation-UL DL resource reservation with slot-level and symbol-level granularity of NB-IoT non-anchor carriers. UL resource reservation with subframe-level, slot-level and symbol-level granularity of NB-IoT non-anchor carriers. | FDD/TDD | Yes |

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Add UE capabilities for resource reservation in UL and DL to enable improved coexistence between NB-IoT and NR, see text proposal.

Proposal 2	Parameters related to resource reservation are configured in dedicated RRC signalling.

# References

R1-1913595 RAN1 agreement list (meetings #98, #98bis, and #99)

R1-1913673 R16 L1 parameter list NB_IOTenh3