3GPP TSG-RAN WG2 Meeting #109e	R2-2002072

Elbonia, Online, 24 February – 6 March 2020

Agenda item:	6.7.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Summary on UE features and capabilities

WID/SID:	NR_IIOT - Release 16

Document for:	Discussion and Decision

# Introduction

The following open issues for UE features and capabilities were identified during the related e-mail discussion prior to RAN2#109-e meeting [1]:

- Ethernet header compression

The need for Ethernet padding removal and Ethernet padding addition features is to be confirmed during RAN2#109.

It is FFS whether Ethernet padding removal capability is needed (if its support is confirmed).

It is FFS whether a capability for EHC context continuation and maxNumberEHC-ContextSessions-r16 are needed (pending the discussion on remaining aspects of EHC).

- TSC scheduling

It is FFS whether “Additional DL SPS periodicities” and “Additional CG periodicities” features require the signalling of maximum value of N supported by the UE.

It is FFS whether a UE supporting [multipleCG-Configs-r16] shall also support lch-ToConfiguredGrantMapping-r16.

- Intra-UE prioritization

It is FFS whether LCH priority-based prioritization covers both data/data prioritization and SR/data prioritization or if separate features are needed.

- PDCP duplication

It is FFS whether “PDCP duplication with more than two RLC entities” feature needs to be separate for UM and AM RLC modes.

# 2	Accurate reference timing summary

## 2.1	Ethernet Header Compression

The related proposals were summarized as part of Summary on EHC and are not discussed here.

## 2.2	TSC scheduling

### 2.2.1	CG and SPS periodicities

| Company (Tdoc) | Proposals | Rationale |
| --- | --- | --- |
| Qualcomm (R2-2001290) | Proposal 1: Support signalling of maximum value of additional SPS periodicity and additional CG periodicity supported by a UE. | Large number of new CG/SPS periodicities (>9000) are supported in Rel-16. Testing/verification of all new periodicities can be challenging. While configurations with lower values of new SPS/CG periodicities (e.g., less than 10ms) are known to be useful, practical relevance of vast majority of new CG/SPS periodicities is unknown. |
| Nokia, Nokia Shanghai Bell (R2-2001049) | Proposal 3: Support CG periodicities of multiple of 2/7 symbols as a separate capability with a cross-slot boundary capability as a pre-requisite. | The main argument against supporting CG periodicities of multiple of 2/7 symbols were potential issues with resource allocations across the slot boundary and the proposal addresses this issue. |
| Nokia, Nokia Shanghai Bell (R2-2001052) | Proposal 4: Do not introduce further fragmentation for “Additional DL SPS periodicities” and “Additional CG periodicities” features. | In our opinion, it will be very hard for implementers to predict which of the periodicities will be needed in real deployments. In addition, it would limit significantly the scheduling flexibility in the presence of multiple IIOT devices supporting multiple TSC flows. Also, there is currently no dedicated solution for support of ‘non-integer’ TSC periodicities and the only solution is to use overprovisioning. With limited periodicities support at the UE, this task would be very hard. |

Summary:

Considering that only two companies expressed their view on the issue, it is proposed to discuss it further:

Proposal 1: Discuss whether signalling of maximum value of additional SPS periodicities and additional CG periodicities supported by a UE is required.

Proposal 2: Discuss whether to support CG periodicities of multiple of 2/7 symbols as a separate capability with a cross-slot boundary capability as a pre-requisite.

### 2.2.2	Relation between multipleCG-Configs and lch-ToConfiguredGrantMapping-r16 capabilities

| Company (Tdoc) | Proposals | Rationale |
| --- | --- | --- |
| Ericsson (R2-2000789) | Proposal 1 UE supporting multipleCG-Configs shall also support lch-ToConfiguredGrantMapping. | No separate capabilities are required in this case as multiple CG configurations and LCH to CG mapping would typically be used together anyway. Multiple CG were introduced to handle TSC flows better, for which also the LCH to CG mapping should be used to schedule resources for certain TSC flows deterministically. |
| Nokia, Nokia Shanghai Bell (R2-2001052) | Proposal 6: Keep the requirement that UE supporting multipleCG-Configs should also support lch-ToConfiguredGrantMapping. | It is very important to keep those two features together as otherwise the management of multiple LCHs which are provided with their dedicated CG configuration becomes very cumbersome. It may often be very challenging or even impossible for the network to achieve the intended mapping only by using legacy LCP restrictions. |
| Qualcomm (R2-2001290) | Proposal 2: A UE supporting multiple CG configurations feature need not support LCH-to-CG mapping restriction. | A UE could support multiple CGs without LCP restrictions (e.g., if associated traffic flows have non-overlapping arrivals) or use other LCP restrictions to restrict LCHs using the CGs. |

Summary:

Two companies indicated that UE supporting multipleCG-Configs should also support lch-ToConfiguredGrantMapping while one company has the opposite view. The following proposal is made:

Proposal 3: FFS if the UE supporting multipleCG-Configs shall also support lch-ToConfiguredGrantMapping.

## 2.3	Intra-UE prioritization

| Company (Tdoc) | Proposals | Rationale |
| --- | --- | --- |
| CATT (R2-2000115) | Proposal 5: It is not necessary to introduce one new parameter for SR/data prioritization. | Separating the features will also add extra-complexity and specification effort, so it is preferred to keep it a one overall functionality configured with a single parameter. |
| Ericsson (R2-2000797) | Proposal 5 SR/data prioritization is separate configurable feature from data/data prioritization feature. | While similarities and overlap in functionality of these features exist, their purpose and impact on UE MAC multiplexing as well as scheduling and gNB UL reception are quite different, e.g. it is not necessary or even not wanted in some implementations that UE prioritized between potentially overlapping grants for data/data. |
| Qualcomm (R2-2001289) | Proposal 1: SR and data prioritization each have separate RRC configuration parameters and capabilities. | SR vs grant prioritization is a feature distinct from and complementary to grant prioritization, and both can be useful on their own for URLLC traffic. |
|  | Proposal 6: Once RAN1 has defined feature/capability related to PHY layer prioritization, RAN2 should discuss whether LCH based prioritization can be supported without PHY prioritization. | RAN2 should discuss whether LCH based prioritization can be implemented without PHY prioritization before introducing any such dependence between the two. For instance, PDU suppression of LCH based prioritization can be used without any PHY-layer actions like pre-emption provided Rel-15 timelines for associated grants are followed by the scheduler. |
| Samsung (R2-2001497) | Proposal 3. A single configuration lch-basedPrioritization is used for both 1) Data-Data prioritization and 2) SR Data prioritization. | Both prioritization between grants and a grant and SR are based on the priority value of the logical channels, so we can say that they have a commonality and are similar to each other. |
| Nokia, Nokia Shanghai Bell (R2-2001052) | Proposal 3: Confirm that lch-PriorityBasedPrioritization-r16 covers both data vs. data and SR vs. data prioritization. | Both prioritization mechanisms are based on comparison of LCH priority of either an LCH carried by a MAC PDU or of an LCH which triggered an SR. At the same time, both of these are equally important when it comes to support of URLLC traffic. In order to avoid too extensive capability fragmentation and specifications complexity, it is proposed to keep those two together. |

Summary:

Some companies seem to mix the issue of separate configurability of data vs. data and SR vs. data prioritization with a capability signalling for those. Two companies (CATT, Samsung) indicate that a single configuration parameter is used, which automatically means these are both a single capability. One company (Ericsson) speaks of separate configurability, but does not refer to capability signalling. Once company (Qualcomm) mentions that both configurability and capability signalling should be separate. One company (Nokia) indicates that these two should be a single capability, but does not mention configurability.

In attempt to reach a common ground, it is proposed to agree on the following proposals:

Proposal 4: Data vs. data and SR vs. data prioritization are configured separately.

Proposal 5: Data vs. data and SR vs. data prioritization are signalled as a single capability.

It has been also raised by one company that we should wait to see the RAN1 definition of feature/capability for intra-UE prioritization to see whether they should be coupled. It is proposed to keep the current definition as in the current 38.306 CR in [2] and re-discuss after receiving feature descriptions from RAN1.

Proposal 6: Once RAN1 has defined feature/capability related to PHY layer prioritization, discuss whether LCH based prioritization can be supported without PHY prioritization.

## 2.4	PDCP duplication

| Company (Tdoc) | Proposals | Rationale |
| --- | --- | --- |
| Qualcomm (R2-2001288) | Proposal 1: PDCP duplication enhancements (support for up to 4 RLC entities, dynamic leg selection, resource efficient PDCP duplication) with RLC entities in mode AM should either - not be supported, or - should be configured for a UE based on a capability signalled by the UE. | PDCP duplication enhancements (support for up to 4 RLC entities, dynamic leg selection, resource efficient PDCP duplication) in scope of this WI are primarily to enhance reliability for use cases requiring low latency and RLC AM mode is not useful for such use cases. Given PDCP duplication enhancements with RLC entities in mode AM don’t add much value, it should be configured for a UE based on a capability to avoid test cases for configurations with limited practical use. |
| Nokia, Nokia Shanghai Bell (R2-2001052) | Proposal 5: Capability of PDCP duplication with more than two RLC entities is common for RLC AM and RLC UM mode. | For two-leg duplication AM and UM modes are not separated and there seems to be no additional complexity of supporting PDCP duplication over RLC AM mode as compared to RLC UM mode. |

Summary:

Only two companies expressed their view and had opposite views.

Proposal 7: Discuss whether separate capabilities are needed for PDCP duplication with more than two RLC entities for RLC AM and RLC UM mode.

# 3	Conclusions

Based on the summary in section 2, the following proposals are made:

Proposal 1: Discuss whether signalling of maximum value of additional SPS periodicities and additional CG periodicities supported by a UE is required.

Proposal 2: Discuss whether to support CG periodicities of multiple of 2/7 symbols as a separate capability with a cross-slot boundary capability as a pre-requisite.

Proposal 3: FFS if the UE supporting multipleCG-Configs shall also support lch-ToConfiguredGrantMapping.

Proposal 4: Data vs. data and SR vs. data prioritization are configured separately.

Proposal 5: Data vs. data and SR vs. data prioritization are signalled as a single capability.

Proposal 6: Once RAN1 has defined feature/capability related to PHY layer prioritization, discuss whether LCH based prioritization can be supported without PHY prioritization.

Proposal 7: Discuss whether separate capabilities are needed for PDCP duplication with more than two RLC entities for RLC AM and RLC UM mode.

# 4	List of referenced documents

- R2-2001053	Summary of e-mail discussion: [108#47][IIOT] UE feature list, Nokia, Nokia Shanghai Bell

- R2-2001054	UE radio access capabilities introduction for NR IIOT WI	Nokia, Nokia Shanghai Bell

- R2-2001290	Open issues in Scheduling Enhancements	Qualcomm Incorporated

- R2-2001049	Remaining issues on TSC scheduling	Nokia, Nokia Shanghai Bell

- R2-2001052	UE feature list and capabilities remaining issues	Nokia, Nokia Shanghai Bell

- R2-2000789	SPS and CG remaining MAC aspects	Ericsson

- R2-2000115	Remaining issues for intra-UE multiplexing and prioritization	CATT

- R2-2000797	Remaining details of intra-UE prioritization	Ericsson

- R2-2001289	Open issues in Intra-UE prioritization	Qualcomm Incorporated

- R2-2001497	Prioritization of SR Transmission	Samsung

- R2-2001288	Open issues in PDCP duplication enhancements	Qualcomm Incorporated