3GPP TSG-RAN WG2 Meeting #109 electronic	R2-2002118

24th Feb – 6th Mar 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.304 | CR | 0784 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of TS 36.304 to introduce IAB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB Core |  |  |  |  |  | Date: |  |  | 2020-02-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN2 agreed to introduce the IAB supporting indication in the SIB1 to indicate whether cell is a candidate cell for IAB node. Both support of IAB node(s) and the cell status for IAB node(s) is combined in the IE iab-Support, i.e. if the IE is present, the cell supports IABs and the cell is also considered as a candidate for IABs; if the IE is absent, the cell does not support IAB and/or the cell is barred for IAB. In the TS 36.304, the deterimination of cell status for IAB cell should be clarified using the new defined IE in RRC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In section 5.3.1: Add “iab-Support” to determine the cell status; Impact analysis Impacted 5G architecture options: EN-DC Impacted functionality: Cell status Inter-operability: - If the UE is implemented according to this CR but the network is not, there is no inter-operability issue foreseen. - If the network is implemented according to this CR but the UE is not, there is no inter-operability issue foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IAB node can not determine whether the cell supporting IAB and whether the cell is barred for IAB node. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 5.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------------- [Start of 1st change] ---------------------------------

3.3	 Abbreviations

For the purposes of the present document, the following abbreviations apply:

1xRTT	CDMA2000 1x Radio Transmission Technology

AS	Access Stratum

AC	Access Class (of the USIM)

ACDC	Application specific Congestion control for Data Communication

BCCH	Broadcast Control Channel

BL	Bandwidth reduced Low complexity

BR-BCCH	Bandwidth Reduced Broadcast Control Channel

BSS	Basic Service Set

CMAS	Commercial Mobile Altert System

CSG	Closed Subscriber Group

DRX	Discontinuous Reception

DL-SCH	Downlink Shared Channel

EHPLMN	Equivalent Home PLMN

EPC	Evolved Packet Core

EPS	Evolved Packet System

ETWS	Earthquake and Tsunami Warning System

E-UTRA	Evolved UMTS Terrestrial Radio Access

E-UTRAN	Evolved UMTS Terrestrial Radio Access Network

FDD	Frequency Division Duplex

GERAN	GSM/EDGE Radio Access Network

HPLMN	Home PLMN

HSDN	High Speed Dedicated Network

H-SFN	Hyper System Frame Number

HRPD	High Rate Packet Data

IAB	Integrated Access and Backhaul

IMSI	International Mobile Subscriber Identity

MBMS	Multimedia Broadcast-Multicast Service

MBSFN	Multimedia Broadcast multicast service Single Frequency Network

MCC	Mobile Country Code

MCCH	Multicast Control Channel

MDT	Minimization of Drive Tests

MM	Mobility Management

MNC	Mobile Network Code

MPDCCH	MTC Physical Downlink Control Channel

MTCH	Multicast Traffic Channel

NAS	Non-Access Stratum

NB-IoT	NarrowBand Internet of Things

NR	NR Radio Access

PLMN	Public Land Mobile Network

ProSe	Proximity-based Services

PSM	Power Saving Mode

PTW	Paging Time Window

PWS	Public Warning System

RAT	Radio Access Technology

RNA	RAN-based Notification Area

RNAU	RAN-based Notification Area Update

RRC	Radio Resource Control

SAP	Service Access Point

SIBX	SystemInformationBlockTypeX

TDD	Time Division Duplex

UAC	Unified Access Control

UE	User Equipment

UMTS	Universal Mobile Telecommunications System

USIM	Universal Subscriber Identity Module

UTRA	UMTS Terrestrial Radio Access

UTRAN	UMTS Terrestrial Radio Access Network

V2X	Vehicle-to-Everything

WUS	Wake Up Signal

--------------------- [End of 1st change] ---------------------------------

--------------------- [Start of 2nd change] ---------------------------------

5.3	Cell Reservations and Access Restrictions

There are two mechanisms which allow an operator to impose cell reservations or access restrictions. The first mechanism uses indication of cell status and special reservations for control of cell selection and reselection procedures. The second mechanism, referred to as Access Control, shall allow preventing selected classes of users or ACDC categories from sending initial access messages for load control reasons. For Access Control based on Access Classes, at subscription, one or more Access Classes are allocated to the subscriber and stored in the USIM TS 22.011 [4]. For Access Control based on ACDC categories, at subscription at least four ACDC categories are allocated to the subscriber and stored in the ACDC MO TS 24.105 [31] or USIM TS 31.102 [32].

5.3.1	Cell status and cell reservations

Cell status and cell reservations are indicated in the SystemInformationBlockType1 message (or SystemInformationBlockType1-BR message or SystemInformationBlockType1-NB message) TS 36.331 [3] by means of the following fields:

-	cellBarred (IE type: "barred" or "not barred") 
This field indicates if the cell is barred for connectivity to EPC.
This field is ignored by the UEs supporting crs-IntfMitig while crs-IntfMitigEnabled is included in SIB1. 
This field is ignored by the BL UEs or UEs in CE supporting ce-CRS-IntfMitig while crs-IntfMigitNumPRBs is included in SIB1-BR.
In case of multiple EPC PLMNs indicated in SIB1/SIB1-BR, this field is common for all EPC PLMNs

NOTE:	For IAB node, it ignores the cellBarred, cellReservedForOperatorUse and cellReservedForOtherUse as defined in TS 36.331 [3].

Editor Notes: This can be updated based on the possible agreements by discussion of “R2-2002058, Summary of 6.1.5.3: SI Broadcast, cell Restrictions/Reservation and Barring, Initial Access, and Connection Setup, Ericsson”.

-	cellBarred-5GC (IE type: "barred" or "not barred")
This field indicates if the cell is barred for connectivity to 5GC.
This field is ignored if the UE does not support E-UTRA connected to 5GC or if the UE supports network-based CRS interference mitigation and nw-BasedCRS-InterferenceMitigation is included in SystemInformationBlockType1.
In case of multiple 5GC PLMNs indicated in SIB1, this field is common for all 5GC PLMNs.

-	cellReservedForOperatorUse (IE type: "reserved" or "not reserved")
This field indicates if the cell is reserved for operator use.
This field is ignored by the UEs supporting crs-IntfMitig while crs-IntfMitigEnabled is included in SIB1. 
This field is ignored by the BL UEs or UEs in CE supporting ce-CRS-IntfMitig while crs-IntfMigitNumPRBs is included in SIB1-BR. 
In case of multiple EPC or 5GC PLMNs indicated in SIB1/SIB1-BR, this field is specified per EPC or 5GC PLMN.

-	cellBarred-CRS (IE type: "barred" or "not barred")
This field indicates if the cell is barred for connectivity to EPC for UEs supporting network-based CRS interference mitigation.
barred means the cell is barred for UEs supporting crs-IntfMitig while crs-IntfMitigEnabled is included in SIB1. For BL UEs or UEs in CE capable of ce-CRS-IntfMitig, barred means the cell is barred while crs-IntfMitigNumPRBs is included in SIB1-BR.
This field is ignored by the UE if the UE does not support CRS interference mitigation or while crs-IntfMitigConfig is not included in SIB1 (SIB1-BR for BL UEs or UEs in CE).
In case of multiple PLMNs indicated in SIB1/SIB1-BR, this field is common for all PLMNs.

-	cellBarred-5GC-CRS (IE type: "barred" or "not barred")
This field indicates if the cell is barred for connectivity to 5GC for UEs supporting network-based CRS interference mitigation.
This field is ignored if the UE does not support E-UTRA connected to 5GC or network-based CRS interference mitigation.
In case of multiple 5GC PLMNs indicated in SIB1, this field is common for all 5GC PLMNs.

-	cellReservedForOperatorUse-CRS (IE type: "reserved" or "not reserved")
This field indicates if the cell is reserved for operator use for UEs supporting network-based CRS interference mitigation.
reserved means the cell is "reserved" for operator use for UEs supporting crs-IntfMitig while crs-IntfMitigEnabled is included in SIB1. 
For BL UEs or UEs in CE capable of ce-CRS-IntfMitig, reserved means the cell is "reserved" for operator use while crs-IntfMitigNumPRBs is included in SIB1-BR.
This field is ignored if the UE does not support CRS interference mitigation or while crs-IntfMitigConfig is not included in SIB1 (SIB1-BR for BL UEs or UEs in CE).
In case of multiple PLMNs indicated in SIB1/SIB1-BR, this field is specified per PLMN.

-	iab-Support (IE type: "true")

Indicated in SIB1 message. In case of multiple PLMNs indicated in SIB1, this field is specified per PLMN. [This field indicates if the cell is barred for IAB node or the cell does not support IAB node, or both. When this field is absent, the IAB node shall treat this cell as if cell status is barred]

Editor Notes: The need of the text in bracket is FFS.

The following description for handling of barred and reserved cells is per CN type. If the UE supports more than one CN type, the UE shall only exclude a cell as candidate for selection/reselection if it is excluded for both CN types.

NOTE:	Fields cellBarred-CRS and cellReservedForOperatorUse-CRS are not indicated in SystemInformationBlockType1-NB

When cell status is indicated as "not barred" and "not reserved" for operator use,

-	All UEs shall treat this cell as candidate during the cell selection and cell reselection procedures.

When cell status is indicated as "not barred" and "reserved" for operator use for any PLMN,

-	UEs assigned to Access Class 11 or 15 operating in their HPLMN/EHPLMN shall treat this cell as candidate during the cell selection and reselection procedures if the field cellReservedForOperatorUse for that PLMN set to "reserved".

-	UEs assigned to an Access Class in the range of 0 to 9, 12 to 14 shall behave as if the cell status is "barred" in case the cell is "reserved for operator use" for the registered PLMN or the selected PLMN.

NOTE:	ACs 11, 15 are only valid for use in the HPLMN/ EHPLMN; ACs 12, 13, 14 are only valid for use in the home country TS 22.011 [4].

When cell status "barred" is indicated or to be treated as if the cell status is "barred",

-	The UE is not permitted to select/reselect this cell, not even for emergency calls.

-	The UE shall consider other cells for cell selection/reselection according to the following rule:

-	If the cell is to be treated as if the cell status is "barred" due to being unable to acquire the MasterInformationBlock (or MasterInformationBlock-NB), the SystemInformationBlockType1 (or SystemInformationBlockType1-BR message or SystemInformationBlockType1-NB), or the SystemInformationBlockType2 (or SystemInformationBlockType2-NB):

-	the UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds.

-	the UE may select another cell on the same frequency if the selection criteria are fulfilled.

-	the UE may select the same cell in normal coverage if the UE was barred in the cell due to being unable to acquire MasterInformationBlock, SystemInformationBlockType1-BR, or SystemInformationBlockType2 in enhanced coverage, but was able to acquire MasterInformationBlock, SystemInformationBlockType1, and SystemInformationBlockType2 in normal coverage, if the selection criteria are fulfilled.

-	else

-	If the cell is a CSG cell:

-	the UE may select another cell on the same frequency if the selection/reselection criteria are fulfilled.

-	If operating as an IAB-MT and the cell is to be treated as if the cell status is "barred" due to iab-Support being absent in SIB1:

-	The IAB-MT may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds;

Editor Notes: FFS on the need of above text.

-	else

-	If the field intraFreqReselection in field cellAccessRelatedInfo in SystemInformationBlockType1 (or SystemInformationBlockType1-BR message or SystemInformationBlockType1-NB) message is set to "allowed", the UE may select another cell on the same frequency if re-selection criteria are fulfilled.

-	The UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds.

-	If the field intraFreqReselection in field cellAccessRelatedInfo in SystemInformationBlockType1 (or SystemInformationBlockType1-BR message or SystemInformationBlockType1-NB) message is set to "not allowed" the UE shall not re-select a cell on the same frequency as the barred cell;

-	The UE shall exclude the barred cell and the cells on the same frequency as a candidate for cell selection/reselection for 300 seconds.

The cell selection of another cell may also include a change of RAT or, if the previous and selected cell are both E-UTRA cells, a change of the CN type.

--------------------------[End of change] ------------------------------