# 3GPP TSG-RAN WG2 Meeting #109e					  R2-200xxxx

# E-meeting, February 24 – March 6, 2020

Agenda item:		6.7.4 (NR_IIOT-Core)

Source:	LG Electronics Inc.

Title: 	Discussion on PDCP duplication enhancements

Document for:	Information

# 1.	Introduction

This document is to facilitate the discussion on AT109e#037 PDCP Duplication Enhancement for IIOT.

[AT109e][037][IIOT] PDCP Duplication Enhancements (LG) Scope: Treat summary on PDCP Duplication Enhancements. Intended outcome: Resolve issues, Describe Open Issues accurately. Deadline: Mar 3 1200 CET (conclusions on “easy agreements” by Feb 27 1200 CET)

The discussion should be based on the summary document submitted to this meeting.

R2-2001286 Summary of PDCP duplication enhancements LG Electronics Inc. report Rel-16 NR_IIOT-Core Late

This document tries to make easy agreements and resolve open issues identified in the R2-2001286.

# 2.	Potential easy agreements

Potential agreements 1

-	Rel-16 PDCP duplication is applied to SRBs.

-	For SRBs, all secondary RLC entities are activated when configured.

-	MAC CE based activation/deactivation of PDCP duplication is not supported for SRBs.

Question 1: Are companies ok with the potential agreements 1?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Nokia | Yes |  |

Proposal 1:

Potential agreements 2

-	When a secondary RLC entity is deactivated (but PDCP duplication is still activated), the UE shall discard duplicated PDCP PDUs in the deactivated secondary RLC entity.

Question 2: Are companies ok with the potential agreements 2?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Nokia | Yes |  |

Proposal 2:

# 3.	Open issues

Open issue 1

Can the Rel-15 Duplication MAC CE be used for Rel-16 Duplication configuration?

- 	Option 1: Rel-15 MAC CE can be used for Rel-16 Duplication configuration.

-	Option 2: Rel-15 MAC CE can be used for Rel-16 Duplication configuration only when the number of leg = 2.

- 	Option 3: Rel-15 MAC CE is not used for Rel-16 Duplication configuration.

- 	Option 4: Rel-15 MAC CE is used to activate/deactivate PDCP duplication, and Rel-16 MAC CE is used to activate/deactivate RLC entities.

Question 3: Which option do you prefer in open issue 1?

| Company | Preferred option | Comments |
| --- | --- | --- |
| Nokia | 1 | Rel-15 MAC CE can be used in the following way: - Activation: Reset to initial state - Deactivation: deactivate PDCP duplication (switch off all secondary RLC entities) |

Proposal 3:

Open issue 2

If Rel-15 MAC CE can be used for Rel-16 Duplication configuration, what if Rel-15 MAC CE indicates “duplication activation”?

- 	Option 1: All secondary RLC entities are activated.

-	Option 2: All secondary RLC entities are set to initial state configured by RRC.

Question 4: Which option do you prefer in open issue 2?

| Company | Preferred option | Comments |
| --- | --- | --- |
| Nokia | 2 | The secondary RLC entities should go to initial state in an absolute manner, when “activation” command is received with Rel-15 MAC CE. |

Proposal 4:

Open issue 3

What if Rel-16 MAC CE indicates all secondary RLC entities are deactivated?

- 	Option 1: Deactivate PDCP duplication.

- 	Option 2: PDCP duplication is kept activated, but only primary RLC entity is used for transmission.

Question 5: Which option do you prefer in open issue 3?

| Company | Preferred option | Comments |
| --- | --- | --- |
| Nokia | 1 |  |

Proposal 5:

Open issue 4

How to set DRBdup ID in Rel-16 MAC CE?

- 	Option 1: 5bits full DRB ID is included.

- 	Option 2: 3~4bits indexed DRB ID is included (similar to Rel-15).

Question 6: Which option do you prefer in open issue 4?

| Company | Preferred option | Comments |
| --- | --- | --- |
| Nokia | 1/2 | No strong view from our side. |

Proposal 6:

# 4.	Summary