3GPP TSG-RAN WG2 Meeting #109 electronic	Tdoc R2-20xxxxx

Electronic meeting, 24 Feb – 6 Mar 2020

Agenda Item:	7.6

Source:	Ericsson (Rapporteur)

Title:		Report of [AT109e][206][LTE16] CR discussion on Rel-16 early security activation (Ericsson)

Document for:	Report

# 1	Introduction

This document contains the summary of email discussion "[AT109e][206][LTE16] CR discussion on Rel-16 early security activation (Ericsson)", please see details below:

[AT109e][206][LTE16] CR discussion on Rel-16 early security activation (Ericsson)

Scope:

Discuss the CRs R2-2000987 and R2-2000988 over offline (email) discussion to solicit opinions from companies on the proposals and CR correctness.

Handle any CRs from discussion 205 that are deemed require further discussion

Intended outcome:

Discuss the CRs and check for correctness and impact to other RRC CRs.

If the CRs can be agreed, provide final CRs (by CR proponents)

Summary of discussions (by email rappporteur)

Deadline for providing comments and for rappporteur inputs:

Companies input: Thursday, Feb. 27th 17:00 CET

Rapporteur summary: Friday, Feb. 28th 17:00 CET (one day for rapporteur to make conclusions)

Updated CRs from each CR proponent: Monday Mar. 2nd 17:00 CET

Comments on CR wording: Tuesday, March 3rd by 17:00 CET

The topic was treated during RAN2#108 and according to the chair notes the outcome is that RAN2 will work on this enhancement and CRs are to be sent to next meeting (i.e. RAN2#109-e) by proponents.

In this document companies are asked to provide feedback based on the submitted CRs, after which a summary will be provided with revised CRs, if possible.

# 2	Discussion

## 2.1 	Early security CRs

### 2.1.1 	RRC CR R2-2000987

Please provide comments on correctness and possible impacts to other RRC CRs in the table below, also if the CR can be agreed or what changes would be needed to make it agreeable. You may also use the draft version in the draft folder for detailed comments. Note that the intention is to produce agreeable CRs, thus detailed comments are very much welcomed.

| Company | Comments (correctness, impacts on other RRC CRs, other suggestions) |
| --- | --- |

Conclusion / summary / proposals related to RRC CR: TBD.

### 2.1.2 	TS 36.306 CR R2-2000988

Please provide comments on correctness and possible impacts to other RRC CRs in the table below, also if the CR can be agreed or what changes would be needed to make it agreeable. You may also use the draft version in the draft folder for detailed comments. Note that the intention is to produce agreeable CRs, thus detailed comments are very much welcomed.

| Company | Comments (correctness, impacts on other RRC CRs, other suggestions) |
| --- | --- |

Conclusion / summary / proposals related to TS 36.306 CR: TBD.

## 2.2. 	Other topics

No other topics are indicated for this offline so far. This is subject to change.

# 3	Summary

TBD

# References

xxxx